*  L S i m  *

The Loudspeaker Simulator

Version 2.X Data File Format

This file is an appendix to the LSIM.DOC file.

This document explains the data file format of LSim.  LSim is a loudspeaker

design program for IBM PCs and 100% compatibles.  LSim simulates a multi-

way loudspeaker.  You explain the driver, cross-over and baffle (together

with the relative driver displacement); LSim plots the amplitude and phase

response graphs. LSim has a good user interface, is very FAST, supports all

major display adaptors and is file compatible with Ralph Gonzales's LMP and

SD-LMP.

LSim is ShareWare. It is available from CompuServe (GO IBMAPP, Library #13

or GO AESNET, Library #6) and from other audio related BBS.

Important!

----------

* LSim v1.X data files (LSim One files) had DIFFERENT format, though LSim

v2.X can read and write data files in that format.

* Although, LSim file format is in public domain, LSim is NOT and it is

copyrighted by its author, Riza Nur Pacalioglu.

LSim data files are pure ASCII text.  Each line ends with a CR and LF and

the file ends with an EOF character (^Z).

Start of MODEL data file

-------------------------

<# of drivers>LSim

<Low cut-off frequency>

<Low cut-off damping ratio>

<High cut-off frequency>

<High cut-off damping ration>_<Order of high frequency roll-off (2|4)>

<Polarity inversion (Y|N)>

<Sensitivity>_<Depth displacement>_<Diffraction boost freq.><X'Over method>

<Diffraction boost>

<X'Over info>

<1st value for X'Over>

...

<Last value for X'Over>

-1

(other drivers' data are repeated, starting immediately after the -1 line

with X'Over info, followed with X'Over data. The first line in the file

shows how many drivers are present, hence for how many drivers the above

data shall be repeated.)

<Note for driver #1>

...

<Note for last driver>

<Manufacturer for driver #1>

<Model name for driver#1>

<File name for driver#1>

<Connection of driver #1 (ON|OFF)>

<X'Over name of driver #1>

(other drivers' data are repeated, starting immediately after the note line

of the last driver, followed with manufacturer, model and file names.

Again, the first line in the file shows how many drivers are present, hence

for how many drivers the above data shall be repeated.)

-------------------------

End of MODEL data file

Start of DRIVER data file

-------------------------

driver

<Manufactures name>

<Model name>

<Note>

<Low cut-off frequency>

<Low cut-off damping ratio>

<High cut-off frequency>

<High cut-off damping ration>_<Order of high frequency roll-off (2|4)>

<Sensitivity>

<Diffraction boost freq.>

<Diffraction boost>

-------------------------

End of DRIVER data file

Explanations:

-------------

* Data will be replaced with the explanations within the <> characters

* Numeric data may be preceded with a SPACE character (ASCII dec.20)

* | means either value will be used

* _ shows a single TAB (ASCII dec.09) or one or more SPACE characters

(exact number of space characters is not important)

* Frequency values are in Hertz

* Sensitivity value is in dB per Watt per meter

(1 Watt equals 2.83 Volts across 8 ohms)

* Dimensions are in inches

* Diffraction boost is in dB

* -1 shows that there are no more components left in the X'Over.

* X'Over method is either of the below:

NOCROSSOVER

COMPONENT

FACTORED

POLYNOMIAL

(Model files can have any combination of X'Over methods for each of its

drivers.)

* X'Over info for COMPONENT method is a single digit and shows the X'Over

type which corresponds to the X'Over names:

0  ->  NO CROSSOVER

1  ->  NOTCH

2  ->  1.ORDER LOWPASS

3  ->  1.ORDER HIGHPASS

4  ->  1.ORDER BANDPASS

5  ->  2.ORDER LOWPASS

6  ->  2.ORDER HIGHPASS

7  ->  2.ORDER BANDPASS

8  ->  3.ORDER LOWPASS

9  ->  3.ORDER HIGHPASS

10  ->  3.ORDER BANDPASS

11  ->  4.ORDER LOWPASS

12  ->  4.ORDER HIGHPASS

13  ->  4.ORDER BANDPASS

* X'Over values for COMPONENT method are according to the X'Over type. Each

value is on a seperate line!

X'Over    First value to last value

Type     (Component names as displayed on LSim screen)

--------   ---------------------------------------------

1       Ro, ...

2       Ro, C1

3       Ro, L1

4       Ro, ...

5       Ro, L1, C1

6       Ro, C1, L1

7       Ro, ...

8       Ro, L2, C1, L1

9       Ro, C2, L1, C1

10       Ro, ...

11       Ro, L2, C2, L1, C1

12       Ro, C2, L2, C1, L1

13       Ro, ...

* X'Over info for POLYNOMIAL method is as shown in the below line:

<Gain factor>_<Power of numerator>_10

(Show gain factor value of 0.0002 as 2D-4)

X'Over names are POLYNOMIAL - N, where N shows the order of the X'Over.

* X'Over values for POLYNOMIAL method are as shown on the below lines

(Total of 19 lines. If a value is 0 then that line will have the value

0):

<Numerator factor of s^0>

<Numerator factor of s^1>

...

<Numerator factor of s^9>

10

<Denumerator factor of s^0>

<Denumerator factor of s^1>

...

<Denumerator factor of s^9>

* X'Over info for FACTORED method is as shown on the below lines:

<Gain factor>_<Power of numerator>_5

(Show gain factor value of 0.0002 as 2D-4)

X'Over names are FACTORED - N, where N shows the order of the X'Over.

* X'Over values for FACTORED method are as shown on the below lines (Total

of 23 lines. If a value is 0 then that line will have the value 0):

<Frequency of 1st 1st order filter at numerator>

<Frequency of 2nd 1st order filter at numerator>

...

<Frequency of 5th 1st order filter at numerator>

5

<Frequency>_<damping ratio of 1st 2nd order filter at numerator>

<Frequency>_<damping ratio of 2nd 2nd order filter at numerator>

...

<Frequency>_<damping ratio of 5th 2nd order filter at numerator>

5

<Frequency of 1st 1st order filter at denomerator>

<Frequency of 2nd 1st order filter at denomerator>

...

<Frequency of 5th 1st order filter at denomerator>

5

<Frequency>_<damping ratio of 1st 2nd order filter at denomerator>

<Frequency>_<damping ratio of 2nd 2nd order filter at denomerator>

...

<Frequency>_<damping ratio of 5th 2nd order filter at denomerator>

* Notes line correspondes to 'driver explanation' data of SD-LMP.

An example MODEL file:

----------------------

3LSim

80

.7142857

5100

.5555556      4

N

81            0             172.3COMPONENT

6

2

8

.003

-1

700

1.25

22000

.5882353      2

N

85            0             344.6POLYNOMIAL

6

4D-16         4             10

1

0

0

0

0

0

0

0

0

0

0

10

1

.00025

.00000006

.00000000001

4D-16

0

0

0

0

0

0

50

1

5000

.5            2

N

84            0             2297.333FACTORED

3

1             0             5

300

0

0

0

0

5

300           1

0             0

0             0

0             0

0             0

5

0

0

0

0

0

5

0             0

0             0

0             0

0             0

0             0

GOOD MID-BASS

high power handling

2.5 dm3 box

KEF Ltd

B110B

B110B.DRV

ON

1.ORDER LOWPASS

Dynaudio

D-28AF

D28AF.DRV

ON

POLYNOMIAL - 2

SEAS

P11RCY

P11RCY.DRV

OFF

FACTORED - 3

KEF Ltd.

B110B

B110B.DRV

ON

NOTCH FILTER

----------------------

An example DATA file:

----------------------

driver

SEAS

11FGX

Good mid-bass

110

1

8500

.6410257 4

82

2650.769

6

-----------------------

For further information about LSim please contact the author:

Riza Nur Pacalioglu

27A Eskibuyukdere, 4. Levent

80650 Istanbul, Turkey

Fax:   90-1-268 6901

Tel:   90-1-268 6900 (office hours, +2 hours GMT)

Telex: 39084 cctv tr

Email: CompuServe 70242,145

AppleLink  BES.TURK

Internet   70242.145@compuserve.com

bes.turk@applelink.apple.com

Happy loudspeakering...and do support audio education!

<end>