SCSIHA.SYS is a buffering device driver, and supports both video memory

and virtual memory buffering.  Video memory buffering can be used if trouble is

encountered running programs that transfer a lot of data directly from the disk

to video memory.  Applications that use the protected mode of the '386 (virtual

memory used by MS Windows 386) may also require the use of SCSIHA.SYS.

SCSIHA.SYS usage:

DEVICE=SCSIHA.SYS /V386 /B:xx

/V386   is used to enable virtual buffering.  If no /B:xx value is

specified, then a default of 16 kBytes is used.

/B:xx	 sets the buffer size where xx can be 16, 32, for 64 in units

of kBytes.

Example:

To run Windows 386, the CONFIG.SYS should include the line

DEVICE=SCSIHA.SYS /V386