Cessna Citation V (C560)

The Cessna Citation V is the big brother to the Citation II and SII. Visually, the three look similar with the SII and V sharing the same wing planform. The most visible difference between the three aircraft is the seven passenger windows per side on the Citation V.

Hopefully, you appreciate its visible fidelity with the real aircraft. I spent many hours taking aircraft Maintenance Manual three-views and transposing them through the grid method to Aircraft Factory. Flying qualities are, I hope, as close to the real thing as possible with the limitations of FS4 taken into account. I haven't had the chance yet to fly the "real" simulator at work (you know, the ones that cost more than the real airplane).

Particulars:

Length: 48.9'

Height: 15.0'

Span:	52.2'

Horizontal Stabilizer: 20' (approx)

Wheel Base: 19.9'

Wheel Track: 17.6'

Powerplant:

2 Pratt & Whitney of Canada JT15D-5A Turbofans

Thrust: 2900 lbs each (takeoff-5 minute limitation)

Thrust: 2900 lbs each (maximum continuous)

Airspeeds:

VMO sea level to 8,000 ft: 260 KCAS (261 KIAS)

VMO 8,000 to 28,890 ft: 290 KCAS (292 KIAS)

MMO above 28,890 ft: .75 Mach (0.755 MIAS)

VA: 201 KCAS (202 KIAS)

VFE (flaps extended):

35 degrees (landing): 174 KCAS (173 KIAS)

15 degrees (T/O and approach): 199 KCAS (200 KIAS)

7 degrees (T/O): 199 KCAS (200 KIAS)

VMCA (min control speed air): 84 KCAS (85 KIAS)

VMCG (min control ground): 86 KCAS (86 KIAS)

VLO (landing gear operating extend): 249 KCAS (250 KIAS)

VLO (landing gear operating retract): 199 KCAS (200 KIAS)

VLE (landing gear extended): 290 KCAS (292 KIAS)

VSB (speed brakes extended): any speed

Max Tire Speed: 165 knots

Weights:

Takeoff: 15,900 lbs

Landing: 15,200 lbs

Zero Fuel: 11,200 lbs

Ramp: 16,100 lbs

Fuel:

Two tanks with 430.5 gallons usable each

Maximum Operating Altitude: 41,000 ft.

Data Sources:

FAA Type Data Certificate Sheet A22CE, Revision 36, April 15, 1992

Cessna 560 Maintenance Manual

Thomas G. Mosher

January 25, 1992

Irving, Texas

Other aircraft created or in work:

Beechcraft B200C Super Kingair

Hawker Siddeley (BAe) 125-700 (TFE731-3-1H engines)