EDIT TRACK

Version 2

26 MAY 1991

Envirocomp

RR1 Baysville

Ontario CANADA

P0B 1A0

EDIT TRACK (or ET) is a drafting aid designed to help you design

model railroad track plans on an IBM PC (or equivalent). It is

the result of my own frustrations with trying to accurately

design complicated trackwork. I simply could not accurately draft

the switches, so this program was born.

ET displays the track plans on the screen as you construct them.

ET also calculates and displays (or prints out) the track

coordinates which can then be used for layout construction. Track

plans and parts of track plans can be saved and retrieved from

disks. They can also be graphically printed on IBM compatible

graphics printers and HP Laserjets.

EDIT TRACK requires that you provide the actual dimensions of the

switches, diamonds etc. that are to be used in your layout.

Approximate dimensions for some code 70 and code 100 Shinohara HO

switches are provided for practice designing layouts. I've also

provided some dimension files for Marklin Z scale and LGB G scale

switches.

Designing trackwork with ET requires some attention to detail and

occasionally some calculations; a calculator can be helpful. You

might also find the "Layout Design Tips" section at the end

useful.

EDIT TRACK is COPYRIGHTED by Envirocomp. You cannot resell this

program for profit and the source code to EDIT TRACK is not

available. However you may use it as much as you like and freely

distribute it. Please, only distribute a copy of the complete

diskette with all subsidiary files and this documentation

included. New versions of ET can be obtained from me. Please send

$5 (U.S. or Canadian) to cover diskettes, mailers, handling and

postage. Please specify whether a 3 1/2" or 5 1/4" diskette is

preferred.

If you have a problem, read this document again. If the problem

persists and you think it's a bug, please send me as much

information as possible, including a diskette with the

troublesome track plan. Any suggestions, comments or criticisms

are also welcome.

IMPORTANT: Please read this document thoroughly!

REQUIREMENTS

------------

EDIT TRACK requires an IBM PC compatible computer with a CGA, EGA

or VGA graphics adapter and a color monitor. Since CGA is

marginal in terms of both the track and font displays, an EGA or

VGA is highly recommended. In general, the higher the video

resolution, the more powerful the computer required to drive the

display at reasonable speeds. If you use VGA resolution and the

display seems sluggish, try EGA instead (see the discussion of

the ET.OPT file below). Some graphics adapters (e.g. VGA) can run

with black and white monitors, however running EDIT TRACK under

these conditions can be difficult.

As intermediate calculations require trigonometric calculations,

an 80x87 math co-processor really speeds things up. If you have

and 8088 or 8086 CPU, a math coprocessor is almost essential;

however, it is not necessary on an 80386 CPU.

Some of ET's best features can only be accessed with a Microsoft-

compatible mouse although it is possible to construct track plans

without one. EDIT TRACK has been successfully tested with PCMOUSE

and LOGITECH mice. Be sure the mouse driver is loaded before

running ET. See your mouse manual about this.

This version of ET supports both IBM (originally EPSON)

compatible graphics printers and HP Laserjets. Most 9-pin dot

matrix printers have an IBM compatible graphics mode, and most

laser printers have an HP Laserjet compatible graphics mode.

Both summary and detailed layout data can be printed on any DOS

compatible text printer.

These printers can be attached to the LPT1, LPT2, COM1 or COM2

ports; a temporary disk file can also be used to store printer

output. If the COM1 or COM2 ports are used, be sure that the MODE

command has been run (usually in the autoexec.bat command file)

to properly set up communications between the port and the

printer. See your DOS manual for more information about the MODE

command. If you print to a disk file, this file can later be sent

to the printer with the following command (assume it is attached

to the LPT1 port) :

COPY FileName LPT1 /b

The /b is not necessary for text only print jobs.

FOR USERS OF EDIT TRACK VERSION 1

---------------------------------

Version 2 is a substantially improved version with many new and

revised pieces of track. However, it can read version 1 track

plans with the following caveats:

- Minimum layout X and Y dimensions are changed from 0 to -100.

- Jumps are discarded.

- Angles in layout and tree files are now degrees not radians,

to make them consistent with config files. However, 360 degree

curves (full circles) in version 1 layout files may now be

converted to 0 degree curves when read in to version 2. You will

have to correct these in EDIT TRACK.

- The old absolute Join is converted to the new relative Join.

- If you've been fiddling with the track specification part of

your layout files or the old TRACK.CFG file, make sure that

characters specifying the track object are all in upper case. For

example:

S 4 8.3300 15.4800 16.2840 0.2444

is ok, but not:

s 4 8.3300 15.4800 16.2840 0.2444

EDIT TRACK version 2 is case sensitive.

The Version 1 TRACK.CFG file has been split into two files in

Version 2: ET.OPT and TRACK.CFG. ET.OPT will be created

automatically for you. To create a Version 2 TRACK.CFG file, read

in a Version 1 layout file with the appropriate track dimensions,

dummy edit a frog switch then save TRACK.CFG when prompted.

SOME GEOMETRY

--------------

Let's start laying track at a given point and in a specific

direction. I'll be working in metric units (centimetres or cm),

but think in inches if you wish. EDIT TRACK supports both. First

we have to decide where to start, how about (100cm 100cm)? This

(X Y) notation gives the flat plane, map, or Cartesian

coordinates: X is the horizontal distance and Y is the vertical

distance from the lower left hand corner (0 0), the origin, of

your layout. Now in what direction will we go? Let's add a

section of track horizontally to the right at an angle of 0

degrees (0d).

IMPORTANT: ET expects angles of direction to be measured in

degrees COUNTER-CLOCKWISE from the horizontal line (X axis)

that extends off to the right.

First add a 30cm section of straight track. Since we started at

(X Y) = (100cm 100cm), the end (or "node") of the straight line

is at (130cm 100cm) and has a direction of 0d, or (130cm 100cm

0d). Next add a curved section of track with a radius of 70cm and

an arc of 90 degrees curving clockwise to the right. The node at

its end will be (200cm 30cm 270d). Recall that angles are

measured in the COUNTER-clockwise direction, so that 90d

clockwise to the right is either -90d or +270d (360d - 90d) as

measured counter-clockwise. Simple, right? Actually, when you do

it on the graphics display, it really is simple.

A switch is then connected at the last node. Assume that it has a

number 4 frog, is a "left-hand" switch, and that the end

connected to that last node is the points end. You will have

already stored the actual dimensions of a #4 switch (including

the lengths of its different arms and the angle between them) in

the program. Alternatively you may opt to enter unique dimensions

for this particular switch alone. The data you provide is used to

calculate the positions of the next two nodes at ends of the two

switch arms. Afterwards, we have the choice of connecting a new

object either to the node at the end of the straight-through arm

of the switch or to the node at the end of the branch curving off

to the left.

I think you get the picture. First define the starting point and

direction, then start attaching objects, either pre-defined

objects like switches and diamonds, or objects defined "on the

fly" like straight and curved lines of arbitrary lengths and

curvatures. EDIT TRACK makes sure that the objects you add are

attached at the proper position and angle. At this point, it's

probably best to try EDIT TRACK.

INSTALLATION

------------

The current drive\directory should be the drive\directory where

ET.EXE is found. For the examples below, make sure that the file

TRACK.CFG that came with the program is in the same

drive\directory. If you have an EGA or VGA adapter, copy the

appropriate file (EGA.OPT or VGA.OPT) to ET.OPT. For example:

COPY  EGA.OPT  ET.OPT

If you have a VGA adapter but would still like to operate in EGA

mode, copy the EGA.OPT file to ET.OPT. There should NOT be an

ET.OPT file in the current drive\directory unless you are sure it

specifies the correct video monitor. If ET.OPT is not present,

EDIT TRACK will assume you have a CGA adapter and create ET.OPT

at the end of your first session.

Now type:

ET

NOTE: ET can be optionally followed by the name of a layout file

(preceed the layout file's name with the disk drive and

subdirectory if necessary). ET automatically adds .LAY to

the layout file name if you give no extension.

To remove the ET Logo, click on the OK with your mouse, or press

the letter O or press ENTER.

IMPORTANT: If the ET Logo did not appear, the incorrect graphics

adapter may have been specified in ET.OPT. Reboot, copy the

correct file to ET.OPT and try again.

You will next have to wait a bit for ET to set up its internal

tables. When the "Please wait ..." message disappears, click on

UTIL or press U. Then click on Aspect or press A. Or, once the

drop down menu has been activated, the arrow keys can be used to

highlight the ASPECT menu item; you then use ENTER to select it.

Using the up and down arrow keys, ensure that the semi-circle is

round. There should be exactly 4 squares from the bottom to the

top of the semi-circle. Use the number keys to adjust the

resolution of movement: 1 = fine, 9 = coarse. Press ENTER when

finished. EDIT TRACK will store the aspect ratio in ET.OPT when

you EXIT.

NOTE: An improper aspect ratio is suspect if your curves do not

appear round or do not line up with attached objects at

their ends. (There will always be a small error especially

with large radius, relatively horizontal curves but it

should not be pronounced).

If you have a printer, select UTIL again and then PRINTER. Enter

the printer type (either IBMGP or HPLASER for IBM graphics

printer or HP Laserjet, respectively). Enter the name of the port

your printer is attached to (it can be LPT1, LPT2, COM1, COM2,

COM3 or any filename).

TAB, SHIFT-TAB and UP/DOWN arrow keys can be used to move between

fields on the form. Within a field, BACKSPACE, DELETE and

LEFT/RIGHT arrow keys can be used for editing purposes. When the

entries are ready, click your mouse on OK, or press ENTER or ALT-

O. Alternatively, it you want to return to the previous menu

without changing anything, click on CANCEL or press ESC or ALT-C.

If you selected IBMGP for printer type, now enter 1.2 and 10 for

the printer aspect ratio and printer height respectively. If you

selected HPLASER, now enter 1.0, 10 and 75 for the printer aspect

ratio, printer height and printer resolution (Dots Per Inch)

respectively. More about these later.

EDIT TRACK will store all this information in ET.OPT when you

EXIT.

GETTING STARTED

---------------

Now click on the FILE menu or press F. Note that your only

options at this point are to create a NEW layout, READ an old

layout file, read ABOUT ET or EXIT.

Click on NEW or press N. ET next presents a form asking for the

minimum and maximum (X Y) coordinates of your layout. The minimum

coordinates can be thought of as the bottom, left corner  of your

layout or model railroad room, and the maximum (X Y) coordinates

as the upper, right corner. Make sure that the minimum and

maximum (X Y) coordinates will enclose your complete track plan,

as ET verifies that the end nodes of every track object fall

between them. The min and max are also used when you tell ET to

print out the whole track plan. They can be changed later, so for

now make (500 500) the maximum (X Y) and (0 0) the minimum.

As before, TAB, SHIFT-TAB and UP/DOWN arrow keys can be used to

move between fields on the form. Within a field, BACKSPACE,

DELETE and LEFT/RIGHT arrow keys can be used for editing

purposes. When all the entries are ready, click on OK, or press

ENTER or ALT-O. Alternatively, it you want to return to the

previous menu without doing anything, click on CANCEL or press

ESC or ALT-C.

ET next asks for the initial starting coordinates, the starting

angle and the tree's title. Enter the coordinates (100 100 0) and

whatever you wish for the tree's title. Wait a minute, what's a

Tree ...? I thought we were talking railroads! Well ..., a layout

can consist of up to 100 "trees", each starting at a particular

place and a particular angle, each having a different name and

each consisting of as many track pieces ("objects") as you like

all linked together at nodes. Trees can be written to disk and

read from disk, are graphically displayed or hidden. Also many of

the cut, copy and paste operations in EDIT TRACK operate on trees

or parts of trees (branches). As will be shown later, it's

usually a good idea to break up your layout into several

different, relatively small trees.

For example, you could break up your railroad into divisions, but

even these might be too big. Alternatively, there could be a

Mainline tree, a Huntsville Yard tree, a Waterford Branch tree,

etc. You could also, for example, have a separate tree for each

electrical block in your layout. For multi-level layouts,

different levels could also be different trees. However you do

it, having several smaller trees will make the display, printing,

editing and changing of different parts of your layout easier.

Note that the clarity of the tree titles is very important when

you have 20 trees and are trying to decide which one is which!

After you've entered all this stuff, click on OK, press ENTER or

ALT-O. Do you see the little cross mark down on the left hand

corner of your screen? Now take look at the status line that

appears under the upper menu bar. You should see:

100.0, 100.0, 0.0 | Start, main | Your title here!

This display tells you all about the current node, current object

and tree. The (X Y) coordinates of the current node are displayed

as well as its angle, just as you entered them. But why is the

current object called a "Start" object? Well ... (here we go

again), Start objects are a little weird. Think of them as place

holders; they don't take up any space on your layout and have no

dimensions; but all trees start with one, and you can attach

objects on either side of them. For example, let's attach an

object to the "main" side of this Start object. Click on ADD or

press A, then choose LINE. Enter 30 then click on OK, press ENTER

or ALT-O.

NOTE: If you have a mouse there is an alternative. Click on (ALT)

MOUSE in the form and then press and hold the mouse's left

button while you drag the line to the length you want. The

current line length is shown on the status line. Dragging

operations go ONLY Left-Right or Up-Down depending upon the

angle of the current node. The status line and long arm of

the cursor tell you which way to drag.

If the operation was successful, you should now see on the status

line:

130.0, 100.0, 0.0 | Straight line, 30.0 end | Your title here!

As before, the status line tells you exactly where you are on the

layout (the current node) and describes the current object: a

straight line, 30cm long in this case. Just as "main" referred to

which side of the start object you were on, "end" tells you here

which end of the line the current node is attached to. To

illustrate this further, type CTRL-RETURN; that is, press the

CTRL key and the RETURN key simultaneously. You should see the

following:

100.0, 100.0, 0.0 | Straight line, 30.0 origin | Your title here!

We are now at the other end of the straight line, the "origin"

end which is attached to the Start object. The "origin" end of

all objects is the end originally attached when you built the

tree. ET remembers this information and maintains a default sense

of direction from the Start object all the way out to the tips of

each tree. Now just type RETURN. You should see next:

100.0, 100.0, 180.0 | Start, branch | Your title here!

RETURN is used to move along your track plan from one object to

the next in the current direction. CTRL_RETURN is used to rotate

from one node to the next of the same object, thus changing the

current direction. We have just arrived at the other side (the

"branch" side) of the Start object.  The (X Y) coordinates of the

current node are the same as the "main" side, but the angle is

180d in the other direction. If we wished, we could start adding

track off in this direction as well. But let's turn around and

return to "end-of-track" in the other direction. Type CTRL-RETURN

and then RETURN again. You should be at:

130.0, 100.0, 0.0 | Straight line, 30.0 end | Your title here!

NOTE: If you are thinking in inches, then instead of having a

display of 130 you might prefer 10'10". Go to the UTIL menu

and toggle "units=Feet" on. The primary unit is still

inches, but for certain displays, inches are converted to

the feet(') inches(") notation. Also, when you enter data in

a form, if the feet(') inches(") notation is not used,

inches are assumed.

Have you noticed how the color of an object and the size of a

node changes when they become current?

Try RETURN one more time. You should get a message telling you

that there's no place to go.

NOTE: Moving around your layout with a mouse is much easier. Just

move the tip of the arrow cursor to the node (not object) of

interest and click on it with the left button. The right

button causes the current object to move just like the ENTER

key does and the middle button (if you have one) rotates

around the nodes of an object like CTRL-ENTER does. Return

to the tip of your tree before continuing.

Select ADD and then CURVE. Answer the questions, giving it a 70cm

radius, 90d arc and clockwise curve to the right.

NOTE: If you have a mouse you can specify left or right

handedness as well as ONLY one of the radius length or arc

angle. The other must be blank. Then click on (ALT) MOUSE in

the form and then press and hold the mouse's left button

while you drag the curve to the size you want. If you left

the radius field blank, then the radius will change with the

mouse motion and the arc angle will remain constant, and

vica-versa. The status line keeps you informed of the

current size of the curve. As with Lines, dragging

operations go ONLY Left-Right or Up-Down depending upon the

angle of the current node. The status line and long arm of

the cursor tell you which way to drag.

You should now be at:

200.0, 30.0, 270.0 | Curve, 70.0 90.0 R (130.0,30.0) end | ...

We have already discussed the coordinates of the current node;

just remember that ET reports angles as measured counterclockwise

from the horizontal line extending off to the right. The curved

line's radius, arc and handedness (clockwise to the Right (R) in

this case) are reported with its name, as well as which end of

the object is attached to the current node (the "origin" or

"end"). Curved Line objects also have in parentheses the (X Y)

coordinates of the centre of the arc. You can use these

coordinates later to draw the curved line on plywood by hand when

coonstructing your layout.

Select ADD again but then select SWITCH then FROG. Enter a frog #

of 4, and tell ET that the switch diverges to the Left (left-

handed). You also must specify which end to attach to the current

node (for example enter A, the points end), and which end to

continue on with next (enter B, the continuation of the straight

track segment through the switch).

In general, for all types of Switches, the switch points end is

referred to as A, the main continuation end opposite the points

is B and the diverging track end is C. Wye switches use A for the

points end but L and R for the left and right hand diverging

branches. You can attach any end to the current node and continue

on at any end. When a switch is the current object, the line

below the menu bar identifies which end is attached to the

current node using A, B or C (A, L and R for wye switches). In

fact, you should now see the following:

200.0 6.2 270.0 | Switch, #4L (A origin) B branch | ...

NOTE: this assumes that the original TRACK.CFG file was found in

your current directory. As will be discussed below,

TRACK.CFG contains the default dimensions for various

switches and diamonds that ET uses for its calculations. The

dimensions provided in the original TRACK.CFG supplied with

EDIT TRACK are for some HO scale Shinohara switches as

measured in centimetres. If you are working in inches, it

will be necessary to provide your own dimensions in inches.

Use CTRL-RETURN or your mouse to rotate around the three nodes of

the Switch to see how the display changes. Note that you are

informed which end is the "origin" (parenthetically when that

node is not the current node).

In general, EDIT TRACK provides two types of objects that you can

add to a tree: those that the USER defines "on the fly", and

those that are predefined and referenced by their FROG numbers.

For example, LINEs, CURVEs, D-XOVERs (double cross-overs),

JOINSs, BOXes and OVALs are individually USER defined when they

are added to the track plan. However SWITCHs, KURVED SWITCHs,

DIAMONDs and SPLINEs can be either USER defined or else you can

specify a FROG number. SPLINE SWITCHs must always be predefined

and referenced with a frog number.

All predefined objects that you use via the FROG number can be

edited, created and saved for future use in other track plans by

selecting UTIL then EDIT FROG. Remember that the units (inches,

centimetres, millimetres, etc.) used when predefining SWITCHs,

etc. must be the same used in USER defined objects such as CURVEs

and LINEs.

ADD can be used to INSERT an object at any node in a Tree, not

just at the empty nodes at the end. Just move to the node of

interest and select ADD.

Now make the Start object the current object and select EDIT

ALTER from the menu. You can change the starting coordinates and

angle of the whole tree. Try changing them to (200 200 90). If

part of the tree is offscreen use the arrow keys to shift it back

in. Note that the line and switch moves right along. Now move the

tree back to where you were and use the arrow keys again to see

it.

NOTE: use EDIT SHIFT to alter ALL trees in the layout at once.

Now make the switch the current object. Select DELETE then OBJECT

(or just use the DEL key). The switch is removed and you should

be back at the end of the previous object:

200.0 30.0 270.0 | Curved line, 70.0 90.0 R (130.0 100.0) end |

DELETE OBJECT (or DEL) can also be used to remove the current

object at any point in the tree if it has only one branch coming

off it. DELETE OBJECT cannot remove a whole "branch" consisting

of several objects. If you want to delete the current object plus

all objects extending out to the tip of the tree (a "branch" of

the tree), select DELETE then BRANCH.

This is where a tree's sense of direction comes into play. As you

build a tree from the Start object outward, ET remembers each

step and knows which objects were added on after others. Thus

when you tell it to DELETE a BRANCH (perhaps Prune would have

been a better word?), ET knows how to go from the current object

to all objects out at the branch's "tips". It deletes all the

objects it finds along the way.

This sense of direction is also used for default navigation using

the RETURN key or right mouse button. If you start at the START

object of a tree, and just keep hitting RETURN, ET will move

outward, choosing at each Switch the node/branch that you

initially selected to add another object to. Conversely, if you

change directions and start to move back from the tip of a tree,

ET will automatically choose the Origin branch at each Switch and

eventually lead you back to the Start object and continue on to

its other side. Of course if you have a mouse, it is usually

easier to just click with the left button on the node you are

going to.

Sometimes ET's sense of direction is not exactly what you want.

How, for example would you delete all the objects from the

current object back to the Start (and beyond)? DELETE BRANCH

deletes all objects outward away from the Start object, not

inward back to it. Also you just cannot delete the old start

object as you need a Start object for every tree. The simplest

solution is to move the Start object to your current node using

EDIT MOVE, then move to the first object in the branch you want

to delete, then delete the branch.

When using DELETE BRANCH, you can optionally save the branch to

be deleted in a scratch buffer and later use EDIT PASTE to attach

it to an empty node somewhere else. DELETE can also be used to

delete a whole TREE, or even your whole LAYOUT. EDIT COPY can be

used directly to copy a branch into the scratch buffer, but

without deleting it.

Now try adding more trees by selecting ADD then TREE. You'll be

asked for the starting coordinates and angle. You may want to

start another tree right at the same spot occupied by current

node, and in the same direction. In this case, instead of

entering the current node's coordinates and angle, just type a *

for the X coordinate. You can now cut and paste the old branch to

the new START object. Remember, its a good idea to break your

layout up into several different trees, even if the actual track

will be continuous from one tree to the next. Mouse users just

enter the new tree's starting angle, click on (ALT) MOUSE, move

the mouse cursor to the new (X Y) coordinates and click the

mouse's left button.

Try adding several new trees with a few simple objects each. Use

EDIT ALTER to modify any existing object.

Now select VIEW then INCLUDE. There you can specify which trees

are in or out of the Current Tree List. This list determines

which trees are visible, printed, summarized, written and

accessible (through the GOTO NEXT command or mouse selection).

The trees currently included are marked by an asterix. If not all

trees appear on the list, use the mouse to click on the up and

down arrows or on the shaded portion of the elevator bar.

Alternatively use the UP/DOWN keys to highlight a selection. To

toggle the asterix on or off, use the mouse to click on the tree

so that it appears in the upper box, or use the UP/DOWN arrow

keys followed by ENTER. Then select OK by clicking with the mouse

or pressing the letter O (OK cannot be selected with the ENTER

key). The asterix next to the tree's name will appear if it was

not present before or vica-versa. Do the same for any tree you

want to change. When finished, select END by clicking with the

mouse or pressing the letter E.

Selecting GOTO then GOTO again permits you to add a tree to the

current tree list and make it the current tree simultaneously. To

do this, first select the tree to make current so that it shows

in the upper box, than select OK by clicking or pressing the

letter O.

Now select FILE then EXIT or just EXIT from the main menu bar.

You will be asked if you really want to leave ET. At this point

you should decide whether you want to save your layout or just

forget it. Let's save it by writing it to a layout file.

Select CANCEL or press ESC or C. Now select FILE then WRITE. You

are first prompted for the path\filename which should be blank

initially as you have not entered a filename yet and ET is using

the current directory as the default path. At this point, edit

the default path to store your file on some other drive or in a

different directory and add a filename. The .lay extension will

be added automatically. Note that you can also just save the

trees in the Current Tree List in a file with the default .tre

extension.

NOTE: When ET saves a layout, is also saves all the current

Switch, Diamond, etc. FROG dimensions in the layout file.

When you next read in that layout file, these dimensions

will replace any dimensions previously loaded in from the

default TRACK.CFG file. You can have different versions of

the configuration file (by default, TRACK.CFG) available:

for example, one with code 70 HO Shinohara Switches and

Diamonds, another with PECO code 100 HO, another with Atlas

or your own etc. Unfortunately, any layout built with one

set of Switch and Diamond dimensions will probably not work

properly with a different set unless all the layout's Line

lengths and Curved Line dimensions are adjusted as well.

This is the rationale for storing the Switch and Diamond

dimensions with each layout file. These dimensions are NOT

stored with a TREE file however. You can thus write out

trees built with one configuration file, then read them into

a layout that is using different Switch and Diamond

dimensions. I'm not sure why anyone would want to do this

...

Now select FILE, READ, LAYOUT. Note that the current layout must

first be deleted. If you have not saved it yet, CANCEL then FILE

WRITE etc. Once that's done you can safely continue. Next you are

asked to edit the file search path before a list of possible

filenames is shown. A blank line means the current

drive/directory. If you wish, enter a different drive and

directory that has .lay files in it. The search routine looks

only for .lay files. Select the desired file so it is displayed

in the upper box and then press O or click on OK.

If you have a standard DOS text printer, select FILE then PRINT.

The whole layout or just the trees on the Current Tree List are

printed much as if you had started out at the START object of

each tree and traversed out to one branch tip, then back-tracked

to successive branches and printed them out, one by one. Note how

the objects in each tree are uniquely numbered successively on

the left margin starting with the Start object, and that a blank

line is printed when the program backs up to the next branch to

be printed out. When that's finished try UTIL then SUMMARY and

SCREEN or PRINT. Note that all USER objects (switchs, etc.) are

added into the total track length but not FROG objects. The track

length of USER SPLINEs is only roughly estimated. Only those

trees in the Current Tree List are included.

It's time to learn through exploration. Try the filled and

unfilled versions of BOX and OVAL objects. These can be coupled

together with JOINs to create a string of structures or major

landscape features in a single tree.

A JOIN's main purpose in life is to change the current

coordinates and angle of a tree in any way that you specify. For

example, to construct a tree that indicates where the windows are

in your layout room, use START to set the coordinates and angle

of the line used to indicate the first window, then add the Line

of appropriate length, then add a JOIN to set the coordinates and

angle of the line to be used for the second window, etc. On my

trackplan, I have a single tree that outlines a stairwell, a

circular stove-pipe and a wall that intrudes into the layout

space. Another such tree can be used to define the outer edge of

the layout itself, again using primarily JOINs, LINEs, CURVEs and

SPLINEs.

JOINs can be specified in three ways: relative to the origin of

the layout (0 0), relative to the (X Y) coordinates of the

current node, and the mouse method. Coordinates relative to the

origin are the default; you simply give the new (X Y) coordinates

and the angle of the new node. To specify coordinates relative to

the (X Y) position of the current node, preface a + or -before

your numbers. For example, if the current node is (100cm 100cm

50d) and you specified (-30cm +30cm -40d), the JOIN would connect

the current node to (70cm 130cm 10d).

NOTE: No matter whether coordinates relative to the origin or

current node are given, JOINs are stored internally in

ANGLE-relative coordinates (these are different! see SPLINEs

below). This means that when a tree with JOINs is rotated

about its START position, the positions of all its "joined"

objects remain in the same relative orientation with the

same distances between them. Also, if you delete an object,

any objects joined further out are moved in. Maybe you

should just try some and see how they behave.

Take a look at SPLINEs which can be both USER and FROG SPLINEs.

USER SPLINEs are entered with ANGLE-relative coordinates by

default but absolute coordinates are possible. FROG splines

require ANGLE-relative coordinates (as do Spline Switches) when

you enter their dimensions under UTIL then EDIT FROG. For example

if the current node is (200 200 45d) and the User spline's ANGLE-

relative coordinates are (+100 +100 +25d), the new node is NOT

(300 300 70d), rather it is (200 341.4 70d). In other words you

go straight out from the node at its 45 degree angle for 100

units (the  ANGLE-relative X axis), then turn left 90 degrees and

go for another 100 units (left is in the positive Y direction in

ANGLE-relative units). Finally you add the 25 degrees to 45.

Perhaps some experimentation is in order to figure out just what

I'm trying to get across here.

Splines can be tricky. If you do not specify a reasonable angle,

the calculated curve will fly all over the screen. The best way

is to practice: start with a relative 90 degree angle and ANGLE-

relative (not absolute) coordinates of (+50 +50 +90d). Use EDIT

ALTER to try different relative angles. Go into UTIL and toggle

the Spline Lines switch on. This permits you to see the two

bounding lines that determine the spline.

FROG SPLINEs can be useful if you want predefined easements for a

few standard curves, although see the discussion under Layout

Design Tips later for an alternative. FROG splines require ANGLE-

relative coordinates when you enter their dimensions.

SPLINE SWITCHES are yet another type of switch; they must be

predefined with a frog number by selecting UTIL then EDIT_FROG

then SPLINE SWITCH. The left hand version is the one you define,

EDIT TRACK will compute the right hand version when you request

it. The switch consists simply of two splines with a common end

(the A or points end). The B end should be that end with the

least curvature to the left (the "outer" spline), and the C end

would be the end with the most curvature (the "inner" spline).

SPLINE SWITCHES require ANGLE-relative coordinates when you enter

their coordinates. These switches are useful general purpose

switches when none of the others can be made to fit.

Perhaps a recap of all the different switch types is in order.

Remember that FROG switches are pre-defined; you edit their

dimensions in the UTIL EDIT_FROG section, then when building your

layout just specify the frog number and orientation. Thus all

switches in your layout with the same frog number have the same

dimensions. Conversely, all information about USER switches is

provided as you build the layout and may be unique for each

switch.

SWITCH (FROG) - defined by the straight line from A to B and the

branch to C. Distances needed are: A to centre, B to centre and C

to centre; the frog angle is also needed. A special case of this

switch occurs when the A to centre and C to centre distances are

equal. Under these circumstances the switch can be equivalently

defined by the full A to B linear distance, as well as the radius

and arc of the curve from A to C (e.g. LGB switches use this

method). I call this the "Effective Radius Method" and you will

be asked if you prefer it whenever entering data for a new

frogged switch or whenever editting a frogged switch with A to

centre equalling C to centre.

NOTE: Switchs with different A to centre and C to centre

distances

SWITCH (USER) - this switch is more useful for those who hand lay

their own track. It uses the "Effect Radius Method" and so is

defined by the full A to B linear distance and the radius and arc

of a curve from A to C.

NOTE: Currently User switches are less flexible than frogged

switches as the A to centre must always equal the C to

centre distance. If you need the additional flexibility,

drop me a note. I'll probably put it in the next version

anyway. However, rather than specifying the A to centre, B

to centre and C to centre distances, it seems to me that

specifying the A to B distance and then the ANGLE-relative

coordinates of the C node would be preferable (just as when

specifying a spline curve). Any preferences?

KURVED SWITCH (FROG) - defined by the radius and angle of an

outer curve (A to B) and an inner curve (A to C).

KURVED SWITCH (USER) - user and frog versions of this switch are

identical in their construction.

WYE SWITCH (FROG only) - defined by the A to centre and L (or R)

arm to centre distance, and the angle between the L and R

branches.

NOTE: When editing standard frog switches and wyes, use the

actual lengths of the B,C and L (R) branches, NOT the length

parallel to the A branch. Some catalogues (e.g. Lambert's)

give the parallel dimensions. Parallel B's and C's can be

converted by dividing them by cosine(angle). For wyes,

parallel L's (or R's) can be converted by dividing by

cosine(angle/2).

SPLINE SWITCH (FROG only) - defined by an outer spline from A to

the B arm and an inner spline from A to the C arm. ANGLE-Relative

X, Y offsets and the relative angle offset are specified for each

spline, assuming a left hand switch.

NOTE: If a User version of the Spline Switch would be useful to

you, drop me a note.

EDIT TRACK does not support 3-way switches, although this may

happen in a future version. For now create a regular switch and

then start a new tree right on top of it, curving off in the

direction of the third branch of your 3-way switch.

Now type X (eXit) to exit. Start up ET again by typing ET

followed by the name of the file you last saved. The .LAY

extension will be automatically added if you do not use a file

extension. Now try the arrow keys. By default the arrow keys move

the screen about 1/3 of a screen length in the direction you

choose. If, however, you now type 9, you will (with subsequent

arrow keys) move 9/9'ths or one full screen in the direction

chosen, or if you type 1, only 1/9th of a screen, etc.

Fool around and try some of the other commands out. For example,

select VIEW then ZOOM. Change the graphics (X Y) origin to (10

10) and the bottom (X) axis width of your window to 50. By

default the bottom window width is set to the maximum X

coordinate specified for the layout (1000 in this example).

Usually this is too big. Make it smaller to ZOOM in and examine

parts of your layout in detail. The mouse can easily be used to

ZOOM in, and you can then UNDO ZOOM and get back to where you

were before. When you specify the bottom (X) width of the window,

ET automatically selects the (Y) vertical height so that curves

are true and circles circular (not ellipses, etc.).

NOTE: if you zoom in too far your curves will replaced by a

series of linked lines that will not join up well with their

end nodes. I hope to have this problem fixed in the next

version of EDIT TRACK.

If you have an IBM compatible graphics printer or an HP Laserjet

or compatible then select FILE then HARDCOPY. The SCREEN option

just prints out the current screen. If you choose LAYOUT, ET

starts with bottom left hand corner of your layout and prints it

out in pieces up to the layout maximum coordinates. The width of

each piece is the same as what you see on the screen blown up to

a standard printer page width (about 7 inches). You defined the

height of each piece (e.g. 10 inches) in the UTIL PRINTER setup

form. All graphics print jobs are first written to internal

memory buffers, so HP Laserjet users will almost certainly run

out of memory if they set their resolution to 300 dpi and printer

height to anything over a couple of inches. 100 dpi should be

good enough for most jobs. If you run out of memory, reduce the

dpi and/or height.

THE CONFIGURATION FILE

----------------------

By default, ET reads in TRACK.CFG (and ET.OPT) every time it

starts up. You do not have to tell it to do this, but you can

force ET to read in a different track configuration file by

giving a second file name on the command line. For example:

ET MyLayout MyConfig

will tell ET to read in MYCONFIG.CFG as the configuration file

and MYLAYOUT.LAY as the layout file. Alternatively, if you

include the .CFG extension in the configuration file name, the

configuration file can be first, second or all alone on the

command line.

These configuration files are simple ASCII files that you can

easily edit to suit your own preferences. Do NOT use a word

processor like WordStar or WordPerfect unless NON-document or

Text-Out mode is used. Preferably, however, the track dimensions

should be edited and saved within ET by using the Edit function

described above. An example TRACK.CFG file follows:

ET 2.0

S 4 8.3300 15.4800 16.2840 0.2444

S 6 8.3300 19.2900 19.3150 0.1658

S 8 13.3350 23.4200 23.0950 0.1248

S 16 6.3300 12.4900 12.7200 0.1658

K 6 60.9600 50.8000 0.5709 0.6672

K 8 91.4400 81.2800 0.4820 0.5372

D 1 7.8800 1.5708

D 2 10.4000 1.0472

D 3 12.0000 0.7854

W 4 9.6840 14.0080 0.2138

Configuration files consist of ET 2.0 on the first line and as

many subsequent lines as necessary to list all the FROG objects

and their dimensions (one per line). Each starts with S, W, K, D,

p and F (for Switch, Wye, Kurved switch, Diamond, Spline switch

and frogged Spline respectively), followed by a frog number

between 0 and 99 (e.g. 4, 6 or 8).

S types then have four more numbers: length of the A, B, C

branches and the frog angle in degrees.

W types have three more numbers: the length of the A, L (or R)

branches and the full angle between the two branches in

degrees.

K types then have four numbers: radius of inner and outer curves,

followed by their respective arcs in degrees.

D types have two more numbers: the full length of each cross-arm

(not the half length) and the smaller of the angles the two

arms in degrees.

p types have six more numbers: the relative angle and X, Y

offsets for the outer then inner spline respectively (X, Y

must be specified in ANGLE-relative coordinates).

F types have three numbers: the relative angle and X, Y offsets

(in ANGLE-relative coordinates).

Be sure to use the same units in your TRACK.CFG file as you plan

to use throughout your layout plan for straight LINEs and

CURVE's. The TRACK.CFG file listed above and the one provided as

the default on the diskette has the dimensions in CENTIMETRES of

Shinohara code 70 Switches and Diamonds. However the Kurved

switches are Shinohara Code 100 and Switch #16 is a code 70

Shinohara #6 cut down to what I think is the minimum practical

dimensions on all three arms. This is useful for tight corners

etc. These dimensions are my best estimates but are by no means

exact.

Anybody who has gone to a lot of trouble to measure and check the

dimensions of other brands of pre-fab track should consider

passing them around as another .CFG file on the distribution

diskette. I would appreciate a copy as well.

On the diskette are two extra configuration files that have been

sent to me by users. One is called MARKLINZ.cfg and is for

Marklin Mini-Club sectional track in units of millimetre. Marklin

provides a curved switch that cannot be exactly represented by

two arcs of constant radius (one is actually a spline curve), so

I converted it to a spline switch. Hopefully my conversion is

accurate. There is one regular switch, one diamond and one spline

switch in this file, all are referenced as frog #1.

The other configuration file is called LGB.cfg. The dimensions

are right out of the LGB catalogue. There are two switches and

two diamonds one each corresponding to LGB's standard radius #1

(60 cm) and standard radius #3 (117.5 cm). The corresponding frog

#s are 1 and 3 as well. Switch #1 corresponds to LGB's #1200,

1205, 1210 and 1215 switches. Switch #3 corresponds to LGB's

#1605 and 1615. Diamond #3 is actually LGB's double x-over #1226.

Diamond #1 is LGB's #1300, but both arms are set to the length of

the shortest (30 cm) arm of this asymetric diamond. You will have

to add a 2.05 cm line to both sides of the other arm to match

LGB's.

LAYOUT/TREE FILES

-----------------

Layout files are simple text files that look like a combination

of the TRACK.CFG file and a printout. Tree files do not have the

dimensional information at the top. Take a look if you are

interested.

THE OPTION FILE

---------------

ET.OPT is a simple ascii text file that EDIT TRACK writes out at

the end of each session. It can also be edited with an  ascii

file editor. If one exists when you start ET, the options it

provides are used as defaults. The file CGA.OPT that is provided

for CGA displays is listed below:

ET 2.0

CGA 0.84200

NONE 0 0.00000

LPT1

0 1

ET 2.0 is always the first line. The second line lists the

display adapter (CGA, EGA or VGA) followed by the aspect ratio.

The third line is for graphics printing. It can be as shown, or

have IBMGP followed by the printer aspect ratio (1.2) and the

height (inches) of the printed graphics; or have HPLASER followed

by the printer aspect ratio (1.0), the height (inches) of the

printed graphics and the resolution in dots per inch. The fourth

line lists the output port for your standard DOS printer (LPT1,

LPT2, COM1, COM2, COM3 or perhaps a path\file name). The next

line lists the default file path. A blank line here means the

current directory. The last line lists the Units=Feet and Spline

Lines toggles (0=off, 1=on) respectively.

LAYOUT DESIGN TIPS

------------------

1. Study John Armstrong's superb books, especially "Track

planning for realistic operation" - Kalmbach Books.

2. EASEMENTS: typically the small change in radii due to spiral

curve easements at the beginning and end of a normal curve are

too small to justify plotting out (though EDIT TRACK does provide

both user and frog splines for this purpose if you want). It is

important however to incorporate the "lead in" section at each

end of the curve (L/2 in John Armstrong's Fig. 8.8) in your

trackplan. L is the length of the transition section from the

straight tangent to the full curve radius. The bigger the L, the

more gradual the easement. For my layout, I chose L = 25 cm; this

is only 1/4 the radius of my wide 100 cm curves but 1/3 the

radius of my minimum 75 cm curves (which is good, as you want a

greater percentage on your sharpest curves). This is the range

recommended by Paul Mallery in his book "Design Handbook of Model

Railroads" - Carstens Publ. However, John Armstrong recommends

L's (his Fig. 8.8) that are twice as large, ranging from just

over 1/2 to 2/3's of the curve's radius. He argues that it is

preferable to have sharper curves with more gradual easements

than vica-versa. You may want to test some alternatives before

choosing. At any rate, once you've decided on an L for a

particular curve, add straight sections of track of length L/2 at

the beginning and end of the curve in the track plan. Then, when

you actually lay the track, gradually "ease in" the curve

starting at the L/2 straight section  and continuing a distance

L/2 into the curve. See John Armstrong's Figure 8.8 for details.

3. REVERSE or S CURVES: usually it's a good idea to have a full

car length of straight track between the back-to-back reverse

curves. However when the curves are "eased", you can get away

with less, the exact amount depending upon your chosen L for

easements. In any case, your track plan will end up having 3

straight sections between the two curves: two L/2 sections with

your chosen reverse curve length in between (or one straight

section with the same length as all three).

4. PARALLEL SIDINGS and PASSING TRACKS: It is a good idea to

predesign a "standard" parallel siding entrance. Create a "Dummy"

tree with a 0d angle just for trying things out like this. First,

decide on the required distance between parallel tracks. I use 5-

6 cm. Now, starting with the branch (C) arm of a switch, add a

straight section followed by a curve with same arc but opposite

curvature as the switch (e.g. 9.5d right for a # 6 left switch).

To be consistent, the radius of the curved piece should be no

smaller than the closure rail radius of the switch (e.g. 43" or

109 cm for a #6; see Fig. 5.4 in Armstrong's book), and the

length of the straight piece in between should be that which

makes the parallel branch the required distance from the main

track. Trial and error is required here. On the layout, the

straight section can be "eased" into the curved section, as can a

section of the parallel siding beyond the curved piece (of course

this shortens the siding a bit). Note that parallel sidings and

passing track entrances are reverse curve situations, just like

cross-overs. They require careful handling.

5. LADDERS: similarly, it is useful to figure out on a "Dummy"

tree the necessary straight section lengths to put together yard

ladders of various types using various switches. Once you have

all the pieces figured out, you can plug them into more

complicated situations. In other words, it is easier to lay out a

yard on a dummy tree with an angle of 0d, rather than on the real

track plan where the yard angle might be 33d, or whatever.

Remember to watch for reverse curve situations.

6. PARALLEL CURVES: The outer parallel curve should have a radius

equal to the inner curve radius plus the between track distance

(BTD). If the angle at the start of the parallel curve is 0d,

then the curves start at the same X value, but the outer curve's

starting Y value is displaced by BTD. If the angle is 90d, then

the curves start at the same Y value but with X displaced by BTD

for the outer curve. In general, the start of the outer curve is

displaced in the X and Y dimensions from the inner curves

starting coordinates by BTD x SIN(angle) and BTD x COS(angle)

respectively. Or else just use trial and error!

7. WHEN TO STOP: It's not necessary to have everything match up

exactly in your track plan. Undoubtedly many small adjustments

will have to be made anyway when the track plan is actually

transformed into a layout. Anyway, translating a track plan into

a layout accurate to one millimeter (or 1/10 of an inch or

whatever) is a near impossibility.