DOS ODI WORKSTATION

August 1990 Edition

Revision 1.0

Novell, Incorporated

122 East 1700 South

P.O. Box 5900

Provo, Utah  84606  USA

�   Copyright 1990 Novell, Inc.  All rights reserved.  No

part of this publication may be reproduced, photocopied,

stored on a retrieval system, or transmitted without the

express prior written consent of the publisher.

Novell Part # 100-000871-001

Disclaimer

Novell, Inc. makes no representations or warranties with respect to the

contents or use of this manual, and specifically disclaims any express or

implied warranties of merchantability or fitness for any particular

purpose.  Further, Novell, Inc. reserves the right to revise this

publication and to make changes to its content, at any time, without

obligation to notify any person or entity of such revisions or changes.

Further, Novell, Inc. makes no representations or warranties with respect

to any NetWare software, and specifically disclaims any express or

implied warranties of merchantability or fitness for any particular

purpose.  Further, Novell, Inc. reserves the right to make changes to any

and all parts of NetWare software, at any time, without obligation to

notify any person or entity of such changes.

FCC warning

Computing devices and peripherals manufactured by Novell generate,

use, and can radiate radio frequency energy, and if not installed and

used in accordance with the instructions in this manual, may cause

interference to radio communications.  Such equipment has been tested

and found to comply with the limits for a Class A computing device

pursuant to Subpart J of Part 15 of FCC Rules, which are designed to

provide reasonable protection against radio interference when operated

in a commercial environment.  Operation of this equipment in a

residential area is likely to cause interference, in which case the user---at

his own expense---will be required to take whatever measures are

necessary to correct the interference.

Some components may not have been manufactured by Novell, Inc.  If

not, Novell has been advised by the manufacturer of the component that

the component has been tested and complies with the Class A computing

device limits as described above.

Copyright 1990 Novell, Inc.  All rights reserved.  No part of this

publication may be reproduced, photocopied, stored on a retrieval

system, or transmitted without the express prior written consent of the

publisher.

Novell, Incorporated

122 East 1700 South

Provo, Utah  84606  USA

August 1990 Edition

Manual Revision 1.0

For NetWare v3.1 and above

Novell Part # 100-000871-001

Table of Contents

How to Use This Manual                    ii

DOS ODI Workstation Installation           1

Prepare the workstations                4

Install the network board               5

Create the master workstation diskette  7

Create the workstation boot disk       11

Boot the workstation and log in

to the file server                     12

NET.CFG Options                           17

Link support                           20

Protocol                               21

Link driver drivername                 23

Link driver LANSUP                     30

Appendix A: Using the IBM Token-Ring

Source Routing Driver                  31

Using the Source Routing Driver

with DOS ODI                           33

Appendix B: System Messages               37

Trademarks                                69

Index                                     71

How to Use This Manual

This manual explains how to set up and install a DOS ODI workstation.

The installation section tells you how to prepare the workstation and

create boot diskettes for use with DOS ODI.

The NET.CFG section contains configuration information for the

NET.CFG file.  Use the information in this section if you are not using

the default options for your workstation.

Appendix A contains information on using the IBM Token-Ring Source

Routing Driver.

Appendix B contains system messages.

DOS ODI Workstation Installation

Novell's interconnectivity strategy for the 1990s, Open Data-Link

Interface (ODI), adds functionality to NetWare and network

computing environments by supporting multiple protocols and drivers.

ODI creates a �logical network board� to send different packet types

over one network board and wire.

You can benefit from ODI in the following ways:

You can expand your network by using multiple protocols (such as

IPX/SPX, AppleTalk, or TCP/IP as they become available) without

adding network boards to the workstation.

You can communicate with a variety of workstations, file servers, and

mainframe computers via different protocol stacks without rebooting

your workstation.

You can protect your investment because protocols send packets

through any LAN driver written to ODI specifications.

You can spend less time and money on support.  With one LAN driver

supporting multiple protocols, you have fewer hardware components

to support.

You can use the NET.CFG file to configure the LAN driver for any

possible hardware configuration, instead of being limited by SHGEN

choices.

You can free up memory by using only the portions of IPX/SPX and

diagnostics that you need.

As new LAN drivers and protocol transports are released, they will be

available on NetWire.

Using DOS ODI workstation software

You can convert a standalone personal computer into a DOS ODI

network workstation by adding one or more network boards, cabling,

and the DOS ODI workstation software.

If you are using a third-party network board, DOS ODI will not

function unless you have a DOS ODI LAN driver for the board.

Check the documentation that came with your network board for

more information.

DOS ODI Workstation software can be stored on a diskette or on a

workstation's hard disk.  Three sets of files allow workstations to

�talk� to each other, to the file server, and to other third-party hosts.

The first file is LSL.COM, the Link Support Layer file.  This file

enables the workstation to communicate over several protocols.

Next are the LAN driver files, such as NE2000.COM and NE2.COM.

Third are the protocol stack files, such as IPXODI.COM for NetWare

networks, that manage communications among the network stations.

In addition, you may need files specific to the networking product

you are using.  For example, if you want to install an ODI

workstation on a NetWare network, you need the shell file

(NETx.COM) that redirects messages from DOS to the network.

This section documents how to install a DOS ODI workstation on a

NetWare network using the IPX protocol stack.  To install a DOS ODI

workstation using another protocol stack or network, use these

instructions as an example.  You will have to modify them to fit your

situation.

You will need to follow the basic procedures of loading the LSL.COM,

the ODI LAN drivers, a protocol stack, and then any other files

specific to your software.

Remote boot is not available for DOS ODI workstations with this

release.

What you will need

To install DOS ODI workstations, you need the following:

The DOS ODI Workstation Installation flow chart (Quick Path) to get

an overview of the process

A working copy of the DOS ODI WORKSTATION SERVICES diskette

Enough DOS system diskettes to make master workstation diskettes

and boot diskettes

A �disk� (formatted diskette or hard disk) for each workstation to

boot from

The DOS ODI Workstation Installation flow chart (Quick Path) is

found at the beginning of this manual.

Table of Contents

Task                                                    Page

Prepare the workstations                                   4

Install the network board                                  5

Create the master workstation

diskette                                         7

Create the workstation boot disk                          11

Boot the workstation and log in

to the file server                              12

Troubleshooting tips                                      13

Where to go from here                                     15

Prepare the workstations

Check the hardware and memory requirements.

Use any of the following computers as workstations:

IBM PC or compatible

IBM PC XT or compatible

IBM PC AT or compatible

IBM Personal System/2 (any model)

NetWare workstations have the following memory requirements:

Up to 80KB of memory to load the NetWare workstation files

At least 512KB of memory to run application programs and NetWare

menu utilities

If your workstations need more memory, install additional memory

boards available from your computer dealer.  Consult the manuals

from your computer manufacturer for installing memory boards.

Record the hardware information for the workstation on the

Workstation Configuration Worksheet.

Set up your workstations.

Install all equipment for the workstation except the network board

and cabling.

Follow the hardware setup instructions in the Guide to Operations or

Quick Reference for each computer.

Boot each workstation with DOS.

Booting each workstation with DOS ensures that the station is

functional as a standalone computer.

When each workstation has booted successfully, turn it off and

continue with installation.

Install the network board

Make sure the network board conforms to your cabling topology (such

as RX-Net or Ethernet).

Select and set network board configurations.

Use the following guidelines.

Refer to the documentation that came with the network board to

select and set the board configuration.

Set the same configuration option on like boards for all your

workstations.  This reduces the number of master diskettes you will

have to make.

Choose the default configuration (shown as option 0 in the network

board supplements) if no other devices are using the settings.  (Many

network boards are factory set to the default.)

Make sure the configuration option of each board is unique from

those of other boards or hardware installed in the computer (network

boards, graphics and modem boards, etc.).

Record the network board configuration settings on the Workstation

Configuration Worksheet.

These settings can be helpful if you need to troubleshoot problems on

your network later.

Install the network board.

Make sure the workstation is turned off before installing

the board.

Create the master workstation diskette

The master workstation diskette is a copy of the files necessary to

boot one type of workstation.  You will copy files from the DOSODI

subdirectory of the DOS ODI WORKSTATION SERVICES diskette to

the master workstation diskette.

Then you will make individual workstation boot diskettes by copying

the files from the master workstation diskette to the boot diskettes

and by customizing the boot diskettes.

Create system diskettes by using the DOS FORMAT command with

the /s parameter.

Format these diskettes using the correct version of DOS registered for

each workstation on the network.

Label the master workstation diskette.

Label the master workstation diskette with the name of the LAN

driver (such as �Master Workstation NE2000�), and list the

configuration settings on the diskette.

Configure software files (such as DXMAID) that came with your

network board (optional).

If you received additional software with your network board (for

example, the DXMAID program for Token-Ring network boards), see

the

installation supplement or accompanying documentation for steps to

configure those files.

Copy the shell file you want from your NetWare diskette to the master

workstation diskette.

(The x in NETx.COM refers to the DOS version that runs on your

workstations.)

If you are using the extended or expanded memory shells, copy the

.SYS files from the diskettes that come with the extended or expanded

memory applications.  Specify those files as device drivers in your

CONFIG.SYS file so that they will be loaded when the workstation

boots.  For example,

DEVICE = HIMEM.SYS

Copy the workstation files from the DOSODI subdirectory of the DOS

ODI WORKSTATION SERVICES diskette to the master workstation

diskette.

The following files are required to communicate with a NetWare

server:

LSL.COM

The LAN driver for your network board (NE2000.COM, for example)

The protocol stack file (IPXODI.COM, for example)

NETx.COM

The LANSUP.COM driver is for Token-Ring networks using the IBM

LAN SUPPORT PROGRAM.  The LANSUP.COM file can be used

with PC Network and with Token-Ring networks.

The following files are optional on the master workstation diskette:

Put the executable files you selected into an AUTOEXEC.BAT file on

the master workstation diskette.

Use a DOS text editor to create the AUTOEXEC.BAT file, and save it

to the master workstation diskette.  It is important to load the files in

the following order: link support layer, LAN driver, protocol stack,

then the shell.

This is a sample AUTOEXEC.BAT file:

LSL

NE2000

IPXODI

NET3

See your DOS manual for ideas.

Create a CONFIG.SYS file on the master workstation diskette using a

DOS text editor (optional).

If you are using memory managers, you need a line in CONFIG.SYS to

specify the memory managers as devices.  See your memory manager

documentation for details.

If you are using the IBM LAN SUPPORT PROGRAM, you must add

two to three device drivers to CONFIG.SYS.  See your DOS manual

for details.

Create a NET.CFG file on the master workstation diskette (optional).

Usually, you do not need a NET.CFG file because the established

defaults are adequate for your network.

Test boot a workstation with the master workstation diskette.

Create the workstation boot disk

Copy the master workstation diskette files to each workstation boot

disk.

If you are copying files to diskette, use the DOS FORMAT command

with the /s parameter to make a system diskette.

Personalize the boot disk with executable files, and update the

AUTOEXEC.BAT file (optional).

If the owner of the bootable files wants any other commands executed

or programs loaded during the boot process, add those files to the boot

area, and make the appropriate changes to the AUTOEXEC.BAT file.

Label the workstation boot diskette (optional).

If you are using a boot diskette, label it with the LAN driver, the

configuration option, and the custom boot files included.  This

prevents the boot diskette from being confused with other diskettes.

Record the boot file information.

This information is helpful for troubleshooting.

Repeat Steps 1 through 4 for each workstation.

Boot the workstation and log in to the file server

Boot the workstation.

Boot your workstation either of the following ways:

Insert the workstation boot diskette into drive A and turn the

workstation on.

Copy the boot diskette files to the workstation's root directory on the

hard disk, and reboot by pressing <Ctrl> <Alt> <Delete>

simultaneously.

Log in to the file server.

If your AUTOEXEC.BAT file does not contain login commands, type

F:  <Enter>

LOGIN SUPERVISOR  <Enter>

Troubleshooting tips

If you get the message �A File Server could not be found,� check to

see if

The network board is seated properly.

The cable is connected properly.  (Run COMCHECK to verify the

connections.)

All cabling is terminated properly.

The workstation node address conflicts with any other node address.

If you want information about the drivers that are loaded, type a

question mark after the driver name.

For example

NE2000 ?  <Enter>

or

LSL.COM ?  <Enter>

or

IPXODI ?  <Enter>

Remove parts of IPXODI.COM to save memory.

The IPXODI.COM file actually contains three parts:

IPX

SPX

Remote Diagnostics Responder

Since many applications do not need SPX or the Remote Diagnostics

Responder (required for third-party applications that gather

diagnostics information), some workstations can save memory by  not

loading these parts of the IPXODI.COM file.

Some NetWare utilities, such as RCONSOLE and NVER,

require SPX and the Remote Diagnostics Responder to be

loaded.

Use the chart below to see how to load IPXODI.COM without the

other two parts of the file.

To unload ODI drivers from the workstation, unload them in reverse

order.

For example, if you work with a protocol that requires a different

shell or if you are working with two protocols that conflict when both

are running on the workstation, unload the first set of files (in

reverse order) before you load the next set.

Unload the workstation files as follows:

NET3 u  <Enter>

IPXODI u  <Enter>

NE2 u  <Enter>

LSL u  <Enter>

If you do not unload the files in reverse order, you get an error

message similar to this: FATAL: There is a TSR above the loaded

IPXODI.

Where to go from here

Notes

NET.CFG Options

The NET.CFG file contains the configuration information for the

workstation.  It is a control file that contains section headings and the

currently defined options that deviate from the established defaults of

the regular workstation boot process.

Usually, you do not need a NET.CFG file because the established

defaults are adequate for your network.

Use any DOS text editor to create the file.  Specify only options that will

change from the defaults.

If you have installed a dedicated IPX workstation, you are probably

familiar with the SHELL.CFG file that also contains network

configuration information.  The information in SHELL.CFG is now

migrating to the more versatile NET.CFG file.  Any options from the

SHELL.CFG file can be addressed in the NET.CFG file.

Use the chart below to determine whether to use both .CFG files or to

combine them into NET.CFG.

If you place any SHELL.CFG options in the NET.CFG file, they will be

ignored.

See your NetWare manual for SHELL.CFG options.

Conventions

The NET.CFG file is left-justified for main section headings and

indented (with either a space or a tab) under each heading.

Keywords and main headings are not case sensitive.

Below are the most common section headings in the NET.CFG file:

Link support

Protocol (protocol_name)

Link driver (drivername)

The heading must precede the options you want to include in that

section.  End each line with a hard return.

Write all numbers in decimal notation except where noted otherwise.

Precede comments by a pound (#) sign.

Sample NET.CFG file

LINK DRIVER NE1000

# Change the interrupt (IRQ) to 5.

INT 5

# Change the DMA channel to 3.

DMA 3

# Change the base memory address (hex) to an

# absolute value of C0000.

MEM C0000

# Change the port to 300 (hex)

PORT 300

The following chart lists the options unique to the NET.CFG file.  Main

section headings are in white print and flush with the left margin.  The

options are listed under each heading and indented.

Following the chart is a brief explanation of each option's function and

some possible reasons for changing the setting.

If you are using third-party protocol or LAN drivers, see their

documentation for any additional parameters.

In the following definitions, these conventions are used:

[ ]                     An optional element

number            A decimal number

hex                     A hexadecimal number

Link Support

BUFFERS communication_number [size]

This option configures the number of receive buffers and their size on

the network.

The number of communication buffers must be large enough to hold all

media headers and the maximum data size.

Default:  0

The IPXODI protocol stack does not use the Link Support Layer

communication buffers.

Refer to the documentation for the third-party protocol stack for

settings.

Buffer size is optional.  The minimum size is 586.  The total buffer space

must fit into approximately 59KB (number times buffer size).

Default:  1130

MEMPOOL number [k]

Some protocols use this option to configure the size of the memory pool

buffers that the Link Support Layer will maintain.

The IPXODI protocol stack does not use the MEMPOOL buffers.

Refer to the documentation for the third-party protocol stack for

settings.

The k notation means multiply by 1024.

Protocol "protocol_name"

BIND name

Usually IPXODI binds to the first network board it finds.  This option

limits the search to the network board you specify.

Replace name with one of the following LAN driver names:

TRXNET (for Novell RX-Net and Turbo RX-Net)

NE2 (for Novell Ethernet NE/2)

NE2-32 (for Novell Ethernet NE/2-32)

NE1000 (for Novell Ethernet NE1000)

NE2000 (for Novell Ethernet NE2000)

LANSUP (for the IBM LAN Support program only)

3C503 (for 3Com EtherLink Series 503)

3C523 (for 3Com EtherLink/MC 3C523)

In a DOS environment, you can bind IPXODI.COM to only one network

board.

For example, if you have an NE1000 board in slot 0 and an NE2000

board in slot 1 in your workstation, IPXODI binds to the NE1000 board

if you specify nothing in the NET.CFG file.  However, if you place

BIND NE2000 in the NET.CFG file, IPXODI binds to the NE2000 board.

DEFAULT name

This option requests the protocol stack to bind with the LAN driver

(MLID) and configures the protocol to a default stack.

IPXODI ignores this parameter.

Replace name with one of the LAN driver names listed for BIND on the

previous page.

PRESCAN name

This option requests the protocol stack to bind with the LAN driver

(MLID) named and configures the protocol to a default prescan stack.

IPXODI ignores this parameter.

Replace name with one of the LAN driver names listed for BIND on the

previous page.

SESSIONS number

This option configures the number of sessions that the protocol stack

will be required to maintain at one time.

See the third-party protocol documentation for the minimum and

maximum values.

IPXODI ignores this parameter.

Link Driver drivername

To use this heading, replace drivername with the name of the driver you

are using; then use one or more of the following options.  The name is

typically the LAN driver's filename.

You can use these options with as many different network boards as you

have, but you must have a separate Link Driver drivername heading for

each board.

Replace drivername with one of the following driver names:

TRXNET (for Novell RX-Net and Turbo RX-Net)

NE2 (for Novell Ethernet NE/2)

NE2-32 (for Novell Ethernet NE/2-32)

NE1000 (for Novell Ethernet NE1000)

NE2000 (for Novell Ethernet NE2000)

LANSUP (for the IBM LAN Support program only)

3C503 (for 3Com EtherLink Series 503)

3C523 (for 3Com EtherLink/MC 3C523)

Place both hardware commands and software commands under the Link

driver heading.

Hardware commands

DMA [#1 | #2] channel_number

This option specifies the hardware setting of the network board used in

the workstation.  This option allows one of two DMA channels to be

configured.

Enter the channel number to be used.  (The driver for the board will

automatically select its default channel number.)

If you do not specify which of the two configurable DMA channels (#1

or #2) to configure, this option defaults to the first configurable channel

(#1).

You do not need to include the #1 option if you are using only the

default option.  For example, if the first configurable DMA channel on

your board uses DMA channel 3, place the following lines in your

NET.CFG file:

Link Driver name

DMA 3

For example, if the first configurable DMA channel on your board will

use channel 3 and the second configurable channel will use channel 4,

place the following lines in your NET.CFG file:

Link Driver name

DMA 3

DMA #2 4

Notice that the second DMA channel requires you to use the pound sign

(#).

INT [#1 | #2] interrupt_request_number

This option specifies which interrupts the network board uses.  Use the

interrupt request number set on the board.

If you do not specify which interrupt line (#1 or #2) to configure, this

option uses the default, which is the first configurable interrupt line

(#1).

Do not include the #1 option if you are using only the default option.

For example, if the first configurable interrupt line on your board will

use IRQ 2, place the following lines in your NET.CFG file:

Link Driver name

Int 2

If your board can use two configurable interrupt lines and you want to

use both of them or only the second one, you must specify the interrupt

line (#1 or #2) to configure.

For example, if the first configurable interrupt line on your board will

use IRQ 2 and the second will use IRQ 3, place the following lines in

your NET.CFG file:

Link Driver name

Int 2

Int #2 3

The second DMA channel requires you to use the pound sign (#).

MEM [#1 | #2] hex_starting_address [hex_length]

This option specifies a memory range to be used by the network board.

Use the hexadecimal physical (absolute) address of the memory used by

the board.  (This starting address must match the starting address

configured on the board.)

Enter the length (in hexadecimal paragraphs) of the memory address

range used by the board.  A paragraph is normally 16 bytes.

For example, to address a board from D0000 to D4000 (bytes), the

starting address is D0000 and the range is 400 (paragraphs in 16KB

decimal length).  In this case, place the following lines in your NET.CFG

file:

Link Driver name

Mem D0000

Usually, the length is not needed.

PORT [#1 | #2] hex_starting_address hex_number_of_ports

Use this option to specify the starting port and number of ports in the

range.

Use the starting hexadecimal I/O port number.

Use the hexadecimal number of ports in the range.  If your network

board can use two ranges and you want to use either the second range or

both ranges, you must specify which range (#1 or #2) to use.

All values must be written in hexadecimal notation.

For example, suppose you want to specify the starting port and the

number of ports in the first range on your board.  The starting I/O port

is 300; 16 ports are in the first range.  Place the following lines in the

NET.CFG file:

Link Driver name

Port 300

The number of ports is optional.

NODE ADDRESS hex_address

This option overrides any hard-coded node address in the network board,

if the hardware allows it.

Use the hexadecimal address number.

Changing the node address on a board can create conflicts with

other network boards.  Stay with the hard-coded node address

whenever possible.

The example below shows how to change the node address on a 3C523

board:

Link Driver 3C523

Node Address 12D43

PS/2 SLOT number

Use this option if you want to access two separate Ethernet backbones.

Use the number of the slot into which you inserted the board.  The slot

number is found on the back of the computer.  The LAN driver will use

the board in this slot.

For example, if you are using two NE/2 boards in the same workstation

and you insert one board into slot 1 and the other into slot 2, place the

following lines in your NET.CFG file:

Link Driver NE2

Link Driver NE2

PS/2 Slot 2

Then place the following lines in your AUTOEXEC.BAT file:

LSL

NE2

NE2

PS/2 SLOT ?

Use this option if you have inserted only one of the same kind of

network board in the workstation.

This option signals the LAN driver to scan for its board.

Software commands

FRAME frame_type

This option specifies the frame type that is used with the network board.

Use this option with boards that support more than one frame type.

For example, to specify an Ethernet board using the Ethernet II frame

type, place the following line in the NET.CFG file beneath the heading

for that driver:

Frame ETHERNET_II

Usually, Ethernet LAN drivers default to Ethernet_802.3; Token-Ring

LAN drivers default to Token-Ring.

Drivers that support more than one frame type (such as Ethernet) allow

multiple "frame" commands.

LOOK AHEAD SIZE number

This option specifies the number of bytes in the packet that the LAN

driver will send to the Link Support Layer to determine how to route the

packet.  The range is 0 to 128 bytes depending on the protocol.  See the

protocol documentation for the recommended size.

If two or more protocols are using a LAN driver, select the highest value

for the look-ahead size.

Default:  18 bytes

PROTOCOL name hex_protocol_ID frame_type

This option enables new protocols to be handled by existing LAN

drivers.

Replace name with the name of the new protocol.

Use the hexadecimal protocol ID number.

Replace frame_type with the frame type that the new protocol ID

applies to.

For example, if you attach a new protocol (XYZ) to your NE2-32

network board, the NET.CFG file could look similar to the following:

Link Driver NE2-32

Frame ETHERNET_SNAP

Protocol XYZ 904A ETHERNET_SNAP

SEND RETRIES number

This option specifies the maximum number of times the network board

driver will try to resend a packet following a communications error.

Use the number of times you want the driver to try resending the packet.

The default value is determined by the driver.

Link Driver LANSUP

The options below are unique to the LANSUP driver.

SAPS number

If you use the LANSUP driver with a network board, you must specify

the number of Service Access Points (SAPs) needed.  Set the option to

allow for all applications using the IBM LAN Support Program.  The

maximum value depends on the type of board used.

Default:  1

LINK STATIONS number

If you use the LANSUP driver with a network board, you must specify

the number of link stations needed.  Set the option to allow for all

applications using the IBM LAN Support Program.  The maximum value

depends on the type of board used.

Default:  1

Appendix A:  Using the IBM Token-Ring Source Routing Driver

This appendix explains the use of the IBM Token-Ring Source Routing

Driver.

The IBM Token-Ring Source Routing Driver enables communication

across IBM Token-Ring network bridges.  Any type of protocol stack can

use this DOS ODI feature to communicate across IBM Token-Ring

network bridges.

All nodes that need to communicate across a source route bridge must be

running the IBM Token-Ring Source Routing Driver.

For more information on using source routing, see the IBM Token-Ring

Network Architecture Reference manual.

The following figure shows an example network configuration using

IBM source routing bridges.

Figure A.1A network configuration using IBM source routing bridges

Using the Source Routing Driver with DOS ODI

To install source routing on workstations, complete the following steps.

Create a boot disk according to the instructions on page 11 of this

manual.

Copy the ROUTE.COM file from the DOS/ODI WORKSTATION

SERVICES diskette to the boot disk.

If you are creating an AUTOEXEC.BAT file for a DOS ODI

workstation, the command to load ROUTE.COM should be added after

LANSUP.COM but before the protocol stack you are using.

Set the parameters for ROUTE.COM.  See "Additional Information" on

the next page for the parameters that ROUTE.COM can be loaded with.

Add the parameter in the AUTOEXEC.BAT file.

The default parameters for ROUTE.COM can be used with most network

communications.

Additional Information

The following parameters can be entered when ROUTE.COM is first

loaded.

BOARD = number    MBR

CLEAR                         NODES = number

DEF                     REMOVE = number

GBR

The parameters can also be changed by loading ROUTE.COM a second

time.  For on-line help, type

ROUTE ?  <Enter>

Most of the parameters have default values that should work with simple

configurations for IBM bridges.  If you have installed parallel IBM

bridges, you can change some of the parameters to reduce traffic on

some of the paths.  Each parameter is described below.

BOARD = number

Use this parameter to specify a board number for the Token-Ring

network board.

If this parameter is not specified, the default of 01 is used.

If you have loaded more than one LAN driver in the workstation, the

board number is determined by the order in which the LAN drivers are

loaded (the first board is 1).  See your AUTOEXEC.BAT file for the

order you have specified.

CLEAR

Use this parameter to manually clear the Source Routing table and force

a dynamic rebuilding of the table by sending a default frame to each

node in the network.  Use this parameter when an IBM bridge on the

network has gone down and an alternate route is available.

DEF

Use this parameter to prevent frames that have unknown (DEFault)

destination addresses from being sent across Single Route IBM bridges.

If this parameter is specified, all frames that have addresses not in the

workstation's Source Routing table are forwarded as All Routes

Broadcast frames.

If this parameter is not specified, all frames that have addresses not in

the workstation's Source Routing table are forwarded as Single Route

Broadcast frames.

If ROUTE.COM has already been loaded with the DEF parameter,

reloading ROUTE.COM with this parameter broadcasts all subsequent

Single Route Broadcast frames as All Routes Broadcast frames.

GBR

Use this parameter to specify that all General BRoadcast frames are to

be sent as All Routes Broadcast frames.

If this parameter is not specified when ROUTE is loaded, all General

Broadcast frames are broadcasted as Single Route Broadcast frames.

If ROUTE has already been loaded with the GBR parameter, reloading

ROUTE with this parameter broadcasts all subsequent General Broadcast

frames as All Routes Broadcast frames.

MBR

Use this parameter to specify that all Multicast BRoadcast frames are to

be sent as All Routes Broadcast frames.

If the parameter is not specified when ROUTE is loaded, all Multicast

Broadcast frames are broadcast as Single Route Broadcast frames.

If ROUTE has already been loaded with the MBR parameter, reloading

ROUTE with this parameter broadcasts all subsequent Multicast

Broadcast frames as All Routes Broadcast frames.

NODES = number

Use this parameter to specify the number of table entries in the Source

Routing table.  This parameter must be entered the first time

ROUTE.COM is loaded.  The parameter cannot be changed by reloading

ROUTE.COM.

Replace number with a value from 8 to 255.  The default is 16.

REMOVE = number

Use this parameter to remove a specified node address from the

workstation's Source Routing table.  Use the parameter when a bridge

has gone down.  Removing the node from the Source Routing table

forces the workstation to determine an alternate path.

Replace number with a 12-digit (6-byte) hexadecimal number.  If fewer

than 9 digits are entered, ROUTE.COM prefixes the address with 4000h.

For example, REMOVE=2 is interpreted as REMOVE=400000000002.

Appendix B:  System Messages

Connector is BNC.

Source:  3C503.COM

Explanation:  The connector selected for the board is for Thin

Ethernet cabling.

Action:  None

Connector is DIX.

Source:  3C503.COM

Explanation:  The connector selected for the board is for Thick

Ethernet cabling.

Action:  None.

FATAL: 3C503 -Card not found.

Source:  3C503.COM

Explanation:  The 3C503.COM driver could not locate a 3C503

board at the specified hardware settings.

Action:  Make sure that a 3C503 board is installed.  Verify the

hardware jumper settings.  If the settings do not match the

defaults, specify the settings in the NET.CFG file.

FATAL: 3C503 -Card will not initialize.

Source:  3C503.COM

Explanation:  A hardware failure occurred.

Action:  Check for possible hardware settings that conflict with

other installed boards.

FATAL: 3C503 -Interrupt is invalid (2,3,4,5,9 valid).

Source:  3C503.COM

Explanation:  An invalid interrupt value was specified in the

NET.CFG file.

Action:  Correct the �INT number� entry in the NET.CFG file;

then try again.

FATAL: 3C503 -I/O port does not match NICs jumper.

Source:  3C503.COM

Explanation:  The specified base I/O port address (the driver

default or the value specified in the NET.CFG file) is different

from the address setting on the 3C503 board.

Action:  Make sure that the value for the driver matches the value

set on the board; then try again.

FATAL: 3C503 -Memory does not match NICs jumper.

Source:  3C503.COM

Explanation:  The specified base memory address (the driver

default or the value specified in the NET.CFG file) is different

from the address setting on the 3C503 board.

Action:  Make sure that the value for the driver matches the value

set on the board; then try again.

FATAL: Board # <number> doesn't provide enough look ahead, IPX

needs 18 or more bytes.

Source:  IPXODI

Explanation:  IPX requires at least the first 18 bytes of incoming

packets for it to preview the packets properly.

Action:  Increase the MLID look-ahead size by specifying �LOOK

AHEAD SIZE 18� in the NET.CFG file under the MLID main

section heading, as in the following example:

LINK DRIVER NE1000

LOOK AHEAD SIZE 18

FATAL: Board failed to initialize correctly.

Source:  MLID

Explanation:  The MLID could not initialize its board correctly.

A hardware failure usually prevents initialization.

Action:  Refer to the MLID documentation for a description of

the error.

FATAL: Could not find a board that supports IPX (See PROTOCOL

keyword).

Source:  IPXODI

Explanation:  IPX could not find a board to bind with.  For IPX

to bind, it must have a protocol ID registered with the Link

Support Layer.  Most MLIDs will register an IPX Protocol ID by

default.  However, if you have specified the PROTOCOL

keyword under an MLID's section heading in the NET.CFG file,

this error may be generated.  When a protocol ID has been

specified for another protocol stack, the MLID does not register

a default protocol ID for IPX.

Action:  If you want to use IPX, register a protocol ID for IPX

along with the other Protocol IDs you are registering.

FATAL: Could not find an enabled 3C523 adapter in any slot.

Source:  3C523.COM

Explanation:  By default, the 3C523.COM driver scans the PS/2

slots to find the 3C523 board it should use.  The driver starts

scanning at slot 1.  The driver was unable to find a 3C523 in any

slot.

Action:  Make sure that the 3C523 board is firmly seated in the

slot.

FATAL: Could not find an NE2 adapter in any slot.

Source:  NE2.COM

Explanation:  By default, the NE2.COM driver will scan the PS/2

slots to find the NE2 board it should use.  The driver starts

scanning at slot 1.  The driver could not find an NE2 in any slot.

Action:  Make sure that the NE2 board is firmly seated in the

machine's slot.

FATAL: Could not find an NE2-32 adapter in any slot.

Source:  NE2-32.COM

Explanation:  By default, the NE2-32.COM driver will scan the

PS/2 slots to find the NE2-32 board it should use.  The driver

starts scanning at slot 1.  The driver was unable to find an NE2-

32 in any slot.

Action:  Make sure that the NE2-32 board is firmly seated in the

slot.

FATAL: Could not find <name> MLID to unload.

Source:  MLID

Explanation:  A request was made to unload the MLID, but the

MLID is not loaded.

Action:  None.

FATAL: Different IPX or a IPX interrupt has been hooked.

Source:  IPXODI

Explanation:  You tried to unload the IPX from memory, but the

IPX detected a condition that would not allow IPX to be removed

from memory safely.  This can occur for two reasons:

The resident IPXODI and the IPXODI used to unload the resident

IPXODI are not the same version.

Another program has been loaded that has hooked one of

IPXODI's interrupt vectors.  IPXODI uses the following interrupt

vectors:  INT 08h, INT 2Fh, INT 64h, and INT 7Ah.

Action:  Complete one of the following:

Use the same version of IPXODI to unload IPX as you used to

load.

Unload the program that has hooked to one or more IPX interrupt

vectors and then unload IPXODI.

FATAL: Different LSL or a LSL interrupt has been hooked.

Source:  LSL

Explanation:  You tried to unload the LSL from memory, but it

detected a condition that would not allow it to be removed safely.

This can occur for two reasons:

The resident LSL and the LSL used to unload the resident LSL

are not the same version.

Another program has been loaded that has hooked one of the

LSL's interrupt vectors.  The LSL uses the following interrupt

vectors:  INT 08h and INT 2Fh.

Action:  Complete one of the following:

Use the same version to unload LSL as you used to load it.

Unload the program that has hooked to the LSL's interrupt

vectors before unloading the LSL.

FATAL: Direct Station 0 already in use by another application.

Source:  LANSUP.COM

Explanation:  The LANSUP.COM driver uses Direct Station 0.

Only one application running on the IBM LAN Support program

can use Direct Station 0.

Action:  Unload the other application.

FATAL: Error initializing board.

Source:  LANSUP.COM

Explanation:  The LAN Support Program could not initialize the

network board.

Action:  Check the installation of the LAN Support software.

FATAL: Error opening board.

Source:  LANSUP.COM

Explanation:  The LAN Support Program could not open the

network board.

Action:  Check the installation of the LAN Support software.

FATAL: Error shutting down <name> MLID, unload operation aborted.

Source:  MLID

Explanation:  The MLID was attempting to remove the resident

MLID from memory.  The MLID was unable to successfully shut

down the resident MLID.  A hardware failure has probably

occurred.

Action:  Run diagnostics on the network board and workstation.

Replace the faulty hardware.

FATAL: Failed to locate MLID Section Heading in NET.CFG file.

Source:  MLID

Explanation:  You tried to load the MLID again.  Normally you

would do this so that you could use two or more boards in the

workstation.  When two or more of the same type of network

boards are installed in the workstation, an associated MLID

section heading must be specified in the NET.CFG file. An entry

in the NET.CFG file for two boards would look similar to the

following:

#Allow the first one to use default values.

LINK DRIVER NE1000

#This section is for the second board.

LINK DRIVER NE1000

INT 4

PORT 320

Action:  Add the commands for both MLID boards to the

NET.CFG file.  Then reboot the workstation.

FATAL: IBM LAN Support Program has not been loaded.

Source:  LANSUP.COM

Explanation:  The LANSUP.COM driver requires that the IBM

LAN Support Program be loaded.

Action:  Load the LAN Support Software and retry the operation.

Add lines similar to the following to the CONFIG.SYS file:

DEVICE=DXMA0MOD.SYS

DEVICE=DXMC0MOD.SYS

FATAL: Init586:  Configure command failed.

Source:  3C523.COM

Explanation:  A hardware failure has occurred.

Action:  Install the 3C523 board in a different slot.  If the driver

still fails to load, the 3C523 board is probably bad and should be

replaced.

FATAL: Init586:  IA_Setup command failed.

Source:  3C523.COM

Explanation:  A 3C523 hardware failure has occurred.

Action:  Install the 3C523 in a different slot.  If the driver still

fails to load, the 3C523 board is probably bad and should be

replaced.

FATAL: Init586:  Initial communication with 82586 failed.

Source:  3C523.COM

Explanation:  A hardware failure has occurred.

Action:  Install the 3C523 board in a different slot.  If the driver

still fails to load, the 3C523 board is probably bad and should be

replaced.

FATAL: Invalid base memory address - Must be C0000 or D0000.

Source:  NE2-32.COM

Explanation:  Because the NE2-32 board is a true 32-bit board, the

board has the option to have its base memory located above the

1MB real mode memory limit.  Since DOS ODI is executing in real

mode, the NE2-32.COM driver cannot access the board's RAM

when it is located above the 1MB address range.

Action:  Use the Reference diskette to change the board's base

memory setting to either C0000 or D0000.

FATAL: Invalid Ethernet node address specified.

Source:  MLID

Explanation:  You used the �NODE� option in the NET.CFG file

to override the node address on the network board.  The number

specified was not a valid Ethernet node address.  An Ethernet

address is 6 bytes in length.  This error occurs if Bit 0 of the first

address byte is a 1.  This bit must always be 0.  For example, if

the first byte has the following address, an invalid Ethernet

address is generated.

First Byte

7 6 5 4 3 2 1 0

���������������Ŀ

�0�0�0�0�0�0�0�1�

�����������������

This byte will produce node addresses in the 0100 0000 0000 to

01FF FFFF FFFF range, all of which will be invalid.

Action:  Change the NET.CFG file so that a valid node address

is specified.

FATAL: Invalid parameter.

Source:  IPXODI

Explanation:  When you tried to load IPXODI, you specified an

invalid parameter on the command line.  IPXODI was not loaded.

The only valid parameters are ? for information and U for

unload.

Action:  Specify a valid parameter.

or

Source:  LSL

Explanation:  When you tried to load the LSL, you specified an

invalid parameter on the command line.  The LSL was not loaded.

The only valid parameters are ? for help and U for unload.

Action:  Specify a valid parameter.

or

Source:  MLID

Explanation:  When you tried to load the MLID, you specified an

invalid parameter on the command line.  The MLID was not

loaded.  The only valid parameters for MLID are ? for

information and U for unload.

Action:  Specify a valid parameter.

FATAL: Invalid Token-Ring node address specified.

Source:  MLID

Explanation:  You used the �NODE� option in the NET.CFG file

to override the node address on the network board.  The number

specified was not a valid Token-Ring node address.  A Token-

Ring address is 6 bytes in length.  This error will occur if Bit 7

of the first address byte is a 1.  This bit should always be a 0.

For example, if the first byte has the following address, an

invalid Token-Ring address is generated.

First Byte

7 6 5 4 3 2 1 0

���������������Ŀ

�1�0�0�0�0�0�0�0�

�����������������

This byte will produce node addresses in the 8000 0000 0000 to

8FFF FFFF FFFF range, all of which will be invalid.

Action:  Change the NET.CFG file so that a valid node address

is specified.

FATAL: IPX already loaded.

Source:  IPXODI

Explanation:  IPX has already been loaded.  It needs to be loaded

only once.

Action:  None.

FATAL: IPX is already registered with the LSL.

Source:  IPXODI

Explanation:  IPX was not removed from the LSL's register when

it was unloaded.  The system may be corrupted.

Action:  Reboot the workstation.

FATAL: IPX is not loaded.

Source:  IPXODI

Explanation:  You tried to unload IPX, but IPX has not been

loaded.

Action:  None.

FATAL: Loading MLID again requires configuration information in

NET.CFG.

Source:  MLID

Explanation:  You tried to load the MLID a second time.

Normally you would do this so that you could use two or more

boards in the workstation.  When two or more of the same type of

network boards are installed in the workstation, an associated

MLID section heading must be specified in the NET.CFG file.

An entry in the NET.CFG file for two boards would look similar

to the following:

#Allow the first one to use default values.

LINK DRIVER NE1000

#This section is for the second board.

LINK DRIVER NE1000

INT 4

PORT 320

Action:  Create a NET.CFG file and add the commands for both

MLID boards to the file.  Then reboot the workstation.

FATAL: LSL already loaded.

Source:  LSL

Explanation:  The Link Support Layer has already been loaded.

The LSL can be loaded only once.

Action:  None.

FATAL: LSL is not loaded.

Source:  LSL

Explanation:  You tried to unload the LSL, but it is not loaded.

Action:  None.

FATAL: Multiplex interrupt 2Fh has no free slots.

Source:  LSL

Explanation:  LSL could not find a free slot for use with INT 2F

(Hex).  When your computer is first booted, approximately 63

multiplex slots are available.  All available slots are already being

used by applications.

Action:  Unload an application that is using a INT 2F multiplex

interrupt; then load the LSL.

FATAL: NE1000 NIC command port failed to respond.

Source:  NE1000.COM

Explanation:  The NE1000 board has malfunctioned, is not

installed, or is not configured to the same port as selected for the

driver.

Action:  Complete one or more of the following:

Make sure that an NE1000 board has been installed and

configured.

Make sure that the configuration for the NE1000 board matches

the configuration for the NE1000 driver.  Configuration options

other than the defaults must be entered in the NET.CFG file.

Replace the board.

FATAL: NE1000 NIC RAM failure.

Source:  NE1000.COM

Explanation:  The NE1000 board's on-board memory has

malfunctioned.

Action:  Replace the board.

FATAL: NE2 NIC command port failed to respond.

Source:  NE2.COM

Explanation:  The installed NE2 board has malfunctioned.

Action:  Replace the board.

FATAL: NE2 NIC RAM failure.

Source:  NE2.COM

Explanation:  The NE2 board's installed on-board memory has

malfunctioned.

Action:  Replace the board.

FATAL: NE2000 NIC command port failed to respond.

Source:  NE2000.COM

Explanation:  The NE2000 board has malfunctioned, is not

installed, or is not configured to the same port as selected for the

driver.

Action:  Complete one or more of the following:

Make sure that an NE2000 board has been installed and

configured.

Make sure that the configuration for the NE2000 board matches

the configuration for the NE2000 driver.  Configuration options

other than the defaults must be entered in the NET.CFG file.

Replace the board.

FATAL: NE2000 NIC is NOT supported in a 8-bit slot.

Source:  NE2000.COM

Explanation:  The NE2000 board is located in an 8-bit slot in the

machine.  The NE2000.COM driver only supports the NE2000 in

a 16-bit AT bus slot.

Action:  Place the NE2000 board in a 16-bit slot and try again.

FATAL: NE2000 NIC RAM failure.

Source:  NE2000.COM

Explanation:  The NE2000 board's on-board memory has

malfunctioned.

Action:  Replace the board.

FATAL: NE2000 NIC was not found at specified hardware settings.

Source:  NE2000.COM

Explanation:  The NE2000.COM driver found a board at the

specified hardware settings, but the board is not an NE2000

board.

Action:  Check the installed hardware.

FATAL: NE2-32 NIC command port failed to respond.

Source:  NE2-32.COM

Explanation:  The installed NE2-32 board has malfunctioned.

Action:  Replace the board.

FATAL: NE2-32 NIC RAM failure.

Source:  NE2-32.COM

Explanation:  The installed NE2-32 board's on-board memory has

malfunctioned.

Action:  Replace the board.

FATAL: No more room in the LSL for another protocol stack.

Source:  IPXODI

Explanation:  The maximum number of protocol stacks have been

registered with the Link Support Layer.  The DOS ODI LSL can

support up to eight protocol stacks.

Action:  Remove an existing protocol stack.

FATAL: RXNet command port failed to respond.

Source:  TRXNET.COM

Explanation:  The TRXNET.COM driver could not find an RX-

Net board in the workstation.

Action:  Make sure that an RX-Net board is installed in the

workstation and that it is firmly seated.  Make sure that the

values for the hardware settings in the driver (the default or the

values set in the NET.CFG file) match the values set on the board.

FATAL: RXNet RAM failure.

Source:  TRXNET.COM

Explanation:  The RX-Net board's RAM has failed the RAM test.

Action:  Replace the board.

FATAL: Specified PS/2 slot contains an NE2 but it is not enabled.

Source:  NE2.COM

Explanation:  The located NE2 board was not enabled.  The NE2

board has probably malfunctioned.

Action:  Replace the board.

FATAL: Specified PS/2 slot contains an NE2-32 but it is not enabled.

Source:  NE2-32.COM

Explanation:  The located NE2-32 board was not enabled.  The

NE2-32 board has probably malfunctioned.

Action:  Replace the board.

FATAL: Specified PS/2 slot does not contain an enabled 3C523 adapter.

Source:  3C523.COM

Explanation:  A �PS/2 SLOT number� option was specified in the

NET.CFG file.  The specified slot does not contain a 3C523 board.

Note that slot numbers are 1 based and correspond to the slot

numbers on the back of the computer.

Action:  Modify the NET.CFG so that the specified slot number

matches the slot that the board is installed in.

FATAL: Specified PS/2 slot does not contain an NE2 adapter.

Source:  NE2.COM

Explanation:  A �PS/2 SLOT number� option was specified in the

NET.CFG file.  The specified slot does not contain an NE2 board.

Note that slot numbers are 1 based and correspond to the slot

numbers on the back of the computer.

Action:  Modify the NET.CFG file to specify the correct slot

number.

FATAL: Specified PS/2 slot does not contain an NE2-32 adapter.

Source:  NE2-32.COM

Explanation:  A �PS/2 SLOT number� option was specified in the

NET.CFG file, but the specified slot does not contain an NE2-

32 board.  Slot numbers are 1 based and correspond to the slot

numbers on the back of the computer.

Action:  Modify the NET.CFG file to specify the correct slot

number.

FATAL: The LSL is not loaded.

Source:  IPXODI

Explanation:  IPX requires that the LSL be loaded first.

Action:  Load LSL.COM; then load IPXODI.COM.

or

Source:  MLID

Explanation:  LSL must be loaded before an MLID can be loaded.

Action:  Load LSL.COM; then load the MLID.

FATAL: There is a TSR above the loaded IPX.

Source:  IPXODI

Explanation:  You tried to unload IPX from memory, but IPX

detected another Terminate-and-Stay-Resident program loaded

above IPX.  For IPX to unload safely, TSRs that were loaded

after it must be unloaded first.

Action:  Either load the TSR before loading IPX or unload the

TSR before trying this operation.

FATAL: There is a TSR above the loaded LSL.

Source:  LSL

Explanation:  You tried to unload the LSL from memory, but the

LSL detected another Terminate-and-Stay-Resident program

loaded above the LSL.  For the LSL to unload safely, TSRs that

have been loaded after it must be unloaded first.

Action:  Either load the other TSR before loading the LSL or

unload the TSR before trying this operation.

FATAL: There is a TSR above the loaded <name> MLID.

Source:  MLID

Explanation:  You tried to unload the MLID from memory, but

the MLID detected another Terminate-and-Stay-Resident program

loaded above the MLID.  For the MLID to safely unload, TSRs

that were loaded after it must be unloaded first.

Action:  Either load the other TSR before loading the MLID or

unload the TSR before trying this operation.

FATAL: This old LSL is not supported.

Source:  IPXODI

Explanation:  IPXODI cannot run correctly using this version of

the LSL.

Action:  Update your LSL.COM to a newer version.

or

Source:  MLID

Explanation:  The MLID cannot run correctly using this version

of the LSL.

Action:  Update your LSL.COM to a newer version.

FATAL: Work Area Exceeded, reduce number of SAPs and/or Link

Stations.

Source:  LANSUP.COM

Explanation:  The number of SAPs or Link Stations specified in

the NET.CFG file exceeded the maximum number of SAPs or

Link Stations that the network board can handle.

Action:  Modify the number of SAPs or Link Stations specified

in the NET.CFG file.

IPX protocol bound to <name> MLID Board # <number>.

Source:  IPXODI

Explanation:  This message is displayed after IPX has successfully

loaded.  It is not a error message but simply tells you which

logical MLID IPX is using.

<name>      The MLID's short name (NE1000, for example).

<number>    The logical board number of the MLID.  This

number is 1 based (for example, the first MLID is

assigned board #1).

Action:  None.

IPX protocol successfully removed.

Source:  IPXODI

Explanation:  A request was made to unload IPXODI, and

IPXODI was removed (unloaded) from memory.

Action:  None.

LSL successfully removed.

Source:  LSL

Explanation:  A request was made to unload the LSL, and the LSL

was removed from memory.

Action:  None.

Number of buffers <number1>, Buffer size <size> bytes, Memory pool

<number2> bytes

Source:  LSL

Explanation:  This information appears when the LSL has read

configuration information from the NET.CFG file.  (LSL does not

display default values when it loads.)

<number1>         The number of communication buffers

allocated by the LSL.  Normally, the LSL

does not need buffers; therefore, none

should be allocated unless directed by a

protocol's documentation.  This number may

be less than requested due to memory limits

within the LSL.

<size>            The size of the LSL's communications

buffers (106 bytes of this value are reserved

for protocol and media headers).  This value

cannot be smaller than 618 bytes.

<number2>         The amount of memory allocated to the

LSL's free memory pool.  This free memory

pool is used by protocols.  This value may be

smaller than requested due to memory limits

within the LSL.  The LSL first allocates the

requested number of communications

buffers and then allocates the free memory

pool from the remaining memory.  Normally,

a free memory pool is not needed and should

not be allocated, unless directed by a

protocol's documentation.

Action:  None.

<name> MLID successfully removed.

Source:  MLID

Explanation:  A request was made to unload an MLID, and the

MLID was removed from memory.

Action:  None.

WARNING: Byte value greater than 255 was truncated

Source:  IPXODI

Explanation:  An IPX or SPX parameter specified in the

NET.CFG or SHELL.CFG file was set to a value greater than 255.

The value used will be 255 (the maximum configurable value).

Action:  Change the NET.CFG file so that the IPX or SPX

parameter has a valid value.

WARNING: Error registering Protocol=<name>, PID=<number>,

Frame=<type>

Source:  MLID

Explanation:  The MLID could not register the specified Protocol

ID.

<name>      The name of the protocol.

<number>    The value of the Protocol ID used to register the

Protocol ID.

<type>      The frame type for which the Protocol ID was

registered.

Action:  Verify the protocol information in the NET.CFG file.

WARNING: Invalid LOOK AHEAD SIZE value, will be set to maximum

(128 bytes).

Source:  MLID

Explanation:  The �LOOK AHEAD SIZE� option was specified

in the NET.CFG for an MLID.  The value specified was greater

than 128 bytes.  The MLID will use 128 bytes for its look ahead

size.

Action:  Change the �LOOK AHEAD SIZE� option in the

NET.CFG file to a valid value.

WARNING: MLID does not support Frame <type> - Protocol keyword

ignored.

Source:  MLID

Explanation:  The �PROTOCOL� option was specified in the

NET.CFG for an MLID.  The specified frame type is not

supported by the MLID.

Action:  Check the �PROTOCOL� line in the NET.CFG file for

possible omissions of required dashes and underscores or any

misspellings.  Check the MLID documentation for supported

frame types.

WARNING: NET.CFG ignored - file length must be less than 4097 bytes.

Source:  IPXODI

Explanation:  The NET.CFG file is larger than 4,096 bytes.  IPX

still loads, but ignores the information in the NET.CFG file.

Action:  Reduce the size of the NET.CFG file.

or

Source:  LSL

Explanation:  The NET.CFG file is larger than 4,096 bytes.  The

LSL still loads, but ignores the information in the NET.CFG file.

Action:  Reduce the size of the NET.CFG file.

or

Source:  MLID

Explanation:  The NET.CFG file is longer than 4,096 bytes.  The

MLID still loads, but ignores the information in the NET.CFG

file.

Action:  Reduce the size of the NET.CFG file.

WARNING: NET.CFG ignored - MLID name cannot be more than 8

characters long.

Source:  IPXODI

Explanation:  The �Bind MLID� option in the NET.CFG file

specified an MLID with a name longer than 8 characters.

Action:  Refer to the MLID's documentation for more information

on the name.

WARNING: Node Address override not supported. Specify override when

loading IBM LAN Support Software.

Source:  LANSUP.COM

Explanation:  A node address was specified in the NET.CFG file

for the LANSUP.COM driver.  The node override must be

specified when loading the IBM LAN Support software.

Action:  Refer to the IBM LAN Support documentation for

instructions.

WARNING: No room in the LSL for another board. Board <number> will

not be activated.

Source:  MLID

Explanation:  The maximum number of boards has been

registered with the Link Support Layer.  The DOS ODI LSL can

support up to eight boards.

Action:  Reduce the number of active boards in the system by

unloading a board.

WARNING: Protocol keyword must have a frame type - entry ignored.

Source:  MLID

Explanation:  The �PROTOCOL� option was specified in the

NET.CFG for an MLID.  The entry failed to specify the

associated frame type for the protocol ID addition.  An entry in

the NET.CFG file for the �PROTOCOL� option would look

similar to the following:

LINK DRIVER NE1000

PROTOCOL IPX 8137 ETHERNET_II

Action:  Specify a frame type with the �PROTOCOL� option.

WARNING: Specified MAX PACKET SIZE too big for this adapter, max

size used.

Source:  LANSUP.COM

Explanation:  The specified maximum packet size in the

NET.CFG file was too large for the network board installed in

the workstation.  The maximum packet size for the network board

was used.

Action:  Modify the maximum packet size in the NET.CFG file.

Notes

Trademarks

Novell, Inc., has made every effort to supply trademark information

about company names, products, and services mentioned in this book.

Trademarks indicated below were derived from various sources.

3Com, 3Com EtherLink, and EtherLink Plus are trademarks of 3Com

Corporation.

AppleTalk is a registered trademark of Apple Computer, Inc.

IBM is a registered trademark of International Business MachinesCorporation.

IBM PC AT/XT are trademarks of International Business MachinesCorporation.

IBM Personal System/2 is a registered trademark of InternationalBusiness Machines Corporation.

Internetwork Packet Exchange (IPX) is a trademark of Novell, Inc.

Micro Channel is a trademark of International Business Machines

Corporation.

MS-DOS is a trademark of Microsoft Corporation.

NetWare and Novell are registered trademarks of Novell, Inc.

NetWire is a trademark of Novell, Inc.

Novell RX-Net is a trademark of Novell, Inc.

OS/2 is a registered trademark of International Business Machines

Corporation.

PC-DOS is a trademark of International Business Machines

Corporation.

Personal Computer AT and Personal Computer XT are trademarksof International Business Machines Corporation.

Personal System/2 is a trademark of International BusinessMachines Corporation.

Token-Ring is a trademark of International Business Machines

Corporation.

Notes

Index

A

AUTOEXEC.BAT

copying to workstation boot   diskettes  11

creating for master workstationdiskette  9

B

BIND (NET.CFG)  21

Boot disk, workstation

booting with  10, 12

creating master  7

creating users' diskettes  11

BUFFERS (NET.CFG)  20

Communication buffers  20

C

Computers, workstation        requirements  4

CONFIG.SYS  10

D

DEFAULT (NET.CFG)  22

DMA (NET.CFG)  23

Drivers. See also LAN drivers

accessing information about  13

unloading ODI  14

E

EMSNETx.EXE, expanded memory              shell file  8

Expanded memory shell  8

Extended memory shell  8

F

File server, logging in to  12

FRAME (NET.CFG)  28

H

Hardware, workstations supported  4

I

IBM Token-Ring Source Routing Driver  31

using with DOS ODI  33

INT (NET.CFG)  24

INT2F.COM file on master workstation diskette  9

IPXODI

copying to master workstation diskette  9

explained  2

removing parts of  13

L

LAN drivers

accessing information about  13

supporting multiple protocols  1

unloading workstation ODI  14

LANSUP.COM driver  9, 30

Link driver LANSUP options  30

Link driver options in NET.CFG  23

LINK STATIONS (NET.CFG)  30

Link support options (NET.CFG)  20

LOOK AHEAD SIZE (NET.CFG)  28

LSL.COM file on master workstation diskette  8

M

MEM (NET.CFG)  25

Memory requirements

workstation  4

Memory settings for NET.CFG  25

MEMPOOL (NET.CFG)  21

N

NET.CFG

conventions  18

creating  10, 17

example  18

options  19

using with SHELL.CFG  17

NETBIOS.EXE

adding to master workstation diskette  9

Network boards

installing in workstations  5

interrupt settings in NET.CFG  24

NETx.COM adding to master workstation diskette  8

NODE ADDRESS (NET.CFG)  26

P

PORT (NET.CFG)  26

PRESCAN (NET.CFG)  22

Protocol

options in NET.CFG  29

using multiple on a workstation  1

PS/2 SLOT (NET.CFG)  27

PS/2 SLOT ? (NET.CFG)  27

R

RAM, workstation memory requirements  4

ROUTE.COM  9

setting parameters for  34

Routing, source. See IBM Token-Ring Source Routing Driver

S

SAPS (NET.CFG)  30

SEND RETRIES (NET.CFG)  29

SESSIONS (NET.CFG)  22

SHELL.CFG, using with NET.CFG  17

Source routing. See IBM Token-Ring Source Routing Driver

T

Troubleshooting, workstation installation  13

W

Workstation

booting  12

hardware requirements  4

installing (DOS ODI)  3

memory requirements  4

troubleshooting  13

X

XMSNETx.EXE, extended memory shell file  8