Date: 7-7-89  1:52pm

From: Chris Love:SC:3COM

To: wtraining

Subj: *** TCOPY.EXE ***

Attach: e:\inbox\451A.msg, e:\inbox\451B.msg

Forwarded: Message from Paul Williams:SPD:3Com of 7-7-89

------------------------------------------------------------------

This utility copies a tape!  I'm in the process of making more 3s400

master tapes, but if you need it, you can now make your own.

--------------------- Forwarded Message Body ---------------------

Date: 7-7-89  9:48am

From: Paul Williams:SPD:3Com

To: Chris Love:SC:3com

Subj: *** TCOPY.EXE ***

Attach: c:\tape\tcopy.exe, c:\tape\tape.sys

------------------------------------------------------------------

Chris, attached are the TCopy.exe and tape.sys files you'll need to

duplicate tapes.

If the tape.sys you have on your 3s/400 is dated sometime in October

of last year, you need not replace it; however, if it's earlier, you'll

need the version I've supplied.  Simply rename tape.sys on your 3s/400

to something like tape.sav, and copy the attached version into the

root on C: of the 3s/400.  After making the copies, you should rename

the new tape.sys to something like tape.new, and rename tape.sav to

tape.sys.  That way, you'll be back to a standard configuration.

Note that this software runs under 3+ ONLY; 3+ Open is not yet

supported.  It will be when they finish the software for the new

Exabyte tape drives.

The steps you must follow (to reiterate our phone conversation),

assuming you've got the correct tape.sys on the 3s/400 are as follows:

1) Unhook the 3s/400 from the network.

2) Use a single, terminated network cable to attach a PC to the 3s/400.

Note that the more horsepower the PC has, the faster things will go.

An AT is abysmally slow in talking to the 3s/400, but a'386 is very

fast.

3) Put the 3s/400 in maintenance mode (position 1 on the thumbwheel),

and boot it.

4) Use the 3+ 286 workstation diskette to boot your PC.  This diskette

will work quite sufficiently for a '386 as well.

5) Remove the boot diskette from drive A, insert the 3+ 3s/400 System

Software diskette in drive A and issue a 3c start command.

6) Once the 3s/400 is up and running, remove the 3s/400 System Software

diskette, and insert the diskette containing TCopy.exe (Yep, you'll

have to put it on diskette before starting these steps.  I just

realized that.)  Copy TCopy.exe to your favorite location on the

3s/400.  I suggest its own directory on the C: drive.

7) Change directories to the partition to contain the image of the

master tape, and issue the TCopy command.  TCopy is most efficient

if the partition which will contain the tape image is large enough

to contain the entire image.  If it isn't, TCopy will attempt to

path to the next partition to continue writing the data.  This

will complicate your interaction with TCopy, as you might be

prompted for the next partition to be used, or TCopy might fail

if it can't find a next partition.

Assuming that you put TCopy on C:\tcopy, that your file to

contain the image is d:\master.img, and that you want five copies

of the tape, the command would look something like

>>c:\tcopy\tcopy -iD:\MASTER.IMG -c5

8) You will be prompted by the software to be sure that the master

tape is in the tape drive. Once this is done, you must press

Carriage Return (or Enter) to start the duplication.

THIS IS IMPORTANT: Once you insert the tape, you must let the tape

driver seek to the beginning of tape before you press Carriage

Return (or Enter)!!!!!!!  This is true for inserting ANY tape,

master, or otherwise.  If you don't wait until the tape settles

down, the attempt by tcopy to open the tape device will fail, and

TCopy will terminate with an error.  Wait about 15 seconds after

inserting the tape just to be safe.  Once the tape drive's light

has gone off, and stays off for 5 seconds or so, it's safe to

proceed.

9) For each megabyte of data on the tape, you've got around 1 minute

during which the tape image is being created, so if it's a 30 mb

tape, you've got 30 minutes before you need to return to the 3s/

400.  Might as well take a break.

10) Once the master image has been created, you'll be prompted to

remove it and place a blank tape in the drive.  The tape doesn't

have to be blank, as any previous data will be overwritten.

Make SURE THAT THE blank tape is NOT WRITE PROTECTED before

inserting it into the tape drive.  Also note the caveat about

allowing the tape drive to settle down before pressing Carriage

Return or Enter above.

11) Repeat step 10 until all copies have been made.

That's it.  Have fun

Paul W.