TIMESYNC.EXE                  4/13/1989

============

* What is it for?

To solve time inconsistencies between netstation and

3S40x servers.

* How does it work?

This is a terminate-stay-resident program which

synchronizes DOS time by the real time clock setting.  It

chains itself with the system timer interrupt INT 1Ch and

maintains a counter of its own.  For every timer tick, it

decrements the count by one.  When it hits zero, it

updates the DOS time by reading from the real time clock

and then reloads the preset count.  The default setting

for synchronization is 15 minutes.  This default setting

can be modified by using the option switches available.

* Available command switches:

/H=hh   to set sync period in hours, where hh = 1 ... 65535.

/M=mm   to set sync period in minutes, where mm = 1 ... 65535.

/?      to prompt user about program usage.

* How do you install it?

1) Establish a console connection with the 3S40x and copy

TIMESYNC.EXE to the root of the C: drive.

2) Edit your AUTOEXEC.BAT file to include TIMESYNC just

before the call to 3plus_on.bat.

For example, suppose the old AUTOEXEC.BAT looks like

this:

echo 3+ Server Boot Up batch file - version 3.1

echo ~   3DOS V3.21 > LCD:

3switch

if errorlevel 17 goto done

\3plus\3config\3plus_on.bat

:done

command

After the modification, the new AUTOEXEC.BAT will look

like this:

echo 3+ Server Boot Up batch file - version 3.1

echo ~   3DOS V3.21 > LCD:

3switch

if errorlevel 17 goto done

TIMESYNC

\3plus\3config\3plus_on.bat

:done

command

3) Reboot your server in operating mode.