CHIPSET.EXE is a small program that simply checks to see if your PC

has one of the memory controllers required to use The Last Byte

Memory Manager.

To see if your PC has one of the supported memory controllers, simply

enter the following command at the DOS prompt and follow the prompts:

A>CHIPSET

If your PC has one of the supported devices, then The Last Byte

Memory Manager will work on your PC, and you will be able to load

device drivers, TSRs, DOS buffers and more into upper memory above

640k.

If your PC uses another kind of chipset, it may not be supported for

one of the following reasons:

(1) No shadow ram hardware:

---------------------------

Some chipsets do not provide shadow ram support.

Examples: VLSI Technologies 82C100 series

Western Digital chipsets

Texas Instruments TACT82300 series

Early Suntac chipsets (ST62BC001-6)

United Microelectronics Corp. UM82C231

(2) No shadow ram read/write mode:

----------------------------------

Some chipsets have shadow ram, but it can only be put into

either write-only mode (for copying rom contents to ram), or

read-only mode (to prevent accidental writes into what's

supposed to be a copy of rom).

Examples: ACC Microelectronics chipsets

Headland Technologies chipsets

G2 chipsets

(3) DMA (Direct Memory Access) can't get to the shadow ram:

-----------------------------------------------------------

Many device drivers and TSRs use either use DMA directly, or

else they employ disk buffers that DOS accesses via DMA.

The hardware design of some memory controller chips prevent

DMA from accessing the shadow ram, and thus loading this kind

of software into shadow ram memory is pointless.

Note: In some cases, a portion of their shadow ram memory may

be made usable by using the chip's hardware support for

expanded memory which typically supports DMA.  However, this

precludes use of The Last Byte Memory Manager together with

an expanded memory manager specifically designed for these

chips.

Examples: VLSI Technologies VL82C286 & VL82C386 TOPCAT

(4) Controller incompatible with design of The Last Byte Memory Manager:

------------------------------------------------------------------------

The Last Byte Memory Manager is table-driven, making it

relatively easy to adapt to different memory controller

architectures, but there are limits!

Examples: Zymos chipsets

United Microelectronics Corp. (UM82C384,'388,'389)

Forex Computer Corp. FRX36C100

Silicon Integrated System Corp. (SIS) chipsets

(5) Shadow ram control can be made inaccessible once configured:

----------------------------------------------------------------

Some controllers are designed so that they can be "locked"

at the end of software initialization, requiring a hardware

reset to unlock, and thus preventing The Last Byte Memory

Manager from accessing the controller.

Example: Intel 82335, 82335SX

Some of these chips, however, have hardware support for LIM 4.0 EMS,

and thus many of their motherboards come with a EMS device driver

designed specifically to take advantage of this capability.  So in

many cases, you can install this driver first, and then The Last Byte

Memory Manager can use it to manage upper memory.

A free demo version of The Last Byte Memory Manager may be downloaded

from our BBS at the number below Monday through Friday between 5pm

and 8am PST, or 24hrs a day on weekends.  The filename is

TLB-Vnnn.ZIP, where the "nnn" is replaced by the latest version

number.  The demo version is fully functional except that it limits

you to 32k of memory and no more than two device drivers and/or TSRs

loaded into upper memory.

Key Software Products

440 Ninth Avenue

Menlo Park, CA 94025

(415) 364-9847