*********************

Copyright has been applied for

BLDPAS

Registration Fee: $20.00

Source Code:      $15.00

Registration of program gets SOURCE

ZOOMCHAR or LOTTERY program for $5.00 off.

DBA TEKSERV

%William A. Phillips

RT 1 BOX 397 Piney Grove

Danbury, Nc 27016     USA

COMPUSERV 75250,1012

**********************

*N*

Build Program is a simple but flexible way to build a screen with

associated cursor contorls and database without programming. Although you

can program changes since a program is generated.

REQUIREMENTS: PC AT 286 or compatible, TURBO Pascal 4.0 or higher compiler.

NOTE: Rapsvars.Pas source code contains the global variables.

The PASCALDB tpu unit sent is a TURBO PASCAL 7.0 Compiled unit. If

you have trouble with an older compile the PASCALDB.T40 is a 4.0

compile. You can rename the PASCALDB.TPU unit to something and

rename the PASCALDB.T40 to PASCALDB.TPU.

*N*

PROGRAM BUILD FOR PASCAL

Example Screen:

XXXXXXXX                          INVOICE                            XXXXXXXX

INVOICE:  XXXXXXXXX

CUSTOMER: XXXXXXXXX

NAME:     XXXXXXXXXXXXXXXXXXXX XXXXXXXXXXXXXXX X

ADDRESS:  XXXXXXXXXXXXXXXXXXXX XXXXXXXXXXXXXXX XX XXXXXXXXX

STOCK   DESCRIPTION                      VENDOR   EACH       TOTAL

_____   _______________________________  ______   _________  _________

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

Sub Total  XXXXXXXXX

Tax        XXXXXXXXX

TOTAL      XXXXXXXXX

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

*N*

USER GUIDE

To begin load the BLDPAS disk onto a directory of your choice.

If you are going to use the "TURBO" Pascal compiler from Borland load the

BLDPAS programs onto the "\PASCAL" directory.

Next load the Database disk onto the "\BLDPAS" directory.

You are then ready to begin building a program by entering "BLDPAS" and

depressing enter. You should see a screen like the following.

PROGRAM BUILD FOR PASCAL

Program Name: ________

Text Color:   B  (R - K)

Back Color:   K  (R - K)

Box  Color:   R  (R - K)

R-Red Y-Yellow G-Green B-Blue C-Cyan W-WHite K-Black M-Magenta

Enter a program name up to 8 characters. The text color is the default

color of the screen text. You can enter a number from "0" thru "9". "1" is the

default for this screen which is blue. The colors are:

K - Black

B - Blue

G - Green

R - Red

M - Magenta

C - Cyan

Y - Yellow

W - White

These text colors can be changed later in the execution for the modifiable

fields.

Enter the Back color or the color of the backround of the screen.

The default is "0" which is Black.

The Box color if present will put a box around the outside of the screen

with defult color Red. If you do not want a box Blank this field out.

Press "ENTER" when done. You will then see an empty screen on which you can

begin entering characters which will comprise you screen.

How this screen WORKS:

The Tab,Cursor, Home, End, Enter, And ESC keys are function keys.

Tab will tab to the end of the line and back. Cursor will move the cursor.

The Home and End key will place the cursor at the top left or bottom left

of the screen respectively.

The 24th line is reserved for action literals for your program and the

25th line is reserved for messages.

To enter a screen you just type the characters just as you would like to

see the screen to appear with one exception. Wherever you plan to enter data

or display it Type an "X" where the field is to start for the length of the

field. THERE MUST BE A BLANK BEFORE THE START OF A DATA FIELD.

For example the screen on the cover is an example of a screen entered to

be built. Notice the "X" characters. These represent fields into which

you will enter or display data.

XXXXXXXX                          INVOICE                            XXXXXXXX

INVOICE:  XXXXXXXXX

CUSTOMER: XXXXXXXXX

NAME:     XXXXXXXXXXXXXXXXXXXX XXXXXXXXXXXXXXX X

ADDRESS:  XXXXXXXXXXXXXXXXXXXX XXXXXXXXXXXXXXX XX XXXXXXXXX

STOCK   DESCRIPTION                      VENDOR   EACH       TOTAL

_____   _______________________________  ______   _________  _________

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

Sub Total  XXXXXXXXX

Tax        XXXXXXXXX

TOTAL      XXXXXXXXX

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

USE THE "ESC" KEY WHEN DONE WITH THIS SCREEN

The two data fields on the top line are for the date and time.

When you have completed your screen depress the "ESC" key. You will then

be ready to enter the Type, Attribute, Color, and Key designations.

At this point you can depress the ENTER key four times without entering

any other data to get the generate screen. By depressing ENTER at the generate

screen you will genereate the program. The data fields will be defaulted to

character fields. The Attributes will be modifiable. The Color will be Blue.

And the first modifiable field will have a an index.

Otherwise you will be prompted to enter the Type of field. The types of

fields are either character or numeric. If the field is a character field

simply leave an "X" in the first byte of the field. If the field is numeric

enter a "N" in the first byte of the field. If you want the field to be an

amount field enter "$" in the first byte of the field.

XXXXXXXX                          INVOICE                            XXXXXXXX

INVOICE:  NXXXXXXXX

CUSTOMER: NXXXXXXXX

NAME:     XXXXXXXXXXXXXXXXXXXX XXXXXXXXXXXXXXX X

ADDRESS:  XXXXXXXXXXXXXXXXXXXX XXXXXXXXXXXXXXX XX XXXXXXXXX

STOCK   DESCRIPTION                      VENDOR   EACH       TOTAL

_____   _______________________________  ______   _________  _________

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

Sub Total  XXXXXXXXX

Tax        XXXXXXXXX

TOTAL      XXXXXXXXX

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

As you can see the INVOICE and Customer fields are numbers and the "EACH"

and "TOTAL" fields are amounts.

Depress "ENTER" when done.

You will then be prompted for the "ATTR" of the field. The Attribute

designates whether the field is to be modifiable by the keyboard or protected

(Display only). Place an "X" in the first byte of the field if the field is to

be modifiable. Place an "P" in the first byte of the field if the field is to

be protected. Since the first two fields are the date and time you can enter a

"P" in these fields. Enter "N" if the field is to be Non Display. There is no

special editing for numeric or amount fields yet. You can inquire MAPTYP to

see what type of field it is. Editing is planned in a later release.

PXXXXXXX                          INVOICE                            PXXXXXXX

INVOICE:  NXXXXXXXX

CUSTOMER: NXXXXXXXX

NAME:     XXXXXXXXXXXXXXXXXXXX XXXXXXXXXXXXXXX X

ADDRESS:  XXXXXXXXXXXXXXXXXXXX XXXXXXXXXXXXXXX XX XXXXXXXXX

STOCK   DESCRIPTION                      VENDOR   EACH       TOTAL

_____   _______________________________  ______   _________  _________

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

Sub Total  XXXXXXXXX

Tax        XXXXXXXXX

TOTAL      XXXXXXXXX

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

Depress ENTER when done.

You will be prompted to enter the color for each data field. The colors

for the fields are:

B - Blue

G - Green

R - Red

C - Cyan

Y - Yellow

W - White

Enter the color letter of your choice in the first byte of each field. You

can leave an "X" in the field and the field color will default to the default

text color you entered on the main screen. The following entries would make the

date and time entries White, the Invoice and Customer numbers Yellow, and the

other fields Green.

WXXXXXXX                          INVOICE                            WXXXXXXX

INVOICE:  YXXXXXXXX

CUSTOMER: YXXXXXXXX

NAME:     GXXXXXXXXXXXXXXXXXXX GXXXXXXXXXXXXXX G

ADDRESS:  GXXXXXXXXXXXXXXXXXXX GXXXXXXXXXXXXXX GX GXXXXXXXX

STOCK   DESCRIPTION                      VENDOR   EACH       TOTAL

_____   _______________________________  ______   _________  _________

GXXXX   GXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  GXXXXX   GXXXXXXXX  GXXXXXXXX

GXXXX   GXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  GXXXXX   GXXXXXXXX  GXXXXXXXX

GXXXX   GXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  GXXXXX   GXXXXXXXX  GXXXXXXXX

GXXXX   GXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  GXXXXX   GXXXXXXXX  GXXXXXXXX

GXXXX   GXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  GXXXXX   GXXXXXXXX  GXXXXXXXX

GXXXX   GXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  GXXXXX   GXXXXXXXX  GXXXXXXXX

GXXXX   GXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  GXXXXX   GXXXXXXXX  GXXXXXXXX

GXXXX   GXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  GXXXXX   GXXXXXXXX  GXXXXXXXX

GXXXX   GXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  GXXXXX   GXXXXXXXX  GXXXXXXXX

GXXXX   GXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  GXXXXX   GXXXXXXXX  GXXXXXXXX

Sub Total  GXXXXXXXX

Tax        GXXXXXXXX

TOTAL      GXXXXXXXX

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

Depress ENTER when done.

You will then be prompted to enter the data fields for which you want

indexes on (you can search on these as keys). Enter the appropriate letter "U"

for a unique index and "K" for an index allowing duplicate entries in the first

byte of any data field you wish to have as a key. By selecting appropriate keys

you can have a very useful and flexible application. The following example

illustrates an application that can have duplicate keys on the Customer, First

Name, Address, State, or Zip code. Also it allows only a Unique key on the

INVOICE number. So that each invoice is unique.

XXXXXXXX                          INVOICE                            XXXXXXXX

INVOICE:  UXXXXXXXX

CUSTOMER: KXXXXXXXX

NAME:     KXXXXXXXXXXXXXXXXXXX KXXXXXXXXXXXXXX K

ADDRESS:  KXXXXXXXXXXXXXXXXXXX KXXXXXXXXXXXXXX KX KXXXXXXXX

STOCK   DESCRIPTION                      VENDOR   EACH       TOTAL

_____   _______________________________  ______   _________  _________

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

Sub Total  XXXXXXXXX

Tax        XXXXXXXXX

TOTAL      XXXXXXXXX

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

Depress the ENTER key when done.

This concludes the buld screen process. You should see the generate screen

which if you now depress ENTER your program will be generated. If you wish to

terminate the process depress the "ESC" key. If you depress the "ESC"

everything will be lost. You will have to start form the beginning.

By depressing the ENTER key you will have a program generated with the

name program.PAS where program is the name you entered on the main screen.

A new directory of the name "program" will be made and your program compiled to

this directory.

You can then change to this directory CD \program and enter the program

name to execute the program. This program was called invoice so you could enter

"CD \INVOICE", depress enter and enter "INVOICE" to execute the program.

XXXXXXXX                          INVOICE                            XXXXXXXX

INVOICE:  XXXXXXXXX

CUSTOMER: XXXXXXXXX

NAME:     XXXXXXXXXXXXXXXXXXXX XXXXXXXXXXXXXXX X

ADDRESS:  XXXXXXXXXXXXXXXXXXXX XXXXXXXXXXXXXXX XX XXXXXXXXX

STOCK   DESCRIPTION                      VENDOR   EACH       TOTAL

_____   _______________________________  ______   _________  _________

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

XXXXX   XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX  XXXXXX   XXXXXXXXX  XXXXXXXXX

Sub Total  XXXXXXXXX

Tax        XXXXXXXXX

TOTAL      XXXXXXXXX

Ent-Get F2-Get Next F3-Get Prev F4-Find F5-List F6-Add F7-Updt F8-Del F10-Util

DEPRESS ENTER TO GENERATE THE PROGRAM

If at any time you wish to rebuild the program you can do so. If

you build the screen with the same length fields and in the same order

you can just re-create the application. If however you change the length of any

fields you will have to delete the data and index files before you start the

building process.

You can enter:

CD \INVOICE

DEL INVOICE.DAT

DEL INVOICE.I*

to delete the data and index files.

THIS WILL DELETE ALL DATA ON THE DATABASE. YOU CAN USE THE UTILTIY OPTION TO

DUMP THE DATA FIRST AND THEN RELOAD.

Note: Since the program is generated as a text file you can use any editor to

edit the program including the "TURBO" editor.

The education part of this process begins now since you will have a Pascal

program that has been generated. Near the end of the program you will see in

comments {USER CODE}. This comment is placed at an appropriate place where the

screen data has been moved to the database record area and you are ready to

change or manipulate the data as desired after which the appropriate action will

automatically take place (retrieve, add, update, delete...). To reference the

database fields you would find the data field sequence from the top to bottom,

lefte to right. For instance the Last Name field would be field sequence 5 since

the datae and time fields are considered data fields. If for instance you wanted

to set the last name to another name you might enter in the "USER CODE" section

Invoice.Datafld5 := 'DAVID JONES".

Or you might want to add to an amount field by entering:

TChr := Invoice.Datafld16;

Val(TempCHr, TempNum, CondCode);

TNum := TNum + 5;

Str(TNum, TChr);

INVOICE.Datafld16 := TChr;

Note: There are 10 temp character and number fields TChr thru

TChr9 and TNum thru TNum9.

*N*

If for instance you wanted to add the amounts in the totals column to

the total you could Enter:

Tchr  := Invoice.Datafld16; Val(Tempchr, Tempnum,  CondCode);

If CondCode <> 0 Then Error := 16;

Tchr1 := Invoice.Datafld17; Val(Tchr1, Tnum1,CondCode);

If CondCode <> 0 Then Error := 17;

Tchr2 := Invoice.Datafld18; Val(Tchr2, Tnum2,CondCode);

If CondCode <> 0 Then Error := 18;

Tchr3 := Invoice.Datafld19; Val(Tchr3, Tnum3,CondCode);

If CondCode <> 0 Then Error := 19;

Tchr4 := Invoice.Datafld20; Val(Tchr4, Tnum4,CondCode);

If CondCode <> 0 Then Error := 20;

Tchr5 := Invoice.Datafld21; Val(Tchr5, Tnum5,CondCode);

If CondCode <> 0 Then Error := 21;

Tchr6 := Invoice.Datafld22; Val(Tchr6, Tnum6,CondCode);

If CondCode <> 0 Then Error := 22;

Tchr7 := Invoice.Datafld23; Val(Tchr7, Tnum7,CondCode);

If CondCode <> 0 Then Error := 23;

Tchr8 := Invoice.Datafld24; Val(Tchr8, Tnum8,CondCode);

If CondCode <> 0 Then Error := 24;

Tchr9 := Invoice.Datafld25; Val(Tchr9, Tnum9,CondCode);

If CondCode <> 0 Then Error := 25;

If Error   <> 0 Then

Begin

InvoiceMsgStr := 'Error on field ';

FldSeq := Error

End;

Tnum := Tnum + Tnum1 + Tnum2 + Tnum3 + Tnum4 + tnum5

+ Tnum6 + Tnum7 + Tnum8 + Tnum9;

Str(Tnum:2, Tchr);

*where "2" is the number of bytes*

Invoice.Datafld65 := Tchr;

The resultant changes would then be available to the Database buffer for

adds or updates.

Every program will have a programMsgStr variable that is written after

processing and before display of screen. You can move messages to it.

The function is held in a variable called "Function_Id".

The function is held in the "Function_Id" as

"S" - Select          (Enter key)

"N" - Select Next     (F2)

"P" - Select Previous (F3)

"F" - Find            (F4)

"L" - List            (F5)

"I" - Insert          (F6)

"U" - Update          (F7)

"D" - Delete          (F8)

The function is held in a variable called "Function_Id".

E.G. If Function_Id = "I"   {If Add (Insert) Function}

Or   Case Function_Id

"I" : <action>;

"U" : <action>;

End;

To Change the field attributes they are kept in MAPLN (line), MAPPOS

(position), MAPLEN (length), MAPATTR (Attribute), MAPCOLR (color) arrays.

To change the color of the 5 field top to bottom left to right you could

MAPCOLR[5] := 'B';

The MAP** arrays are initialized in program_INIT once. After that you can

change them.

If you want to make a permenant change to the field change the constants

at the beginning of the program. That's where INIT copies the data to the

MAP** arrays.

There are two main screen/ database record format procedures you can use

anytime. programFMT_SCRNBUF and programFMT_RECBUF.

You can add database files and indexes following the open/create and access

routines in the program.

SYSTEM FIELDS: The Date And Time are held in the ScreenDate And ScreenTime

Fields. To Refresh The date or time there is a Procedure GetDateTime you can

Execute. The ScreenDate field is 10 bytes and the ScreenTime Field Is 11

Bytes.

The F10 key will change the prompts at the bottom of the screen. You can

re-build an index by placing the cursor on an index field and depressing

F1. You can dump the database with F2 and after deleting the records of

the .dat and i* files re-load with F3 and rebuild the indexes.

This software can be used to build a simple but efficient program

for viewing as a model or for use.

Another thing that can be done is to combine programs by making

procedures out of them and executing them from a mainline program.

ZOOMCHAR and FEDERAL BALANCE THE BUDGET LOTTERY were written using

BLDPAS software.

*N*

OTHER SOFTWARE

ZOOMCHAR/BIGCHAR is a set of two programs. BIGCHAR is for displaying/ Creating

big letters/figures and scrolling or displaying. ZOOMCHAR will scroll these

3" letters across your screen with phrases you enter. Both contain their own

database.

FEDERAL BALANCE THE BUDGET LOTTERY is a lottery GAME that allows lottery

entries. You can draw random selections or use a DRAW TO WIN feature that will

Generate random numbers up to a Number of draws entry. If there is a completed

match with an entry it will stop and let you know. If not it will stop at the

pre set Number of draws with the entry greater than or equal to the set drawn.

You can use 1 to 8 numbers for a range of numbers from 1 to 99.

*N*

ADDITIONAL COPIES

__ 3.5" Disk    __ 5.25"

#

__ OF BLDPAS   x $20.00 = _____   __ SOURCE  x $15.00 = ______

__ OF LOTTERY  x $15.00 = _____   __ SOURCE  x $10.00 = ______

__ OF ZOOMCHAR x $10.00 = _____   __ SOURCE  x $7.00  = ______

Registration or purchase of one gets $5.00 off any other.

(You must have sent questionaire in or SSN so it is on Database for sponsor)

Name:    __________________________

Address: ___________________________

City:    _______________  ST: _____  ZIP: ___________    COUNTRY: _________

Sponsor Number: _______________ Name: ____________________________

(Enter SSN and pass around. Fill out questionaire. Every order gets you $1.00)

DBA TEKSERV, %William A. Phillips, RT1 BOX 397 Piney Grove Rd., Danbury, Nc

27016

*N*

COPYRIGHT

A Copyright has been applied for and DBA TEKSERV claims all rights to this

product. However you are encouraged to use this product for any activity

including making money as long as the money made is not for the sale of this

product. This product is not to be copyied for other persons PC's nor to be

given away to others. It is intended for the owner's PC's.  The owner is allowed

to operate the product on multiple PC's that the owner owns in a private sense.

Buisiness, Corporate, and Government agencies are only allowed one copy per

machine. However a discount can be applied for multiple purchases by these

agencies.

*N*

SPONSOR PROGRAM

If you pass the following additional copies request on to a friend put

your SSN in the Sponsor field. Then if you tell DBA TEKSERV how you liked the

product and if there are any problems or IDEAS you have and include your SSN

on this Questionaire you will receive a dollar if the additional copies form

comes in with an order. Make as many copies as you wish with your sponsor

number. For every order with the Sponsor number you will get a dollar.

SSN:  _________________

NAME: _________________________

ADDR: _________________________ ST: ____ ZIP: __________

How well did you like BLDPAS Program?

________________________________________________________________________

Compare it to other software as far as program productivity Good, Bad, or

Ugly.

_________________________________________________________________________

How easy is BLDPAS to use? Any New Ideas for it?

___________________________________________________________________________

Would you be interested in purchasing a 3" LETTER SCROLLING game. It is

a phrase scrolling program that has a database. It can also scroll ASCII

Text files like a billboard.

___ YES  ___ NO

Do you have an interest in a Lottery game which allows you to make entries

and then you can have an auto random draw to win?

___ YES  ___ NO

AGE: ____

SEX: ____

OCCUPATION: ________________________

*N*

DISCLAIMER

This program is intended for program development. DBA TEKSERV is not

responsible for the illegal use or misuse of this program.  DBA TEKSERV is not

responsible for any such accusitory "DEFECTS" in program design or operation.

This product is sold "AS IS".

RETURN POLICY

Any returns of this product will be subject to a 20%

restocking Charge to the customer.

HELP POLICY

DBA TEKSERV encourages correspondence to DBA TEKSERV concerning the

appearance, operation, or problems associated with this product. Although no

guarantee is given for "FIXING" any inherent problems, "IRRITATIONS", etc. All

available effort and time will be given to HELP the customer enjoy this game if

posslible.

HEALTH POLICY

DBA TEKSERV is not responsible for any damage the customer may incur from

the computer or the operation of the game in any financial, or health related

area. The operation of you computer and software is considered by DBA TEKSERV to

be the responsibility of the customer. The color combinations of the game were

chosen to attract the customers attention in an entertaining way. So the

customer is to use this product at their own risk. This policy is stated

due to allegations of damage due to radion of the computer montitor to the

eyes and electomagnetic waves. This game is assumed by DBA TEKSERV to be no

more damaging than any other game presently available.

*N*

_____________________                                              ________

_____________________

_____________________                                               STAMP

________

DBA TEKSERV

RT1 Box 397 Piney Grove

Danbury, NC 27016

*N*

Federal Balance the Budget Lottery Promotional Entry

This is an entry for you to enter 4 numbers from 1 to 36. DBA TEKSERV

is going to hold a PROMOTIONAL lottery by DEC 31, 1993. The winner(s)

will get/split $100.00 in prizes. The lottery will only be held if there

are more than 100 participants. The lottery will be using the Draw to Win for

an exact match. The first entry(s) that match will be the winners. This

entry is an incentive for owners of the FEDERAL BALANCE THE BUDGET LOTTERY

or ZOOMCHAR/BIGCHAR. However all entries are welcome.

NAME    ___________________________ SSN ____________________ Phone: __________

ADDRESS ___________________________ ST ____ ZIP ____________

(4 numbers 1 thru 36 - Do not use leading zeroes)

__ __ __ __

*N*

ZOOMCHAR/BIGCHAR

ZOOMCHAR/BIGCHAR are programs that allow the use/creation of big

letter's and figures. Zoomchar will scroll these letter's/figures across

your screen in color. You can enter phrases on the database and chain them

You can also scroll ASCII text files. Use for Messaging, Education, Screen

saver. Normal registration Fee $10.00.

*N*

ORDER FORM

Orders of 10 or more $15.00 each. Owners of LOTTERY/ZOOMCHAR/BIGCHAR $15.00

__ Copies BLDPAS  At $20.00 Total = ($20.00 times number of copies) = _______

Orders of 20 or more or owners of FEDERAL BALANCE BUDGET LOTTERY $5.00

__ Copies Zoomchar At $7.50 Total = ($7.50 times number of copies)  = _______

Every 10 copies of Lottery,Zoomchar or BLDPAS get 1 free.

__ 3.5" disk  __ 5.25" disk

Send to: name    ________________________________________ SSN: ___________

address __________________________________

city    __________________________________  ST ___ ZIP __________

DBA TEKSERV,RT1 BOX 397 PINEY GROVE RD,DANBURY, NC 27016

Sponsor Number: _______________ Name: ____________________________

(Enter SSN and pass around. Fill out questionaire. Every order gets you $1.00)