ELCUT

Finite Element Analysis System

Version 3.0

User's Guide

Copyright TOR Cooperative Enterprise, 1989-1992

All rights reserved.

TOR Cooperative Enterprise

6 Uglovoj per.,

St.Petersburg 198052,

Russia

Telephones:	(812) 292 1965,

(812) 292 7210

Fax:	        (812) 110 1334

E-mail:	prigogin@tor.spb.su

ELCUT is a trademark of TOR Cooperative Enterprise.

286, 386 and 486 are trademarks of Intel Corporation.

IBM PC/AT and PC-DOS are trademarks of International Business Machines

Corporation.

Microsoft, Microsoft Word, MS-DOS and Windows are registered trademarks

of Microsoft Corporation.

1.	Introduction

Welcome to ELCUT FEA System.

In this chapter, you will find:

*	A description of ELCUT system and its configurations for

various fields of application.

*	ELCUT hardware requirements.

*	An explanation of how to use this manual.

1.1.	What Is ELCUT FEA System?

ELCUT is PC-oriented interactive environment for electromagnetic,

thermal and stress analysis. Standard analysis types include:

*	Electrostatics.

*	Linear and nonlinear magnetostatics.

*	Linear, nonlinear and time-dependent heat transfer and

diffusion.

*	Linear stress analysis.

During a 15-minute session, you can describe the problem (geometry,

material properties, sources and other conditions), obtain solution with

high accuracy and analyze field details looking through full color picture.

With ELCUT, complicated field problems can be solved at your PC

instead of large mainframes or workstations. At usual PC with standard

configuration, ELCUT is able to solve problems of electrostatics,

magnetostatics and heat transfer with up to 100,000 nodes, and for stress

analysis up to 50,000 nodes (100,000 degrees of freedom.)

1.2.	ELCUT Configuration

The ELCUT system includes several modules destined for:

SHELL	-	the main module of the system;

TRIA	-	the geometrical model editor, automatic mesh

generator;

SHOW	-	the postprocessor for problems of

electrostatics, magnetostatics and stationary

heat transfer;

SHOWHD	-	the postprocessor for time-dependent heat

transfer problems;

SHOWS	-	the postprocessor for stress analysis;

SOLVL, SOLVLD -	matrices building and solving utility used with

electrostatics, linear magnetostatics and

stationary heat transfer problems;

SOLVMNL	-	matrices building and solving utility for

nonlinear magnetostatics;

SOLVHNL	-	matrices building and solving utility for

nonlinear heat transfer;

SOLVHD	-	matrices building and solving utility for time-

dependent heat transfer;

SOLVS	-	matrices building and solving utility for stress

analysis problems;

GRAD	-	derivatives calculation utility for electrostatics,

magnetostatics and heat transfer;

GRADS	-	derivatives calculation utility for stress analysis

problems.

ELCUT system implementation for use with particular class of

problems could include some subset of listed modules, e.g., the

configuration for electrostatics, nonlinear magnetostatics and stationary

heat transfer problems includes SHELL, TRIA, SHOW, SOLVL,

SOLVLD and GRAD modules. Being destined for nonlinear

magnetostatics, ELCUT includes, in addition, the SOLVMNL module,

and for nonlinear heat transfer-the SOLVHNL module. ELCUT for

stress analysis problems includes SHELL, TRIA, SHOWS, SOLVS and

GRADS modules.

1.3.	Required Hardware Configuration

Computer:

A 286, 386 or 486 based IBM PC/AT compatible.

Coprocessor:

Intel 80287 or 80387 math coprocessor.

Memory:

640K minimum, 2MB recommended (1MB extended),

4-8MB recommended for very huge problems.

Display:

EGA, VGA color or LCD monochrome.

Mouse:

Microsoft mouse or 100% compatible.

Operating System:

PC-DOS or MS-DOS (Rev. 3.3 or above).

1.4.	Keyboard Formats

In this manual we use SMALL CAPITALS to specify the names of keys on

your keyboard. For example, ENTER, ESC, or ALT. Four arrows on the

keyboard, collectively named the DIRECTION keys, are named for the

direction the key points: UP ARROW, DOWN ARROW, RIGHT ARROW, and

LEFT ARROW.

A plus sign (+) between key names means to hold down the first key

while you press the second key. A comma (,) between key names means

to press the keys one after the other.

1.5.	How to Use this Manual

This manual is divided into seven chapters:

Chapter 1, "Introduction", briefly discusses ELCUT capabilities.

Chapter 2, "Theoretical Description" contains mathematical formulations

for all problem types that can be solved with ELCUT. Read this chapter

to learn if ELCUT can solve your particular problem.

Chapter 3, "Getting Started", describes first steps of using ELCUT. In this

chapter, you will learn how to install and start the package.

Chapter 4, "Basic Skills", tells you about working patterns with ELCUT.

Chapter 5, "Model Geometry Definition", explains how to describe

geometry of the model, build the mesh, and define material properties

and boundary conditions.

Chapter 6, "Problem Parameters Description", introduces non-geometric

data file organization, and the way to attach this file to the model.

Chapter 7, "Obtaining and Analyzing the Results", describes how to start

the solver and introduces ELCUT Postprocessor, its features and

capabilities.

Appendix A, "Example Problem from Start to End", contains full

step-by-step instructions how to solve the real problem from scratch.

2.	Getting Started

2.1.	Installing ELCUT

The BIN directory contains all the necessary to run the package. These

files are to be copied to the special directory on the hard disk, for

example, C:\ELCUT\BIN. For your convenience, we recommend to

add the name of this directory to DOS PATH environment variable.

The EXAMPLES directory contains a number of problems solved with

ELCUT. We suggest you to look through the examples in the area of

your interest, before you start with your own problems.

To operate with large amount of data, ELCUT creates temporary file in

the current working directory. You can specify alternate place for this

file by assigning path of the preferred directory to ELCUTTMP

environment variable, e.g., to store temporary files in the C:\TEMP

directory, you could include SET ELCUTTMP=C:\TEMP line in

your AUTOEXEC.BAT. We highly recommend to have enough free

space for the ELCUT working files. About 1.3 MB is required for every

ten thousand nodes.

ELCUT can use the extended memory of PC. This feature is available

through the XMS driver, e.g., HIMEM.SYS distributed with

MS-DOS 5.0 and Microsoft Windows 3.x.

Note. ELCUT uses part of the extended memory that is

not occupied by RAM disks, disk caching systems or

any other resident program.

The BIN directory of ELCUT distribution kit contains four files with

.CFG extension. These files contain optional color tables:

SCREENC.CFG	-	color table for VGA and EGA color monitors;

SCREENL.CFG	-	color table for LCD monitor of Laptop;

SCREENG.CFG	-	gray scale color table, can be used when

obtaining screen hard copy on a monochrome

printer;

SCREEN.CFG	-	default color table, the same as

SCREENC.CFG.

You can change the default by replacing SCREEN.CFG on your hard

disk with a copy of the color table you prefer.

2.2.	Starting ELCUT

The ELCUT system is protected with hardware lock key, which must be

attached to parallel port LPT1: of your computer. This key does not

affect the work of your printer and other lock keys.

To start ELCUT, go to the directory you reserved for this work and

enter ELCUT at your system prompt. The command line may include

the problem file name. If it does not, the name is taken from

ELCUT.INI file of the previous session. If ELCUT.INI is absent in the

current directory or does not contain the problem file name, ELCUT

will ask for the name of the problem to work with.

2.3.	Quitting ELCUT

To exit from ELCUT to operating system environment, choose Exit in

the Files menu (ALT+F, X) or press ALT+F4.

3.	Basic Skills

This chapter explains common working patterns you will use with ELCUT.

ELCUT is a menu driven system. The meaning of the selected menu

item is explained by the prompt message occupying the bottom line of

the screen. The same line is used for other messages.

In most context the ESC key may be used to cancel or to interrupt the

current action. The right mouse button is completely equivalent to the

ESC key. The left mouse button is used to click objects.

3.1.	Terminology

The following terms are used to describe your actions when working

with ELCUT.

Choose	-	To use a mouse or key combination to pick an item that

begins an action. For example, choosing a menu item

usually causes the execution of ELCUT command.

Click	-	To press the mouse button while the tip of the mouse

pointer rests on the item of choice.

Select	-	To mark an item by highlighting it with key combinations

or by clicking it with a mouse. Selecting does not initiate an

action.

3.2.	Working with Menus

To choose a menu item click it with a mouse. You can also use the UP

and DOWN ARROW keys to select the item you want; then press ENTER.

If the item name has an underlined letter, you can type it to choose the

menu item with one step. To select an item on the horizontal menu bar

press its underlined letter while holding down the ALT key.

Pressing the ESC key or clicking the right mouse button returns to the

previous menu level. If you press ESC while in main menu, you will be

asked about exiting to DOS.

3.3.	Working with Dialog Boxes

ELCUT uses dialog boxes to request information from you and provide

information to you. For example, when ELCUT needs additional

information to carry out a command you have chosen, a dialog box

requests the information. You complete the dialog box by providing the

missing information. Whenever you see an ellipsis (...) after a menu

command, another menu or a dialog box follows.

For example, when you choose Open from the File menu, ELCUT

displays a dialog box asking for the filename of the file you want to open.

Most dialog boxes contain options, each asking for a different kind of

information. After you supply all the requested information, you choose

a command button to carry out the command.

Often you need to move around within a dialog box to make several

selections. The current option is marked by a highlight or dotted

rectangle (or both) around the name of the option or button. To move

within a dialog box:

*	Click with a mouse the option you want to move to.

*	Press TAB to move forward (generally from left to right and top

to bottom) or SHIFT+TAB to move in opposite direction.

*	You can use the DIRECTION keys to move in desired direction.

*	Or, while you hold ALT, you can type the underlined letter in the

option name or group name.

The options that are unavailable for some reason are dimmed.

The next few sections describe each kind of dialog box option and how

to move and select options.

3.3.1.	Command Buttons

Command buttons initiate an immediate action. One command button

in each dialog box carries out the command you choose, using the

information supplied in the dialog box. This button is usually named

OK. Other command buttons let you cancel the command or choose

from additional options.

Command buttons marked with an ellipsis (...) open another dialog box

so you can provide more information. The currently selected, or default,

button has a highlighted green name or, in a monochrome mode, a

darker border than the other buttons. You can choose the selected

button by pressing ENTER.

You can close the dialog box without completing a command by

choosing Cancel button.

To choose a command button:

*	Click it with a mouse.

*	Move to the command button you want. A dotted rectangle

around the button text marks the selected button. Press the

SPACEBAR (or ENTER) to choose the button.

*	Or, while you hold ALT, you can type the underlined letter in the

button name.

One command button in each dialog box carries out the command you

choose, using the information supplied in the dialog box. This button is

usually named OK. Other command buttons let you cancel the

command or choose from additional options.

Some dialog boxes are so small that does not contain any command

button. Such dialog boxes are usually arise at the right-hand side of the

screen and have gray background. In spite of missing the OK command

button, it is still possible to use a mouse to carry out the command you

choose. Click the dialog box background anywhere outside options. The

effect will be the same as if using the OK command button.

3.3.2.	Text Boxes

A text box is a rectangle into which you type information.

When you move to an empty text box, a text cursor appears at the left

side of the box. The text you type starts at the cursor position.

If the box already contains text when you move to it, all the text in the

box is automatically selected and any text you type replaces it. Or, you

can erase the existing text by pressing DEL. To discard the selection

simply move the cursor to the point where you want to enter or erase

text. Use LEFT and RIGHT ARROW, HOME or END keys to move the cursor.

The text exceeding the length of the text box is scrolled automatically.

3.3.3.	List Boxes

The list box shows a column of available choices. If there are more

choices than can fit in the list box, a scroll bar is provided so that you can

use your mouse to move up and down quickly through the list.

To scroll one line click one of the scroll arrows. To scroll one window up

or down click the gray background of the scroll bar above or below the

white rectangle.

When the required item is already visible in the list box, you can select it

by clicking it with a mouse. Clicking the previously selected item is

equivalent to pressing the ENTER key.

To select an item using a keyboard press UP or DOWN ARROW key until

you reach your choice. You also can use PAGE UP and PAGE DOWN to

move one window a time, and HOME or END to move to the first or to

the last item of the list. Or, type the first letter of the item you want, the

highlight will be moved to the first item that starts from that letter.

3.3.4.	Drop-Down List Boxes

A drop-down list box appears initially as a rectangular box with the

current choice (default) displayed in the box. The arrow in a square box

at the right opens into a list of available choices when you select it. If

there are more choices than can fit in the drop-down list box, a scroll bar

is provided.

A selected drop-down list box can be opened without a mouse by

pressing ALT+DOWN ARROW. An opened drop-down list box is closed

when you select an item in it, select other option in the dialog box, or

press ALT+DOWN ARROW.

3.3.5.	Option Buttons

Option buttons appear in dialog boxes as a list of mutually exclusive

items. You can select only one option from the list at a time. You can

change a selection by selecting a different button.

The selected option button contains a black dot.

To select an option button:

*	Click it with a mouse.

*	Press TAB to move into the option group you want; then use the

DIRECTION keys to select the option button you want.

*	If the option name contains an underlined letter, you can hold

down ALT and press the underlined letter from anywhere in the

dialog box to select an option button.

3.3.6.	Check Boxes

Check boxes offer a list of options you can switch on and off. You can

select as many or as few check box options as are appropriate. When an

option in a check box is selected, it contains X. Otherwise, the box is

empty.

To select or clear check box options:

*	Click with a mouse each empty check box you want to select.

Click a selected box again to clear the selection.

*	Press TAB to move to the empty check box you want to select.

Press the SPACEBAR to enter an X. Press the SPACEBAR again if

you want to clear the selection.

*	If the check-box name has an underlined letter, hold down ALT

and press the underlined letter for each check box you want to

select or clear.

3.4.	Selecting the Geometric Objects

When editing the model geometry or analyzing the results you may need

to enter the coordinates of a point. The plus sign cursor arises to

indicate the point locating mode. This cursor can be moved by mouse or

using the DIRECTION keys. The HOME, END, PAGE DOWN and PAGE UP

keys move the cursor in four diagonal directions. You can control the

keyboard cursor step by the MINUS and PLUS keys. The MINUS key

approximately halves the cursor step, the PLUS key increases it back.

You also can get very fine cursor movement by holding down CTRL

while pressing the DIRECTION keys.

To select a point of the model move the cursor to the position of choice

and click left mouse button or press the ENTER key. ESC or the right

mouse button cancels the operation. If you prefer numerical form of the

coordinates input press TAB and you will get a dialog box with two text

boxes for coordinates typing. Press ENTER or click the dialog box

background to complete the dialog and carry out the command.

When working with the model you often need to select some of its

constituent geometric objects. The picking mode is indicated by the X-

shaped cursor. You can move this cursor the same way as while locating

a point. To pick a massive object like a block place the center of the

cursor on that object and click the left mouse button or press the ENTER

key. To pick a vertex or an edge it is not necessary to point cursor

exactly on the object. The selected object is always the closest to the

cursor.

3.5.	Using the Rubber Band Rectangle

The rubber band rectangle is used to specify the rectangular part of the

model or of the X-Y plot, which is to be enlarged to occupy all available

screen area in the result of zoom-in operation. The rubber band

rectangle is controlled using a mouse or the DIRECTION keys. First, posit

the lower left corner, then the upper right one.

4.	Problem Description

4.1.	Structure of Problem Database

A special database is built for each problem solved with ELCUT. The

kernel of the database is the problem description, which is stored in file

with the extension .PBM. The problem description contains the basics of

the problem: its subject, plane, precision class, etc., and also references

to all other files, which compose the problem database. These files are

the model file, with standard extension .MOD, and physical data files,

with extension .DES, .DMS, .DHT, or .DSA, with respect to the subject

of the problem.

The problem description may refer to one or two physical data files.

Both files have the same format, and differ only in purpose. Usually, the

first data file contains specific data concerning the problem, as the

second file is a library of standard material properties and boundary

conditions, which are common for a whole class of problems.

Owing to the problem is distributed among several files, you may use

single model file or single data file in several similar problems at once.

While solving the problem, ELCUT creates one more file-the file of

results with the extension .RES. This file always has the same name with

the problem description file, and is stored in the same directory.

4.2.	Creating New Problem Description

To create new problem description, choose New in the Files menu

(ALT+F, N), and then enter name for new problem. While created, new

problem inherits settings of the preceding problem. To change these

settings, choose Problem in the Edit menu (ALT+E, P). The dialog box

appears, containing problem description parameters. The problems of

time-dependent heat transfer have additional parameter set. To edit

these values, choose the Time Params button. To exit from problem

description editing, choose OK. You can cancel editing by choosing

Cancel button, or pressing ESC, or clicking right mouse button.

Choosing the Browse button allows you to select the file from the list of

files and directories when defining the model or data filename. The

button acts on that type of file, which box is currently selected.

When the file chosen, you can immediately edit that file by choosing the

Open button. It acts upon currently selected file.

4.2.1.	Choosing the Length Units

ELCUT allows to use different metric units for coordinates when editing

model's geometry. You can use millimeters, centimeters, meters and

even kilometers. To set the units, choose Length Units in the Options

menu (ALT+O, U).

Chosen unit is stored with the problem description, you can choose it for

each problem independently. The choice of length units does not affect

units for other physical parameters, which always use SI units. E.g., the

current density is measured in   independently of length units. The

only physical value that is measured in chosen units of length, is

displacement vector in stress analysis problems.

5.	Model Geometry Definition

This chapter describes how to define the region geometry and build the

mesh using ELCUT preprocessing utility-the Model Editor.

5.1.	Terminology

Vertex, edge, and block are three basic types of geometric objects, which

the Model Editor operates with.

Vertex is a point on the plane with coordinates defined by the user or

calculated automatically as intersection of the edges. For each vertex you

can define mesh spacing value and the label. The mesh spacing value

defines approximate distance between mesh nodes in the neighborhood

of the vertex. The label is used, for example, to describe a singular field

source.

Edge is a line segment or a circular arc connecting two vertices. It can't

intersect any other edge of the region. If an edge being created contains

an existing vertex, two adjacent edges are created. New vertices are

automatically created in all points where new edge intersects the existing

ones and all intersected edges are split by these vertices. While

discretizing the region, edges are divided into elementary fragments

according to the spacing values defined in the vertices. The edge can be

labelled, for example, to specify the boundary condition.

Block is a contiguous subregion with the boundary consisting of edges

and possibly isolated vertices. A block may contain holes which can be

formed by chains of edges or by isolated vertices. Each block has to be

labelled to describe material property. Labels of the blocks are also used

to define distributed field sources. Unlabelled block is not included in

calculation of field even it is covered by the mesh. The Mesh is created

block by block according to the mesh spacing value defined in particular

vertices.

The Label is a string literal up to 16 characters length, which establishes

the correspondence between geometrical objects of the model and

physical values assigned to those. Any printable characters including

letters, digits, punctuation marks, space character are permitted, except

for asterisk (*) and question mark (?) characters. The label cannot begin

with space character; trailing spaces are ignored. Labels are case-

sensitive.

The Mesh Spacing value is the parameter of the units of length associated

with the vertex. The spacing defines the density of the mesh around the

vertex. Changing these values, you can control the accuracy of the

solution.

5.2.	How to Create the Model

Model development consists of three stages:

*	region geometry definition;

*	region discretization;

*	definition of properties, field sources and boundary conditions.

To describe model geometry you define vertices and edges which form

boundaries of all subregions having different physical properties. You

can create or delete a vertex; create, delete or copy an edge.

To get the finite element mesh you have to define the spacing values at

few specific vertices. Spacing values for other vertices are calculated

automatically to make the division smooth. Once the spacing value is

defined at any vertex, the edges are divided automatically. To build the

mesh it is necessary to define the spacing value at least at one vertex.

5.2.1.	Starting and Quitting the Model Editor

To start the Model Editor, choose Geometry from Edit menu (ALT+E,

G) or while editing the problem description select the model filename

and choose Edit button.

The Model Editor uses interactive graphics. Two graphic windows are

displayed on screen permanently. The small window presents general

view of the problem region, while the large one provides more detailed

view. Below the graphic windows is a prompt line. Upper right screen

area is normally occupied by the menu or a temporary window used for

editing values and labels.

The Model Editor has hierarchical structure of menus. Pressing ESC or

right mouse button causes returning to preceding menu level or quitting

the Model Editor in the main menu.

To quit the Model Editor select Exit from the main menu or press ESC

while in main menu. You will be prompted to save the model.

5.2.2.	Region Geometry Definition

With ELCUT Model Editor, you can avoid many routine operations

while describing the geometry. We recommend to start with look throw

the draft to escape from inserting vertices, which could be obtained with

intersection of edges. Also you might detect the objects which could be

built by copying some other objects, think over the auxiliary elements,

which would simplify the procedure and be removed afterwards. We

recommend not to avoid creating edges over existing ones, although you

should take care especially with arcs, because sometimes it takes more

time to remove incorrect edge thrown over the whole region than to

rebuild the model from scratch. We proclaim saving the model to file

regularly to be able to continue with that state if such an error occurs.

It happens that improper elements rest out of window. Zoom Natural

arranges window limits to contain all the elements of the model.

When new model is created, the default window is set to correspond the

unit square. It is convenient to assign the window with region's

dimensions and create the outward boundary of the model at once, and

than describe the details. To change default window dimensions choose

Keyboard from Zoom submenu. The default window dimensions are

saved with the model to be restored in later editing sessions.

Use the Model submenu to change the geometry. To create new vertex

choose Add Vertex. Then locate required position with the cursor or

press TAB to enter coordinates from the keyboard. New vertex appears

in the window. Then you can continue creating vertices or return to

Model menu by pressing ESC or right mouse button.

If at least two vertices are defined in the region you can connect them

with an edge. To do this, choose Add Edge and type in angle size in

degrees for new edge. Zero value corresponds to the line segment.

Positive value defines an arc directed from the first vertex to the second

counter-clockwise, or clockwise if negative. After that pick the vertices

to be connected by the edge in corresponding order. New edge appears

in the window. Picking vertex by vertex you can create several edges of

equal angle size. To break the chain and start new one, press ESC. To

return to menu press ESC twice.

If you want to copy some edges using parallel displacement, choose

Copy Edge. Then edit values of shift vector components and pick the

edges to copy. To return to Model menu press ESC twice. After the first

ESC you can change shift vector and continue copying.

You can remove auxiliary or incorrectly defined vertices and edges by

choosing Remove Vertex or Remove Edge. When some vertex is

removed, all adjacent edges are deleted.

5.2.3.	Building the Mesh

After you get ready with geometry of the model or its part, you can

proceed with building the finite element mesh. The Model Editor allows

to build meshes of uniform or essentially non-uniform density of

elements, e.g. the mesh can be densed to some vertices or subregions

and rarefied to the outer boundary. The mesh have to be finest where

the field changes most rapidly, and also where you need the most

precision.

The mesh density is controlled by spacing values in vertices. The spacing

value defines approximate distance between mesh nodes around that

vertex. You never need to define the spacing in all model's vertices. To

obtain uniform mesh you can set the spacing in any one vertex. This

value is spread among all other vertices automatically. If you need the

non-uniform mesh, define spacing values only in those vertices where

you need finest and roughest mesh. The spacing values are automatically

interpolated to other vertices to smooth the mesh density distribution.

The group selection mechanism allows to assign the value to several

vertices at once.

If the spacing value is defined at least in one vertex, you can proceed

with the mesh building. The mesh is built block by block. You may

order to build the mesh in one block or in selected blocks or in all region

at once.

Changing the density of pre-built mesh (e.g. if solution results show that

you need more precision somewhere in the region) obey some rules:

*	when you change the spacing value in some vertex, the mesh is

removed automatically in those blocks which contact that vertex;

*	the mesh that is not removed, freezes spacing values along its

boundary from recalculation as if those values were defined

manually; so whether you need radical changes to the mesh

density, first remove the mesh in the whole region.

All operation with spacing values and the mesh is performed in the

Mesh submenu. To define the spacing, choose Set Spacing, then pick the

vertex and type in the spacing value. Then you can alter spacing at any

other vertex or return to menu by pressing ESC. If there are some

previously selected vertices, the spacing values are set for all of them at

once. If some edges are selected, the spacing values are set for all

vertices situated at them. The same way, if there are some selected

blocks, the spacing values are set for all vertices situated at their

boundaries or inside them.

To build the mesh in separate blocks, choose Build Mesh and pick the

blocks by turn. Use Build All to build the mesh in whole region except

already meshed blocks.

Remove Mesh removes meshes in chosen blocks, while Remove All

removes all the mesh at once.

If the spacing visibility switch is on (Show Spacing in the Options

menu), the explicitly set spacing values are shown as small circles

around the vertices. You can check out mesh building process when

Show Breaking or Show Mesh switches in the Options menu are on.

5.2.4.	Labelling Vertices, Edges and Blocks

The correspondence between geometrical objects and physical

properties, boundary conditions or field sources is established by use of

labels. All operation with assigning, editing and checking the labels is

done in the Label menu.

To assign a label, choose one of Label Block, Label Edge or

Label Vertex, and then pick the object which you need to label. The

dialog box will appear, which allows to type in the label from keyboard

or to select one from already defined in the model or in data files

assigned to the problem. After you define the label, you can continue

picking the objects or return to the menu by pressing ESC key.

You may assign the equal labels to several objects of similar type at

once. To do so, select those objects and choose Label Selected. How to

select objects by picking them is described in following division.

To check labels assignment or to select objects possessing the same

label, use Find Label. The dialog box will appear, which allows to select

the type of objects, and contains the full list of labels as appears in the

model for the given object type. Picking some label in the list causes

selection of all objects carrying that label.

5.2.5.	Objects Selection

The Model Editor provides possibility to make some group operation

upon several objects at once if those have been previously selected. To

enter select mode, choose Select and then one of  Select Blocks,

Select Edges or Select Vertices in submenu that will appear.

While in select mode pick an object to select or unselect it. All selected

objects are highlighted on the screen. To leave the select mode press ESC

or right mouse button. Objects of different types cannot be selected

simultaneously.

Unselect All cancels all previous selection.

5.2.6.	Graphics Window Management

There are two graphic windows on the screen while editing the model.

The small window always displays the general view of the model. The

large window is used to present more delayed picture of the whole

model or its selected part. Zoom menu provides several options to

control which part of the model will be seen in each graphic window.

Keyboard allows you to type in dimensions of the visible region.

Specified region is displayed in both windows and becomes the default.

This is a normal way to extend the default visible region. Limits of the

visible region can be automatically adjusted to preserve equal X and Y

scales.

Natural sets minimum visible region large enough to contain the whole

model. Resulting region is displayed in both windows and becomes the

default.

Default sets visible region dimensions to the default values. The region

visible in the large window becomes the same as in small one.

Large Window specifies graphic input of visible region dimensions using

rubber rectangle in the large window. The rubber rectangle is controlled

using mouse or cursor keys. First, posit lower left corner, then upper

right one.

Small Window does the same, but rubber rectangle appears in the small

window. This mode is recommended when whole or part of the required

region lies outside the large window.

5.2.7.	Obtaining Model Information

You can get the detailed information on the current model state entering

Info submenu. After you pull it down, choose the object type to deal

with (vertices, edges or blocks). The complete information about that

type will appear (amount of objects, presence of labels, mesh

parameters, etc.). Then pick an object you are interested in to get local

information. Pressing ESC returns to menu. This mode is used to get

total number of mesh nodes, to check label values, to examine the mesh

spacing values and so on.

5.3.	Additional Options

5.3.1.	Saving Model

The Save item of File menu saves model to disk. The Save As does the

same but lets you change the name of output file. It is wise to save mesh

not only when editing is over, but regularly during the session to avoid

troubles due to errors or supply failure.

Note Changing output file name with Save As does not

cause changes in problem description.

5.3.2.	Opening Model File for Edit

To create new empty model choose New in the File menu. To open

existing model file choose Open in the same menu, and then type or

select the file name. If the active model was changed you will be

prompted to save it before transition to the new one.

With Open, you can import old geometry model files created with the

first or second versions of ELCUT and having .TRI extension. To do so,

you need only to specify .TRI extension when selecting the file.

Note While loading old-formatted model, all digital labels

are transferred to literal form automatically.

5.3.3.	DXF File Import

You can import model geometry or its fragments from the DXF file

produced with any major CAD system. To do so, choose Import DXF in

the File menu and then type or select required file name. The visible

region is automatically extended if needed to assure visibility of all

imported geometric objects. If the model is not empty when reading the

DXF file, it is recommended to save current model state before the

operation. This will give you a chance to return back if the imported

objects incidentally overlap the existing part of model.

5.3.4.	Discretization Visibility Options

There are three switches Show Spacing, Show Breaking and Show Mesh,

which affect discretization visibility level. Those are accessible in

Options menu. By default all the switches are on (ticked off in the

menu). When all these switches are off, region is displayed without

discretization. This mode is useful for region geometry description and

label setting. If the Show Spacing mode is switched on, all explicitly set

spacing values are shown as circles with the appropriate radii.

When Show Breaking switch is on, you can see the result of automatic

edges fragmentation. It is convenient to use both Show Spacing and

Show Breaking when specifying the mesh spacing values. Show Mesh

lets you see triangular mesh where it exists. Turn it on to check mesh

building.

5.3.5.	Attraction Distance Parameter

To avoid very fine unrecognizable inaccuracies in geometry definition

new vertices or edges cannot be created very close to existing ones. The

creation of new geometric objects is controlled by the E parameter also

called the attraction distance.

The following rules concern creating new vertices and edges.

*	Creating a new vertex is prohibited within 2E-neighborhood of

the existing one.

*	A new edge cannot be added if it joins the same vertices as an

existing edge and the maximum gap between them does not

exceed E.

*	If the distance between a vertex to add and some edge is less

than or equal E, the vertex is attracted by the edge and the edge

is automatically split into pair of new edges to incorporate the

vertex. The same is true when new edge is added, but in this case

the new edge may be attracted by existing vertex.

The default value of E is 0.5 per cent of the visible region size. You can

set different value by choosing Epsilon position in Options menu.

Decreasing E would allow you to describe very fine details of the model.

But the most convenient way to get the same result is to zoom in the

window.

6.	Problem Parameters Description

To solve the problem it is needed to describe the material properties,

field sources and boundary conditions. These parameters are stored in

the property description file. The correspondence between records of

these files and subdomains or boundaries of the region is established by

the labels assigned to geometrical objects during editing the model.

Labelling vertices, edges and blocks is described in "Model Geometry

Definition".

Physical values for a problem may be defined in one or two data files

attached to the problem. Both files have the same format and distinguish

only in purpose. The first file is the basic data file of the problem and is

assumed to contain the data specific for the problem. The second is the

library file, that contains common material properties and standard

boundary conditions for a class of problems.

To edit the basic data file, choose Data in the Edit menu (ALT+E, D) to

edit the library file, choose  Library (ALT+E, L.) The alternate method to

start editing data is to select the name of one of the data files and to

choose the Open button  while editing the problem description.

When entering data file editing mode, the dialog box appears, containing

three lists of labels, which corresponds to blocks, edges and vertices.

Option buttons above the list boxes allow to choose which group of

labels is currently dealt with. One label in the list may be highlighted. In

this case it appears also in the text box below the list boxes. The label in

the text box is the current label, so all the immediate actions are done to

this label.

When editing the basic data file of the problem (not library), label lists

contain not only labels defined in the data file, but also those labels the

model file refers to although not defined in neither basic nor library data

files. Such labels are marked with asterisk in the list.

Labels of blocks, excluded from consideration (i.e., having empty

material properties,) are marked with double exclamation mark.

Options for editing the data file could be divided into three groups. The

first group considers actions on creating new labels and editing data for

existing ones. Operations on copying, renaming and removing some data

set, which corresponds to some label, belong to the second group. The

third group assumes the actions on saving the current state of data file

under another file name and merging two data files.

To exit from data file editing, choose the Close command button or

press the ESC key. You will be prompted to save the changes to file.

6.1.	Creating New Label

To create new label:

*	choose appropriate type of geometrical object-block, edge or

vertex;

*	type the name of the label in the text box titled Label. You may

use the INS key in addition to usual methods to move to text box.

If the label's name already exists in the list, but is marked with

asterisk (which means that the data for the label is not defined,)

you need not type the name, but simply select it in the list using

keyboard or mouse;

*	choose Add button or press ENTER to start editing the data.

After you define the data, new label appears in the list of existing labels.

If data editing was canceled, new label is not created.

6.2.	Editing Label Data

To edit the data assigned to some label, select that label and choose Edit

button or press ENTER. The dialog box appears, its view depends on the

class of current problem and on the type of geometrical object which the

label corresponds to.

To finish label data editing, choose OK button. Choosing Cancel button

will end the editing with discarding all changes to the values.

6.2.1.	Editing Data in Electrostatics

Block label data for electrostatics problem contain two components of

electric permittivity and possibly distributed charge density.

When creating new label, the text boxes for electric permittivity

components contain None instead of numbers. The word None in these

boxes or absence of value means that the block containing such label is

excluded. If you want to define the material properties (and therefore

include the block into consideration,) simply type in the value of electric

permittivity, which will replace highlighted None.

If you need to define two components different from each other, first

check the Anisotropic box.

The data for the edge label allow to assign one of possible boundary

conditions. Select the type of condition and then type in the values.

The vertex in the problem of electrostatics may have known potential or

concentrated charge. Check one of these options and then enter a value.

6.2.2.	Editing Data in Magnetostatics

With problems of magnetostatics, block label data contain two

components of magnetic permeability tensor, the current density and

two components of coercive force, if the subregion is the permanent

magnet.

With nonlinear materials, you need to define the magnetization curve,

instead of magnetic permeability. In this case the dialog box contains the

B-H Curve button. Choose the button to edit the curve. Check the

Nonlinear box to switch between linear or nonlinear material properties.

Editing the magnetization curve is discussed in "Editing the Curves".

When creating new label, the text boxes for magnetic permeability

components contain None instead of numbers. The word None in these

boxes or absence of value means that the block containing such label is

excluded. If you want to define the material properties (and therefore

include the block into consideration,) simply type in the value of

magnetic permeability, which will replace highlighted None.

If you need to define two components different from each other, first

check the Anisotropic box.

The data for the edge label allow to assign one of possible boundary

conditions. Select the type of condition and then type in the values.

The vertex in the problem of magnetostatics may have known potential

or the concentrated current may flow through the vertex. Check one of

the options and then enter a value.

6.2.3.	Editing Data with Heat Transfer Problems

The data for block label contain two components of thermal conductivity

tensor and, possibly, the volume power of heat source. With time-

dependent problems, the value of specific heat per unit volume is

significant.

To describe the thermal conductivity as a function of temperature, check

the Nonlinear box. The  l = l(T) button will appear, choose it to edit

the dependency.

Also the volume power of heat source could be described as a function

of temperature. To do so, check the Function of Temperature box

related to the heat source field. To edit the dependency, choose the

q = q(T) button which will appear. The templates for editing the

dependencies are described in "Editing the Curves".

When creating new label, the text boxes for thermal conductivity

components contain None instead of numbers. The word None in these

boxes or absence of value means that the block containing such label is

excluded. If you want to define the material properties (and therefore

include the block into consideration,) simply type in the value of thermal

conductivity, which will replace highlighted None.

If you need to define two components different from each other, first

check the Anisotropic box.

The data for edge label allow to describe boundary conditions. Check the

condition which you need, and then type in the parameters. The

condition of the second kind and the convection and radiation condition

could be combined together which means that the heat flow through the

surface is compounded from several components.

The vertex in heat transfer problem may have known temperature or

string source. Check one of these possibilities, and then enter the

numeric parameter.

6.2.4.	Editing Data with Stress Analysis Problems

When editing the data for the block label with stress analysis problem,

there are two sets of properties to be edited simultaneously. To switch

from one set to another, use option buttons at the top of dialog box or

press PAGE UP or PAGE DOWN keys.

When creating new label, the text boxes for Young's moduli contain

None instead of numbers. The word None in these boxes or absence of

value means that the block containing such label is excluded. If you want

to define the material properties (and therefore include the block into

consideration,) simply type in the value of the Young's modulus, which

will replace highlighted None.

The Anisotropic boxes, which applied to elastic moduli or coefficients of

thermal expansion, allow to describe anisotropic properties in each set

independently.

The values for allowable stresses do not affect the solution. Those are

only used in postprocessing stage to calculate the Mohr-Coulomb and

Drucker-Prager criteria. You need not define allowable stresses, if you

are not interested in these criteria.

The data defined for some edge label may include restraints along one or

both coordinate axes and the surface force described as surface pressure

or Cartesian components. To apply fixed displacement along the axis,

check the appropriate box and then enter a value of displacement.

The node label data may define rigid or elastic support along one or both

coordinate axes, or concentrated external force. To describe rigid

constraint along some axis, check the appropriate box, and then enter

the value of fixed displacement.

6.2.5.	Editing the Curves

Curveous functions, which describe some physical dependencies, are

implemented as tables containing two columns: an argument and a

function, e.g., magnetic field intensity and flux density or temperature

and thermal conductivity. Editing the table is supported with graphical

presentation of the dependency, which is interpolated with cubic spline

among the entered points. The solver uses just the same curve as you

see on your screen.

To add the new point to the dependency, type in two values (B and H in

shown example,) and press ENTER key or choose Add button. If the

argument of a new point coincides with the argument of existing one,

new point replaces the old one.

To remove the point, select it in the table and choose the Delete button

or press the DEL key.

You may control the scaling of the graph with use of the Zoom In or

Zoom Out buttons.

To exit from editing the curve, choose the Close button or press ESC.

Note that subsequent canceling label data editing with ESC key or the

Cancel button will discard all changes including the curve editing.

6.3.	Copying, Renaming and Deleting the Label

In order to copy or rename the label (to be precise, the data set related

to the label,) select the label in the list and choose the Copy or Rename

button. The new name is entered in a dialog box, which appears when

you choose the button.

To remove the label, select it and choose Delete or press DEL.

6.4.	Merging and Copying Data Files

The Merge command button allows you to expand the contents of the

data file being edited with the labels contained in some other data file for

the same type of problems. The labels with coinciding names will not be

replaced.

To copy current state of edited data under some name, use the Save As

button. This function does not affect the name of the file being edited.

7.	Obtaining and Analyzing the Results

This chapter tells how to solve the problem and how to use the

postprocessor to analyze  the results.

7.1.	Solving the Problem

Several conditions have to be met to solve the problem. The problem

type, plane, required precision and other parameters have to be specified

in the problem description file. The model geometry file must contain

complete model with mesh and labels. Each label referred by the model

file is to be defined in the problem's private or library data file.

To obtain the problem solution choose Solve Problem from the Results

menu (ALT+R, S). You may skip this action and directly proceed to the

results analysis by choosing Analyze from the Results menu (ALT+R, A).

If the problem has not been solved yet or its results are out of date, the

solver will be invoked automatically.

Special bar indicator lets you see the progress of the solution process.

Linear problems are solved by using the preconditioned conjugate

gradients method. The preconditioning based on the domain

decomposition technique guaranties very high speed and very weak

dependence between number of nodes and the required number of the

CG iterations. Nonlinear problems are solved using the Newton-

Raphson method. The Jacobian matrix arising at the each step of the

Newton-Raphson method is inverted the same way as it is done for

linear problems.

The solver writes the results to the file with the extension .RES, and

with name and directory matching the problem description file.

7.2.	The Postprocessor

This section explains how to arrange the results for detailed examination

using the ELCUT postprocessing utility.

The Postprocessor provides various ways of results presentation:

*	field pictures,

*	local field values,

*	integral quantities,

*	X-Y plots,

*	tables.

7.2.1.	Interpreted Quantities and Forms of Presentation

The set of the physical quantities which can be displayed by the

Postprocessor depends on the problem type.

For the electrostatic problem these quantities are:

*	scalar electric potential;

*	vector of electric field intensity;

*	vector of electrostatic induction;

*	electric permittivity

For the magnetostatic problem:

*	vector magnetic potential;

*	vector of magnetic flux density;

*	vector of magnetic field intensity;

*	magnetic permeability.

For heat transfer problem:

*	temperature;

*	vector of heat flux density;

*	thermal conductivity.

For stress analysis problems:

*	displacement vector;

*	stress tensor and its principal values;

*	Von Mises, Tresca, Mohr-Coulomb and Drucker-Prager criteria.

Several methods possess drawing of 2D field picture:

*	color map of the distribution of a chosen scalar quantity. The

color map is accompanied by the legend showing the

correspondence between colors and numerical values.

*	field lines. Those are isotherms for temperature fields, lines of

equal potential in electrostatics and flux lines for magnetostatic

problems;

*	family of line segments showing magnitude and direction of the

vector quantity. The base point of each vector is marked by dot;

The following methods are used only for stress analysis problems:

*	stress tensor display as a pair of eigenvectors reflecting the

direction of principal axes, magnitudes and signs of principal

stresses (blue color denotes tension, red color-compression);

*	deformed boundary drawing;

*	deformed shape indicated by means of deformed and original

rectangular grid.

The Postprocessor draws vector and tensor symbols suited in the nodes

of equidistant rectangular grid with user-controlled cell size. The same

grid is used for deformed shape display.

It is possible to combine several visualization methods in the same

picture to obtain the most expressive one.

7.2.2.	Starting and Quitting the Postprocessor

To examine the results choose Analyze from the Results menu (ALT+R, A).

Screen layout when working with the Postprocessor is very similar to

one of the Model Editor. There are two graphic windows displayed on

the screen. The small one presents general view of the model, while the

large one shows detailed field picture or an X-Y plot. The message bar is

at the screen bottom. Upper right screen area is normally occupied by

the menu, legend box, or a temporary window used to display text

information.

When you decided to leave the Postprocessor and exit to the ELCUT

main menu, choose Exit from the main Postprocessor menu or press

ESC key in that menu.

7.2.3.	Field Picture Constructing

When entering the Postprocessor, the default form of the field picture

appears on the screen. You may use View menu to select other methods

and quantities to display by checking menu positions. If the color map is

checked, the quantity to map may be chosen from the lower part of the

menu. You may select several methods of presentation, but only one

quantity at once. If the list of available quantities is too long to fit in the

menu, some quantities are accessible through Others submenu.

Note. Selecting the Shape position (deformed shape

display) for stress analysis problems will turn on the

Boundary position automatically.

The picture will be re drawn on exit from the View menu. If you select

none of the visualization methods, drawn will be the contours only.

7.2.4.	Editing Scales and Ranges

To adjust the scale of the field picture choose Zoom from the menu. This

command is very similar to the analogous command of the Model

Editor, but the default window dimensions cannot be altered, they retain

values set in the Model Editor.

The default values of color scales, ranges of X-Y plots, field lines

spacing, scales of vectors, tensors and displacements are calculated

automatically when the results are loaded by the Postprocessor. For

dynamic problems all the defaults are suited to the final field picture, i.e.,

the last time step. By choosing the Scaling command you can adjust

those parameters that affect the current field picture.

The density of the field lines is controlled by the distance between two

neighboring lines. This distance is measured in potential units. The color

scale is determined by two parameters-the lower and upper limits of

the range of the shown quantity. The specified range is mapped to the

available set of colors.

Vector and tensor symbols are drawn in the nodes of the regular

rectangular grid. The same grid is used with deformed shape display.

You can change the grid cell size and the scaling factor for desired

feature.

Sizes of the vector symbols of all vector quantities except the

displacement vector are determined by the corresponding physical value

multiplied by the scaling factor and by the cell size. Similar method is

used for stress tensor components. Unlike other interpreted physical

quantities, the size of the displacement vector on the screen does not

depend on the cell size. It is determined by the dimensionless scaling

factor, the unit value of which means that the displacement is shown in

its natural scale.

7.2.5.	Local Data Display

The Postprocessor displays local field data in Values mode. Click the

point where you need to know the values of the field quantities, or press

TAB and then enter the coordinates of the point with the keyboard. Once

you choose the point, the values at this point are displayed on the screen.

To leave the Values mode use ESC key, or press right mouse button.

The values of the local physical quantities obtained in the Values mode

can be logged to the table file. This file has self explaining ASCII format,

its default extension is .TBL. The table file can be used for printing

numerical results, or to pass them to other application program, e.g., a

spreadsheet program to produce the report.

To open the table file choose Table File from the Options menu. You

will be asked about name of the table file, and when you provide it the

file will be opened. Existing files may be appended or overwritten. Every

point you click in the Values mode causes writing a line to the table file.

To close the table file choose Table File from the Options menu again.

7.2.6.	Editing Contours for X-Y Plots and Integrals

The drawing of the X-Y plot or a calculation of the integral quantity is

based on the to be evaluated.

The contour is needed when the X-Y plot is to be drawn or the integral

quantity is to be calculated. The contour is the line consisting of the

edges of the model and/or other line segments and arcs. Closed

contours are used for integrals only, while the open ones have sense for

X-Y plots and for some integrals. The contour can be specified in the

Edit Contour mode and preserved until next modification. The following

operations change the current contour state:

Add Line	 -	attaches a line segment or an arc to the

contour. The arc is specified by its degree

measure (zero means line segment) and two

end points. The contour may be initiated by an

arbitrary line, but only adjacent lines are

accepted later. The line cannot be added to the

closed contour. Adding lines is terminated by

pressing ESC or when the contour becomes

closed.

Add Edge	 -	append the contour with an edge of the model.

The contour may be initiated by an arbitrary

edge, but only adjacent edges are accepted

later. The edge cannot be added to the closed

contour. Adding edges is terminated by

pressing ESC or when the contour becomes

closed.

Add Block	 -	considers the current closed contour as a

frontier of the plane region and updates this

region by adding (or subtracting) a block of the

model in the sense of set theory. Adding blocks

is terminated by pressing ESC.

Change Direction -	alters the contour direction. The direction is

shown by the arrows at the contour elements.

Undo	         -	reverses the last action done with the contour.

Clear	         -	deletes the contour.

The contour may not intersect itself. Open and closed contours are

discerned. Depending on the current contour state some editing

operations may be prohibited.

The direction of the contour is significant in the following cases:

*	for volume integrals the domain of integration lies to the left

from the contour. It corresponds to the contour's interior if the

contour is directed counter-clockwise, or to the contour's

exterior otherwise.

*	for surface integrals the positive normal vector points to the right

relatively to the contour direction.

*	the starting point of the contour corresponds to zero point at the

x-axis of the X-Y plot.

*	if the plotted or the integrated function has different values to the

left and to the right of the contour, the right-hand value is used.

7.2.7.	X-Y Plots

In X-Y Plot mode you can examine the distribution of some field

quantities along the selected contour. The X-Y Plot mode menu allows

you to edit the contour, to select the shown quantity, to adjust the scale

of the current plot, or to tabulate the field quantities along the current

contour.

To select a quantity to display choose View from the menu. Selecting the

quantity for the X-Y plot is similar to selecting the quantity for color

map. See "Field Picture Constructing" earlier in this chapter.

Zoom option allows you to change the y-scale of the plot with the

procedure similar to zooming the field picture. The default displayed

range of the shown quantity can be set by choosing the Scaling option.

Note. When entering plotting mode, ELCUT draws the

quantity last selected for color map. Altering the quantity

shown on the X-Y plot causes modification of the quantity

for color map. Default ranges of the quantities are also

common for X-Y plots and for color maps.

To tabulate the field quantities along the current contour choose

Tabulate from menu. Each element of the contour is divided into 20

equal pieces and data for all these points are written to the file. Existing

files may be appended or overwritten. Output format of the Tabulate

command is the same as for the table file output described earlier in

"Local Data Display." This command does not affect state of the table file

if it was opened to log the values in the selected points.

7.2.8.	Integral Quantities

The set of calculated integral quantities depends on problem type.

The quantities available for electrostatic problems are:

*	electric charge;

*	electrostatic force;

*	torque of electrostatic forces;

*	electric field energy.

For magnetostatic problems:

*	magnetostatic force;

*	torque of magnetostatic forces;

*	magnetic field energy;

*	flux linkage.

For heat transfer problems:

*	heat flux.

No integral quantities have sense for stress analysis.

Equations for above integral quantities are provided in chapter

"Theoretical Description". For plane-parallel problems all integral

quantities are considered per unit length in z direction.

Torque is calculated relative to the origin of the coordinate system and

only for plane problems.

To get the integral quantities choose Integrals from menu. In the

Integrals mode you can edit the contour and select the required quantity

from menu. Some integrals require closed counter-clockwise oriented

contour, otherwise they have no physical sense.

Apart from the main list of integral quantities some additional integrals

are available in Line/Surf. Int. menu (line and surface integrals) and in

Volume Int. menu (volume integrals). All these integrals are defined by

equations provided in the menu.

When the electrostatic or magnetic force, torque, electric charge, or heat

flux are to be calculated the domain of integration may be chosen by

many different ways. The only requirement for the surface of integration

is to contain all the necessary bodies, but to avoid any extra bodies or

field sources. It is significant to understand that the precision will be the

best if you choose the integration surface as far as possible from the

places with strong inhomogeneity of field, e.g., field sources or

boundaries of conducting or ferromagnetic bodies.

When calculating the flux linkage the domain of integration must exactly

fit the cross section of the coil.

7.2.9.	Saving the Postprocessor State

Current state of the Postprocessor can be saved to the special .SST file

and restored from it later. The current state includes: chosen method of

presentation, selected quantity, scales, ranges, current contour state,

color table, etc. If you analyze several similar problems or the results of

the same problem several times, you can save a lot of work by reusing

the same Postprocessor parameters once saved in the .SST file.

Choose Save Setup from the Options menu to save current

Postprocessor state and Load Setup from the same menu to restore this

state from the file. In both cases you will be inquired to supply the file

name. The default file name is constructed from the current problem

name and .SST extension.

7.2.10.	Printing the Results

ELCUT supports special color scheme suited for creating monochrome

screen hardcopies. You can select this color scheme  with Options

Colors command, which is available in main ELCUT and Postprocessor

menus.

If you are running MS-DOS 5.0 and the GRAPHICS program is resident

you can obtain the screen hardcopy by pressing SHIFT+PRINT SCREEN on

the keyboard.

Many word processing programs, e.g., Microsoft Word, provide way to

obtain screen image in the file, and then include it into the text

document. This is the most convenient and common way of producing

high quality reports.

Appendix A. Example Problem from Start to End

Title:

Microstrip Transmission Line Capacitance.

A shielded microstrip transmission line consists of a substrate, a

microstrip, and a shield.

Problem Type:

Plane-parallel problem of electrostatics.

Geometry:

The transmission line is directed along z-axis, its cross section is

shown at the sketch. The rectangle ABCD is a section of the shield,

the line EF represents a strip.

a

D  |<------------------------------>| C

|________________________________|_________

I                                I        |

I                                I        |

I                                I        |

I               Air              I        |

I                                I        |

I                                I        | a

I         E    Strip    F        I        |

I___________=====w=====__________I______  |

I            Substrate           I     |b |

I________________________________I_____|__|

A                                     B

Given:

Relative permittivity of air  e = 1;

relative permittivity of substrate  e = 10;

dimensions: a = 10 cm, b = 1 cm, w = 1cm.

Problem:

Determine the capacitance of a transmission line.

Solution:

There are two approaches to calculate the capacitance of the line:

*	to apply some distinct potentials to the shield and the strip

and to calculate the charge that arises on the strip;

*	or, to apply zero potential to the shield and to describe the

strip as having constant but unknown potential and carrying

the charge, and then to measure the potential that arises

on the strip.

Both these approaches make use of the equation for capacitance C = q / U.

The first approach needs to get the charge as a value of integral

along some contour, and the second one uses a local value of

potential. Therefore, we recommend the second way of calculating

the capacitance (or partial capacitances, if the number of

conductors exceeds two.)

The first approach is illustrated in the STRIP1.PBM problem on

your EXAMPLES disk, and the STRIP2.PBM explains the second

approach.

The method of calculating the capacitance via the energy of electric

field could not be applied to this problem, since the energy integral

does not converge in theory due to infinite singularity on the ends

of strip.

Results:

Theoretical result:	C = 178.1 pF/m.

Approach 1:		C = 186.4 pF/m.

Approach 2:		C = 186.5 pF/m.

Step-by-step Description

Let us learn, how to solve this problem from scratch, using the

second approach. We'll forget the solution made in STRIP2.PBM,

and start a new problem, STRIP3.PBM.

To create new problem:

*	Choose New in the Files menu (ALT+F, N); the dialog box

appears, asking for the filename for new problem.

*	Change, if needed, the drive and directory in the Directories

list box.

*	Type strip3 in the Filename box.

*	Choose OK.

The extension .PBM will be added automatically.

To select convenient length measurement units (centimeters):

*	Choose Length Units in the Options menu. A dialog box

appears.

*	Select Centimeters.

*	Choose OK.

To assign the problem with appropriate features:

*	Choose Problem in the Edit menu (ALT+E, P). The

Problem Description dialog box appears.

*	Select Electrostatics in the Problem Type drop-down list

box.

*	Select XY Plane.

We'll agree with suggested model and data file names

(STRIP3.MOD and STRIP3.DES.) If the Library Data filename

box is not empty, clear it, since we'll define all the labels in the local

data file (STRIP3.DES.)

We can start editing the model or the data directly in the Problem

Description dialog box. To edit the model:

*	select the Geometry text box (click anywhere in the box

with a mouse, or press ALT+G, or press TAB until the

selection reaches the box.)

*	choose the Open button.

The Model Editor starts.

The first step with the new model in Model Editor is to adjust

window dimensions fitting the problem's region. As the problem

has vertical axis of symmetry, it is convenient to set zero of x-axis at

the axis of symmetry. So the region fits the square (-5 <= x <= 5,

0 <= y <= 10). To assign these values to the window limits:

*	Choose Keyboard in the Zoom menu.

*	Type the values in appropriate text boxes.

*	Press ENTER or click the dialog box background anywhere

outside options.

Now we can proceed with defining the geometry itself. To define

the vertices which correspond to points labeled with letters A

through F in the sketch:

*	Enter Add Vertex mode in the Model menu. The plus sign

cursor (*) arises in the large window indicating the point

locating mode.

*	Use DIRECTION keys or a mouse to move cursor from point

to point and press ENTER or click left mouse button where

you want the vertex to arise. New vertices immediately

appear in the window.

Or, press TAB, type coordinates, and press ENTER for each

new vertex. Don't worry of making mistakes--you can

remove erroneous vertices later.

*	Press ESC to return to the Model menu.

If you have created excess points, you can remove it now:

*	Choose Remove Vertex. The X-shaped cursor appears to

indicate the picking mode. Consequently pick excess

vertices, which immediately disappear.

*	Press ESC to return to the Model menu.

Now we can create edges connecting the vertices:

*	Choose Add Edge. A dialog box appears asking the arc

angle for new edges.

*	Press ENTER (or click gray background of the dialog box) to

agree with suggested zero value, which means creating the

straight lines. The picking mode (X-shaped) cursor appears

in the window.

*	Pick points A, B, C, D, A consequently to create edges,

which constitute the rectangle ABCD. The edges

immediately appear on the screen.

*	Press ESC to break the chain of edges and start a new one.

*	Pick point E and then F to create the corresponding edge.

*	Press ESC twice to return to the Model menu.

You can remove erroneous edges, using the Remove Edge

command.

We need some additional constructing to create edges which

separate substrate from air. The easy way to create them is to copy

the edge AB shifting it 1 cm upper. No doubt that new edge will

partly coincide with already created edge EF--while creating, the

coincidence is checked out, and only non-existent parts of new edge

are really created. To make a copy:

*	Choose Copy Edge. A dialog box appears asking for the

components of the shift vector.

*	Press TAB or DOWN ARROW to move to the text box

corresponding to y-component.

*	Type 1.

*	Press ENTER. The picking mode (X-shaped) cursor appears

in the window.

*	Pick the edge AB to make its copy. New vertices and edges

arise on the screen.

*	Press ESC to cancel copying mode and return to the Model

menu.

We've done with the model's geometry. Now we can assign labels

to geometrical objects to describe material properties, sources and

boundary conditions.

The model contains two blocks having different material properties:

the air and the substrate. To be clear, we can use the word Air to

label the upper block and Substrate for the lower one. To assign

these labels to blocks:

*	Press ESC to close the Model menu and return to the main

menu of the Model Editor.

*	Choose Label Blocks in the Label menu. The picking mode

cursor arises.

*	Pick the upper block. It becomes highlighted and the dialog

box appears asking for the label value.

*	Type Air and press ENTER. The X-shaped cursor appears

again.

*	Pick the lower block, type Substrate and press ENTER.

*	Press ESC to return to the Label menu.

Edge labels are used to define specific boundary conditions on

inner and outward boundaries of the region. In our case, we need

to specify boundary conditions for the shield (rectangle ABCD) and

for the strip (line EF). To assign labels to edges:

*	Choose Select Edges from the Select menu (we'll select

several edges to assign a label to them at once.) The picking

mode cursor arises.

*	Pick consequently six edges, which constitute the rectangle

ABCD. Those edges become highlighted that indicate the

selection. If you have selected excess edge, pick it once

more to unselect it.

*	Press ESC to cancel selection mode.

*	Choose Label Selected. A dialog box appears asking for the

label value.

*	Type Shield and press ENTER to assign the label to selected

edges.

*	Choose Label Edges. The picking mode cursor appears.

This mode is convenient for assigning labels edge-by-edge.

*	Pick the edge EF. A dialog box appears.

*	Type Strip and press ENTER to assign the label.

*	Press ESC to return to the Label menu.

We also need to assign vertex label to any vertex contacting the

strip, to specify that the strip is charged. No matter which vertex

you choose, the charge will be distributed through all the

conductor. To assign the label to a vertex:

*	Choose Label Vertices. The picking mode cursor arises.

*	Pick any of vertices E or F. A dialog box appears asking for

the label value.

*	Type Charge and press ENTER to assign the label to vertex.

*	Press ESC to return to the Label menu.

Now we have finished with assigning labels to geometrical objects.

You can check their values in the Find Label mode.

We can proceed with building a mesh of finite elements. To define

the mesh density, we need to define spacing parameters in several

vertices of the model. We suppose that the electric field is most

non-homogeneous near the ends of the strip, so the mesh there

must be maximum dense. Therefore, we'll assign the spacing value

of 0.2 cm to the vertices E and F and the value of 0.8 cm to the

vertices A, B, C and D to build the mesh of approximately 400

nodes. To define spacing values:

*	Press ESC to return from the Label menu to the main menu.

*	Choose Mesh to open the mesh building menu.

*	Choose Select Vertices from the Select menu, which allows

to assign one spacing value to several vertices at once.

*	Select vertices A, B, C and D and press ESC to return to the

Mesh menu.

*	Choose Set Spacing. A dialog box appears asking for the

spacing value.

*	Type 0.8 and press ENTER. This value is now assigned to

selected vertices.

*	Choose Unselect All from the Select menu to unselect all

previously selected objects.

*	Select vertices E and F.

*	Choose Set Spacing, type 0.2 and press ENTER. The model

is now ready to build the mesh.

*	Choose Build All to build the meshes for both blocks at

once.

Now the model is ready. To exit the Model Editor with saving the

file:

*	Press ESC twice to close the Mesh menu and quit. The box

appears prompting you to save the model file.

*	Choose Yes to confirm saving operation.

You have returned to the Problem Description dialog box. Let us

continue with defining data for material properties and boundary

conditions. To start editing data file:

*	Select the Data text box.

*	Choose the Open button. A dialog box appears warning you

that the file STRIP3.DES does not exist.

*	Choose OK to create new data file.

The Properties Description File dialog box appears. It contains

labels, which you have just defined in the model. The label names

are marked with asterisks to outline the fact, that the data for these

labels are not yet defined. Now we need to select the labels one-by-

one and to define the data for them.

To define the data for block label Air:

*	Select its name in the list box (click it with a mouse, or

press ALT+B and use DOWN key to highlight the name's field.)

*	Choose Open button (or press ENTER as Open is the default

button, or click a mouse once more on the name's field.)

A dialog box appears, prompting to enter material properties and

distributed source for block label Air. To assign values:

*	Type 1 in any text box for components of electric

permittivity tensor.

*	Choose OK.

Repeat last actions for the label Substrate. The value of relative

permittivity of substrate is 10.

Now we'll continue with edge labels' data. We'll define the label

Shield as homogeneous Dirichlet boundary condition (U = 0) and

the label Strip as conductor (U = const) condition.

To define the data for edge label Shield:

*	Select its name and choose Open. A dialog box appears,

allowing to assign to edge label any of possible boundary

conditions.

*	Select the Dirichlet Condition box. Zero value of pre-

defined potential will be suggested.

*	Choose OK.

To define the data for edge label Strip:

*	Select its name and choose Open.

*	Select the Conductor box.

*	Choose OK.

We need to define the vertex label Charge to assign the charge to

the strip. While determining the capacitance, no matter what exact

value of the charge to specify. We'll issue the value of 1.

To define the data for vertex label Charge:

*	Select its name and choose Open. A dialog box appears,

allowing to specify the charge, or to assign the Dirichlet

boundary condition.

*	Select the Electric Charge box.

*	Type 1 in the text box for charge value.

*	Choose OK.

All the data needed to solve the problem is now defined. To exit

from data editing mode:

*	Choose Close button in the Properties Description File

dialog box. A dialog box appears, prompting you to save

changes to data file.

*	Choose Yes to save changes. Now you return to the

Problem Description dialog box again.

*	Choose OK.

At last, we can solve the problem and analyze the solution. To do

this in one step:

*	Choose Analyze from the Results menu. You will be

suggested to solve the problem first, as the results are

absent.

*	Choose OK.

The solver starts, showing you the process in dynamics. After

finishing, the Postprocessor starts automatically. There are many

possibilities to analyze the field in the Postprocessor. We will show

only those steps needed to determine the capacitance:

*	Choose Values from the menu. Cross-shaped cursor

appears allowing you to pick points to determine the local

field data.

*	Move the cursor to the point (0.0, 1.0) (exactly; the

convenient way is to use the DIRECTION keys, or to press

TAB and type coordinates from keyboard.)

*	Press ENTER.

The box appears, showing you the local field data. The potential of

the strip is 4.98E9 Volts. The capacitance is

C = q / U = 1 / 4.98E9 = 2.01E-10 F/m = 201 pF/m

Note that, with plane-parallel problems, we specify the sources as

specific values per unit depth (e.g., the charge of the strip), and the

result is specific capacitance per unit depth, measured in F/m.

If you'll now look through the model provided with STRIP2.PBM,

STRIP.MOD, you find some hints that allow to get more fine mesh

near the vertices E and F--the points of singularity.

As the Model Editor cannot squeeze the mesh spacing inside the

edge, but only from one end to another, two extra vertices have

been added in central points of edges AB and EF. The first of these

vertices allows the Model Editor to adjust automatically the spacing

around it, since the strip is very close to this point. The second

vertex is aimed to specify manual spacing value for it, which allows

to decrease the total number of nodes in the mesh, without loss of

precision.