=================================================================

Version 1.00                  Piper              January 18, 1993

(C) Copyright 1993, Yves Beaudoin

All Rights Reserved

DOCUMENTATION

=================================================================

TABLE OF CONTENTS

~~~~~~~~~~~~~~~~~

1. OVERVIEW . . . . . . . . . . . . . . . . . . . . . . . . .   2

2. PREPARING A DATA FILE  . . . . . . . . . . . . . . . . . .   3

2.1  Title . . . . . . . . . . . . . . . . . . . . . . .   5

2.2  Plotter-device code . . . . . . . . . . . . . . . .   5

2.3  Output orientation  . . . . . . . . . . . . . . . .   5

2.4  Legend code . . . . . . . . . . . . . . . . . . . .   6

2.5  Grid code . . . . . . . . . . . . . . . . . . . . .   6

2.6  Identification-label code . . . . . . . . . . . . .   7

2.7  Statistics code . . . . . . . . . . . . . . . . . .   7

2.8  Concentration-units code  . . . . . . . . . . . . .   8

2.9  Tolerance on the charge balance error . . . . . . .   8

2.10 Chemical analyses . . . . . . . . . . . . . . . . .   8

2.10.1 Symbol code  . . . . . . . . . . . . . . . .   8

2.10.2 Pen code . . . . . . . . . . . . . . . . . .   9

2.10.3 Identification label . . . . . . . . . . . .   9

3. RUNNING PIPER  . . . . . . . . . . . . . . . . . . . . . .  10

APPENDIX: ERROR MESSAGES  . . . . . . . . . . . . . . . . . .  13

_GetOptionInteger  . . . . . . . . . . . . . . . . . . .  13

_GetOptionReal . . . . . . . . . . . . . . . . . . . . .  13

_GetOptionString . . . . . . . . . . . . . . . . . . . .  13

_GetParamFilename  . . . . . . . . . . . . . . . . . . .  14

_PopModule . . . . . . . . . . . . . . . . . . . . . . .  14

ComputeTernaryStats  . . . . . . . . . . . . . . . . . .  14

DrawLegend . . . . . . . . . . . . . . . . . . . . . . .  14

DisplayOptionHeader  . . . . . . . . . . . . . . . . . .  14

GetTextRecord  . . . . . . . . . . . . . . . . . . . . .  14

HPGL2Output  . . . . . . . . . . . . . . . . . . . . . .  15

HPGL2Pen . . . . . . . . . . . . . . . . . . . . . . . .  15

HPGL2Plot  . . . . . . . . . . . . . . . . . . . . . . .  15

HPGL2Symbol  . . . . . . . . . . . . . . . . . . . . . .  15

ModuleTraceOn  . . . . . . . . . . . . . . . . . . . . .  15

ParseTextRecord  . . . . . . . . . . . . . . . . . . . .  15

Piper  . . . . . . . . . . . . . . . . . . . . . . . . .  15

PrepTextInput  . . . . . . . . . . . . . . . . . . . . .  17

PrepTextOutput . . . . . . . . . . . . . . . . . . . . .  17

SkipTextRecord . . . . . . . . . . . . . . . . . . . . .  18

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

1. OVERVIEW

~~~~~~~~~~~

Piper generates  HP-GL/2 instructions for  plotting chemical

water data on  a Piper diagram.  HP-GL/2 is  the new standardized

version  of  the  Hewlett-Packard   Graphics  Language.    It  is

available, for example, with HP's series of LaserJet III printers

and DraftMaster plotters.

The program  exploits an important feature  of this graphics

language,  namely  that  the  plot  size  can  be  made  strictly

dependent on the output  media size.  The same  plotting commands

can be  directed to an HP LaserJet  III with letter-size paper or

to  an HP  DraftMaster  SX+ with  paper  of any  acceptable  size

mounted.    The resulting  Piper  diagram  will be  automatically

adjusted to  fit up to  the hard-clip  limits of the  media, and,

accordingly, be expanded or contracted.

Piper also  reports the charge-balance errors.   It computes

the results  in epm or epL  units depending on the  units used to

define the input mass concentrations.  Water analyses exceeding a

user-specified error tolerance on the electroneutrality condition

will be excluded from the plot.

The charge-balance  report can be  directed to a  printer or

redirected to a file  for later viewing.  Similarly,  the HP-GL/2

commands  can  be directed  to a  compatible plotter  or printer.

They can also be redirected to a file for importing, for example,

into a print/plot-utility or drawing program.

Each data point can have a symbol and a color/pen associated

with  it.   There  are eight  possible  pen selections  and eight

symbols to chose from.  A point can be further  identified with a

user-supplied label.   The amount of total dissolved solids (TDS)

for  each  data point  is  represented by  the  area of  a circle

centered  on the  point.   The TDS  representation for  any given

point can be suppressed by defining its symbol-code as a negative

value.

The user can specify a title  of up to thirty characters.  A

legend  describing  the  number  of data  points  plotted  with a

specified symbol and  pen code  can be optionally  selected.   If

there is  enough room,  it can  be replaced  or augmented with  a

listing of the  amount of  total dissolved solids  for each  data

point.   The resolution  of the grid system  can be selected from

six possibilities or omitted entirely.

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                               2

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

The  sample  mean  and  standard deviation  of  the  ternary

components of  the diagram can be drawn:  the result is a polygon

whose  central point, marked by  an asterisk, represents the mean

and whose  boundaries are  one component standard  deviation away

from the  mean.   In turn,  these means  are interpolated  to the

"diamond"  area  of the  Piper diagram  and  the result  is again

marked by  an asterisk.   Moreover, the average TDS  value of the

plotted  data is represented by  a circular area  centered on the

interpolated mean.

The total number of data points plotted must not exceed 750.

2. PREPARING A DATA FILE

~~~~~~~~~~~~~~~~~~~~~~~~

An input data file for Program Piper must be a DOS text file

conforming to a specific structure.  There are two major sections

to any data  file.  The first section  consists of the processing

options for  the program.  The second section groups the ion mass

concentrations and their representation codes.

An example is  provided by  TEST.DAT, one of  the many  data

files  used to develop Piper.  If  you examine this file, you can

see  the two major sections.   Moreover, you'll  notice that each

record has  a row  or column  heading associated  with  it.   The

headings help interpret the  subsequent alphanumeric entries and,

therefore, make the data file more understandable.

In order to help  you set up  a data file, the  distribution

package  includes the  file TEMPLATE.DAT.   As  suggested by  its

name, the file is a data-entry template containing all the record

field headings mentioned above.

To illustrate its use, suppose that we want to set up a data

file  to be called MYDATA.  The first step is to use the DOS COPY

command to duplicate the contents of the template file to MYDATA.

After making  the  Piper directory  the  current one,  enter  the

following command:

COPY TEMPLATE.DAT MYDATA /V

where  the  /V switch  instructs DOS  to  verify that  MYDATA was

written correctly.  The next  step is to load MYDATA into  a text

editor,  such as the  MS-DOS EDLIN or  EDIT program,  in order to

enter the plotting  parameters and  your data.   Note in  passing

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                               3

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

that if you use a word processor instead, it should be configured

to emulate a text processor.  The left margin must  correspond to

Column  1 and the  right one to Column  80 or more.   The top and

bottom margins must  be inactive.   Headers and  footers are  not

allowed.   Finally select a  monospace font, such  as Courier, to

ensure that your entries are aligned properly and remain so after

you save them to disk.

If  the aforementioned  operations  have  been  carried  out

successfully,  the  file MYDATA  consists  of  the following  ten

lines:

Title (30 chars. max):

Plotter-device code (0-1):

Output orientation (-90,0,90):

Legend code (0-3):

Grid code (0,5,10,20,40,50,100):

Identification-label code (0-1):

Statistics code (0-1):

Concentration-units code (1-6):

Tolerance on charge balance error:

Ca++  Mg++  Na+  K+  HCO3-  CO3--  Cl-  SO4--  TDS Symbol Pen Id

Each  line  of the  data  file makes  up  a record.   Information

enclosed  between  parentheses   represents  the  constraints  or

admissible values for a particular record or field.

The first nine records make up the first major section.  The

first  record is for the title of the plot.  Records #2 to #7 are

for specifying the plotting options.  The eighth line is used  to

designate  the  concentration  units  of  the chemical  analyses.

Record  #9  allows the  user to  set  an error  tolerance  on the

electroneutrality condition.   Analyses exceeding  this value are

excluded from the  diagram and  are flagged in  the error  report

with the message "<--Rejected" at  the end of the line.   For all

these records, input always begins following the colon.

The  second major section begins with Record #10.  This line

is present  solely for  providing the column  headings describing

the subsequent data entry sequence.  The concentration masses and

their display  codes  are entered  in  a tabular  fashion,  after

Record #10.  Each analysis takes up one  and only one line in the

data file.  The horizontal order  is dictated by the order of the

column headings.  Note  in passing that there are  no constraints

on the vertical  spacing of  the headers and  the chemical  data.

You can expand or  contract the distance between columns  as much

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                               4

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

as you wish so long as the above constraints are respected.

Descriptions and constraints for each record are as follows:

2.1  Title

~~~~~~~~~~

The first  record is for  the title  of the  plot that  will

appear above  the top  apex of the  "diamond" area  of the  Piper

diagram.  There is a limit of thirty characters.

2.2  Plotter-device code

~~~~~~~~~~~~~~~~~~~~~~~~

The second record is for specifying the plotter-device type.

The possible values and their corresponding meanings are:

Code | Interpretation

------+------------------------------------------------

0  | Color device such as a plotter (with a maximum

|  capacity of 8 pens),

1  | Monochrome device such as a laser printer.

The  purpose of  this  option is  as follows.    As explained  in

Section 2.10, the data points on the diagram can be distinguished

by  symbol  and by  color  when using  a  plotter.   If  you take

advantage of this feature when you create your data file but then

send the output to a laser printer, the color distinction will be

lost of course.   With the  plotter-device code set  to 1 on  the

other  hand, Piper will override  all color settings  to "Pen #1"

(which  designates black on a LaserPrinter) and adjust the legend

accordingly by  grouping the  data points  by their symbol  codes

only.  It is faster  to change this one record than to modify all

the color codes for the data points.

2.3  Output orientation

~~~~~~~~~~~~~~~~~~~~~~~

The  third  record is  for assigning  an orientation  to the

plot.  The following table defines the possible rotations:

Code | Interpretation

------+---------------------------------

-90  | Rotate the plot by -90 degrees,

0  | No rotation,

90  | Rotate the plot by +90 degrees.

This  option allows  you  to select  a  portrait or  a  landscape

orientation to the diagram.

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                               5

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

2.4  Legend code

~~~~~~~~~~~~~~~~

The  fourth record is for designating what to include in the

legend or for  suppressing it.  The associated code is defined as

follows:

Code | Interpretation

------+---------------------------------------------------

0  | No legend,

1  | Include symbol frequencies only, i.e., a listing

| of the colored symbols used along with a

| frequency count of the corresponding data points,

2  | Include identification labels & TDS values only,

3  | Include frequencies, ID labels, & TDS values.

The legend  is drawn to  the left  of the Piper  diagram and

consists of the following additional features:

o  the name and version number of the program,

o  the date and time the plot was created,

o  a listing of the pens mounted in the carousel if the

output device code is set to 0.

Finally  a frame  is  drawn which  encloses  the legend  and  the

diagram.

A  request to include the TDS values is conditional on there

being  enough room  left to  draw them  in the  legend's reserved

area.    Otherwise,  Piper  will  signal   a  violation  of  this

constraint and halt execution.

2.5  Grid code

~~~~~~~~~~~~~~

The  next record is  for choosing  a predefined  grid system

applicable  to  both ternary  diagrams  and  the "diamond"  area.

Admissible values and their actions are:

Code | Interpretation

------+-------------------------------------------------

0 | No grid,

5 | Quick reference grid (5 subintervals per edge),

10 | Very coarse grid (10 subintervals per edge),

20 | Coarse grid (20 subintervals per edge),

40 | Medium grid (40 subintervals per edge),

50 | Fine grid (50 subintervals per edge),

100 | Very fine grid (100 subintervals per edge).

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                               6

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

The corresponding annotation is  controlled by Piper.  Regardless

of  the value  of  the  grid  code, there  will  always  be  four

annotation labels per edge  with two exceptions.  The  right edge

of the cations  ternary diagram and the  left edge of  the anions

ternary diagram are not annotated in order to avoid overcrowding

of  the  neighbouring  labels.   However,  the  grid-line  values

involved can be read from the annotation of the "diamond" area.

2.6  Identification-label code

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The  sixth record is for toggling the annotation of the plot

points with  their identification  labels.  Possible  code values

are:

Code | Interpretation

------+--------------------------------

0  | Omit identification labels,

1  | Include identification labels.

2.7  Statistics code

~~~~~~~~~~~~~~~~~~~~

Record  #7 toggles  the  plotting of  the  sample means  and

sample standard deviations of the data:

Code | Interpretation

------+-----------------------------

0  | No statistics,

1  | Plot the sample statistics.

The resulting plot in each ternary diagram is a six-sided polygon

whose central point represents  the sample mean and is  marked by

an asterisk.    Each boundary  is  located one  component  sample

standard deviation away from the mean.   In turn, these means are

interpolated to the "diamond"  area of the Piper diagram  and the

result is again  marked by  an asterisk.   Moreover, the  average

value  of the  total  dissolved solids  of  the plotted  data  is

represented by a circular area centered on the interpolated mean.

A  minimum of  two data  points with  charge  balance errors

below  the  user-specified tolerance  are  required  to define  a

statistical sample.

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                               7

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

2.8  Concentration-units code

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Record  #8 allows  you to  specify which  mass concentration

units the chemical analyses are expressed in:

Code | Interpretation

------+----------------

1  | kg/m3 or g/L,

2  | g/m3 or mg/L,

3  | epL,

4  | mepL,

5  | ppm,

6  | epm.

2.9  Tolerance on the charge balance error

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The  purpose of  the  ninth  record  is  to  set  the  error

tolerance on the electroneutrality condition.   This value can be

entered  as a fixed- or  floating-point real number.   A chemical

analysis  exceeding the tolerance is omitted from the plot and is

flagged,  at the  end of  the corresponding  line in  the charge-

balance report, with the message "<--Rejected" .

2.10 Chemical analyses

~~~~~~~~~~~~~~~~~~~~~~

Record  #10 consists  of  column headings  for the  chemical

analysis  data: it  is provided  solely to  facilitate subsequent

data entry.

Record #11 is  associated with the first data  point.  It is

made  up of eleven numeric fields and one alphanumeric field that

correspond to the twelve headings of Record #10.  The first eight

fields  are for  the major  ion mass  concentrations.   The ninth

field is  for the  TDS value.   The next two  fields are  for the

symbol and pen numeric codes.  The last field is  an alphanumeric

one allowing up to eight characters  for an identification label.

All  field entries are separated from each other by blank spaces:

there can be as many blanks as you wish.

2.10.1 Symbol code

~~~~~~~~~~~~~~~~~~

The  magnitude  of the  code  will result  in  the following

representation of a data point:

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                               8

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

Code | Representation

------+-----------------------------------------

1  | Circle

2  | Square

3  | Triangle

4  | Diamond

5  | Dot, i.e. filled circle

6  | Filled square

7  | Filled triangle

8  | Filled diamond

When the code is entered as a positive  value, the associated TDS

value will be represented by the area of a circle centered on the

data point.  If the code is negative, the TDS representation will

be  suppressed  for  the  point.   Piper  will  draw  a  scale of

diameters to the left of the "diamond" area as a consequence of a

single positive symbol code in the whole data set.

2.10.2 Pen code

~~~~~~~~~~~~~~~

The integer pen code designates the pen stall from which the

pen will be selected to draw a symbol.  For a plotter,  the limit

is normally eight.   Thus, Piper conforms to this  maximum value.

On  a monochrome output device,  only "Pen #1"  is available and,

thus,  all code values yield  the same color.   However, remember

that  Piper will automatically override a pen code greater than 1

if the  plotter-device code has  been set  to 1.   The difference

between  the two scenarios is that the symbol frequencies will be

readjusted in  the  latter  case  only  because  Piper  has  been

forewarned.   Note also that "Pen #1"  is always used to draw the

grid system, the annotation, etc.

2.10.3 Identification label

~~~~~~~~~~~~~~~~~~~~~~~~~~~

The last record field  is for an identification label  of up

to eight alphanumeric characters.   This is the only  field which

may be left empty.

Record #12 and subsequent records are for the remaining data

points, each structured as explained previously.  Note in passing

that  you can delimit groups  of data records  using blank lines:

these extra lines will be ignored by Piper.

Once you have specified all the plotting options and entered

your  data, save  the  information back  to disk  overwriting the

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                               9

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

original file, e.g., MYDATA.  If you are  using a word processor,

insure  that the file is  saved as a DOS text  file.  You are now

ready to process your data file.

3. RUNNING PIPER

~~~~~~~~~~~~~~~~

Piper is launched from the DOS prompt as follows:

PIPER DataSource ReportDestination PlotDestination

where,

DataSource is the specification  for the file containing the

required plot parameters and the water chemical data,

ReportDestination is  either the specification for  the file

or  the name of the  device port to  which the computed

charge-balance   errors   will   be    written,   e.g.,

C:\TEST.CBE for a  file or LPT1 for a printer connected

to the first parallel port.

PlotDestination is either the  specification for the file or

the  name  of  the device  port  to  which  the HP-GL/2

commands will be sent, e.g., A:\EXAMPLE\TEST.PLT for  a

file  or  COM1 for  a  plotter connected  to  the first

serial port.   If required,  use the  DOS MODE  command

beforehand to configure the port.

A  similar  explanation of  the  command-line  parameters can  be

obtained by just entering  PIPER at the DOS prompt.  Moreover, it

will be generated if the number of parameters is incorrect.

The following  names are  reserved for the  operating system

device ports:

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                              10

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

Name | Device

------+-------------------------------

AUX  | Communications port (COM1)

COM1 | Communications port #1

COM2 | Communications port #2

CON  | Keyboard and screen

LPT1 | Line Printer port #1

LPT2 | Line Printer port #2

LPT3 | Line Printer port #3

NUL  | Null device

PRN  | Printer (LPT1)

The connection is always  to the associated device: they  can not

be  used as file names,  regardless of the  directory path and/or

extension  that  is  added.  Consult  your  DOS  manual  for more

information if these port designations are unfamiliar.

As an example  of the usage  of output file  specifications,

the command

PIPER TEST.DAT ..\TEST.CBE B:\TEST.PLT

instructs Piper to  (1) retrieve  the data file  TEST.DAT on  the

current directory of the currently logged-on drive, (2) write the

charge-balance error  report to the  file TEST.CBE on  the parent

directory of the current one, and (3) store the resulting HP-GL/2

commands to the file TEST.PLT on  the root directory of Drive B:.

If TEST.CBE or TEST.PLT exist beforehand, Piper will in each case

request  permission to  overwrite the  file  with the  new output

results.

On  the  other hand,  to process  the  data file  MYDATA, to

suppress   the  error   report,  and   to  direct   the  plotting

instructions  to  a laser  printer  connected  through the  first

parallel port, the following command would normally be issued:

PIPER MYDATA NUL LPT1

If your HP  laser printer  should generate a  "21 PRINT  OVERRUN"

error message, you'll  have to set the  "Page Protection" feature

(see  the printer's  user manual  for more  details).   Once page

composition is finished, you must issue a form-feed command.

As Piper  sequentiallly reads  and processes the  data file,

the  various  options will  be echoed  to  the screen.   Wherever

applicable, short interpretations will also be included.

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                              11

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

Many error  traps  have  been  built into  Piper  to  detect

invalid  entries in the  data file or  problems with disk-related

operations (see  the appendix  for more details).   Corresponding

messages  will be  displayed on  the screen.   An  erroneous data

record  will be  echoed  to the  screen.   Moreover,  Piper  will

continuously update the total number of data records successfully

read.   Thus the location of  an incorrect record can be inferred

readily.  During debugging,  you can direct either output  to NUL

(the black-hole of DOS  device-drivers) to suppress it or  to CON

to  see  the  results.     Although  the  display  will   not  be

aesthetically  pleasing in the later  case, it can  assist you in

isolating the problem.

If everything goes smoothly,  you'll see the message "Done!"

appear as the last line on the screen before the DOS prompt.

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                              12

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

APPENDIX: ERROR MESSAGES

~~~~~~~~~~~~~~~~~~~~~~~~

When  a  fatal execution  error  occurs, Piper  issues  a message

describing the  error from its perspective.   Input/output errors

are also  accompanied  by a  second  message that  describes  the

problem from the operating system's point of view.

This  section details  the  various primary  error messages  that

Piper  can issue.  Each error description  is made up of 3 parts:

the name of the  program module that reports the error, the error

message, and an explanation of the probable causes.

_GetOptionInteger:

Cannot read integer in "device" record!

Cannot read integer in "grid" record!

Cannot read integer in "identification" record!

Cannot read integer in "legend" record!

Cannot read integer in "orientation" record!

Cannot read integer in "statistics" record!

Cannot read integer in "units" record!

Piper expected  to encounter  an integer number  in the

indicated  record.    Check  for  the  presence  of  an

extraneous decimal point or alphanumeric character.

Integer value in "device" record is too big!

Integer value in "grid" record is too big!

Integer value in "identification" record is too big!

Integer value in "legend" record is too big!

Integer value in "orientation" record is too big!

Integer value in "statistics" record is too big!

Integer value in "units" record is too big!

Integer  numbers  must  be  between  -32768  and  32767

inclusively.  Check the  preceding pages for admissible

values.

_GetOptionReal:

Cannot read real in "tolerance" record!

Piper  expected  to  encounter  a real  number  in  the

indicated  record.    Check  for  the  presence  of  an

extraneous alphanumeric character.

_GetOptionString:

Data string in "title" record is too long!

The length of the plot title exceeds thirty characters.

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                              13

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

_GetParamFilename:

Filename/device port  is missing from command line parameter

#n!

The   n-th   command-line  specification   is  probably

incomplete as in C:\PIPER\DATA\.

_PopModule:

The error-trace stack is empty!

This  error message  is intended  for the  author only.

Please report the problem.

ComputeTernaryStats:

Insufficient number of plot points for statistics!

The computation  of the  sample  statistics requires  a

minimum   of   two  data   points   that   satisfy  the

electroneutrality criterion.

DrawLegend:

Not enough room to generate the requested legend!

There  are too many IDs  and TDS values  to list within

the designated area of the plot.  Consider omitting the

symbol-frequency table.   If the problem persists,  the

legend  itself can be suppressed or the data set can be

partitioned.

DisplayOptionHeader:

Next header is too long!

This  error message  is intended  for the  author only.

Please report the problem.

GetTextRecord:

Expected to read the "device" record next!

Expected to read the "grid" record next!

Expected to read the "identification" record next!

Expected to read the "legend" record next!

Expected to read the "orientation" record next!

Expected to read the "statistics" record next!

Expected to read the "title" record next!

Expected to read the "tolerance" record next!

Expected to read the "units" record next!

The next record in  the data file does not  contain the

keyword that Piper is looking for.  The record  heading

is  either  missing or  corrupted.   Alternatively, the

order  of the  data records  might not  conform to  the

format of  the supplied  template file.   Check against

the latter for any discrepancies.

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                              14

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

HPGL2Output:

Unable to write the plotting instructions!

This  error  message  is  accompanied  by  a  secondary

message  generated  by  the  DOS  error  handler  which

indicates the probable cause.  The most likely cause is

insufficient disk space.

HPGL2Pen:

Invalid pen-stroke width!

This  error message  is intended  for the  author only.

Please report the problem.

HPGL2Plot:

Invalid pen mode!

This  error message  is intended  for the  author only.

Please report the problem.

HPGL2Symbol:

Invalid symbol code!

This  error message  is intended  for the  author only.

Please report the problem.

ModuleTraceOn:

Out of memory for the error-trace stack!

To  recover more  memory, close  down any  suspended or

background tasks.   Also consider  removing any memory-

hungry TSRs.

ParseTextRecord:

Next text record has an invalid field delimiter!

A  field header in the input file is separated from the

field data by a  colon at column position 35.  So check

for its presence at the proper location.

Unable to read the next record of the text file!

This  error  message  is  accompanied  by  a  secondary

message  generated  by  the  DOS  error  handler  which

indicates the probable cause.

Piper:

Data record #nn: A negative ion concentration was detected!

A  minus  sign  was  encountered  in  front of  a  mass

concentration value in the nn-th record.  Check Piper's

display to locate the erroneous record.

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                              15

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

Data record #nn: A non-positive TDS value was detected!

A  total-dissolved-solids  count  has  been  entered as

negative or zero  in the nn-th  record.  Check  Piper's

display to locate the erroneous record.

Data record #nn: Invalid pen code detected!

Data record #nn: Invalid symbol code detected!

The  integer code value for  a pen or  symbol option is

not  in its correct range  in the nn-th  record.  Check

the preceding pages for admissible values.

Data record #nn: The anion concentrations are all zero!

Data record #nn: The cation concentrations are all zero!

The anion  or the cation mass  concentrations have been

found  to be  all  zero in  the  nn-th record.    Check

Piper's display to locate the erroneous record.

Data record #nn: The Id label is too long!

An identification label can have up to eight characters

inclusively.    Check  Piper's  display  to locate  the

erroneous nn-th record.

Data source and report destination must differ!

Data source and plot destination must differ!

Report and plot destinations must differ!

The  command-line parameters  must point  to different,

unique files or device ports.  For  example, the device

ports 'PRN'  and 'LPT1' are equivalent.   Restart Piper

with non-equivalent specifications.

Invalid graphics-device code!

Invalid grid code!

Invalid identification-label code!

Invalid legend code!

Invalid mass concentration units code!

Invalid plot orientation!

Invalid statistics code!

The integer code value for a particular plotting option

is not in its correct range.  Check the preceding pages

for admissible values.

Maximum number of plot points exceeded!

There  are more than 750  points to be  plotted!  Split

the data file into smaller subsets.

There are no points to plot!

Either  (1) the data file is empty or corrupted, or (2)

all  the  charge-balance  errors  exceed  the specified

tolerance.  Check  if the number  of data records  read

matches the number of records rejected.  If the numbers

differ, then verify the  integrity and the  correctness

of your data file.

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                              16

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

Unable to close the files and/or devices!

Unable to read the next record of the input file!

Unable to write to the report destination!

Check the DOS error message for the probable cause.

PrepTextInput:

A DOS device cannot be used for input!

The DOS device  names are AUX, COM1,  COM2, LPT1, LPT2,

LPT3, CON,  NUL, and PRN.   None may be used  as a file

name nor be part of a file specification as in AUX.DAT.

Input file name matches a directory name!

The input file specification is probably incomplete  as

in  C:\PIPER\DATA  where  DATA  is  a  subdirectory  of

\PIPER.

Input file name matches the volume ID!

Check the spelling of the file name.

The input file is a hidden file!

The input file is a system file!

Hidden files are normally  system files, such as IO.SYS

and MSDOS.SYS, and are not meant to be read by a user's

application.  So consider  (a) checking the spelling of

the  file name,  or  (b) changing  the  attribute of  a

genuine input file to un-hide it.

The input file is empty!

Check the  spelling of the file name first.  Otherwise,

read your text processor's  instructions on how to save

a DOS text file.

Unable to locate the input file!

Check  the spelling of the file name.  In addition, the

associated   or   implied  directory   path   might  be

incorrect.

Unable to open the input file!

Check the DOS error message for the probable cause.

PrepTextOutput:

Output file name matches a directory name!

The output file specification is probably incomplete as

in C:\PIPER\PLOTS  where  PLOTS is  a  subdirectory  of

\PIPER.

Output file name matches the volume ID!

Check  the spelling of the file name or use a different

file name.

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                              17

Piper                     Documentation              Version 1.00

-----------------------------------------------------------------

The output file is a hidden file!

The output file is a system file!

Hidden files are normally  system files, such as IO.SYS

and  MSDOS.SYS, and  system  files  are definitely  not

meant to be overwritten by an application.  So consider

(a) checking  the spelling  of  the file  name, or  (b)

using a different name.

The output file is a read-only file!

A  file with the same  name already exists  and has its

read-only attribute bit set.  Consider (a) changing the

file attribute  back to  read-and-write if you  want to

overwrite the  contents, or (b)  specifying a different

file.

Unable to create the output file!

Unable to open the device port for writing!

Check the DOS error message for the probable cause.

SkipTextRecord:

Unable to skip the next record of the text file!

Check the DOS error message for the probable cause.

-----------------------------------------------------------------

(C) Copyright 1993, Yves Beaudoin                              18