These utilities provide the basic functions to allow the remote operation

of a FLUKE/PHILIPS Model 97 ScopeMeter. The three main utilities are:

PS  Program Setup: sends an earlier saved setup file to the 97.

QS  Query Setup: save the setup data to a file.

QW  Query Waveform: saves captured data to a file (ASCII).

All programs operate through comm ports 1 or 2 at 1200 baud (97 default)

Entering the command without any qualifiers will result in the following

help information being displayed.

**************************************************

* ScopeMeter Utilities - V1./0 12/5/92 (NBS)     *

* Format:                                        *

*       PS [/n] srcname <Enter>                  *

* where n=1 for COM1 and n=2 for COM2            *

* (default is COM1)                              *

* Programs ScopeMeter setup data from file       *

* 'srcname.sms'                                  *

**************************************************

**************************************************

* ScopeMeter Utilities - V1./0 12/5/92 (NBS)     *

* Format:                                        *

*       QS [/n] destname <Enter>                 *

* where n=1 for COM1 and n=2 for COM2            *

* (default is COM1)                              *

* Saves ScopeMeter setup data to file            *

* 'destname.sms'                                 *

**************************************************

**************************************************

* ScopeMeter Utilities - V1./0 12/5/92 (NBS)     *

* Format:                                        *

*       QW [/n] [/sss] destname <Enter>          *

* where n=1 for COM1 and n=2 for COM2            *

* (default is COM1)                              *

* where sss is one of the following              *

*    101 = ChA                                   *

*    102 = ChB                                   *

*    103 = A+-B                                  *

*    104 = Temp1                                 *

*    105 = Temp2                                 *

*    106 = Temp3                                 *

*    107 = Mem1                                  *

*    108 = Mem2                                  *

*    109 = Mem3                                  *

*    110 = Mem4                                  *

*    111 = Mem5                                  *

* (default is channel A = 101)                   *

* Saves ScopeMeter waveform data to file         *

* 'destname.smw'                                 *

**************************************************

There are a number of other commands available and I may implement them in

a similar fashion so that the ScopeMeter may be operated from a batch file.

These include setting comms parameters, other setup options, reading the

DVM functions, arming the trigger, programming the waveform etc.

All normal disclaimers apply regarding responsibilty for any outcome

resulting from the use of these utilities.

There is no charge but I would appreciate feedback, and of course donations

of money, favourite programs, recipes etc. would never be refused!

I would like to consider writing a graphical interface for the 97 at some

stage so would particularly like to get feedback on that. I haven't seen

the commercial (US$500?) package yet but at that price I'm sure I won't be

buying it!

Regards

Bruce Spedding

IDW		(soon to be Industrial Research Ltd...will I be there?)

DSIR

Box 31310	Ph. +64+4+569-0788	Fx: +64+4+569-0132

Lower Hutt	Email: SRGXNBS@GFV.DSIR.GOVT.NZ

New Zealand