- 1 -

ZMODEM PROTOCOL REFERENCE

The

ZMODEM

Inter Application

File Transfer Protocol

Chuck Forsberg

Please distribute as widely as possible.

Questions to Chuck Forsberg

Omen Technology Inc

17505-V Northwest Sauvie Island Road

Portland Oregon 97231

Voice: 503-621-3406

Modem (Telegodzilla): 503-621-3746 Speed 1200,300

Compuserve: 70715,131

UUCP: ...!tektronix!reed!omen!caf

- 2 -

1.  ACKNOWLEDGMENTS

Encouragement and suggestions by Stuart Mathison, Thomas

Buck, John Wales, Ward Christensen, and Irv Hoff are

gratefully acknowledged.

2.  ROSETTA STONE

Here are some definitions which reflect the current

vernacular in the computer media.  The attempt here is

identify the file transfer protocol rather than specific

programs.

Frame   A ZMODEM frame consists of a header packet and 0 or

more data packets.

Response Time This is the maximum expected delay between the

receiver sending a packet to the transmitter and

receiving the beginning of a response from the

transmitter.

XMODEM  refers to the original 1979 file transfer etiquette

introduced by Ward Christensen's 1979 MODEM2

program.  It's also called the MODEM or MODEM2

protocol.  Some who are unaware of MODEM7's unusual

batch file mode call it MODEM7.  Other aliases

include "CP/M Users's Group" and "TERM II FTP 3".

This protocol is supported by every serious

communications program because of its universality,

simplicity, and reasonable performance.

XMODEM/CRC replaces XMODEM's 1 byte checksum with a two byte

Cyclical Redundancy Check (CRC-16), giving modern

error detection protection.

YMODEM  refers to the XMODEM/CRC protocol with the

throughput and/or batch transmission enhancements

described in YMODEM.DOC.

ZMODEM  Zmodem is a second generation streaming protocol for

text and binary file transmission between

applications running on microcomputers and

mainframes.

Chapter 3              DRAFT 3-24-86                       2

DRAFT ZMODEM Protocol Ref 02-23-86                         3

3.  YET ANOTHER PROTOCOL, AGAIN?

Since its development half a decade ago, the Ward

Christensen modem protocol has enabled a wide variety of

computer systems to interchange data.  There is hardly a

communications program that doesn't at least claim to

support this protocol now called XMODEM.

Advances in computing, modems and networking have spread the

XMODEM protocol far beyond the close coupled micro to micro

environment for which it was designed.  These application

have exposed some weaknesses.

o#+ The short block length causes throughput to suffer when

used with timesharing systems, packet switched

networks, satellite circuits, and buffered (error

correcting) modems.

o#+ The 8 bit arithmetic checksum and other aspects allows

line impairments to interfere with dependable, accurate

transfers.

o#+ Only one file can be sent per command.  The file name

has to be given twice, first to the sending program and

then again to the receiving program.

o#+ The transmitted file accumulates as many as 127

extraneous bytes.

o#+ The modification date of the file is lost.

A number of other protocols have been developed over the

years, but none have displaced XMODEM to date.

o#+ Lack of public domain documentation and example

programs have kept proprietary protocols such as MNP,

Blast, and others tightly bound to the fortunes of

their suppliers.

o#+ Hardware and/or software complexity discourages the

widespread application of BISYNC, SDLC, HDLC, X.25, and

X.PC protocols.

o#+ Link level protocols such as X.25, X.PC, and MNP modems

do not manage application to application file

transfers.

o#+ The Kermit protocol was developed to allow file

transfers in environments hostile to XMODEM.  The

performance compromises necessary to accomodate non

transparent environments limit Kermit's efficiency.

Chapter 3              DRAFT 3-24-86                       3

DRAFT ZMODEM Protocol Ref 02-23-86                         4

Even with completely transparent channels, Kermit

control character quoting limits the efficiency of

binary file transfers to about 75 per cent.

Kermit Sliding Windows ("SuperKermit") improves

throughput over networks at the cost of increased

complexity.  SuperKermit state transitions are encoded

in a special language "wart" which requires a C

compiler.  The SuperKermit C code requires full duplex

communications and the ability to check for the

presence of characters in the input queue, precluding

its implementation on some operating systems.

A number of extensions to the XMODEM protocol have been made

under the collective name YMODEM.

o#+ YMODEM-k reduces the overhead from transmission delays by

87 per cent compared to XMODEM, but network delays can

still degrade performance.

o#+ The handling of files that are not a multiple of 1024 or

128 bytes is awkward, especially if the file length is

not known, or changes during transmission.

o#+ YMODEM-g is essentially insensitive to network delays.

Because it does not support error recovery, YMODEM-g must

be used hardwired or with a link level protocol.

Another XMODEM "extension" is protocol cheating, referred to

as "Turbo Download" and OverThruster.  These improve XMODEM

throughput at the expense of error recovery.

The ZMODEM Protocol is proposed as a means of addressing the

weaknesses described above while maintaining as much of

XMODEM's simplicity and prior art as possible.

4.  ZMODEM Protocol Design Criteria

The design of a file transfer protocol is an engineering

compromise between conflicting requirements:

4.1  Throughput

ZMODEM is designed for optimum performance with minimum

degradation caused by delays introduced by packet switched

networks and timesharing systems.

ZMODEM is optimized for best throughput where line hits

occur infrequently.  This assumption markedly reduces code

Chapter 4              DRAFT 3-24-86                       4

DRAFT ZMODEM Protocol Ref 02-23-86                         5

complexity and memory requirements.

It is assumed that many transfers will originate from a

timesharing system connected to a packet switched network.

ZMODEM provides features to allow for simple, efficient

implementation on timesharing hosts.

File transfers begin immediately regardless of which program

is started first, no 10 second delay.

4.2  Integrity and Robustness

All packets are protected with 16 bit CRC.

4.3  Ease of use

File names need be entered only once.  Wild Card names may

be used with batch transfers.  Minimum keystrokes required

to initiate transfers.  ZMODEM steps down to X/YMODEM if the

other end does not support ZMODEM.

4.4  Ease of implementation

ZMODEM accomodates a wide variety of systems:

o#+ Microcomputers that cannot overlap disk and serial i/o

o#+ Microcomputers that cannot overlap serial send and

receive

o#+ Computers and/or networks requiring XON/XOFF flow control

o#+ Computers that cannot check the serial input queue for

the presence of data without having to wait for the data

to arrive.

Although ZMODEM provides "hooks" for multiple "threads",

ZMODEM is not intended to replace link level protocols such

as X.25.  ZMODEM provides a near optimal general purpose

application to application file transfer protocol to be used

with link level protocols such as X.25, MNP, Fastlink, etc.

5.  PACKETIZATION

ZMODEM uses packets somewhat different from those used in

X/YMODEM.  X/YMODEM type packets are not used for the

following reasons:

o#+ Block numbers are limited to 256

Chapter 5              DRAFT 3-24-86                       5

DRAFT ZMODEM Protocol Ref 02-23-86                         6

o#+ No provision for variable length blocks

o#+ Line hits corrupt protocol signals, causing failed file

transfers.  In particular, modem errors sometimes

generate false block numbers, false EOTs and false ACKs.

False ACKs are the most troublesome as they cause the

sender to lose synchronization with the receiver.

State of the art X/YMODEM programs such as Professional-

YAM overcome some of these weaknesses with clever

proprietary code, but a newer protocol is still useful.

o#+ It is difficult to determine the beginning and ends of

X/YMODEM blocks when they are corrupted by line hits.

This discourages rapid error recovery.

5.1  Link Escape Encoding

ZMODEM acheives data transparency by extending the 8 bit

character set (256 codes) with escape sequences based on the

ZMODEM data link escape character ZDLE.1

Link Escape coding permits variable length data packets.  It

allows the beginning of packets to be detected without

special timing techniques, facilitating rapid error

recovery.

Link Escape coding does add some overhead.  The worst case,

a file consisting entirely of ZDLE characters, would incur a

50% overhead.  The particular ZDLE character was chosen

after examining the character distributions of many types of

files used with personal computers.

The ZDLE character is special.  ZDLE represents a control

sequence of some sort.  If a ZDLE character appears in the

data sent within a binary packet, it is prefixed with

another ZDLE.  An escaped ZDLE is counted once in the CRC.

The current value for ZDLE is exclamation point (!).  Use of

a printing character as ZDLE allows application programs to

recognize HEX Header Packets.  This particular character was

chosen because it does not appear often in many types of

files likely to be transferred with this protocol.  In

__________

1. This and other constants are defined in the _#z_#m_#o_#d_#e_#m._#h

include file.  Please note that constants with a leading

0 are octal constants in C.

Chapter 5              DRAFT 3-24-86                       6

DRAFT ZMODEM Protocol Ref 02-23-86                         7

addition, no known timesharing system uses it for editing.

5.2  Header Packet Information

All ZMODEM frames begin with a header packet which may be

sent in binary or HEX form.  ZMODEM uses a single routine to

recognize binary and hex header packets.  Either form of the

header packet contains the same raw information:

o#+ A type byte 2 Future extensions to ZMODEM may use the

high order bits of the type byte to indicate thread

selection.

o#+ Four bytes of data indicating flags and/or numeric

quantities depending on the packet type

Order of Bytes in Header Packet

T:  packet Type

F0: Flags least significant byte

P0: file Position least significant

P3: file Position most significant

T  F3 F2 F1 F0

--------------

T  P0 P1 P2 P3

5.3  Binary Header Packet

A binary header packet is only sent by the sending program

to the receiving program.

A binary header packet begins with the sequence ZPAD, ZDLE,

ZBIN.

The frame type byte is ZDLE encoded.

The four position/flags bytes are ZDLE encoded.

A two byte CRC of the frame type and position/flag bytes is

ZDLE encoded.

0 or more binary data packets will follow depending on the

__________

2. The packet types are cardinal numbers beginning with 0

to minimize state transition table memory requirements.

Chapter 5              DRAFT 3-24-86                       7

DRAFT ZMODEM Protocol Ref 02-23-86                         8

frame type.

The function _#z_#s_#b_#h_#d_#r transmits a binary header packet.  The

function _#z_#g_#e_#t_#h_#d_#r receives a binary or hex header packet.

5.4  HEX Header Packet

The receiver sends responses in hex header packets.

Hex encoding is required to support XON/XOFF flow control.

Use of Kermit style encoding for control and paritied

characters was considered and rejected because of increased

possibility of interacting with some timesharing systems's

line edit functions.  Use of HEX packets from the receiving

program allows control characters to be used to interrupt

the sender when errors are detected.  Except for header

packet types that imply data packets to follow, a HEX header

packet may be used in place of a binary header packet.

A hex header packet begins with the sequence ZPAD, ZPAD,

ZDLE, ZHEX.  The _#z_#g_#e_#t_#h_#d_#r routine synchronizes in the ZPAD-

ZDELE sequence.  The extra ZPAD allows other parts of the

program to detect a ZMODEM packet and then call _#z_#g_#e_#t_#h_#d_#r to

receive the packet.

The type byte, the four position/flag bytes, and the CRC

thereof are sent in hex using the character set

01234567890abcdef.  Upper case hex digits are not allowed;

they would false trigger X/YMODEM programs.

A carriage return, line feed, and XON are appended to the

HEX header packet but are not considered to be part of it.

The CR/LF aids debugging from printouts.  The XON releases

the sender from spurious XOFF flow control characters

generated by line noise, a common occurrence.

0 or more HEX data packets will follow depending on the

frame type.

The function _#z_#s_#h_#h_#d_#r sends a hex header packet.

5.5  Binary Data Packets

Binary data packets immediately follow the associated binary

header packet.  A binary data packet contains 0 to 1024

bytes of data.  Recommended length values are 256 bytes

below 4800 bps, 1024 above 4800 bps or when the data link is

known to be relatively error free.

No padding is used with binary data packets.  The data bytes

Chapter 5              DRAFT 3-24-86                       8

DRAFT ZMODEM Protocol Ref 02-23-86                         9

are ZDLE encoded and transmitted.  A ZDLE and frameend are

then sent, followed by two ZDLE encoded CRC bytes.  The CRC

accumulates the data bytes and frameend.

The function _#z_#s_#b_#d_#a_#t_#a sends a binary data packet.  The

function _#z_#r_#b_#d_#a_#t_#a receives a binary data packet.

5.6  HEX Data Packet

The format of HEX data packets is not currently specified.

These would be used for server commands, etc.

6.  PROTOCOL TRANSACTION OVERVIEW

As with the XMODEM recommendation, ZMODEM timing is receiver

driven.  The transmitter should not time out at all, except

to abort the program if no packets are received for an

extended period of time, say one minute.

To start a ZMODEM file transfer session, the sending program

is called with the names of the desired file(s).

The sending program sends the string "rz\r" and a single HEX

ZRQRINIT packet with data = 0.  The "rz" followed by

carriage return activates a ZMODEM receive program or

command if it were not already active.  If the receiving

program receives the ZRQRINIT packet, it totally ignores it

as the sending program will be responding to the RINIT

packet sent by the receiver.  The sending program should

also ignore this packet type, which would be an echo of its

own packet.

Since the ZRQRINIT sequence contains no exotic control

characters, it can be accepted by the application program as

a command to begin ZMODEM receive.  The sequence prints as

"rz^M**!B00000000000000^M^J" where ^M represents CR and ^J

represents LF.

The sending program awaits a command from the receiving

program to start file transfers.  If a "C" or NAK is

received, an XMODEM or YMODEM file transfer is indicated,

and file transfer(s) use the X/YMODEM protocol.  Note: With

ZMODEM and YMODEM Batch, the sending program provides the

file name, but not with XMODEM.

When the ZMODEM receive program starts, it immediately sends

a ZRINIT packet to initiate ZMODEM file transfers.  The

receive program resends the ZRINIT packet at _#r_#e_#p_#s_#o_#n_#s_#e _#t_#i_#m_#e

intervals for a suitable period of time (40 seconds typical)

Chapter 6              DRAFT 3-24-86                       9

DRAFT ZMODEM Protocol Ref 02-23-86                        10

before falling back to X/YMODEM protocol.  If the receiving

program receives a ZRINIT packet, it is an echo indicating

that the sending program is not operational.

If the receiving program receives a ZRQRINIT packet, it

ignores it.

Eventually the sending program correctly receives the ZRINIT

packet.

The sender may respond with an optional

ZCRYPT packet, which the receiver

acknowledges with a suitable frame.  (Details

to be worked out later)

The sender may respond with an optional ZSINIT frame to set

the receiving program's Attention string.  The receiver

sends a ZACK packet in response.

The sender then sends a ZFILE packet containing the file

name, file length, modification date, and other information

identical to that used by YMODEM Batch.

The receiver may respond to this with a

ZGETCRC packet, which requires the sender to

permorm a CRC on the file and transmit same

with a ZCRC packet.  The receiver may use

this information to determine whether to

accept the file or skip it.

The receiver may respond with a ZSKIP packet, which causes

the file to be passed over.

A ZRPOS packet from the receiver initiates transmission of

the file data starting at the offset in the file specified

in the ZRPOS packet.  Normally the receiver specifies the

data transfer begin begin at offset 0 in the file.

The receiver may start the transfer further

down in the file.  This allows a file

transfer interrupted by a loss or carrier

or system crash to be completed on the next

connection without requiring the entire

file to be retransmitted.  If downloading a

file from a timesharing system that becomes

sluggish, the transfer can be interrupted

and resumed later with no loss of data.

The sender sends a ZDATA header packet (with file

position) followed by one or more data packets.

The receiver compares the file position in the ZDATA

Chapter 6              DRAFT 3-24-86                      10

DRAFT ZMODEM Protocol Ref 02-23-86                        11

header with the number of characters successfully

received to the file.  If they do not agree, a ZRPOS

error response is generated to force the sender to the

right position within the file.

A data packet terminated by ZCRCGO and CRC does not

elicit a response unless an error is detected; more data

packet(s) follow immediately.

ZCRCQ data packets expect a ZACK response

(with the file offset) if no error,

otherwise a ZRPOS response (with the last

good file offset).  Another data packet

continues immediately.  ZCRCQ packets are

not used if the receiver does not indicate

FDX ability with the CANFDX bit.

ZCRCW data packets expect a response before the next

frame is sent.  If the receiver does not indicate

overlapped I/O capability with the CANOVIO bit, the

sender uses the ZCRCW to allow the receiver to write its

buffer before sending more data.

A zero length data packet may also be

used as a sending idle packet to prevent

the receiver from timing out in case data

is not immediately available to the

sender.

In the absence of fatal error, the sender encounters

end of file.  If the end of file is encountered within

a frame, the frame is closed with a ZCRCE data packet

which does not elicit a response except in case of

error.

The sender sends a ZEOF packet with the file ending

offset equal to the number of characters in the file.

The receiver compares this number with the number of

characters received.  If the receiver has received all

of the file, it closes the file and responds with

ZRINIT.  Otherwise the receiver sends ZRPOS with the

current file offset, forcing the sender to send the

missing data.

After all files are processed, any further protocol

errors should not prevent the sending program from

returning with a success status.

The sender closes the session with a ZEXIT header

packet.  The receiver acknowledges this with its own

ZEXIT packet.

Chapter 6              DRAFT 3-24-86                      11

DRAFT ZMODEM Protocol Ref 02-23-86                        12

When the sender receives the acknowledging packet, it

sends two characters, "OO" (Over and Out) and exits to

the operating system or application that invoked it.

The receiver waits briefly for the "O" characters, then

exits whether they were received or not.

7.  STREAMING TECHNIQUE

ZMODEM allows a choice of data streaming methodmethods

selected according to the limitations of the sending

program operating environment, receiving program

operating environment, and the transmission channel(s).

If the computers can overlap serial I/O with disk I/O,

the sender begins data transmission with a ZDATA header

and continuous ZCRCG data packets.  When the receiver

detects an error, it sends the Attn sequence and a

ZRPOS packet to force the sender back to the correct

position within the file.

At the end of each transmitted packet, the sender

checks for the presence of a error packet from the

receiver.  To do this, the sender may sample the

reverse data stream for the presence of a ZPAD

character.

If the reverse data stream cannot be sampled without

entering an I/O wait, the receiver can interrupt the

sender with a control character, break, or combinations

thereof, as specified in the ZSINIT frame sent by the

sending program.

If the receiver cannot overlap serial and disk I/O, it

uses the ZRINIT frame to specify a buffer length which

the sender will not overfill before sending a ZCRCW

packet.

8.  ATTENTION SEQUENCE

The receiving program sends the Attn sequence whenever

it detects an error and needs to interrupt the sending

program.

The default Attn string value is empty (no Attn

sequence).  The characters in the Attn string are

terminated with a null.  Two characters perform special

functions:

Chapter 8              DRAFT 3-24-86                      12

DRAFT ZMODEM Protocol Ref 02-23-86                        13

o#+ \335 Sends a break signal

o#+ \336 Pauses one second

9.  PACKET/FRAME TYPES

The numeric values for the values shown in boldface are

given in _#z_#m_#o_#d_#e_#m._#h.

9.1  ZRQRINIT

Sent by the sending program to call up the receiving

program.  P0...P3 are zero.

9.2  ZRINIT

Sent by the receiving program.  ZF0 and ZF1 contain

receiver capability flags:

#define CANFDX  1 /* Rx can send and receive FDX */

#define CANOVIO 2 /* Rx can receive during disk I/O */

#define CANBRK  4 /* Rx can send a break signal */

#define CANCRY  8 /* Receiver can decrypt */

ZP0 and ZP1 contain the size of the receiver's buffer

in bytes, or 0 if overlapped I/O is allowed.

9.3  ZSINIT

Sender sends capability flags (none currently defined)

followed by a binary data packet terminated with ZCRCW.

Data contains the Attn string, maximum length 32 bytes.

The ZSINIT is only sent to programs that indicate the

ability to overlap serial data and disk I/O (CANOVIO).

9.4  ZACK

Acknowedgement to ZSINIT header packet or ZCRCW data

packet.  ZP0 to ZP3 contain file offset.

9.5  ZFILE

This packet indicates the beginning of a file

transmission attempt.  ZF0 and ZF1 contain special file

processing flags:

o#+ ZBIN This is a binary file

A ZCRCW data packet follows with file name, file

length, modification date, and other information

described in a later chapter.

Chapter 9              DRAFT 3-24-86                      13

DRAFT ZMODEM Protocol Ref 02-23-86                        14

9.6  ZSKIP

Sent by the receiver in response to ZFILE, makes the

sender skip to the next file.

9.7  ZNAK

Indicates last packet header was garbled.  (See also

ZRPOS).

9.8  ZABORT

Sent by receiver to terminate batch file transfers when

requested by the user.  Sender initiates a ZFIN

sequence.1

9.9  ZFIN

Sent by sending program to terminate ZMODEM.  Receiver

responds with ZFIN.

9.10  ZRPOS

Sent by receiver to force file transfer to resume at

file offset given in ZP0...ZP3.

9.11  ZDATA

ZP0...ZP3 contain file offset.  One or more data

packets follow.

9.12  ZEOF

ZP0...ZP3 contain file offset.  Sender reports End of

File.

9.13  ZFERR

Error in reading or writing file, equivalent to ZABORT.

__________

1. Or ZCOMPL in case of server mode.

Chapter 9              DRAFT 3-24-86                      14

DRAFT ZMODEM Protocol Ref 02-23-86                        15

9.14  ZCRC

Request (receiver) and response (sender) for file CRC.

ZP0 and ZP1 contain 16 bit file CRC.

9.15  ZCRYPT

Negotiation for encryption.

9.16  ZCOMPL

Server request now completed.

10.  Transaction

A simple transaction, one file, no errors, overlapped

I/O:

Sender          Receiver

ZRINIT

ZFILE

ZRPOS

ZDATA data ...

ZEOF

ZRINIT

ZFIN

ZFIN

OO

11.  PERFORMANCE

Some tests of ZMODEM protocol performance have been

made.  A PC-AT with SCO SYS V Xenix or DOS 3.1 was

connected to a PC with DOS 2.1 either directly at 9600

bps or with dial-up 1200 bps modems.  The ZMODEM

software was configured to use 1024 byte packet lengths

above 2400 bps, 256 otherwise.

Because no time delays are necessary in normal file

transfers, per file negotiations are much faster than

Chapter 11             DRAFT 3-24-86                      15

DRAFT ZMODEM Protocol Ref 02-23-86                        16

with YMODEM, the only observed impidiment being the

time required by the program(s) to update logging

files.

During a file transfer, a short line hit seen by the

receiver usually induces a CRC error.  The interrupt

packet is usually seen by the sender before the next

packet is sent, and the resultant loss of data

throughput averages about half a packet.  At 1200 bps

this is would be about .75 second lost per hit.  At

10-5 error rate, this would degrade throughput by about

9 per cent.  The throughput degradation increases with

the channel delay, as the packets in transit through

the channel are discarded on error.

A longer noise burst that affects both the receiver and

the sender's reception of the interrupt packet usually

causes the sender to remain silent until the receiver

times out in 10 seconds.  If the round trip channel

delay exceeds the receiver's 10 second timeout,

recovery from this type of error may become difficult.

Noise affecting only the sender is usually ignored,

with one common exception.  Spurious XOFF characters

generated by noise stop the sender until the receiver

times out and sends an interrupt packet which concludes

with an XON.

In summation, ZMODEM performance in the presence of

errors resembles that of X.PC and SuperKermit.  Short

bursts cause minimuml data loss.  Long bursts (such as

pulse dialing noises) often require a timeout error to

restore the flow of data.

12.  TABLES

Figure 1.  Protocol Overhead Information

Figures are calculated for round trip delay times of 40

milliseconds and 5 seconds.  A 102400 byte randomly

distributed binary file is sent at 1200 bps 8 data

bits, 1 stop bit.  The calculations assume no

transmission errors.  For each of the protocols, only

the per file functions are considered.  Processor and

I/O overhead are not included.  YM-k refers to YMODEM

with 1024 byte packets.  YM-g refers to the YMODEM "g"

option.  ZMODEM uses 256 byte packets for this example.

SuperKermit uses amximum packet size, 8 bit transparent

Chapter 12             DRAFT 3-24-86                      16

DRAFT ZMODEM Protocol Ref 02-23-86                        17

transmission, no run length compression.

Protocol     dump   XMODEM   YM-k    YM-G   ZMODEM   S-Kermit

Round-Trips   -      803      103     5      5        5?

Time@40ms     -      32s      4s      -      -        -

Time@5s       -      4015s    515s    25s    25s      25

Chars-Ovhd    -      4803     603     503    2000     7460

Time@0s       853s   893s     858s    857s   870s     1135s

Time@40ms     853s   925s     862s    857s   870s     1135s

Time@5s       853s   5761s    1373s   882s   905s     1160s

Figure 2.  Transmission Time Comparision

(5 second delay)

**********************************************************

XMODEM

************** YMODEM-K

************ Kermit Sliding Windows

********* YMODEM-G

********* ZMODEM

Figure 3.  Y/ZMODEM Header Information usage

Program     Batch   Length   Date   Mode   S/N   YMODEM-g   ZMODEM

Unix rb/sb   yes     yes      yes    yes    no    sb only    no

Unix rz/sz   yes     yes      yes    yes    no    sb only    yes

VMS rb/sb    yes     yes      no     no     no    no         no

Pro-YAM      yes     yes      yes    no     yes   yes        yes

CP/M YAM     yes     no       no     no     no    no         no

KMD/IMP      yes     no|#-      no     no     no    no         no

MEX          no      no       no     no     no    no         no

13.  ZFILE FRAME FILE INFORMATION

Only the pathname (file name) part is required for

batch transfers.

Pathname The pathname (conventionally, the file name)

is sent as a null terminated ASCII string.  This

is the filename format used by the handle oriented

MSDOS(TM) functions and C library fopen functions.

An assembly language example follows:

DB        'foo.bar',0

No spaces are included in the pathname.  Normally

only the file name stem (no directory prefix) is

transmitted unless the sender has selected YAM's f

option to send the full pathname.  The source

drive (A:, B:, etc.) is never sent.

Chapter 13             DRAFT 3-24-86                      17

DRAFT ZMODEM Protocol Ref 02-23-86                        18

Filename Considerations:

o#+ File names should be translated to lower case

unless the sending system supports

upper/lower case file names.  This is a

convenience for users of systems (such as

Unix) which store filenames in upper and

lower case.

o#+ The receiver should accommodate file names in

lower and upper case.

o#+ The rb(1) program on Unix systems normally

translates the filename to lower case unless

one or more letters in the filename are

already in lower case.

o#+ When transmitting files between different

operating systems, file names must be

acceptable to both the sender and receiving

operating systems.

If directories are included, they are delimited by

/; i.e., "subdir/foo" is acceptable, "subdir\foo"

is not.

Length The file length and each of the succeeding

fields are optional.1 The length field is stored

as a decimal string counting the number of data

bytes in the file.

With ZMODEM, the receiver uses the file length

only for display (progress reporting) purposes;

the actual length is determined by the data

transfer.

Modification Date A single space separates the

modification date from the file length.

The mod date is optional, and the filename and

length may be sent without requiring the mod date

to be sent.

The mod date is sent as an octal number giving the

time the contents of the file were last changed

measured in seconds from Jan 1 1970 Universal

__________

1. Fields may not be skipped.

Chapter 13             DRAFT 3-24-86                      18

DRAFT ZMODEM Protocol Ref 02-23-86                        19

Coordinated Time (GMT).  A date of 0 implies the

modification date is unknown and should be left as

the date the file is received.

This standard format was chosen to eliminate

ambiguities arising from transfers between

different time zones.

Two Microsoft blunders complicate the use of

modification dates in file transfers with

MSDOS(TM) systems.  The first is the lack of

timezone standardization in MS-DOS.  A file's

creation time can not be known unless the timezone

of the system that wrote the file2 is known.  Unix

solved this problem (for planet Earth, anyway) by

stamping files with Universal Time (GMT).

Microsoft would have to include the timezone of

origin in the directory entries, but does not.

Professional-YAM gets around this problem by using

the z parameter which is set to the number of

minutes local time lags GMT.  For files known to

originate from a different timezone, the -zT

option may be used to specify T as the timezone

for an individual file transfer.

The second problem is the lack of a separate file

creation date in DOS.  Since some backup schemes

used with DOS rely on the file creation date to

select files to be copied to the archive, back-

dating the file modification date could interfere

with the safety of the transferred files.  For

this reason, Professional-YAM does not modify the

date of received files with the header information

unless the d parameter is non zero.

Mode A single space separates the file mode from the

modification date.  The file mode is stored as an

octal string.  Unless the file originated from a

Unix system, the file mode is set to 0.  rb(1)

checks the file mode for the 0x8000 bit which

indicates a Unix type regular file.  Files with

the 0x8000 bit set are assumed to have been sent

from another Unix (or similar) system which uses

the same file conventions.  Such files are not

__________

2. Not necessarily that of the transmitting system!

Chapter 13             DRAFT 3-24-86                      19

DRAFT ZMODEM Protocol Ref 02-23-86                        20

translated in any way.

Serial Number A single space separates the serial

number from the file mode.  The serial number of

the transmitting program is stored as an octal

string.  Programs which do not have a serial

number should omit this field, or set it to 0.

The receiver's use of this field is optional.

The file information is terminated by a null.  If only

the pathname is sent, the pathname will be terminated

by two nulls.  The length of the file information

packet, including the trailing null, must not exceed

1024 bytes; a typical length is less than 64 bytes.

14.  MORE INFORMATION

More information may be obtained by calling

Telegodzilla at 503-621-3746.

UUCP sites can obtain the nroff/troff source to this

file with

uucp omen!/usr/caf/public/zmodem.mm /tmp

A continually updated list of available files is stored

in /usr/spool/uucppublic/FILES.

The following L.sys line calls Telegodzilla (Pro-YAM in

host operation).  Telegodzilla waits for carriage

returns to determine the incoming speed.  If none is

detected, 1200 bps is assumed and a greeting is

displayed.

In response to "Name Please:" uucico gives the Pro-YAM

"link" command as a user name.  The password (Giznoid)

controls access to the Xenix system connected to the

IBM PC's other serial port.  Communications between

Pro-YAM and Xenix use 9600 bps; YAM converts this to

the caller's speed.

Finally, the calling uucico logs in as uucp.

omen Any ACU 1200 1-503-621-3746 se:--se: link ord: Giznoid in:--in: uucp

Chapter 15             DRAFT 3-24-86                      20

DRAFT ZMODEM Protocol Ref 02-23-86                        21

15.  ZMODEM Programs

A demonstration version of Professional-YAM is

available as YAMDEMO.LQR (A SQueezed Novosielski

library).  This may be used to test ZMODEM and YMODEM

implementations.

Unix programs supporting the YMODEM protocol are

available on Telegodzilla in the "upgrade" subdirectory

as RBSB.SHQ (a SQueezed shell archive).  Most Unix like

systems are supported, including V7, Sys III, 4.2 BSD,

SYS V, Idris, Coherent, and Regulus.

A version for VAX-VMS is available in VRBSB.SHQ, in the

same directory.

A CP/M assembly version is available as MODEM76.AQM and

MODEM7.LIB.

Irv Hoff has added YMODEM 1k packets and basic YMODEM

batch transfers to the KMD and IMP series programs,

which replace the XMODEM and MODEM7/MDM7xx series

respectively.  Overlays are available for a wide

variety of CP/M systems.

MEX and MEX-PC also support some of the YMODEM

extensions.

Questions about YMODEM, the Professional-YAM

communications program, and requests for evaluation

copies may be directed to:

Chuck Forsberg

Omen Technology Inc

17505-V Sauvie Island Road

Portland Oregon 97231

Voice: 503-621-3406

Modem: 503-621-3746 Speed: 1200,300

Usenet: ...!tektronix!reed!omen!caf

Compuserve: 70715,131

Source: TCE022

Chapter 15             DRAFT 3-24-86                      21

CONTENTS

1.  ACKNOWLEDGMENTS....................................   2

2.  ROSETTA STONE......................................   2

3.  YET ANOTHER PROTOCOL, AGAIN?.......................   3

4.  ZMODEM Protocol Design Criteria....................   4

4.1   Throughput...................................   4

4.2   Integrity and Robustness.....................   5

4.3   Ease of use..................................   5

4.4   Ease of implementation.......................   5

5.  PACKETIZATION......................................   5

5.1   Link Escape Encoding.........................   6

5.2   Header Packet Information....................   7

5.3   Binary Header Packet.........................   7

5.4   HEX Header Packet............................   8

5.5   Binary Data Packets..........................   8

5.6   HEX Data Packet..............................   9

6.  PROTOCOL TRANSACTION OVERVIEW......................   9

7.  STREAMING TECHNIQUE................................  12

8.  ATTENTION SEQUENCE.................................  12

9.  PACKET/FRAME TYPES.................................  13

9.1   ZRQRINIT.....................................  13

9.2   ZRINIT.......................................  13

9.3   ZSINIT.......................................  13

9.4   ZACK.........................................  13

9.5   ZFILE........................................  13

9.6   ZSKIP........................................  14

9.7   ZNAK.........................................  14

9.8   ZABORT.......................................  14

9.9   ZFIN.........................................  14

9.10  ZRPOS........................................  14

9.11  ZDATA........................................  14

9.12  ZEOF.........................................  14

9.13  ZFERR........................................  14

9.14  ZCRC.........................................  15

9.15  ZCRYPT.......................................  15

9.16  ZCOMPL.......................................  15

10.  Transaction........................................  15

11.  PERFORMANCE........................................  15

- i -

12.  TABLES.............................................  16

13.  ZFILE FRAME FILE INFORMATION.......................  17

14.  MORE INFORMATION...................................  20

15.  ZMODEM Programs....................................  21

- ii -

LIST OF FIGURES

Figure 1.  Protocol Overhead Information................  16

Figure 2.  Transmission Time Comparision................  17

Figure 3.  Y/ZMODEM Header Information usage............  17

- iii -

offse###############################################################################################################################