April 10, 1991

1.0  INTRODUCTION

1.1  PURPOSE

The purpose of this document is to aid professional programmers who

wish to write software to use the high resolution modes supported by

the VGAWONDER video board. Working knowledge of EGA/VGA graphics and

assembly language programming is required and assumed. The document

describes several features unique to the VGAWONDER and how each high

resolution mode is similar or is different compared to standard VGA

modes. It does not contain any programming information about standard

EGA/VGA modes. Refer to the Reference list in the appendix for

recommended EGA/VGA programmer's references.

1.2  CONVENTIONS

The code segments contained in this document follow the conventions of

the MicroSoft MASM assembler. Numbers given in hexadecimal end with a

lower case h.

2.0  FEATURES OF THE VGA BOARD

2.1  STANDARD VIDEO MODES

AH = 0  ;set video mode

AL =    MODE/             RESOLUTION      DIM/          START

TYPE                              COLOR         ADDRESS

---------------------------------------------------------------

0h      color/alpha       640x200         40x25/ BW     b800:0h

1h      color/alpha       640x200         40x25/ 16     b800:0h

2h      color/alpha       640x200         80x25/ BW     b800:0h

3h      color/alpha       640x200         80x25/ 16     b800:0h

0*      color/alpha       640x350         40x25/ BW     b800:0h

1*      color/alpha       640x350         40x25/ 16     b800:0h

2*      color/alpha       640x350         80x25/ BW     b800:0h

3*      color/alpha       640x350         80x25/ 16     b800:0h

0+      color/alpha       720x400         40x25/ BW     b800:0h

1+      color/alpha       720x400         40x25/ 16     b800:0h

2+      color/alpha       720x400         80x25/ BW     b800:0h

3+      color/alpha       720x400         80x25/ 16     b800:0h

4h      color/graphics    320x200         40x25/  4     b800:0h

5h      color/graphics    320x200         40x25/ BW     b800:0h

6h      color/graphics    320x200         80x25/ BW     b800:0h

7h      mono/alpha        720x350         80x25/ BW     b000:0h

0dh     color/graphics    320x200         40x25/ 16     a000:0h

0eh     color/graphics    640x200         80x25/ 16     a000:0h

0fh     mono/graphics     640x350         80x25/ BW     a000:0h

10h     color/graphics    640x350         80x25/ 16     a000:0h

11h     color/graphics    640x480         80x30/ BW     a000:0h

12h     color/graphics    640x480         80x30/ 16     a000:0h

13h     color/graphics    320x200         80x25/256     a000:0h

2.2  ENHANCED VIDEO MODES

23h     color/alpha       1056x350        132x25/16     b800:0h

27h     mono/alpha        1056x350        132x25/BW     b000:0h

33h     color/alpha       1056x352        132x44/16     b800:0h

37h     mono/alpha        1056x352        132x44/BW     b000:0h

53h     color/graphics    800x560         16            a000:0h

54h     color/graphics    800x600         16            a000:0h

55h     color/graphics    1024X768        16            a000:0h

61h     color/graphics    640x400         256           a000:0h

62h     color/graphics    640x480         256           a000:0h

63h     color/graphics    800x600         256           a000:0h

64h     color/graphics    1024x768        256           a000:0h

65h     color/graphics    1024x768        16            a000:0h

67h     color/graphics    1024x768        4             a000:0h

2.3  REQUIRED CONFIGURATIONS

The enhanced modes have differing memory requirements. Some enhanced

modes are supported only on certain monitor types or hardware revisions.

The following chart describes the requirements for each enhanced mode.

Mode#   Resolution      Colors/         Memory          Monitors

(hex)                   Palette         Required        Supported

-----------------------------------------------------------------

23      132x25          16/64           512K            1,2,5

27      132x25          B&W             512K            1,2,5

33      132x44          16/64           512K            1,2,5

37      132x44          B&W             512K            1,2,5

54      800x600         16/256K         256K            5

55*     1024X768        256/256K        256K            4,5

61      640x400         256/256K        512K            3,4,5

62      640x480         256/256K        512K            3,4,5

63      800x600         256/256K        512K            5

64#     1024x768        256/256K        1M              4,5

65      1024x768        16/256K         512K            4,5

67      1024x768        4/16            256K            4,5

Monitors: (1) TTL mono (2) EGA (3) VGA (4) IBM 8514 (5) Multisync

* not supported on VGAWONDER V3

# only supported on VGAWONDER+ 1Mb

3.0  DETERMINING THE STATUS OF THE CARD

The VGA BIOS features some special functions which will allow the

programmer to determine if a compatible VGA board is installed and how

the card is configured within his program.

3.1  IDENTIFYING THE VIDEO CARD

All EGA and VGA display adapters manufactured by ATI Technologies will

contain the character string: "761295520" starting at address: C000:31.

VGA boards which are based on the ATI VGA chip set are identified by the

character string "31" at address C000:40. The card is further identified

by 2 bytes. The definition of the byte at address C000:42 is:

bit 0   = 0 16-bit VGA board

bit 0   = 1  8-bit VGA board

bit 1   = 1 mouse port present on display adapter

bit 2   = 1 supports non-interlaced 1024x768

bit 3   = 1 microchannel product

bit 4   = 1 contains programmable video clock chip

bit 5-7 = not currently defined

The character at address C000:43 contains the revision of the VGA gate

array. Currently, the following models exist and other revisions

may follow in the future.

'1' = 18800

'2' = 18800-1

'3' = 28800-2

'4' = 28800-4

'5' = 28800-5

3.2  DETERMINING THE MONITOR TYPE

The following assembler code segment can be used to determine for

which monitor the VGA board is currently configured.

cli

mov     dx,EXTENDED_REG

mov     al,DATA_I

out     dx,al

inc     dx

in      al,dx

sti

and     al,0fh

returns  al =       0       EGA monitor

1       PS/2 monochrome monitor

2       TTL monochrome monitor

3       PS/2 color monitor

4       RGB color monitor

5       Multisync monitor

7       PS/2 8514 monitor

8       Seiko 1430

9       NEC MultiSync 2A

a       Crystalscan 860/Tatung 1439

b       NEC MultiSync 3D

c       TVM 3M

d       NEC MultiSync XL

e       TVM 2A

f       TVM 3A

where

EXTENDED_REG = see section 4.41 in this document

DATA_I = 0bbh

Note : We reserve the right to add more monitor types at any time. We

recommend that if an undefined value is returned that the program

assumed that a multisync is installed.

3.3  DETERMINING MEMORY SIZE

In terms of memory determination, ATI VGA chipsets fall into 3 groups. The first

group is identified as types 1 and 2 (see section 3.31). The second group is identified as

type 3 and the third group as type 4 or 5. All types can be equipped with 256K or 512K

of RAM. Only types 4 or 5 may be equipped with 1M of RAM.

The following code segment will determine the type of chipset and use the appropriate

method to determine memory size.

.MODEL small,C

.CODE

PUBLIC RAMSIZE

; --------------------------------------------------------------------------

; CALL VALUES: none

; RETURNS VALUES: AX = 256 (100h) for 256K bytes

;                      512 (200h) for 512K bytes

;                      1024 (400h) for 1024K bytes

RAMSIZE PROC NEAR

push bx                      ; Save used register values

push dx

push ds

mov  ax, 0c000h

mov  ds, ax

xor  ax, ax

mov  dx, word ptr ds:[0010h] ; Get value of ATI extended register

mov  bl, byte ptr ds:[0043h] ; Get value of ATI chip version

pop  ds

mov  bh, 1                   ; Assume 256K ramsize (100h)

cmp  bl, '3'

jae  ramsize_V6_up           ; Use different method to determine

mov  al, 0bbh                ;  memory size of chip version is 3 or

cli                          ;  higher

out  dx, al

inc  dx

in   al, dx                  ; Get ramsize byte for chip versions

sti                          ;  1 & 2

test al, 20h

jz   eo_ramsize

mov  bh, 2                   ; Set ramsize return value to 512K (200h)

jmp  short eo_ramsize        ;  and exit

ramsize_V6_up:                       ; Method used for newer ATI chip versions

mov  al, 0b0h

cli

out  dx, al

inc  dx

in   al, dx                  ; Get ramsize byte for versions 3-5

sti

test al, 10h                 ; Check if ramsize byte indicates 256K

jz   ramsize_V7_up           ;  or 512K bytes

mov  bh, 2                   ; Set ramsize return value to 512K (200h)

ramsize_V7_up:                       ; Check for ramsize for ATI chip

cmp  bl, '4'                 ;  versions 4 & 5

jb   eo_ramsize

test al, 8                   ; Check if version 5 ATI chip has 1024K

jz   eo_ramsize

mov  bh, 4                   ; Set ramsize return value to 1024K (400h)

eo_ramsize:

xor  bl,bl                   ; Zero lower byte of BX

mov  ax, bx                  ; Return actual ramsize value in AX

pop  dx                      ; Restore BX and DX registers

pop  bx

ret

RAMSIZE ENDP

; --------------------------------------------------------------------------

END

3.4  DETERMINING WHETHER A MODE IS SUPPORTED

Your VGA board implements an extended video BIOS call which can be

used to determine if a particular mode is actually available in the

user's current configuration. The following assembler routine

demonstrates this technique:

push      bp

mov       bp,sp

push      es

mov       al,ARG

mov       ah,12h

mov       bx,5506h

mov       bp,0ffffh

int       10h

mov       ax,bp

pop       es

pop       bp

ret

where ARG = mode desired in hexadecimal

If this routine returns ax = 0ffffh, that mode is not supported in the

current configuration. If the mode is supported, ax will contain the

offset start address of the CRTC parameter table for that mode.

Note : We recommend this technique as the only reliable means of

determining if a particular mode is supported on our EGA or VGA boards.

4.0  PROGRAMMING THE HIGH RESOLUTION MODES

4.1  SETTING THE VIDEO MODE

The VGA card can be set into a video mode through the BIOS by using

the standard IBM method: AH is set to 0, AL is set to the video mode

number, and a call to software interrupt 10h is executed. See sections

2.1 and 2.2 of this manual for a list of available modes. The

following example will perform this function:

mov ah,0            ;set mode function

mov al,MODE         ;select video mode

int 10h             ;make BIOS call by using the video

;interrupt

We recommend that programmers select the video mode through BIOS

whenever possible rather than programming the mode via the registers.

For any particular video mode, the configuration of the internal

registers may differ depending on the type of monitor installed.

Also, we reserve the right to change the product in a way which would

be transparent to BIOS but which would have an impact on software

which writes to the registers.

4.2  VIDEO MEMORY ADDRESSING

4.21 ADDRESS RANGE OF VGA BOARD

The VGA board uses the RESERVED area designated by IBM for video

RAM and the video BIOS (A000 to C7FF). The organization is as follows:

Mono text video RAM:  B000-B7FF

Color text video RAM: B800-BFFF

Graphics video RAM:   A000-AFFF (upto eight 64k banked pages with 512K

RAM)

Video BIOS:           C000-C7FF

I/O Ports:            1CE-ICF, 3B0-3DF

4.22 PAGES and PLANES:

There are two memory mapping systems used with VGA graphics modes:

Packed pixel and Planar organization.

The Packed pixel mapping scheme is associated with pages and uses

contiguous memory to describe each pel. IBM VGA mode 13h uses this

system. Extended modes 61, 62, 63, and 65h also use the Bit-packed

scheme but they use a unique method to select which page is

available for reading and writing.

The Planar organized memory mapping scheme is associated with

planes and uses a bit from each plane to describe a pel. IBM VGA

mode 12h uses this system as does Extended modes 54h, 55h and 67h.

4.23 SELECTING MEMORY PAGES:

The memory organization of extended modes 61h, 62h, 63h, 64h and 65h are

arranged into 64k page segments. The data in each page is displayed on

a specific region of the screen image. For example, mode 63h has eight

pages. The first page is displayed on the top eighth of the screen

image, the second page is displayed on the second eighth of the screen

image, and so on. Data can be read or written to one page at a time.

The default active page is Page 0. A special code segment is required

to be executed before data can be read or written to a specific page.

A page selecting code segment is provided below. VGA cards with 256K

of RAM have four, those with 512K have eight and those with 1M have 16

pages available. Modes 61h, 62h, 63h and 64h require 1 byte per pel and

mode 65h requires 1/2 byte per pel.

The following code segment can be used to select the required page:

cli

mov     dx,EXTENDED_REG

mov     al,PAGE_SELECT

out     dx,al

inc     dl

in      al,dx

mov     ah,al

and     ah,PAGE_MASK

shl     ch,1

or      ah,ch

mov     al,PAGE_SELECT

dec     dl

out     dx,ax

sti

where    ch = required page value (0 to 7 for mode 63h for example)

PAGE_SELECT  = 0B2h

PAGE_MASK    = 0E1h

EXTENDED_REG = value of extended register stored at memory

address C000:0010h. See section 4.31

The following is a table that describes the visible range for

each extended mode which uses the bit-packed memory organization:

Mode 61h (640 x 400 - 256 colors):

Page            Visible range  (column,row)              Memory range

0               (0,0)      -->  (255,102)                A0000 - AFFFF

1               (256,102)  -->  (511,204)                A0000 - AFFFF

2               (512,204)  -->  (127,307)                A0000 - AFFFF

3               (128,307)  -->  (639,399)                A0000 - AE7FF

Mode 62h (640 x 480 - 256 colors):

Page            Visible range  (column,row)              Memory range

0               (0,0)      -->  (255,102)                A0000 - AFFFF

1               (256,102)  -->  (511,204)                A0000 - AFFFF

2               (512,204)  -->  (127,307)                A0000 - AFFFF

3               (128,307)  -->  (383,409)                A0000 - AFFFF

4               (384,409)  -->  (639,479)                A0000 - AAFFF

Mode 63h (800 x 600 - 256 colors):

Page            Visible range  (column,row)              Memory range

0               (0,0)      -->  (735,81)                 A0000 - AFFFF

1               (736,81)   -->  (671,163)                A0000 - AFFFF

2               (672,163)  -->  (607,245)                A0000 - AFFFF

3               (608,245)  -->  (543,327)                A0000 - AFFFF

4               (544,327)  -->  (479,409)                A0000 - AFFFF

5               (480,409)  -->  (415,491)                A0000 - AFFFF

6               (416,491)  -->  (351,573)                A0000 - AFFFF

7               (352,573)  -->  (799,599)                A0000 - A561F

Mode 64h (1024 x 768 - 256 colors):

Page            Visible range  (column,row)              Memory range

0               (0,0)      -->  (1023,63)                 A0000 - AFFFF

1               (0,64)     -->  (1023,127)                A0000 - AFFFF

2               (0,128)    -->  (1023,190)                A0000 - AFFFF

3               (0,191)    -->  (1023,255)                A0000 - AFFFF

4               (0,256)    -->  (1023,319)                A0000 - AFFFF

5               (0,320)    -->  (1023,383)                A0000 - AFFFF

6               (0,384)    -->  (1023,447)                A0000 - AFFFF

7               (0,448)    -->  (1023,511)                A0000 - AFFFF

8               (0,512)    -->  (1023,575)                A0000 - AFFFF

9               (0,576)    -->  (1023,639)                A0000 - AFFFF

10              (0,640)    -->  (1023,703)                A0000 - AFFFF

11              (0,704)    -->  (1023,767)                A0000 - AFFFF

12                       spare

13                       spare

14                       spare

15                       spare

Mode 65h (1024 x 768 - 16 colors):

Page            Visible range  (column,row)              Memory range

0               (0,0)      -->  (1023,127)               A0000 - AFFFF

1               (0,128)    -->  (1023,255)               A0000 - AFFFF

2               (0,256)    -->  (1023,383)               A0000 - AFFFF

3               (0,384)    -->  (1023,511)               A0000 - AFFFF

4               (0,512)    -->  (1023,639)               A0000 - AFFFF

5               (0,640)    -->  (1023,767)               A0000 - AFFFF

6                        spare

7                        spare

4.3  DETAILS OF EACH EXTENDED VIDEO MODE

4.31 132X25 - Color text (Mode 23h)

132x25 - B&W text (Mode 27h)

132X44 - Color text (Mode 33h)

132X44 - B&W text (Mode 37h)

Video Memory Organization:

The memory organization of very similar to standard text modes. Only

one page of 6,660 bytes (132X25 modes) or 11,616 bytes (132X44 modes)

is availible.

Video Data Organization:

The video data organization is identical to standard text modes.

BIOS support

All BIOS calls supported on standard text modes are supported.

NOTE! It is a good idea to avoid DOS calls when using the 132X44 text

modes. Most versions of ANSI.SYS are not compatible with text modes

which have more than 25 lines. This is particularly a problem when

scrolling the screen.

4.32 800x600 - 16 COLORS (Mode 54h)

Video Memory Organization:

The memory organization is identical to 16 color IBM EGA/VGA modes

and consists of 256K organized into 4 maps of 64K each. One bit from

each of the 4 maps is used to compose each pel. The IBM EGA/VGA Map

Mask Register is used to select any or all the maps to be updated.

A0000:0 -> ----------

|          |----------

|          |          |----------

|----------|          |          |---------- <===\=======\

| Reserved |----------|          |          |  60,000    |

----------| Reserved |----------|          |    |       |

Plane 0  ----------| Reserved |----------|<===/      64K

Plane 1  ----------| Reserved |            |

Plane 2   --------- <===========/

Plane 3

Video Data Format:

The video data format is identical to the 16 color IBM EGA/VGA modes.

Color Mapping:

Color selection and palette changing is identical to the 16 color IBM

EGA/VGA mode

4.33 1024x768 - 16 COLORS PLANAR (Mode 55h)

Video Memory Organization:

The memory organization is similar to 16 color IBM EGA/VGA modes

and consists of 2 pages of 256K each organized into 4 maps of 64K. One

bit from each of the 4 maps is used to compose each pel. The IBM

EGA/VGA Map Mask Register is used to select any or all the maps to be

updated.

Address                                 Page #

A000:0 ==> +-----+-----+-----+-----+

|     |     |     |     |

|     |     |     |     |

|     |     |     |     |      0

|     |     |     |     |

|     |     |     |     |

A000:0 ==> +-----+-----+-----+-----+

|     |     |     |     |

|     |     |     |     |

|     |     |     |     |      1

|     |     |     |     |

|     |     |     |     |

+-----+-----+-----+-----+

Map #         0     1     2     3

Selecting Memory Page:

The following code segment demonstates how to switch between pages

when in this mode.

To change the write plane

cli

mov     dx,ATI_REG

mov     al,PLANE_SELECT

out     dx,al

inc     dl

in      al,dx

mov     ah,al

and     ah,WPLANE_MASK

shl     ch,1

or      ah,ch

mov     al,PLANE_SELECT

dec     dl

out     dx,ax

sti

To change the read plane

cli

mov     dx,ATI_REG

mov     al,PLANE_SELECT

out     dx,al

inc     dl

in      al,dx

mov     ah,al

and     ah,RPLANE_MASK

shl     ch,1

shl     ch,1

shl     ch,1

shl     ch,1

rol     ch,1

or      ah,ch

mov     al,PLANE_SELECT

dec     dl

out     dx,ax

sti

Where ch = page number required

ATI_REG = store in location c000:10h

PLANE_MASK = 0e1h

WPLANE_MASK = 0e1h

RPLANE_MASK = 01eh

PLANE_SELECT = 0b2h

DIFF_RW_PLANE = 008h

MISC2_REG = 0beh

DATA_I = 0bbh

The read and write planes can be changed simultaneously.

4.36 800x600 - 256 COLORS (Mode 63h)

Video Memory Configuration:

This enhanced resolution mode uses a page memory system unique to this

VGA board. The memory is organized into contiguous 64K blocks located

at A0000H to AFFFFH. The VGA board must be equipped with 512K of RAM

to support this mode. Mode 63h uses eight 64K pages. For information

on selecting the active page, refer to section 4.23 "SELECTING MEMORY

PAGES" of this document.

A0000:0 -> ----------<==============================================\

|          |----------                                     |

|          |          |----------                          |

|          |          |          | \                       |

|          |          |          |  \                      |

----------|          |          |   \                     |

Page 0  ----------|          |    \                    |

Page 1  ----------       ---------- <=\  480,000

Page 3  \    |          |  |     |

\   |          | 64K    |

\  |          |  |     |

\ |          |  |     |

---------- <=/=====/

Page 7

Video Data Format:

The video data format is identical to IBM VGA mode 13h (320x200-256

color). Each pel is represented by 1 byte.

Color Mapping:

Color selection and palette changing is identical to VGA mode 13h.

Video Data Format:

The video data format is identical to the 16 color IBM EGA/VGA modes.

Color Mapping:

Color selection and palette changing is identical to the 16 color IBM

EGA/VGA mode

4.34 640x400 - 256 COLORS (Mode 61h)

Video Memory Configuration:

This enhanced resolution mode uses a page memory system unique to this

VGA board. The memory is organized into contiguous 64K blocks located

at A0000H to AFFFFH. A VGA board equipped with 256K or 512K of RAM

will support this mode. Mode 61h uses four 64K pages.  For information

on selecting the active page, refer to section 4.23 "SELECTING MEMORY

PAGES" of this document.

A0000:0->-----------<===========================================\

|          |----------                                |

|          |          |----------                     |

|          |          |          |---------- <==\=====|

|          |          |          |          |   |     |

----------|          |          |          |  64K    |

Page 0    ----------|          |          |   |   256,000

Page 1  ----------|          |   |     |

Page 2  ---------- <==/=====/

Page 3

Video Data Format:

The video data format is identical to IBM VGA mode 13h (320x200-256

color). Each pel is represented by 1 byte.

Color Mapping:

Color selection and palette changing is identical to VGA mode 13h.

4.35 640x480 - 256 COLORS (Mode 62h)

Video Memory Configuration:

This enhanced resolution mode uses a page memory system unique to this

VGA board. The memory is organized into contiguous 64K blocks located

at A0000H to AFFFFH. The VGA board must be equipped with 512K of RAM

to support this mode. Mode 62h uses five 64K pages. For information on

selecting the active page, refer to section 4.23 "SELECTING MEMORY

PAGES" of this document.

A0000:0 -> ---------- <=============================================\

|          |---------                                      |

|          |         |---------                            |

|          |         |         |---------                  |

|          |         |         |         |--------- <=\    |

----------|         |         |         |         |  |    |

Page 0   ---------|         |         |         | 64K   |

Page 1  ---------|         |         |  | 307,200

Page 2  ---------|         |  |    |

Page 3  --------- <=/====/

Page 4

Video Data Format:

The video data format is identical to IBM VGA mode 13h (320x200-256

color). Each pel is represented by 1 byte.

Color Mapping:

Color selection and palette changing is identical to VGA mode 13h.

4.36 800x600 - 256 COLORS (Mode 63h)

Video Memory Configuration:

This enhanced resolution mode uses a page memory system unique to this

VGA board. The memory is organized into contiguous 64K blocks located

at A0000H to AFFFFH. The VGA board must be equipped with 512K of RAM

to support this mode. Mode 63h uses eight 64K pages. For information

on selecting the active page, refer to section 4.23 "SELECTING MEMORY

PAGES" of this document.

A0000:0 -> ----------<==============================================\

|          |----------                                     |

|          |          |----------                          |

|          |          |          | \                       |

|          |          |          |  \                      |

----------|          |          |   \                     |

Page 0  ----------|          |    \                    |

Page 1  ----------       ---------- <=\  480,000

Page 3  \    |          |  |     |

\   |          | 64K    |

\  |          |  |     |

\ |          |  |     |

---------- <=/=====/

Page 7

Video Data Format:

The video data format is identical to IBM VGA mode 13h (320x200-256

color). Each pel is represented by 1 byte.

Color Mapping:

Color selection and palette changing is identical to VGA mode 13h.

4.37 800x600 - 256 COLORS (Mode 64h)

Video Memory Configuration:

This enhanced resolution mode uses a page memory system unique to this

VGA board. The memory is organized into contiguous 64K blocks located

at A0000H to AFFFFH. The VGA board must be equipped with 512K of RAM

to support this mode. Mode 64h uses sixteen 64K pages. For information

on selecting the active page, refer to section 4.23 "SELECTING MEMORY

PAGES" of this document.

A0000:0 -> ----------<==============================================\

|          |----------                                     |

|          |          |----------                          |

|          |          |          | \                       |

|          |          |          |  \                      |

----------|          |          |   \                     |

Page 0  ----------|          |    \                    |

Page 1  ----------       ---------- <=\  786,432

Page 3  \    |          |  |     |

\   |          | 64K    |

\  |          |  |     |

\ |          |  |     |

---------- <=/=====/

Page 15

Video Data Format:

The video data format is identical to IBM VGA mode 13h (320x200-256

color). Each pel is represented by 1 byte.

Color Mapping:

Color selection and palette changing is identical to VGA mode 13h.

4.38 1024x768 - 16 COLORS (Mode 65h)

Video Memory Organization:

This enhanced resolution mode uses a page memory system unique to this

VGA board. The memory is organized into contiguous 64K blocks located

at A0000H to AFFFFH. The VGA board must be equipped with 512K of RAM

to support this mode. Mode 65h uses eight 64K pages. For information

on selecting the active page, refer to section 4.23 "SELECTING MEMORY

PAGES" of this document.

A0000:0 -> ---------- <=============================================\

|          |----------                                    |

|          |          |----------                         |

|          |          |          | \                      |

|          |          |          |  \                     |

----------|          |          |   \                    |

Page 0   ----------|          |    \                   |

Page 1  ----------       ---------- <==\ 402,432

Page 2   \    |          |   |   |

\   |          |   |   |

\  |          |  64K  |

\ |          |   |   |

---------- <==/===/

Page 7

Video Data Format:

The video data format is unique to this mode. Each pel requires 4 bits

or half a byte.

____________________________________________ High Order

|    |    |    |    |                              Pixel

|____|____|____|____|____ ____ ____ ____

|  H |  H |  H |  H |  L |  L |  L |  L |

|____|____|____|____|____|____|____|____|

|    |    |    |    |

|____|____|____|____|________ Low Order

Pixel

Color Mapping:

This mode has a different memory organization than standard EGA mode

10h. If users want to use colors other than the standard 16 colors,

users are required to re-program the external palettes (3C8h/3C9h). Re-

programming the internal palettes (3C0h) will not work the same as it

would in EGA mode 10h because the color mapping in mode 65h is

different. It is not recommended to change the internal palettes at

all for mode 65h. The following table shows the mapping of colors and

external palettes.

Palette registers available for re-programming are:

Palette register   Palette register       default color   color

(low order pels)   (high order pels)      assignment      value

----------------   -----------------      -------------   -----

00h                 00h                Black           0h

01h                 10h                Blue            1h

02h                 20h                Green           2h

03h                 30h                Cyan            3h

04h                 40h                Red             4h

05h                 50h                Magenta         5h

06h                 60h                Brown           6h

07h                 70h                White           7h

08h                 80h                Dark Gray       8h

09h                 90h                Light Blue      9h

0Ah                 A0h                Light Green     Ah

0Bh                 B0h                Light Cyan      Bh

0Ch                 C0h                Light Red       Ch

0Dh                 D0h                Light Magenta   Dh

0Eh                 E0h                Yellow          Eh

0Fh                 F0h                Bright White    Fh

For example, to change the default color assignment for palette

register 4 from Red to Bright White, palette register 4 and 40h would

need to be re-programmed with the values:

Red Intensity = 3Fh

Green Intensity = 3Fh

Blue Intensity = 3Fh

so that both the low order AND high order pels are affected. The only

exception is palette register 0 (Black); re-programming this register

will affect both the low order and high order pels.

An Example:

The following example uses a standard IBM EGA/VGA BIOS call. If more

information is required, consult the IBM EGA/VGA programmers reference

list included in this document.

Example segment to re-program a palette register:

mov    ah,10h                ;EGA/VGA Set Palette Register BIOS call

mov    al,10h

mov    bx,PALETTE_REG_LOW    ;Palette register to be re-programmed

; for low order pels.

mov    ch,GREEN_INTENSITY    ;Load new values

mov    cl,BLUE_INTENSITY

mov    dh,RED_INTENSITY

int    10h

mov    ah,10h                ;EGA/VGA Set Palette Register BIOS call

mov    al,10h

mov    bx,PALETTE_REG_HIGH   ;Palette register to be re-programmed

; for high order pels.

mov    ch,GREEN_INTENSITY

mov    cl,BLUE_INTENSITY

mov    dh,RED_INTENSITY

int    10h

where:

PALETTE_REG_LOW  = low  pel palette register (00,01,02,..,0F)

PALETTE_REG_HIGH = high pel palette register (00,10,20,..,F0)

GREEN_INTENSITY

BLUE_INTENSITY  = 6 bit intensity values for defining the default color

RED_INTENSITY

4.39 1024x768 - 4 COLORS (Mode 67h)

Video Memory Organization:

The memory organization is identical to the IBM 16 color EGA and VGA

modes. 256K of RAM is divided into 4 maps of 64k. The IBM EGA/VGA Map

Mask Register is used to select the maps to be updated.

A0000:0->/------\

|      | /------\

|  C0  | |      | /------\

\------/ |  C1  | |      | /------\ <---+

Plane 0  \------/ |  C0  | |      |     49152 bytes/plane

^        Plane 1  \------/ |  C1  |     |

|              ^  Plane 2  \------/ <---+

|              |  ^         Plane 3

\---odd pels---/  |               ^

|               |

\---even pels---/

Video Data Format:

This mode uses a unique data format where 2 bits are required per pel.

Plane 0 and 1 contain the data for odd numbered pels and Planes 2 and

3 contain the data for the even numbered pels.

Color Mapping:

This mode supports four palette selections. The palette or color set

is selected by setting bits 0 & 1 of the IBM EGA/VGA Color Select

Register.

Color Values (C0, C1)

Color set #        C0=0,C1=0  C0=1,C1=0  C0=0,C1=1  C0=1,C1=1

0              black        white      grey     bright white

1              black        cyan       red      white

2              black        green      red      yellow

3              black        cyan       magenta  white

Examples:

The following examples use standard IBM EGA/VGA registers. If more

information is required, consult the EGA/VGA programmers reference

list included in this document.

Segment example for selecting the active palette or color set:

mov   dx,03dah      ;Load Input Status Register One

;

wait:

in    al,dx         ;Wait for vertical retrace bit to be set

test  al,8          ; (bit 3 = 1)

jnz   wait          ;It is good practise to wait for the vertical

; retrace bit before directly modifying the

; palette registers

mov   ah,ARG        ;Arg = color set # (0 to 3)

mov   cl,4          ;Multiply arg by 8 so that bits 4 & 5 of the

shl   ah,cl         ; palette registers can be modified

mov   dx,03c0h      ;Load Palette Control Register

mov   cx,16         ;16 Palette registers to be modified

xor   bl,bl         ;Start from 0

;

repeat:               ;Modify bits 4 & 5 of all 16 palette registers with

mov   al,bl         ; Arg value (color set #)

out   dx,al         ;Select palette register

or    al,ah

out   dx,al         ;Write modified data to selected palette register

loop  repeat

mov   al,20h        ;Restore internal registers access to palette data

out   dx,al         ; bit 5 must be zero during palette data writing

; and one after data writing is complete

Segment example for selecting Bit Planes to be modified during a

memory write:

mov   dx,03c4h      ;Load Sequencer Address Register

mov   al,2          ;Load index of Map Mask Register

out   dx,al         ;Select Map Mask Register

inc   dl

mov   al,ARG        ;Value to select which bit planes are to be

; modified during a memory write

out   dx,al         ;Select EGA/VGA planes to be modified

IBM EGA/VGA Map Mask Register (3C5h, index 2) organization:

Bits:   0     Controls access to Bit Plane 0

1     Controls access to Bit Plane 1

2     Controls access to Bit Plane 2

3     Controls access to Bit Plane 3

4-7   Not used

A bit value of 0 disables access to the corresponding Bit Plane and a

bit value of 1 enables it.

4.4  SPECIAL NOTES

4.41 NOTE ON EXTENDED REGISTER ADDRESS

Hard-coding this value is not recommended since it is programmable

and may be changed in subsequent BIOS releases. The value of

EXTENDED_REG should be retrieved from the BIOS at location C000:0010h.

The 'mov dx, EXTENDED_REG statement in the page selecting segment can

be replaced with code to load the value of EXTENDED_REG from its stored

location. The registers that need to be saved will depend greatly on

the particular application:

push es         ;save registers es and bx

push bx

mov  ax,0c000h  ;define storage location of EXTENDED_REG

mov  es,ax

mov  bx,10h

mov  dx,es:[bx] ;get the value of EXTENDED_REG from contents

;of the storage location C000:0010h

pop  bx         ;restore registers es and bx

pop  es

4.42 NOTE ON DAC REGISTER PROGRAMMING

In IBM VGA mode 13h (320x200 - 256 color), each color is controlled by

a corresponding DAC register. There are a total of 256 18-bit DAC

registers. Therefore, each DAC register can be assigned a color value

from 0 (BLACK) to 262,143 (BRIGHT WHITE). The first 16 DAC registers

have assigned color values that correspond to the 16 colors used in

mode 12h (640x480 - 16 color).  DAC register zero, which has an

assigned color value of 0 (BLACK), also controls the color of the

overscan border. This function is also supported in ATI modes 61, 62,

63, and 65h. On some analog monitors, a non-BLACK overscan border may

cause line wrapping at either the top or bottom of the screen image

due to the video bandwidth required. For this reason, it is not

recommended to change the color value of this DAC register.

4.43 BIOS FUNCTION SUPPORT

Many versions of the VGAWONDER do not support certain video BIOS

function calls in some extended video modes due to EPROM space

limitations. Programmers should assume that the following functions

are not supported in the indicted modes:

MODE #              BIOS FUNCTION

61h, 62h, 63h,      ah = 06h - Scroll active page up

65h, 67h            ah = 07h - Scroll active page down

ah = 09h - Read Character at Cursor

ah = 0ch - Write Dot

ah = 0dh - Read Dot

ah = 0eh - Write TTX

All extended modes have only one active page regardless of the amount

of memory installed.

5.0  APPENDIX

5.1  LIST OF SUGGESTED REFERENCES

TITLE:       Programmer's Guide to the EGA/VGA *

AUTHOR:      George Sutty and Steve Blair

PUBLISHER:   Brady Books / Simon and Schuster, Inc.

DATE:        1988

TITLE:       Advanced Programmer's Guide to SuperVGAs *

AUTHOR:      George Sutty and Steve Blair

PUBLISHER:   Brady Books / Simon and Schuter, Inc.

DATE:        1990

TITLE:       Programmer's Guide to IBM PC and PS/2 Video Systems

AUTHOR:      Richard Wilton

PUBLISHER:   MicroSoft Press

DATE:        1987

TITLE:       Programmer's Guide to the EGA and VGA Cards

AUTHOR:      Richard F. Ferraro

PUBLISHER:   Addison - Wesley Publishing Co.

DATE:        July, 1988

TITLE:       EGA/VGA A Programmer's Reference Guide

AUTHOR:      Bradley Dyck Kliewer

PUBLISHER:   McGraw Hill

DATE:        1988

TITLE:       Personal System/2 Model 50 and 60 Technical Reference

AUTHOR:      unknown

PUBLISHER:   IBM

DATE:        April, 1987

TITLE:       Personal System/2 and Personal Computer BIOS Interface

Technical Reference

AUTHOR:      unknown

PUBLISHER:   IBM

DATE:        April, 1987

TITLE:       Programmer's Reference Manual for the VGAWONDER  *

AUTHOR:      various

PUBLISHER:   ATI Technologies (order from Tech Support $40)

DATE:        1990

* recommended

TO OBTAIN A PRINT COPY OF THIS DOCUMENT ON YOUR SYSTEM

PRINTER, ENTER THE FOLLOWING COMMAND AT THE DOS PROMPT:

TYPE PROGINFO.DOC>PRN <rtn>