This is information taken from literature sent by Brown-Wagh Publishing,

distributers of the Sound Blaster Pro.

Sound Blaster Pro card- "The Multimedia sound standard"

Creative Labs, Inc.

2050 Duane Ave.

Santa Clara, CA 95054

Tel: (408) 986-1461  Fax: (408) 986-1777

Distributed by:

Brown-Wagh Publishing

130-D Knowles Dr.

Los Gatos, CA 95030

Tel: (408) 378-3838  (800) 451-0900

Hardware features:

Stereo DACs - 8 bit

- Sound Blaster compatible in Mono mode

- ADPCM Decompression in hardware(2:1, 3:1, and 4:1)

- DMA transfer (shareable) on channel 0, 1, or 3 selectable by jumper

- Selectable sampling rates:  4K - 44.1KHz

Stereo ADCs - 8 bit

- Sound Blaster compatible in Mono mode

- DMA transfer (shareable) on channel 0, 1, or 3 selectable by jumper

- Selectable sampling rates:  4K - 44.1KHz

- Support recording from Microphone, Stereo Line-IN and CD-Audio

Stereo Digital/Analog Mixer

- Mixing of all audio sources: Stereo DAC, Stereo FM, Microphone, Stereo

Line-In, CD-Audio and PC Internal Speaker

- Full software control of fade-in, fade-out, pan, volume, left/right

steering and Mic mixing

- Multiple level volume controls on digitized sound:

DAC            FM Music     Microphone

CD-Audio       Line-In      Master Volume

Stereo FM Music Syntesizers

- Twin FM chips giving maximum 22 voices

- Power-on default to Mono mode for full compatibility with Sound Blaster

and Adlib

MIDI Interface

- Sound Blaster Compatible

- MIDI time-stamp for Multimedia Extension

- 64-byte FIFO

- Sharing Joystick Port

- MIDI Adaptor and cables for standard MIDI cabling included

CD-ROM Interface

- 8-bit DMA transfer on channel 1 or 3 selectable by jumper

- Internal connector for CD-Audio

- Support internal and external CD-ROM drives

Joystick Port

- Standard PC Joystick Port for 1 or 2 joysticks(Y-cable Optional)

Microphone Automatic Gain Control (AGC) Amplifier

- Hardware AGC for optimum Microphone sensitivity

- Input Impedance 600 ohms

Sensitivity: 10 - 100m V

Output Power Amplifier

- 4 watts per channel

- Load Impedance 4 ohms

- Mini stereo jack to RCA conversion cable included

Manual Volume Control

- Volume control knob to set output level

- Easily accessible and durable

Jumper Selections

- Interrupt selection        : IRQ 2, 5, 7, 10

- DMA selection              : channel 0, 1 or 3

- I/O address selection      : 220H or 240H

On-board Connector Pins

- PC Internal Speaker Input

- Line-Out

- Microphone Input

- CD-Audio Input

- CD-ROM Interface

Software Included:

Bundled Software 1: SB VOICE EDITOR

This is a powerful program that supports stereo recording, compression and

editing of voice file of unlimited size.  It displays the waveform with

zooming and allows cut-and-paste.  It creates echo and reverb.  It can also

insert marker and repeat commmands.  Playback of the voice files in

background is possible using the utilities.

Bundled Software 2: FM INTELLIGENT ORGAN

This powerful, easy to use organ software lets you play and learn to play

orchestra-like music using only ONE finger on your PC keyboard.  Background

accompaniments and rhythms are added automatically.  No music knowledge

required.  And now play from your MIDI keyboard too.

Bundled Software 3: SBTALKER

A high value memory resident module that makes your computer talk directly

from ASCII text supplied.  It speaks English from an unlimited vocabulary

dictionary.  SBTALKER comes with the insrutable DR SBAITSO AI software.

Consult DR SBAITSO about any of your problems and hear him reply you.

Loads of Fun.

Bundled Software 4: MIDI Sequencer Sp Pro

Sp Pro includes all of the essential features you'll need to create serious

music composition.  More importantly, it offers a wide range of options so

that you won't lose interest!  In fact, Sp Pro's extensive list of features

includes many of those found in the "Pro" versions of other programs.

Bundled Software 5: CD MUSIC PLAYER

Controls the playing of standard compact disc with CD-ROM drive.  Program

comes with a full feature control panel - Play, Next/Last Track, Pause,

Fast-Forward, Rewind and Stop.

Bundled Software 6: MMPLAY PRESENTATION

MMPlay integrates graphic animations with the sound capabilities of SB Pro:

FM Music, Digitized Voice and CD Audio.  THis utility supports

synchronisation, fading and mixing.

Suggested Retail Price : $299.95

Upgrade from Sound Blaster to Sound Blaster Pro : $199.95 + old Sound

Blaster

Release date: Late August - Early September

DOS software drivers and Windows DLL are included.

All specifications are subject to change without prior notice.