Hard Disk Secure

V1.05

Copyright TeraTronics Consulting

Stuart Macdonald

1991

Shareware Notice:

I have undertaken a considerable effort to create both this Security

utility and the corresponding installer utility and to painstakingly ensure

that it works flawlessly on a multitude of configurations. As with any Shareware

utilities that alter your system, I strongly suggest that you perform regular

backups of your system on tape or disk(s).

If you find this software useful then please help out my future efforts

by sending $20.00 per copy to:

Stuart Macdonald

107 Horizon Road

Palmdale, CA 93551 USA

Documentation for disk security program(s).

Programs:

Install.exe    - 	Self-contained Installation utility

Secure.sys   - 	Config.sys-based Device Driver for password protection upon IPL

Secure.doc  - 	This Help file

SetVar.Exe  - 	Utility to dynamically change the Disk protection options Format and Write

protection.

This Security utility, combined with the secure.sys driver, offers a substantial

increase in hard disk security over other security mechanisms. This utility

comes with the following features:

1) Prevent unauthorized access to Hard disk by using an

eight character sign on password(secure.sys). This

utility will allow the user to enter up to an eight

character password. If incorrect, the user will be informed

and the system will halt. If correct then the system will

continue loading the operating system. The User/intruder

can not break out of this prompt by pressing CTRL-C.

2) Prevent unauthorized access to the Hard disk if the system

is booted from the floppy drive. DOS will not be able to

find a valid Hard drive. This ability is gained through

running the Install.exe utility.

3) Prevent accidental Format of Hard disk. This option, if

enabled, will prevent the DOS Format command from formatting

any Tracks on the Hard Disk.

4) Prevent any Writes to Hard disk. This option, if enabled,

will prevent any BIOS and DOS -level Writes to the Hard

Disk. This utility can be useful in the event that LAN

Gateway or Bridge PC's are placed in a "non-trusted" area.

This utility could also offer some protection against

viruses that attempt to install themselves onto the Hard

disk(assuming that the utility is first loaded to the

Hard disk or executed from the Floppy disk).

Compatibility Notes:

This software has been verified to work without any problems with the

following LAN workstation shells: Banyan VINES, Novell Netware and LAN Support

Program.

This software has been verified to work with:

IBM DOS 3.0

IBM DOS 3.3

Microsoft DOS 3.3

Microsoft DOS 5.00.449

Compaq DOS 3.31

Compaq DOS 4.01

Microsoft Windows 3.0.

QEMM 5.11

Banyan VINES 4.00, 4.02 Redirectors

Novell Netware 2.15 Redirectors

The only requirement is DOS 3.0 or greater since the Installer uses

features not found in earlier versions of DOS.

This software has been verified to work on the following systems:

IBM PC, PC-XT, PC-AT(all), PS/2(30,50,55SX,60,70,80(386)) ESDI and MFM drives.

NCR (all i286 and i386 workstation models)

COMPAQ Portable 386.

PC-AT Clones running Phoenix, Award and Chips and Technologies BIOS.

*** NOTE ***

Will Not Work on an NCR i486 system with SCSI drives. (Current Incompatibility)

However, if installed from A: drive, the system can be restored to its

original state by running the install utility to remove the security

feature.

Please not that with some Virus detection software you will get a message "Invalid Disk Media Access" -

Don't worry, this is normal as the Install.exe utility modifies the Disk partition

table.

Memory Requirements:

Security Feature    1,024 Bytes of NON-DOS Memory. DOS Memory Reduced by 1KB

Secure.sys          94 Bytes of DOS Memory.

INSTALL.EXE         Requires DOS 3.0 or greater and at least 128KB of Memory.

HOW TO INSTALL THE SECURITY UTILITY:

*** NOTE ***

For your own protection, please install this utility from the A: drive

on a bootable diskette (format a:/s...). This is your protection if

you are running on a non-tested platform. Resolving a problem is as simple

as re-running the Install utility to automatically restore the original

partition.

1) Type INSTALL S <enter>  - You should get a message that Install.EXE can be

distributed. If Not then you have a copy of Install.exe that has already been installed

somewhere and therefore you can't use it.

2) Type INSTALL H <enter>  - This will provide you with a help screen in case

you decide to blaze through this document and forget/overlook the option switches.

3) The parameters for the Install.exe are as follows:

INSTALL option(s)

Where Options are:

H   	:  Help Screen that reviews options and commands

A   	:  About screen that advertises the product

P:Password   :  Up to eight character case-sensitive password

Default password is "PASSWORD"

O:Options    :  Security options:

F :  Disable BIOS and DOS Format commands for

Hard Disk only.

W :  Disable BIOS and DOS Write commands for

Hard Disk only.

Example:

Install P:Hello O:FW   : Password = "Hello" which is not equal to

"HELLO" or "hello", Options = Format and Write

protection ON.

Install O:F            : Password defaults to "PASSWORD", Options =

Format protection ON, Write protection OFF.

4) Run Install.exe with the appropriate command line parameters. Please note

that input redirection is not supported. Please note that Install.exe

will check to see if it, or another copy, has already been installed on

the Hard disk. If so, then Install.exe will terminate with the appropriate

message. For the upmost in protection, Install.exe should be run from

the A: drive on a formatted system diskette.

5) After Install.exe has completed, you will be prompted to press any key to

reboot. After rebooting, the system will load as usual with no apparent

changes to the end-user. If there are any error messages then please note

them down and contact me via E-mail on Compuserv.

6) Save a copy of the Install.exe file since it is now specific to your system(

specifically, your hard disk). The Install.exe file, after running, will

contain a copy of the pre-existing partition table. Once installed, you will not

be able to run Install.exe on any other system except the one that it was

originally installed on. Furthermore, Install knows when it has been installed,

and if run again on the same Disk, will attempt to remove itself. If you save

this file to a floppy(strongly suggested) then please ensure that the floppy is

"self booting" (format a:/s). Doing this will allow you to restore your system

to its original state from the Floppy disk. Naturally, leave this diskette in

a very safe place.

7) At this point, the Secure.sys utility can be placed into the Config.sys

file. This driver should(but doesn't have to be) be the first driver in the

Config.sys file. Also, this driver is only required if one wishes to have

Password protection before gaining access to the Hard Disk. It is not

required for Format or Write protection or for preventing a user from

booting from the A: drive and accessing the Hard disk.

8) After installing the Secure.sys utility, manually reboot the PC.

9) When the PC is processing the Config.sys file, the user will be prompted

for a password in order to gain access to the system. Type the same

password as was done for the Install.exe utility.

HOW TO REMOVE THE SECURITY UTILITY:

1) Remove or comment out the Secure.sys utility from the Config.sys file. If

you accidentally remove the security system before removing or commenting

out the Secure.sys file, don't worry - Secure.sys will sense that the

security system is absent and will not interfere with the DOS loading

process.

2) Take the earlier-saved Install.exe file and run it from either the Hard or

Floppy disk. You can even boot from the floppy disk and then run

install.exe.

3) Type Install P:password

Where P: Password is the earlier-mentioned password used with

Install.exe. if you did not use a password when Install.exe

was previously run then this field can be omitted.

example : Without a password type INSTALL <enter>

Note:    If a password was used during the installation phase then

one must be entered in order to remove the security system.

4) The Install.exe utility will check the Disk to ensure that it is the same

one used during the Installation phase. If the disk is the same then Install.exe

will replace the current Partition sector with the original partition sector.

You must use the Install.exe, or a copy of, that was used in the installation

process - this is software enforced and there are no exceptions.

5) When complete, Install will prompt you to press any key to Reboot.

6) After rebooting, the system will be restored to its original pre-installation

state.

Since Install.exe does not support StdIn or StdOut, a DOS return code is

provided to indicate program completion status or a failure point.

Install.exe will display Text messages indicating the cause of any failure.

HOW THE SECURITY SYSTEM WORKS

The Install.exe utility saves a copy of the current partition sector which is

comprised of both partition loader code and the actual partition tables.

Install.exe then copies an internally-generated, serialized, Partition image

to the hard Disk. Each Partition image is virtually guaranteed to be unique due

to a 32-bit random number that is seeded off of the internal BIOS clock.

Install.exe, once installed, will never allow itself to be installed on another

system. Further, it will not attempt to remove a partition image that it did

not specifically create. Install.exe will only remove itself from the system

that it was originally installed on. As an added measure of security, if a

password was specified during installation then one will be required for

removal. This strict enforcement is critical since the original Partition table

is stored in the Install.exe file in case the software needs to be removed/upgraded.

Only after Install.exe has completed an Install/Remove cycle can the

Install.exe file again be freely distributed to other co-workers. If you attempt to

run a copy of install.exe that has already been installed but not removed, it

will automatically terminate. To check the state of your copy of install.exe, type

Install S <enter>

This Security application does not modify DOS in any way. It is a BIOS-level

utility that is completely transparent to DOS. This means that when you

Format system diskettes, they are no different than they were before

Install.exe was installed.

This Security utility will allow you to re-partition your Hard Disk,

provided that the Write protect option is disabled. It will also allow you

to Format your Hard disk, provided that the Format protect option is disabled.

Formatting or running Fdisk will have no effect on the Security utility as long

as the system is booted from the C: drive.

When you boot from the Floppy drive, you loose this Write/Format protection.

However, DOS applications can not correctly read the partition sector when

booted from a Floppy.

Fdisk or Format  can be run at any time and will not effect, or be effected by the

presence of this utility.

The security that this utility provides is that the Hard Disk is not accessible

if the system is booted from a Floppy Disk. Provided that the Secure.sys

utility is used, the Hard Disk is virtually safe from unwanted "guests."

Like any form of security, this system, is not completely invincible to the serious

Hacker who has nothing better to do than to break into systems. However, it is felt

that this form of security is adequate enough to protect against break-ins from

the majority of PC users/programmers.

KNOWN PROBLEMS:

Although I have done my best to fully test this product, to prevent any

serious problems, I insist that the best protection is to run Install.exe

from a Floppy drive and keep the file in a safe place(perhaps make several copies

using the following command:

COPY INSTALL.EXE A:INSTALL.EXE /V

In this manner, if there is ever a problem, I could E-mail you a Partition

table restoration utility within hours that would restore your Partition to

its original state.

As mentioned above, the software will not currently run on NCR i486 systems

with SCSI due to an incompatibility problem.

Have Fun and feel free to drop me a line on how you have used this software in your

environment and any suggestions/product enhancements that you would like to see

in a future release.

I can be reached on Compuserve as: "Stuart Macdonald"

Have Fun!

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 3,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

Orders only:

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.