****************************************************************************

*                                                                          *

*       DDDD   IIIIII    A    LL     TTTTTT  OOOO   NN   NN EEEEEE         *

*       DDDDD  IIIIII   AAA   LL     TTTTTT OOOOOO  NN   NN EEEEEE         *

*       DD  DD   II    AA AA  LL       TT   OO  OO  NNN  NN EE             *

*       DD  DD   II   AA   AA LL       TT   OO  OO  NN N NN EEEE           *

*       DD  DD   II   AAAAAAA LL       TT   OO  OO  NN  NNN EE             *

*       DDDDD  IIIIII AA   AA LLLLLL   TT   OOOOOO  NN   NN EEEEEE         *

*       DDDD   IIIIII AA   AA LLLLLL   TT    OOOO   NN   NN EEEEEE         *

*                                                                          *

*           95LX System Manager Compliant (DTMF) Phone Dialer              *

*               (c) Ivo Steklac, Toronto, Ontario CANADA                   *

*                               March 1992                                 *

*                                                                          *

****************************************************************************

DialTone - The HP 95LX Phone Dialer

------------------------------------------------------------------------------

DialTone  is  the  one  simple  utility  that HP/Lotus omitted from the 95LX's

repertoire of built-in applications.  DialTone dials any number stored in your

phone book(s)  by generating  the proper  Dual Tone  Multifrequency (DTMF), or

Touchtone, tones which by playing them into the microphone end of a  telephone

receiver.

Anyone having  seen an  Atari Portfolio  or any  other electronic phone dialer

already knows  that a  Touchtone phone  can be  dialed in  this manner.   This

telephony feature is possible because the DTMF tones are contained within  the

telephone voice  band (0.3-3.0kHz).   The  circuitry within  your phone simply

generates these tones and applies them directly to the phone line.

When I first uncrated my 95LX and tore through the manual (yech!)  I found, to

my disappointment, that this marvel of technology did not have the  capability

of generating DTMF tones.   After hearing its  beeps and moans I  surmised the

unit used a piezo  electric beeper for tone  generation and I gave  up hope of

ever endowing  the device  with this  capability.   Several months later David

Marsh was kind enough to write  and release into the public domain  SND.EXE, a

program that  played digitized  sounds (including  speech) through  the 95LX's

built-in digital to analog converter (DAC) and piezo speaker.

My spirits lifted.  Not only was  this little beauty a pocketable PC -  it was

an improvement on  the original design.   At this  point I conducted  a little

religious ceremony in honor of the "hardware weenies" (to implore a  favourite

of Everet Kaser, who I'm sure is using the praiseworthy connotation of  weenie

as defined in The New Hackers Dictionary) at Corvalis.  After pestering  David

for the source code and clogging  his mail basket with questions about  how to

generate proper sounds I managed to write DialTone.

What is it and what does it do?

------------------------------------------------------------------------------

DialTone is both a  System Manager compliant program  and a DOS program.   The

DOS program is used to calculate the DTMF tones and store them in a data  file

for the  System Manager  version.   The DOS  version also  allows you  to dial

manually entered phone  numbers.  The  System Manager version,  using the data

file created by the DOS version, dials either manually entered phone  numbers,

or phone numbers extracted from the contents of the clipboard.  Obviously this

feature supports pasting phone numbers copied from SYSMGR phone books.

There are several reasons  for having 2 programs.   To generate the  DTMF data

file I needed floating point math which is generally unavailable under  SYSMGR

by any reasonable means (yes I know  both Lotus 123 & HP Calc employ  floating

point math, but I have it  under good authority that they implement  their own

libraries  which  are  inaccessible).    Also  once the Touchtone data file is

created, and fine  tuned, it becomes  a waste of  valuable memory to  drag the

creation code along.

Installation

------------------------------------------------------------------------------

DialTone consists of the following files:

DIALTONE.DOC - this file

DIALTONE.EXE - DOS setup & config file

DIALTONE.EXM - SYSMGR phone dialer

DIALTONE.DAT - sample data file

DIALTONE.ENV - generated by DIALTONE.EXM

The executable  files, DIALTONE.EXE  and DIALTONE.EXM,  can be  copied to  any

drive and  directory.   According to  95LX fashion,  however, the  data files,

DIALTONE.DAT and DIALTONE.ENV, must reside in "C:\_DAT".  Note the environment

file, DIALTONE.ENV, is created by the SYSMGR version in "C:\_DAT".

You may use  PKLITE to compress  DIALTONE.EXE while you  experiment with phone

dialing, but you must not compress DIALTONE.EXM.

Setup & Configuration - DIALTONE.EXE

------------------------------------------------------------------------------

The DOS program "DIALTONE.EXE" is invoked from the command line with the "new"

argument to create the  "DIALTONE.DAT" DTMF data file.   In creating the  data

file, the following questions must be answered:

1. Sampling frequency (in Hz)

2. Cumulative frequency error (in %)

3. Tone duration (in # of samples)

4. Inter-digit pause (in ms.)

The sampling frequency is the rate  at which the program quantizes sine  waves

to generate the primary DTMF tone data.  Frequencies between 23,000 and 24,000

Hertz have worked best for me, although differences between 95LX's and various

phones  may  account  for  other  values  working  better.    Determining  the

appropriate frequency takes a bit of experimentation.  Surprisingly, the  best

method is put a phone against one ear and the 95LX against the other (hold  it

a ways out though - it can be LOUD).

The cumulative frequency error is a percentage figure that denotes a threshold

for the tone generation algorithm.  It is a measure of how close the generated

tone is  to a  pure tone.   Obviously  the lower  the threshold the better the

tonal accuracy.   The telephone specification  can tolerate 1.5%  error.  I've

used figures in the range of 0.20% to 0.50% successfully.  These lower  values

take into account the inherent errors in the piezo electric speaker and in the

free air transmission of the  tones.  Note that the  size of the data file  is

directly proportional to the error threshold.  The smaller the threshold,  the

larger the data file - so  again experimentation will probably yield the  best

results.    To  keep  things  reasonable  DIALTONE.EXE will terminate, without

generating DIALTONE.DAT, if any  one tone exceeds 500  bytes in length.   This

limits the tone file to a maximum of 4,500 bytes.

The tone duration  is given in  samples and denotes  how long each  individual

tone will sound.  The telephone specification calls for a minimum duration  of

50  ms.    In  practice  I've  found  that  the  95LX  must generate tones for

significantly longer than this for them to be recognized.  Durations of  4000

or more samples should  be sufficient.  Note  that the duration does  not have

any impact on the length of the data file.

The last parameter, the inter-digit pause, denotes the duration in ms. of  the

pause that is inserted between subsequent digits.  The telephone specification

calls for a minimum of 50 ms.   In practice I've found that 75 ms. has  worked

well.

The key to determining proper values for these parameters is  experimentation.

Try different phones, home  and work.  I've  found that new electronic  phones

are best, the  older traditional phones  with the carbon  microphones are very

particular.  From these observations it became obvious why HP did not  include

a DTMF dialer in the 95LX - it is simply not possible to provide it in a  form

that "works right out of the box".  If you don't mind fooling around a bit you

can get it to work, if you can't tolerate this kind of hacking, then  DialTone

is not your kind of application.

To assist  the experimentation  effort DIALTONE.EXE  allows you  to dial phone

numbers manually.  This mode is entered after the tone data file is generated,

if the program was invoked with the "new" command line argument, or as soon as

the the program is invoked if  an existing DIALTONE.DAT file is found  and the

"new" keyword  is left  out.   In this  mode the  program excepts phone number

strings similar to those  used in the Hayes  AT command set.   All numbers 0-9

are signaled,  while number  separators such  as "()-"  are ignored.   The ","

provides a pause  for PBX systems.   The pause  is equivalent to  20 times the

inter-digit pause.   In addition to  these, the program  excepts the following

single character commands:

"x":  exits the program

"r":  redials the last number

"l":  same as "r", only closer to the Enter key

"d":  modifies the tone duration

"p":  modifies the inter-digit pause

A final few words about DIALTONE.EXE; the program only runs on an HP 95LX, and

is by no means a robust application.  By this I mean it uses "scanf" and other

such user hostile 'C'  standard library calls -  so be careful, please!   Once

you have the data file configured  you can remove DIALTONE.EXE from your  95LX

to conserve precious RAM.

System Manager Phone Dialer - DIALTONE.EXM

------------------------------------------------------------------------------

The  System  Manager  (SYSMGR)  compliant  program  "DIALTONE.EXM"  makes  use

"DIALTONE.DAT" to dial either manually entered phone numbers, or phone numbers

extracted from the SYSMGR Phone Book application.  As with any other  external

SYSMGR  application   a  reference   to  DialTone   must  placed   within  the

C:\_DAT\APNAME.LST application loader name list file.  To invoke DialTone from

the  C:\_DAT\  subdirectory  with  the  {ALT-Phone} hotkey place the following

entry into APNAME.LST (without the quotes):

"C:\_DAT\DIALTONE.EXM,B700,DialTone"

To register DialTone with SYSMGR save the APNAME.LST file and reboot your 95LX

(make sure to save any open files first), then press {ALT-Phone} to  activate.

After a brief plea for support you'll be presented with the following  opening

screen:

+----------------------------------------+

Application -> |DialTone v1.0      YY-MM-DD hh:mm:ss xm | <- Date/Time in

Status Line -> |                                        |    your format

|========================================|

|                                        |

|                                        |

Screen -> | DDD  III   A   L   TTT  OO  N   N EEEE |

Filler    | D  D  I   A A  L    T  O  O NN  N E    |

| D  D  I  A   A L    T  O  O N N N EEE  |

| D  D  I  AAAAA L    T  O  O N  NN E    |

| DDD  III A   A LLLL T   OO  N   N EEEE |

|                                        |

|  (c) 1992 Ivo Steklac, Toronto CANADA  |

|                                        |

|                                        |

Function Key -> |   Paste    LDC                     Quit|

Definitions  -> |Dial    PBX                             |

+----------------------------------------+

DialTone attempts to follow  the 95LX style of  applications with only one  or

two slight deviations, the most notable being that there is no help, and that

you  can  exit  the  application  using  a  function  key  rather  than a menu

selection.    The  visible  function  keys  provide the majority of DialTone's

functions.

Function key F1 allows  manual entry of a  phone number for dialing;  function

key F2 dials  a phone number  copied from a  Phone Book entry  by means of the

clipboard; function keys F3  and F4 are toggles  that enable, or disable,  the

PBX and  LDC dialing  prefixes; and  function key  F10 removes the application

from memory.  After dialing a number  using either F1 or F2, function key  F9,

redial,  becomes  available  to  enable  you  to  redial the last number.  The

following list summarizes the function keys:

F1 : manual dial

F2 : paste from clipboard & dial

F3 : enable/disable PBX dial prefix

F4 : enable/disable LDC dial prefix

F9 : redial last number

F10: quit application

Function keys F3  & F4 require  a little more  explanation.  If  you work in a

office where you must dial 9 to get an outside line, or you've been in a hotel

where you must dial 8 or 9  to get an outside line, you've experience  using a

PBX, or  personal branch  exchange.   In such  situations you  must prefix the

phone number  you wish  to dial  with a  "9," or  a "8,"  (remember the  comma

provides the necessary delay  for the 95LX to  allow the PBX to  switch to the

outside line before it continues with the rest of the dial string).   Function

key F3 toggles the PBX prefix on and off.  Similarly automatic long  distance

code handling (explained  below) is toggled  with function key  F4.  The  PBX

and long distance code prefixes can be set from DialTone's menu.

Pressing the 95LX MENU key brings up the following screen:

+----------------------------------------+

Menu -> |PBX Prefix  Local LDC  Home LDC  Quit   |

Edit Line -> |                                        |

|========================================|

Status -> |Current DialTone Settings:              |

|                                        |

|->PBX Prefix := 9,                      | <- default values

|->Local LDC  := 416                     |

|->Home LDC   := 416                     |

|                                        |

|                                        |

|                                        |

|                                        |

|                                        |

|                                        |

|                                        |

|                                        |

+----------------------------------------+

The menu behaves  like all other  95LX menus.   The right and  left arrow keys

navigate the menu, ENTER, or the first letter of an option selects the option,

and ESC,  or the  Quit option,  exits the  menu.   The screen  also shows  the

current settings for the  three menu options.   Shown above are their  default

settings prior  to any  modification.   The local  and home  LDC, like the PBX

prefix, are dialing prefixes for long distance codes.  The home LDC represents

the long distance code  of your home town,  the local LDC represents  the long

distance  code  of  the  town  you're  currently  in.  Each option accepts ten

characters, but only recognizes "0-9" and the "," as valid dialing characters.

For clarity DO NOT include any parenthesis, "()", in the LDC prefix codes.

Selecting an option  replaces the menu  with a prompt  message followed by  an

edit line containing the current setting.  Any change is reflected immediately

in  the   status  area   of  the   screen.     The  settings   are  stored  in

C:\_DAT\DIALTONE.ENV according to  95LX convention.   The environment file  is

saved (or created if it does not exist) only upon exiting DialTone (F10).

DialTone employs the following rules when dialing a phone number extracted

from a Phone Book entry (via the clipboard) when LDC prefixes are enabled:

1. If the local LDC is not the same as the home LDC and the phone number to be

dialed does not have any LDC, DialTone prefixes the phone number with the

home LDC. The assumption is that you're away from home and dialing a home

phone number, so you need to include the long distance code.

2. If the LDC of the phone number to be dialed is the same as the local LDC,

the LDC is removed from the phone number. The assumption is that you're

presently in the town to which an LDC would apply if you were at home.

3. Any phone number with a LDC is prefixed with "1-" prior to dialing.

4. If the PBX prefix is enabled all phone numbers are prefixed with this

prefix prior to dialing (the PBX prefix, prefixes the LDC prefix).

NOTE: These rules do not apply to manually entered phone numbers (ie. those

entered via F1).

The  LDC  and  PBX  prefix  handling  also  assumes some basic rules have been

applied in the creation of your Phone Book.  These are:

1. All local phone numbers entered in the Phone Book MUST NOT BE proceeded by

any long distance codes, or other dialing prefixes (such as "9,").

2. All non-local phone numbers entered in the Phone Book MUST BE proceeded by

the appropriate long distance code bracketed by parenthesis, ie. in the

form "(xxx)", but MUST NOT BE proceed with a "1-" prefix.

3. Any numbers not conforming to 1. and 2. above will not be correctly handled

by DialTone, thus it is suggested to disable LDC prefix handling for these

numbers (overseas numbers for example that require longer dial prefixes are

not currently supported).

4. Only phone numbers from the 30 character Number field of Phone Book entries

are currently supported (any numbers found within the address field area

are ignored). Note only the first phone number from this field is

extracted.

Finally to enable quick dialing from a SYSMGR Phone Book, it is recommended

that the following keyboard macro be defined using SYSMGR Setup:

"{F2}{Alt-PHONE}{F2}{PHONE}"

The macro can be bound to any unused  function key (I use F10).  Once set,  it

allows quick  dialing of  the currently  highlighted Phone  Book entry  without

leaving the Phone Book  (well, in principle only,  in actuality you leave  but

come right back to where you left).  Note, if you do not bind DialTone to  the

ALT-PHONE hotkey,  substitute whatever  binding you  used in  the above  macro

definition.

The Bottom Line

------------------------------------------------------------------------------

DialTone (EXE & EXM) and its documentation are copyright (c)1992 Ivo  Steklac,

all rights reserved.  They may not be circulated in any incomplete or modified

form, nor sold for profit, without written permission of the author.

Version  1.0  of  DialTone  is  hereby  released  as  user supported freeware.

DialTone is fully  functional with only  a slightly annoying  plea for support

preceeding  each  invokation  (initial  launching,  and  at  each  SYSMGR task

switch).    If  you  find  DialTone  useful,  and  would  like  to support the

development of 95LX software, please register it.  Single user registration is

a very reasonable $10 US ($15 CDN).  All registered users will receive:

1. A disk copy of DialTone (specify 3 1/2" or 5 1/4" disk) and a certificate

of registration.

2. One free upgrade to the next released version of DialTone.

3. 50% discounts on all subsequent releases of DialTone or any other 95LX

software written by the author.

-----------------------------------cut here-----------------------------------

DialTone v1.0 Registration

Please send all registrations along with a cheque or money order for

$10 US or $15 CDN to:

Ivo Steklac

205 Wynford Drive #2503

Toronto, Ontario

CANADA

M4G 3X1

Name           :

-------------------------------------------------------------:

Address        :

-------------------------------------------------------------:

City           :

-------------------------------------------------------------:

Country        :

-------------------------------------------------------------:

Postal/ZIP Code:

-------------------------------------------------------------:

+-+           +-+

Disk Size      : 3 1/2" | | or 5 1/4" | |

+-+           +-+

-----------------------------------cut here-----------------------------------

Disclaimer

------------------------------------------------------------------------------

You undertake to use DialTone at your  own risk.  The author does not  warrant

the  suitability  of  DialTone  for  any  particular  purpose,  and assumes no

liability for damages of any kind resulting from its use.

Ivo Steklac

CompuServe [70404, 1073]

Canada Remote Systems (CRS)