HamComm

Version 2.1

June 11th 1992

W. F. Schroeder

DL5YEC

HamComm 2.1                Manual              June 11th 1992

1.  Introduction

HamComm  is  a program for ham radio communications.  Version

2.1  supports  reception  and  transmission  of amateur radio

teletype  and  Morse code signals.  A converter or modem chip

is  not  required.   The  audio  output  of  the  receiver is

connected  to  the  serial  port  of  any PC/XT/AT compatible

computer  thru  a very simple and low-cost circuit.  Only one

IC  is  needed  (Op-Amp  LM741  or similar) and a few diodes,

capacitors  and  resistors.  The supply current is drawn from

the  serial port.  For transmission the audio signal from the

speaker  or  serial port is connected to the microphone input

of   the  transmitter  thru  a  passive  R/C  filter.   Audio

frequency generation and decoding, serial/parallel conversion

and all other signal processing is done by the program.

2.  License

HamComm  2.1  is  not in the public domain.  The author keeps

the copyright and all other rights.

You are NOT allowed to:

- make any modifications to the program

- distribute the program without this file

- take more than $10 for distribution including all costs

You ARE allowed to:

- use this program freely for any non-commercial purpose

- make as many copies as you like and give them away

If you like HamComm please send a contribution of $10

or more to the following address:

Django Schroeder

Augsburger Weg 63

D-4790 Paderborn

Germany

- 1 -

HamComm 2.1                Manual              June 11th 1992

3.  Exclusion of warranty

Because  this  program is licensed free of charge, the author

provides  absolutely  no warranty, to the extent permitted by

applicable  state  law.   Except  when  otherwise  stated  in

writing, the author and/or other parties provide this program

"AS  IS"  without  warranty  of any kind, either expressed or

implied,   including,   but   not  limited  to,  the  implied

warranties  of  merchantability  and fitness for a particular

purpose.   The  entire risk as to the quality and performance

of  the  program  is  with  you.   Should  the  program prove

defective,  you  assume  the cost of all necessary servicing,

repair or correction.

In no event unless required by applicable law will the author

and/or  any  other  party  who may modify and/or redistribute

this program be liable to you for damages, including any lost

profits,   lost  monies,  or  other  special,  incidental  or

consequential  damages arising out of the use or inability to

use  (including but not limited to loss of data or data being

rendered inaccurate or losses sustained by third parties or a

failure  of  the  program to operate with any other programs)

this   program,   even  if  you  have  been  advised  of  the

possibility  of  such  damages, or for any claim by any other

party.

4.  System requirements

HamComm  2.1  will  run  under  MS-DOS  3.x  or higher on any

PC/XT/AT-compatible  computer  with  at  least  320KB of free

memory.   A harddisk is not required.  Because of the size of

the  program  file  there may be not enough space to copy all

required  files onto a 360KB diskette.  On slow systems, e.g.

8088  CPU, some functions may not work as expected or may not

work at all.

HamComm  will  automatically detect the type of video adapter

in  use.  MDA, CGA, EGA, VGA and Hercules are supported.  The

graphics  display  functions  are not available on MDAs since

the  MDA  has  no graphics mode.  No attempt has been made to

avoid screen flicker (snow) on cheap CGAs.

HamComm  will probably not run under any kind of multitasking

software like Desqview, Windows or OS/2 since it needs direct

control  of  the  interrupt controller, timer chip and serial

I/O hardware.

- 2 -

HamComm 2.1                Manual              June 11th 1992

5.  Installation

For   installation   on   a  harddisk  you  should  create  a

subdirectory  HAMCOMM  and  CD to that directory.  HamComm is

distributed  as  a  .ZIP or .EXE file.  To unpack a .ZIP file

you  need  a program like UNZIP or PKUNZIP.  The .EXE version

is  a  'self-extracting  archive' that contains all the files

and  a build-in unpack program.  In both cases the files will

be created in the current directory.

Please read the files README, HC.CFG and CHANGES !

6.  Startup

To  start  HamComm  type  HC at the DOS prompt and hit ENTER.

The  interface circuit is not needed if you just want to play

with the program.

HamComm  normally uses the currently selected video mode.  If

you  have  an EGA or VGA card with an EGA/VGA monitor you can

also  switch to 43 or 50 lines.  Try option -L43 to switch to

43 lines on EGA/VGA cards or -L50 for 50 lines on VGA cards.

Many  SuperVGA  cards  have special text modes e.g.  80x60 or

132x44.   Activate  the desired mode before starting HamComm.

Most  cards come with a utility program to do this.  Only the

modes  where  the video display buffer starts at segment B800

are supported.

HamComm  assumes  that  the original PC display character set

known as "codepage 437" is active.

On  startup the program identifies the video card and monitor

in  use  and  automatically  selects  the  graphics  mode for

highest   resolution.    Some  program  functions  require  a

graphics   mode   for  display  of  the  input  signal.   The

identification  may  fail  because of compatibility problems.

By selecting one of the following commandline options HamComm

can be forced to use the specified mode:

Option  Mode      Resolution

-cga    6        640x200  2-color

-ega   10h       640x350 16-color

-vga   12h       640x480 16-color

-herc  Hercules  720x348  2-color

- 3 -

HamComm 2.1                Manual              June 11th 1992

If  you  have  a computer with LCD display and readability is

poor try the following DOS command before starting HamComm:

mode bw80

The  'mode'  program is supplied with MS-DOS and is used here

to switch to black and white mode.

7.  Screen layout

HamComm has an SAA-like user interface with pull-down menues,

dialog  boxes and online helptexts.  The menue bar at the top

of  the  screen  is  always visible if the program is in text

mode.   The  bottom  line has information about the currently

selected menue item.

Every  name  on  the  menue  bar has a highlighted character.

Press  and hold the ALT key and type that character to select

the  corresponding  menue.   The cursor-left and cursor-right

keys  can  now  be used to switch to the previous/next menue.

Use the cursor-up and cursor-down keys to move to the desired

function and hit the ENTER key.

Every  menue  line  also has a highlighted character.  Typing

that character (without the ALT key) will get you directly to

the corresponding function.

Some  frequently  used  menue  entries  have been assigned to

function  keys.   The  F8 key for example activates the SCOPE

function.

The ESC key can be used any time to abort menue selection.

8.  Help system

The  ALT-H and F1 keys both start the help system.  The ALT-H

key  displays  general  information about the program, the F1

key displays a help text specific to the current situation.

- 4 -

HamComm 2.1                Manual              June 11th 1992

9.  Interface schematics

The  interface  schematics  are  included  in the online help

texts.

1. Start the program,

2. Hit ALT-H to start the help subsystem,

3. Hit the letter 'O' to select topic 'Overview',

4. Hit the TAB key to select topic 'Converter' and hit return,

5. Scroll the help text using the cursor-up/cursor-down keys

and read the text.

Hit ESC to leave the help system.

10.  Receive circuit

The  operational amplifier (OpAmp) is used to bring the audio

signal  from  the  receiver  up  to  RS232 level.  The supply

current  is  drawn  from  the  DTR and RTS pins of the serial

port.   The  four  diodes  (1N4148 or similar) of the receive

circuit form a standard bridge rectifier.  The 1uF capacitors

are used for buffering.

The  input  signal amplitude should be at least 100mVpp.  The

100nF  capacitor  removes  any DC bias.  Since the OpAmp runs

with  maximum gain there will be a (more or less) rectangular

waveform  at  its  output.  It should have an amplitude of at

least +/-5V to reliably drive the RS232 input.

The  operational  amplifier  LM741  was choosen because it is

inexpensive   and  widely  available.   However  with  todays

technology its electrical characteristics are not impressive.

An LF356 or TL071 for example is much faster and has a higher

input  resistance.   If  you  want  to substitute a different

opamp  type,  keep  in  mind  that  the  serial port can only

deliver a supply current of a few milliamperes.

- 5 -

HamComm 2.1                Manual              June 11th 1992

11.  PTT circuit

The  RTS  output  of the COM port is not only used to provide

the  supply  current  for  the  opamp,  but  also  to key the

transmitter.   A Diode is used to protect the base of the PTT

transistor  against the negative voltage of the RTS output in

receive  mode.   The  resistor  is required to limit the base

current.   In  transmit  mode  the  RTS  and  DTR pins change

polarity.   RTS  is now positive and the transistor pulls the

PTT  line  to  ground.   This circuit has been tested with an

FT747, FT757GXII and a TS440S.  On some old rigs there may be

a  high voltage on the PTT line.  The required current to key

the  transmitter may also be too high for a small transistor,

so better check BEFORE connecting it to the computer.

12.  Transmit circuit

AFSK  tone  signals  for  transmission  are  available at two

places:

1. at the speaker connector

2. at the serial port.

With  many  motherboards one side of the speaker is connected

to  the  +5Volt  supply  thru  a  resistor, the other side is

pulled  to ground by a transistor.  In contrast to most other

areas  of  the  PC's hardware there is no standard design for

the  speaker  output, so there may be variations.  In general

one  side of the speaker will be near ground or +5Volt, while

the  other side has a square-wave signal.  This signal is fed

to two R/C filters to smooth it out.  The microphone input is

very sensitive so a variable resistor is used for attenuation

and a capacitor to remove any DC.

If  you don't want to make any modifications to the PC or the

speaker  output  is  not accessible, you can also get an AFSK

signal from the TxD pin of the COM port.  The frequencies are

not  as  accurate  as  at  the  speaker output and will often

deviate  by  5  to  10Hz, but this usually gives no problems.

The  signal  level  at the TxD pin is much higher than at the

speaker  output, so you probably have to change the component

values for the R/C filter and the attenuator a bit.

- 6 -

HamComm 2.1                Manual              June 11th 1992

13.  External converter

An  option  "External Converter" is available on the "Keying"

menue.  It can also be controlled by the ALT-F9/ALT-F10 keys.

If  this  option  is  enabled,  a  mark/space  signal from an

external  converter  or modem chip is expected at the CTS pin

of  the  COM  port.  Please note that the normal interface is

still  required.   There  will  be  no reception if it is not

connected  or doesn't get the audio signal from the receiver.

Connect  the  input  of  the  converter  and the input of the

HamComm interface in parallel to the receiver output.

Filter   converter   for   RTTY   are  normally  designed  to

differentiate  between two tones.  For CW reception a kind of

tone decoder is required that can tell the tone from noise.

14.  External AFSK

AFSK  tones  for  transmission  are normally available at the

speaker  output or the TxD pin of the COM port.  If you don't

want  to  use those signals for whatever reason, you can find

an  FSK  signal  at  the  DTR  pin of the COM port.  For RTTY

transmission  DTR  is  negative for 'mark' state and positive

for  'space' state.  In CW mode DTR is negative for 'no-tone'

and  positive  for  'tone'.   During  reception DTR is always

positive.

Note  that in receive mode the RTS pin is negative and always

positive  during  transmissions.  It can thus be used to gate

the DTR signal for keying a transmitter in CW mode.

If  you attach any additional components to these pins please

keep  in  mind  that the DTR/RTS signals supply the power for

the OpAmp.

15.  Signal decoding

The  amplified  audio  signal  is  connected to the DSR modem

status  input  of  the  serial port where every zero crossing

generates  an interrupt.  HamComm determines the time between

successive interrupts using the PCs timer chip and calculates

the   corresponding   tone   frequency.   Due  to  the  timer

resolution  of  about  one  microsecond  the  result is quite

accurate  and  constitutes  the  base  for all further signal

processing.

- 7 -

HamComm 2.1                Manual              June 11th 1992

For  RTTY  decoding  the  tone  is  compared to the currently

selected  center  frequency to decide wether this is a 'mark'

or  'space'  signal.  The mark/space signal is sampled at the

proper time according to the current baud rate collecting all

bits of a character.

For  CW  decoding  the  program has to differentiate the tone

from noise.  To be regarded a valid signal the tone has to be

between  the  currently selected mark/space frequencies for a

certain  amount  of  time.   The program maintains a floating

average  of  the  length  of the dots and dashes to adjust to

varying speeds.

When  the character is complete it is converted to ASCII code

and displayed in the RX window.

16.  Macro keys

A  macro  key  can  be  used  to replay a previously recorded

sequence  of  keystrokes.   The  digits on the top row of the

keyboard  are  used  as  macro  keys  while  the  ALT  key is

depressed.    They  are  normally  located  right  above  the

QWERTY...   row.   Do not confuse these keys with the numeric

keypad!

In  the  beginning  there  are  no keystrokes assigned to the

macro  keys, so pressing ALT-1, ALT-2...ALT-9 or ALT-0 has no

effect.   To  start  recording,  select item 'Macro recorder'

from  the  FILE  menue.  A message box will appear confirming

that  recording has been started.  When the ENTER key is hit,

the  message  box  will  go  away  and  from now on up to 250

keystrokes  will be recorded along with normal processing.  A

blinking  "! RECORDING !" message is shown at the lower right

corner of the screen to remind you that the recorder is still

active.

Now hit one of the macro keys to stop recording and to assign

the  keystroke  sequence to that key.  This also replaces any

previous  assignment.   A  message box will appear to confirm

that  macro  recording  has terminated.  Every time the macro

key  is  hit the recorded keystroke sequence will be replayed

just like it had been typed again.

To  terminate  recording  without changing any macro key just

select 'Macro recorder' again from the FILE menue.  A message

box  will  appear  to  confirm  that macro recording has been

canceled.

- 8 -

HamComm 2.1                Manual              June 11th 1992

To  clear  a  macro  key start recording in the usual way and

directly  after  that  hit the key to clear.  This will erase

any  previous  recording assigned to that macro key.  Again a

message box will confirm the result.

17.  Macro files

Macro  definitions  can be saved to a file by selecting "Save

macros"  from the FILE menue.  The usual file select box pops

up  with  a suggested .MAC file type.  All recorded keystroke

sequences will be saved to the selected file.

To load a .MAC file select "Load macros" from the FILE menue.

All  macro  definitions  found  in  the selected file will be

added to or replace the current assignments.  Macro keys that

didn't  hold any keystrokes at the time the file was written,

will not be changed when the file is loaded.

There  is  also  a  way  to automatically load your favourite

macros  definitions.  When HamComm is started it searches for

the   default   macro  file  HC.MAC,  first  in  the  current

directory, then in every directory along the PATH.

18.  Scope function

In  RX  mode  the tone decoder routine calculates the current

signal  frequency for use by other parts of the program.  The

SCOPE  function  uses  this  value  to display a graph of the

input  frequency  vs.   time.   This  is the blue line on the

SCOPE screen.

The tone decoder also maintains a floating average frequency.

The  effect  is very similar to a low-pass filter and is used

for  noise  reduction.  The result is displayed at the top of

the screen as a red line.

The  solid  green  line  marks  the currently selected center

frequency  and the dotted lines above and below show the mark

and  space  tones.  For RTTY reception the center line should

be  in  the  middle between the mark and space tones.  For CW

reception  the signal should match the center frequency.  All

signals  outside  the  area  marked  by  the dotted lines are

ignored.

- 9 -

HamComm 2.1                Manual              June 11th 1992

The display can be controlled with the following keys:

F1  display help screen

F10  activate menue bar

ESC  same as F10

'+'  toggle display of decoded signal

'*'  toggle grid display

'B'  toggle position of red line

HOME  reset sample rate to 1ms

PG-UP  sample rate faster

PG-DOWN  sample rate slower

ENTER  hold display immediately

The  right  mouse  button holds the display at the end of the

current sweep.  The left mouse button activates the menue bar

just like F10.

The  red  line  (input  signal  after  low-pass)  is normally

displayed  at the top of the screen to not interfere with the

blue  line.   In its second position it will overlay the blue

line to compare the two signals.

Display  of  the decoded signal is normally disabled.  It can

be found at the bottom of the screen and shows the mark/space

states for RTTY and tone/no-tone states for CW reception.

With  some  practice it is quite easy to determine the signal

you  are  listening  to  by  watching the SCOPE display.  CW,

AMTOR   ARQ  and  FEC,  Packet  Radio  and  RTTY  show  their

characteristic  patterns and you can also watch unstable VFOs

sweeping slowly across the band.

19.  Spectrum function

On  entry  to the Spectrum function a graphics mode according

to the video card and monitor in use is activated.

Every  millisecond  the  current  input frequency is checked.

For  every  frequency  there  is  a  counter and the one that

corresponds  to  the  current  frequency is incremented.  All

counter  values  are  displayed  as  vertical lines with each

sweep  from  left to right.  The higher the count, the longer

the  line.   What  you see on the screen is therefore not the

true  audio spectrum but the result at the output of the tone

decoder  routine.   An  opamp  and  a few lines of code can't

substitute a real spectrum analyzer.

- 10 -

HamComm 2.1                Manual              June 11th 1992

When set to 1Hz per pixel SPECTRUM has the best resolution of

all display functions.  For example, the actual shift used by

an RTTY station can be determined quite accurately.

The display can be controlled with the following keys:

F1  display help screen

F10  activate menue bar

ESC  same as F10

'-'  toggle raw/filtered data

'+'  toggle 'shaddow' (color displays only)

'*'  toggle dot/line mode

HOME  reset, start at 200Hz, 3Hz per pixel

PG-UP  zoom in

PG-DOWN  zoom out

LEFT  lower frequencies

RIGHT  higher frequencies

SPACE  clear display

ENTER  hold display

The  right  mouse  button holds the display at the end of the

current sweep.  The left mouse button activates the menue bar

just like F10.

If  you  have a slow PC then don't use the SPECTRUM function,

you  might  get  frustrated.   There  simply isn't enough CPU

power  to keep the display in motion.  AT class maschines and

real fast XTs should be ok.

The   SPECTRUM   function   has   been   found  to  be  quite

entertaining, especially with a color display.  Next time you

got  visitors  in  your  shack who don't know what's going on

just  start  this  function,  turn  the  lights low and sweep

across a crowded band.

- 11 -

HamComm 2.1                                 Table of Contents

Table of Contents

1 Introduction  ........................................... 1

2 License ................................................. 1

3 Exclusion of warranty ................................... 2

4 System requirements ..................................... 2

5 Installation  ........................................... 3

6 Startup ................................................. 3

7 Screen layout ........................................... 4

8 Help system ............................................. 4

9 Interface schematics  ................................... 5

10 Receive circuit  ....................................... 5

11 PTT circuit  ........................................... 6

12 Transmit circuit ....................................... 6

13 External converter ..................................... 7

14 External AFSK  ......................................... 7

15 Signal decoding  ....................................... 7

16 Macro keys ............................................. 8

17 Macro files  ........................................... 9

18 Scope function ......................................... 9

19 Spectrum function  .................................... 10

i

HamComm 2.1                                             Index

AFSK output................6

CGA........................2

Converter, external........7

Converter..................5

CW.........................8

Desqview...................2

EGA........................3

ESC........................4

Exclusion of warranty......2

External AFSK..............7

external converter.........7

F1.........................4

Floppy.....................2

Function keys..............4

Harddisk...................2

Help system................4

Installation...............3

Interface schematics.......5

Introduction...............1

Laptop.....................4

LCD........................4

License....................1

Macro files................9

Macro keys.................8

Menue bar..................4

Multitasking...............2

Operational Amplifier......5

PTT circuit................6

Receive circuit............5

RTTY.......................8

Scope function.............9

Screen layout..............4

Selection..................4

Signal decoding............7

Speaker....................6

Spectrum function.........10

Startup....................3

SuperVGA...................3

System requirements........2

Transmit circuit...........6

VGA........................3

Video adapter..............2

Windows....................2

XT.........................2

i