Trantor Systems, Ltd.

5415 Randall Place

Fremont, CA  94538

510-770-1400

510-770-9910 FAX

510-656-5159 BBS

June 16, 1992

These drivers are for Trantor T128, T228, T338 and T130B SCSI host

adapter cards and OS/2 2.0.  Earlier versions of OS/2 are not supported.

These drivers are Copyright 1992 by Trantor Systems, Ltd.  All rights

reserved.  They may be freely distributed to owners of Trantor products

as long as this documentation file is included with the files.  Latest

revisions of these files can be obtained from the Trantor BBS system or

they may be ordered on disk from the Trantor office for a small charge.

Files on this release:

README.DOC          This file

T128SCSI.ADD        T128 / T228 driver

T13BSCSI.ADD        T130B driver

T338SCSI.ADD        T338 MiniSCSI driver

T128SCSI.DDP        T128 / T228 Install script

T13BSCSI.DDP        T130B Install script

T338SCSI.DDP        T338 MiniSCSI Install script

All files are being distributed in a self-extracting archive file called

OS2-TSL.EXE for use with BBS systems.

AUTOMATIC INSTALLATION

Copy all the above files to a floppy.  This floppy is now a device

support diskette.  Insert it in A: and run DDINSTAL from the OS/2

prompt.  The Device Driver Install icon can be found by selecting the

OS/2 System icon on the desktop, then the System Setup icon in that

window.  Insert your floppy in a disk drive and double click on the

Device Driver Install icon.

MANUAL INSTALLATION

The desired driver must be copied to the OS/2 2.0 directory:

COPY A:*.* C:\OS2

This driver should be installed in the config.sys file as follows:

....

BASEDEV=T338SCSI.ADD

BASEDEV=OS2DASD.DMD

IFS=C:\OS2\CDFS.IFS /Q

BASEDEV=OS2SCSI.DMD

....

All of the above drivers are necessary for CD and HD support. They

should be inserted in your CONFIG.SYS.  Some of these drivers

might already be in your CONFIG.SYS file.  All drivers except for

TxxxSCSI.ADD are placed in the OS2 directory during the initial OS2

installation.

OS2DASD.DMD is already in the CONFIG.SYS, but the other drivers may not

be there.  Be sure you add them either manually or with the OS/2

Installer.

The OS/2 .ADD driver we supply is fully compatible with any driver that

interfaces with the .ADD specification for OS/2 2.0.  It will work

transparently with any hard disk supported by OS/2.

OPTICAL DRIVES:

IBM supplies a driver called OPTICAL.SYS for use with their 3.5"

Magneto-Optical drive and other Magneto-Optical drives.  We have

confirmed that it works correctly with this .ADD release.

In addition, we have implemented an optional command line switch that

will enable some Magneto-Optical drives to be treated like a hard drive

when OPTICAL.SYS is not available.  Adding the command line parameter of

/o to the .ADD driver forces the operating system to treat an MO drive

as a standard hard drive.

CD-ROM DRIVES:

IBM recommends using CDROM.SYS (which is supplied with OS/2 2.0) with

the /i option for non-IBM CD-ROM drives.  This driver may not support

audio commands on all CD-ROM drives.  IBM is planning to release an

updated driver that supports audio commands for most CD-ROM drives.

Please contact IBM for more information on their CD-ROM support and

future drivers.

To support CD-ROM drives that are not supported with the /i option of

CDROM.SYS, we have implemented a command line switch of /t in our .ADD

that emulates the IBM (Toshiba) CD-ROM drive and allows standard data

read commands to work with most SCSI CD-ROM drives.  NOTE: Audio

commands are not supported and may cause unpredictable results with some

CD-ROM drives.

Add the command line parameter in the following manner:

BASEDEV=T338SCSI.ADD /t

In order to have a CD-ROM drive show on your desktop with a CD-ROM icon,

you must have a CD-ROM disc in your drive when the system is booted.  If

you boot with an empty CD-ROM drive, the drive icon will appear as a

floppy disk instead of a CD-ROM disc.  This will not affect the

operation of the CD-ROM drive itself.

NOTES:

- Removable media is supported. It was tested with a SyQuest drive.

- OPTICAL.SYS provided by IBM will support MO drives.

- option /o supports MO drives by treating them like HD's.

- option /t supports non-IBM CD-ROM drives for data only by

emulating the IBM CD-ROM drive.