Chips and Technologies

DS8221 EMM.SYS EMM Driver Documentation

I.Introduction:

This document describes the MS-DOS device driver

NEATEMM.SYS, when in conjunction with a Chips and

Technologies 8221 "NEAT" ChipSet provides support for LIM

EMS 4.0

II.  Device Driver Installation

This EMS driver is supplied as a device driver that can be

loaded at DOS boot time.  The user is required to edit the

CONFIG.SYS file so that it contains a line in the form of:

DEVICE = d:[path]\EMM.SYS [options]

where d: and [path] represent the drive and path where the

driver is located.

The driver is designed so that, under normal circumstances,

it does not need any command line options in order to give

satisfactory performance.  However, command line options are

provided so that the driver may be tailored to a user's

specific needs.

Before installing the driver, you should run your system

SETUP and make sure to set aside sufficient memory for your

needs.  This memory should be designated as expanded memory.

III. Command Line Parameters

The following command line parameters are available for

knowledgeable users who wish to tailor the driver for their

own requirements.  Option names may be shortened to their

first two characters.  Options should be separated by one or

more spaces.  Spaces are optional before and after the '-'

and '=' symbols.

FRAME = aaaa

Specifies the page frame address (the address of

the 64K segment through which expanded memory is

mapped.  "aaaa" must be either C000, C400, C800,

CC00, D000, D400, D800, DC00, or E000.  Specifying

FRAME will override the automatic choice of page

frame by the driver.

EXCLUDE = xxxx [- cccc]

Specifies an area of memory which should not be

under the control of the EMM.SYS driver. "xxxx"

and "cccc" specify the address or range of

addresses of memory to be excluded and should be in

multiples of 16K (i.e. end in 00).  EXCLUDE may be

used several times on the driver command line to

exclude multiple blocks of memory, though at least

64K of memory should remain above 640K (A000).  The

driver will find all RAM/ROM in the range D000 -

EFFF, and will adjust for them.

INCLUDE = xxxx [- cccc]

Specifies memory to be included under the control

of the EMM.SYS driver.  Parameters are similar

to EXCLUDE above.

HANDLES = hhh

Specifies the number of handles and names

available.  "hhh" must be in the range of 16 to

255, with the default being 64.

IOADDR = aaa

Overrides the default base I/O address at which the

"NEAT" ChipSet should be configured.  aaa must be

either 208, 218, 258, 268, 2A8, 2B8, or 2E8.

DIAGS = xxx

Specifies whether EMS memory diagnostics are performed

during initialization of the driver.  "xxx" must be

ON or OFF.  The default is OFF.

IV.  Error Messages

"Initialization Error. Memory Manager not Installed"

This is a generic error message that is accompanied

by another error message.  This means that for some

reason (given by the other error message) the EMM

driver could not be installed correctly.

"Cannot detect mapping hardware"

The EMM driver could not find a Chips 8221 "NEAT"

ChipSet in the system.  Make sure that your EMS is

enabled.  This error may indicate that you have the

wrong driver for your system.

"Problems encountered with memory initialization"

The EMM driver could not set up the page frame

correctly.  Make sure that there is at least one

contiguous 64K segment of unused memory space above

640K (A000).

"Interrupt 67 Vector already allocated"

Some other driver has already allocated the EMS

interrupt vector.  Remove all other EMS drivers

from your CONFIG.SYS file.

"I/O address specified is not one of the permitted

values"

The EMS mapper I/O address you indicated using the

IOADDR command line option is an invalid address.

Make sure that it is either 208, 218, 258, 268,

2A8, 2B8, or 2E8.

"An equals (=) sign was expected"

One of your EMM.SYS command line options was

not immediately followed by an equals '=' sign.

Check your CONFIG.SYS file to make sure the

EMM.SYS command line is formatted correctly.

"Invalid HEX number or not divisible by 16"

One of your INCLUDE or EXCLUDE "xxxx" command line

arguments is incorrect.  Each "xxxx" argument for

these options must be divisible by 16 (ending in

00).

"Number of handles invalid or out range"

The value you specified for the HANDLES command

line option is invalid.  Make sure it is a decimal

number between 16 and 255.

"Invalid Page Frame Address"

The page frame address you specified is invalid.

The page frame must start at either C000, C400,

C800, CC00, D000, D400, D800, DC00, or E000.

"No working memory can be found. EMM aborted"

The driver cannot find any memory set aside as

expanded memory.  Make sure to run your system

SETUP, and set aside some memory for expanded

memory.

"Unrecognized command/option in command line"

The string displayed after this error message was

found to be invalid.  Edit your CONFIG.SYS to make

sure this invalid option is corrected.

"End address is less than starting address"

One of the "cccc" addresses specified for either

the INCLUDE or EXCLUDE options is less than the

"xxxx" value for that option.

"The command line is incomplete"

Options are incomplete on the EMM.SYS command

line.  Make sure that all command line options are

correctly formatted.

"Problems encountered in setting default page frame

address"

The default page frame address found in your system

CMOS is invalid.  This is not a fatal error, if

this occurs the driver will use the default base

address.  Make sure to run your system SETUP and

make sure that your EMS base address is valid.

"Decimal number input overflow error"

The number of handles specified is an invalid

number.  Edit your CONFIG.SYS and make sure that

the parameter for HANDLES is correctly formatted.

"Error EMS I/O not enabled"

EMS has not been enabled on the Chips 8221 "NEAT"

ChipSet. Run your system SETUP and make sure EMS

has been enabled.

"The EMS hardware is disabled or not functional"

This is a generic error message that is accompanied

by another error message.  This means that for some

reason (given by the other error message) the EMS

hardware is not operating properly.