TN210-9.DOC

SYSOP COMMAND LIST

**********************

Besides the USER command listing, there are  a  special set of commands for

SYSOP use.  To be able to use these,  one must be recognized  by  the node as a

SYSOP.    The method for doing this is  to  answer correctly  the random set of

numbers given in response to a (S)YSOP command as described previously.

INFO  (I) - Allows the SYSOP to enter text into the soft coded INFO section

on the  node.  The INFO section has available a maximum of 160 characters.   Up

to  80  characters  per line  are allowed.    To use,  type:  "I  (space)  text

<enter>"  for  the  first line.   If one prefers NOT having text appear on  the

first  line,  then type "I (space) (Control G) <return>."  This gives a  "beep"

along  with  a  blank  first  line.  For text to be  entered  on  the  2nd  and

subsequent  lines,  type:   "I (space) + <text> <return>" until the  total  160

character limit is reached.

An example for setting INFO:

SYSOP action:  I (control G) <return>

Node response:  SVATST:NJ7P-4}

SYSOP action:  I +Sierra Vista, AZ

Node response:  SVATST:NJ7P-4}

Sierra Vista, AZ

SYSOP action:  I +145.01 MHz

Node response:  SVATST:NJ7P-4}

Sierra Vista, AZ

145.01 MHz

SYSOP action:  I +CARA elections coming up soon, be prepared to help!

Node response:  SVATST:NJ7P-4}

Sierra Vista, AZ

145.01 MHz

CARA elections coming up soon, be prepared to help!

SYSOP action:   I +BISBEE node will soon QSY to 145.11 MHz.

Node response:  SVATST:NJ7P-4}

Sierra Vista, AZ

145.01 MHz

CARA elections coming up soon, be prepared to help!

BISBEE node will soon QSY to 145.11 MHz.

This process can be repeated until the maximum of 160 characters, including

non-typing  and  punctuation,  has  been reached.  To  erase  the  entire  INFO

section,  just type:  "I <Control G>."  If the INFO section is frequently to be

changed,  an  INFO  file  could be prepared in  advance  with  the  appropriate

commands and text included.  After SYSOPing the node, the file can be uploaded.

Another  advantage  of preparing the file in advance is it doesn't  require  as

much on-the-air time.

While  160  characters are  available,  good  network  management  practice

dictates only enough text is used to get the  message across.   Although it may

seem  "distinctive"  to fill the remainder with spaces, line feeds,  or Control

G's,  please  don't  do it.     Users  find  such  practices  annoying  plus it

unnecessarily congests the network.

Examples of INFO text and format are:

WTEX:W5CDM-3}

Located at Notrees, Tx.  Ant 600 ft. courtesy of WD5THB & W5CDM...

RDG:WA6YNG-1> From 18 Miles West of Redding

On Shasta Bally at 6200 Ft. - 145.05 Mhz.

With a Motorola Micor 60 watt radio on battery and float supply.

YKA:VE7RKA>

Greenstone Mountain Packet System

Kamloops, B.C.

SysOps- VE7EHL - VE7EJE

Mark your calendars, Nov 22, Packet meeting at the Lotus, C U There...

KEY  TONE  (K T) - Operates the PTT line of the TNC and turns on  the  Mark

(low) tone or the Space (high) tone for approximately 22 seconds.

KEY  DIDDLE (K D) - Operates the PTT line of the TNC and alternates rapidly

between Mark and Space for approximately 22 seconds.

The  purpose of the "KEY" commands are to make the NodeOp's job of  setting

deviation  and  RF  frequency  much easier.  Previously  it  was  necessary  to

reinstall  the original TNC firmware chip and perform the CALIBRA procedure  to

set FM deviation.    Now, if the NodeOp has appropriate equipment and is within

radio  range  of  the node,  he can  routinely  check  frequency  and deviation

remotely.  At the site, this same procedure can be done via the HOST mode port.

Once  entered,  there is NO way the KEY  command  can  be terminated  until the

internal timer runs its course.  If the TNC-2  watchdog timer  is  set for less

than a 22 second duration, the watchdog will unkey  the PTT.  However, the node

will  continue the  KEY  command until the remaining time has expired.   During

this period, the node will not execute any other commands.

Due  to the  nature  of  the  SIO chip  in the TNC-2,  it's necessary it be

"toggled"  to switch  from high to the low tone, or vice versa.   This was done

in the  TAPR  modem alignment  procedure by tapping the  space bar.   Of course

this isn't feasible when remotely SYSOPing and some experimentation is required

if one wants to change tones.    One  way would be to perform a K T command and

if  the  tone  is  not  the  one desired, do a "CQ" and repeat the K T command.

Oftentimes one or more  "CQs" will toggle to the opposite tone.  This procedure

is also required in HOST SYSOP mode.

ON - OFF - A simplified remote control capability as compared with the HIGH

LOW commands  found in TheNet version 1.0.  "ON" turns on the CON LED and "OFF"

turns  the  LED  off.   In the MFJ 1270B, the voltage sense from  the  CON  LED

appears on pin 8 of the DB-25 connector.  The voltage sense also appears on the

base  of Q16, a 2N3904, the output of which goes to pin 2 of the TTL connector.

The  main caution here is that the controlled device  not draw  more than a few

milliamps either source or sink.   In the MFJ-1270B, pin 2 (NOT-DCD) of the TTL

connector  is low  when  ON and  high when OFF.

A solid state  SPST  relay such  as the Teledyne p/n 611-2  can switch up to

25 amps at 250 VAC.   It requires a DC control  of 3 - 28  volts.   In the MFJ-

1270B,  replace Q16  with a 2N3906.    Replace  R103  with  a 220 ohm  1/4 watt

resistor.   On the bottom of the board add a 470 ohm 1/4 watt resistor  between

the  base of Q16 and the ground tab of radio port connector J2.   These changes

will  source 4 volts at 4 milliamps  through pin 2 of the TTL connector.    The

input  of the relay is connected between pin 2 (+) and pin 3 or 6 (-) on the J5

TTL  connector.    This satisfactorily  switches the relay and provides correct

"sense" for the CON LED.  It will glow when the relay is ON and be extinguished

when  the  relay  is OFF.    Other relay types may  require a transistor buffer

between  pin 2  and the controlled device.    As with any higher voltage wiring

projects,  be sure to comply  with applicable  electrical safety  practices and

codes.

PARMS  (P) - Allows the NodeOp to make changes, as necessary.  To use, type

"P space  parameter  number space new value <enter>" to change any of the parms

to a new value.   For example, to change parameter 14 from a "0" to a "1," type

"P 14 1" to  effect the change.   Parameters are  numbered  starting  with "1,"

from left to right.   Multiple parameter changes in one command are disallowed.

NOTE: The above is a new procedure with version 2.09/2.10.

LOCKING NODES - Occasionally a need may arise to modify node entries in the

NODES table.  The SYSOP command for this is:

N [LOCKED NODECALLSIGN] + ALIAS QUALITY OBS PORT [LOCKED NODECALLSIGN]

For example:  N K7WAA-1 + CAHTO 0 0 0 K7WAA-1

Assuming  the  path  to  neighbor node CAHTO is  unreliable,  good  network

management  practices would dictate CAHTO be locked out.  In the above example,

setting  the  QUALITY to 0 will prevent CAHTO from appearing as a direct  path.

Setting the Obsolescence count to 0 makes CAHTO be listed as a permanent locked

entry.   If  the Obs count was set to a value between 6 - 1, this locked  entry

would  then  be  deleted  at  a   time  as  established  between  Parameters  4

(Obsolescence counter) and 6 (Broadcast interval).

To manually unlock CAHTO, the command is reversed:

N K7WAA-1 - CAHTO 0 0 0 K7WAA-1

Here  one  will  note,  the minus  sign is  used for this purpose.    It is

essential  the  unlock  command  be  the  same  as the lock  command,  with the

exception of the minus sign.

Setting  the  Obsolescence to zero permanently locks the  destination  node

into the nodes  table.   Even if the locked node fails, it will still be listed

in the nodes  table.   For this reason, the locked NODES procedure is not often

used as it's counterproductive network practice to broadcast failed nodes.   It

should  be  noted  this  procedure  only  works  with  "Net"  nodes  capable of

generating valid Level 4 Transport Layer responses.  If one attempts to connect

from the node to a locked  TNC or a  KA-node,  one will initially see a connect

made.   But when TheNet fails to receive a return Level 4 acknowledgement after

two or three interrogations, the circuit is disconnected.

The LOCKED NODES command to use if a node should NOT have an alias is:

N [LOCKED NODECALLSIGN] + * QUALITY OBS PORT [LOCKED NODECALLSIGN]

Usage example: N AK7Z-1 + * 143 0 0 AK7Z-1