ZipNews v0.97 (5/08/93)

(c) Copyright 1993 by J.C. Kilday Associates

Developed at the Northern Lights BBS

207-761-4782   v.32bis/VHST

Table of Contents

-----------------

Introduction ..................... 1

Description ...................... 2

Installation Overview ............ 3

Installation Details ............. 4

Operating/Using ZipNews .......... 9

Distribution and Restrictions .... 9

After You Register ZipNews ...... 10

Support ......................... 10

ZipNews v0.97                       Page 1

Introduction

------------

ZipNews is a door for use on PCBoard, Wildcat!, and Waffle BBSs.  When

operating on a Waffle system  it is run as Waffle  "external command".

It's purpose  is fast  delivery of  Usenet news  articles for  offline

reading.  It greatly simplifies the process of reading Usenet news  by

allowing the caller  to download new  postings in compressed  form for

reading  offline.    Callers  may  download  their news in plain ASCII

format, compressed using an archiver of their choice.  But the primary

function of this door is to provide news "packets" for offline reading

through use of the ZipNews Reader.  The ZipNews Reader is a  companion

program to the door and usually  may be obtained from the same  source

as this door.

The  ZipNews  door  allows  a  broad  selection of DOS-based archiving

utilities for compressing the news packet and it supports a variety of

file transfer protocols (use of Forsberg's DSZ file transfer  protocol

module  is  recommended).    The  selection of archiving utilities and

protocols  is  determined  by  the  system  operator.   The individual

command  line  specifications  are  also  established  by  the  system

operator using a configuration text file.  A sample configuration file

is included in this package.

First-time users entering the  ZipNews Door are prompted  to establish

their personal choices of archive utility and file transfer  protocol.

User choices are maintained  by the system and  may be changed by  the

user during subsequent calls.  First-time users of the door on PCBoard

and Wildcat! systems are  prompted to choose an  Internet ID of up  to

8-characters in length.  The door does not allow a caller to choose an

Internet ID  that is  already in  use by  another caller  to the  same

system.  On Waffle-only systems,  users are registered per the  normal

Waffle conventions.

During their  first use  of the  ZipNews door  on PCBoard and Wildcat!

systems, users are  prompted to download  a list of  newsgroups within

their first  packet.   If the  caller is  using the  DOS-based ZipNews

Reader,  newsgroup  selections  and  settings  of  last  article  read

pointers are established offline, and the results are uploaded back to

the door.  Callers to Waffle-only systems may also use this  procedure

(first-time  callers  are  not  automatically  prompted  to  do  this,

however).

If the caller is NOT using the ZipNews Reader, when accessing the door

on other-than-Waffle  systems (such  as PCBoard  or Wildcat! systems),

there is presently no provision  for selecting newsgroups.  Perhaps  a

friendly sysop could help such  callers by modifying their JOIN  files

via a standard  text editor.   The reader is  required, when accessing

such systems, for selecting newsgroups (unless the sysop is willing to

lend a hand).  On Waffle  BBS systems or on systems where  Waffle runs

as  a  door,  however,  callers  without  ZipNews readers, may use the

normal Waffle commands to join the desired groups.

ZipNews v0.97                       Page 2

When download of a news packet is requested, the user is presented  an

active display indicating the newsgroups being scanned, the number  of

articles  being  extracted,  and  other  progress information.  At the

conclusion of the scan, the user  has the option of NOT accepting  the

packet and returning to the Waffle system to alter groups JOINed, etc.

Once the  user affirms  that an  assembled packet  should be sent, the

packet  is  compressed  and  the  transfer  of  the  resulting file is

initiated.  If the normal  process is interrupted before a  successful

conclusion, the "last article read"  pointers in the user's JOIN  file

are restored to their previous settings.  Otherwise, upon a successful

download,  these  pointers  are  appropriately  updated.  (Note:  this

operation  is  dependent  upon  the  archiver  and  protocol   modules

returning appropriate "success/fail" DOS ERRORLEVELs.  Utilities  such

as PKZIP and DSZ are known to work correctly in this regard.)

ZipNews is SHARE-compatible and can support multi-node operations from

a single configuration file (ZN.CFG).  It can handle non-standard comm

port specifications and speeds  to 57,600 bps.   FOSSIL communications

are not used.   ZipNews disables FOSSIL supported  comm when run.   On

Waffle systems,  FOSSIL support  is "reset"  per the  normal way  when

returning  to  Waffle.     On  other   systems,  ZipNews  leaves   the

communications set up  such that the  FOSSIL can resume  upon when the

door exits back to the main board.

ZipNews  is  being  distributed  as  a  fully  functional package (not

crippled in any way).  In its unregistered form, ZipNews DOES  display

on-screen,  and  in  the  files  produced,  notices  informing  system

operators and users that it is an unregistered, evaluation copy.   The

executable  is  self-registering  in  that  when  a valid registration

number  is  input  these  notices  are  removed  and,  thereafter, the

registrant's name is displayed.

The complete ZipNews v0.97 package includes the following files:

ZIPNEWS.DOC  - this file

ZN.EXE       - executable module

ZN.CFG       - sample ZipNews configuration file

ZIPNEWS      - the Waffle external command file for ZipNews

ZNORDER.FRM  - order form for registering ZipNews

READ.ME      - quick review and features summary

WHATS.NEW    - major enhancements and bug fixes in this

release.

ZIPNEWS.PCB  - sample ZipNews door batch file for use on

PCBoard (and Wildcat!) BBS systems

NLBBS.GPS    - Sample newsgroups file

Description

-----------

ZipNews is  designed for  operation on  MS-DOS 3.O  and later systems.

Operation with earlier DOS versions is not supported.

ZipNews is being released for  widespread use while in its  final BETA

test stages.  Operation of the door is very sound, but there are  some

original requirements  of the  package that  are not  yet implemented.

Until such requirements  have been completed,  it is planned  that the

ZipNews door will remain a product in BETA testing.

Zipnews has been tested  primarily in a DOS  5.0/DESQview environment,

on a system  running 4 nodes  (PCBoard as the  primary BBS and  Waffle

1.65  as  a  Door).    It  has  not been tested with other versions of

Waffle.

ZipNews v0.97                       Page 3

Installation Overview

---------------------

This section attempts to suumarize the major steps in setting up.  The

next section covers installation in greater detail.

ZipNews requires the installation of  at least portions of the  Waffle

BBS package if you are not running the full Waffle BBS.  The  required

parts of  Waffle are  basically the  files necessary  to implement the

uucp transport mechanism and to establish the structure and control of

newsgroups and mailboxes.  This document is not intended to serve as a

substitute for Waffle documentation or as a guide in its installation.

Suffice  it  to  say,  that  the  Waffle  docs and configuration files

pertaining  to  the  following  executables  are  needed for the basic

(non-Waffle BBS) setup:

UUCICO.EXE

COMPRESS.EXE

UUX.EXE

UUXQT.EXE

BATCH.EXE

RMAIL.EXE

RNEWS.EXE

The first two files listed above are actually public domain  programs,

but the  remainder and  the needed  docs are  copyrighted by  Darkside

International, and require a registration fee of $30 payable to Thomas

E. Dell.  The  complete Waffle package can  be obtained at most  large

BBSs or via anonymous FTP from major Internet archive sites under  the

name WAF165.ZIP (the name as of this writing).

Once Waffle (or Waffle uucp operation) is installed:

1. Place ZN.EXE and the sample ZN.CFG into a directory in your PATH

(\waffle\bin will do quite nicely).

2. Customize ZN.CFG as needed using an ASCII text editor.  This also

involves naming and specifying command lines for archive utilities

and file tranfer protocols you wish to make available to callers.

3. a. For ZipNews as a Waffle external command:

Make (or modify) the ZIPNEWS external command file and place

it in your \waffle\external directory.  See the sample external

command file included in this package -- filename is ZIPNEWS.

b. For ZipNews as a door under PCBoard or Wildcat!:

Using the file ZIPNEWS.PCB as a guide, set up the necessary

door batch file for your system (possibly multiple files if

running multi-node).

4. Provide for setting the required DOS ENVIRONMENT variables.  A very

important one is the DSZPORT variable.

5. If running a multi-node system, provide for uniquely named work

directories (one for each node) for temporary file storage.  This

will require an environment variable also.

ZipNews v0.97                       Page 4

6. If running a multi-node system, set the read-only (write protect)

file attribute bit on ZN.EXE (avoids SHARE violations while

program loading during normal multi-node operations).

Installation Details

--------------------

ZipNews (ZN.EXE) should be placed in a directory contained in your DOS

PATH (e.g., \waffle\bin).    Your ZipNews  configuration file (ZN.CFG)

should be placed in the same directory.  WARNING:  ZN.EXE locates  the

directory in which it resides, no matter how started, and you  MUST(!)

have ZN.CFG located in that same directory for proper operation.

If you are running ZipNews as a Waffle external command, the  external

command file should be quite similar to that provided as part of  this

package.  It should be  placed in your \waffle\external directory  (on

Waffle  v1.65  systems).    An  example  of this one-line file appears

below:

zipnews /local /reset /command="d:\waffle\bin\zn %A"

Note that the only command line parm required on startup is the Waffle

account (caller's ID).

If you are running ZipNews as  a door under PCBoard or Wildcat!:   the

sample door batch  file contained in  ZIPNEWS.PCB should be  used as a

guide in setting up the  necessary door batch file(s) on  your system.

The sample file shows how a multi-node system might be set up using  a

single  door  batch  file  (through  use of PCBoard's door environment

variables).

Also it should be noted that  two methods of providing user access  to

ZipNews on PCBoard systems are  documented in ZIPNEWS.PCB.  The  first

is now  the preferred  method and  the one  that MUST  be employed  if

setting up for use with Wildcat! systems.  We'll get into the  details

of the two methods shortly.

Here's the syntax for invoking the ZipNews door:

zn @UserListName | Username [local] [pcb | wc]

Where:

zn is the executable name

@UserListName indicates a list is specified, and UserListName

is the full pathname to the list of ZipNews door users (created

and maintained by the door).  This option, rather than the

alternative specification of UserName is the recommended method

for invoking the door from PCBoard or Wildcat!.

UserName is the Internet ID, or the Waffle "username" of the

caller (for whom a directory of that name is maintained on the

system containing personal configuration data).  This option is

the required method for invoking from Waffle as an external

command.

(You must specify either the user list path prefixed by the @

symbol OR the username when invoking the door.)

ZipNews v0.97                       Page 5

"local" may be specifed for sysop testing of the setup without

having to go online to test.   (Note: new user registration

processes are not functioning well in local mode in v0.97 BETA;

but all other online door operations are pretty well emulated in

the local mode.)

"pcb" or "wc" must be specified as the 2nd or 3rd parm (depending

on whether the "local" parm was given) if the door is NOT being run

as an external command under Waffle.  (pcb for PCBoard; wc for

Wildcat!).

Examples:

zn @c:\user\zn_users.lst local pcb

Above, the system operator invokes the door locally to simulate

operation as a PCBoard door, and points it to the list of valid

users of the door located in c:\user .   (By the way, that list

of users will be searched for a match against the caller name

extracted from PCBoard's drop file, PCBOARD.SYS.   If a match is

found, the caller's Internet-style login ID is extracted from the

file.  If a match is not found, the caller will be handled as a

first-time ZipNews user.)

zn @c:\user\zn_users.lst wc

Similar to the first example, but here the door is invoked in

online mode (comes up as a communications application) and

looks for the Wildcat! door drop file, USERINFO.DAT.  It will

extract the caller name from line 1 of this file.

zn johndoe pcb

This is an older form of invoking the door that was the only one

available for operations with PCBoard before v0.97 BETA.   It is

still available for any PCBoard operators that might prefer it,

OR be unwilling to change their current setup.  The method

requires the caller to invoke the door by following the PCBoard

door command with his/her Internet-style login ID, thusly:

zipnews johndoe    OR   door 1 johndoe

More will be said about this method next, but it is no longer the

recommended method for invoking ZipNews.

With basic understanding of the  door's command syntax out of  the way

let's return to the door batch file (the sample being ZIPNEWS.PCB) and

discuss how the door is accessed.  The line invoking the door uses the

preferred method  of accessing  the door  from PCBoard  (and the  only

method for Wildcat!) in  that the first parm  points a file that  is a

list of users.  This file in d:\users\zn_users.lst per the example  is

completely maintained by the  door.  If no  such list exists when  the

door is first installed, it is created when the door handles its first

caller.  When a caller uses  the ZipNews door for the first  time, the

door adds the caller's BBS name and requested Internet-style login  ID

to the list.

ZipNews v0.97                       Page 6

The ZipNews users list is in standard ASCII text format.  A few  lines

of the list list might look like this:

JOHN DOE jdoe

MARY DOE maryd

SYSOP jkilday

The words in all caps on each line are the full caller name  extracted

from the door drop file (PCBOARD.SYS, USERINFO.DAT, etc.).   Separated

by a space from  the full caller name,  and in all lower  case, is the

Internet-style ID which the caller chose during his/her first  session

with the door.

IMPORTANT NOTE:

If you are upgrading from a prior release of the ZipNews door

used as a door under PCBoard, OR if you are adding ZipNews as

a PCBoard  or Wildcat  door in  a system  that also  has been

running Waffle as a door, there is an important task you must

perform in order to allow former users of the door (or  users

of the Waffle system) access  to this version of the  ZipNews

door.   You must  add your  current Waffle  door users to the

ZipNews users list manually with a text editor!

Reason:    Beneath  the  "user"  directory  specified in your

ZN.CFG file (and the same  directory used by the Waffle  BBS,

if used on your  system) individual callers' directories  are

created  to  contain  personal  setup and status information.

When ZipNews identifies  a new, first-time  user of the  door

(caller name not  on the ZipNews  users list), the  door asks

the "new user" to specify their desired login ID.  THE CALLER

IS NOT  PERMITTED TO  USE ANY  ID THAT  CORRESPONDS TO A USER

DIRECTORY ALREADY IN EXISTENCE.

Therefore, any users of a Waffle system already installed  on

your  system  (or  of  a  ZipNews  door  formerly  used under

PCBoard)  must  be  added  manually  to the new ZipNews users

list.   Otherwise, they  will NOT  be granted  access to this

implementation of the door with their current or former ID.

If you are operating ZipNews as  a door under PCBoard, you may  choose

to set up access by your callers  in a different way.  This method  is

ONLY available for PCBoard systems,  and was used in earlier  releases

of ZipNews.   If you examine  the ZIPNEWS.PCB file  (sample door batch

file provided with this package,  you will see a door  invocation line

that has been commented out.  It makes use of the PCBoard  environment

variable named PCBDOOR (and whose  value is denoted in the  batch file

by %pcbdoor%).  This method requires your caller to invoke the door by

following the  door command  with his/her  login ID,  thusly:  zipnews

johndoe ; OR door 1 johnndoe ; OR open 1 johndoe

This form of access is similar to that used for invoking from  Waffle.

But, if used for PCBoard (with the "pcb" parm on the command line) the

door does  not unconditionally  allow access  even if  it finds a user

directory corresponding to the login ID.  It must also find a file  in

that directory named PCB.ID and that file must contain the caller name

as  contained  in  the  PCBoard  door  drop file, PCBOARD.SYS.  So, if

johndoe logs in under this access method,  ZipNews would  verify  that

ZipNews v0.97                       Page 7

the PCB.ID  file in  the johndoe  user directory  contains  the single

line  containing  "JOHN  DOE"  (assuming  this  caller  name  was   in

PCBOARD.SYS).  This form of addressing scheme works well,  and handles

first-time user logins  just as effectively  as does the  ZipNews user

list method (using  the @users.lst approach),  but it does  force your

callers to always enter their ID  following the door command.  Not  as

"seamless" perhaps as the newer  method, but just as effective.   This

method remains  available in  ZipNews to  accommodate those  who might

have set it up using earlier releases on their PCBoard systems.

To configure ZipNews to your system, use a standard ASCII text  editor

to modify the ZN.CFG provided.  The basic ZN.CFG format rules are:

1) Lines beginning with a space or a # symbol are ignored.

2) The 12 configuration subject titles listed below MUST be

present in the finished .CFG file and each must be followed

by a colon (:).  There MAY be intervening whitespace.

newsgroups

user

temporary

bbs

abbrev

maximum

archivers

protocols

formats

organization

hostname

list

3) The lines may be in any order, but the sub-lines for archivers

and protocols must be grouped within their respective sections,

and with the "prompt" lines immediately preceding the

corresponding command line specifications.

4) The command lines for the protocols contain one character in

the first position which is not a part of the command line.

This is the "selection character", that by which a caller may

express his/her protocol choice.

Additional conventions used in the ZN.CFG file follow.

If you need to specify more than one top level news directory on  your

system, indicate the  full pathname to  each (without a  trailing "\")

and with a space separating each pathname.

If you have  sufficient RAM available  for use of  a RAMDISK, such  is

heartily recommended -- not only  for speed gains, but for  preventing

wear and tear on your hard drive.   However, you may name a hard  disk

and directory  in the  "temporary" parameter  as your  work directory.

This is the directory in which the extracted news articles are  placed

and compressed for download.  If you are running a multi-node  system,

you will need to specify multiple temporary work directories (one  for

each node) using this parameter . Here's how to do it:

ZipNews v0.97                       Page 8

1) Set an ENVIRONMENT variable for each node.  In each node's

ENVIRONMENT a new variable should be defined, NODE, for example.

Assign the number of the node to each one as required.  Thus

in the ENVIRONMENT controlling node 1, you would set NODE=1,

for node 2, NODE=2 , etc.

(On PCBoard systems you could use the standard PCBNODE variable,

and on Wildcat!, WCNODEID, etc.)

2) Specify your "temporary" parameter such that it contains

the variable name with a % before and after it (just as in

DOS batch files).   So you might specify in your ZN.CFG line:

temporary  :    g:\work%node%

This would be interpreted by ZipNews running on node 1 as

g:\work1, on node 2 as g:\work2, etc.

This method  will allow  you to  run any  number of  BBS nodes  from a

single copy of ZN.COM and ZN.CFG.

It is recommended  that Forsberg's ubiquitous  DSZ protocol module  be

used to support file transfers.  In fact, ZipNews has not been  tested

with any other.  Normally protocol modules must have the comm port and

speed communicated to them when invoked.  DSZ automatically determines

the speed, so this  spec is not necessary.   To specify the  comm port

number, or to  specify a non-standard  comm port, you  set the DSZPORT

environment variable.  (Set DSZPORT=1, for COM1:, for example.  Or for

a non-standard port at  hex address 3e8 and  using IRQ 5, it  would be

necessary to specify DSZPORT=3e8,5.  See DSZ docs for further info  as

necessary.)

WARNING:  If  you do not  SET DSZPORT=SomeCommPort, ZipNews  will exit

with an error message.   The DSZPORT setting  is required even if  you

opt to use a different protocol module.

(A future release of ZipNews may allow specification of port and speed

via substitutable parameters,  such as %1,  %2, etc.   If DSZ is  used

with  this  release,  however,  capability  for handling substitutable

parameters is not necessary.)

Another required environment  variable for the  running of ZipNews  is

the WAFFLE variable.   But you're all using  that already, right?   :)

ZipNews requires that the WAFFLE variable be present, but the  present

release does not actually access your STATIC file.

If you're running a multi-node system, there's one last step necessary

for reliable operations.  Switch to the directory in which you  placed

ZN.EXE  and,  using  the  DOS  ATTRIB  command or other file attribute

setting/resetting utility, set the  READ ONLY attribute bit  ON (write

protect).  This will  prevent SHARE violations during  program loading

under normal multi-node operations.

This completes the installation process.

ZipNews v0.97                       Page 9

Operating/Using ZipNews

-----------------------

There is not much more to say re:  operations; ... the first time  you

fire up ZipNews, it will be pretty obvious how it is used.  You should

be reminded of the local mode of operation.  You might start it up on

a Wildcat! system like this:

zn @d:\waffle\user\zn_user.lst local wc

The above assumes you are PATHed to the directory in which you  loaded

ZN.EXE and that  there is a  USERINFO.DAT (Wilcat! door  drop file) in

the current directory.

In local mode you may operate the door pretty much as you would if  it

were online.   There currently are  two areas of  exeception.  If  you

request a download, a ZipNews  packet (not compressed) may be  created

and left in the work directory (specified by "temporary" in the ZN.CFG

file).  This should prove helpful in testing parts of your setup.  The

second area is the new user login process, the prompting for new  user

ID,  etc.    This  functional  area  of  the code not been finished to

operate well in local mode in this release.

Re:  advising your users in use of the ZipNews door and its output  --

ZipNews' ASCII text  format can be  viewed quite conveniently  using a

text viewer such as Vern Buerg's LIST utility.  If, for example, you'd

like to quickly move from one  message to another, set LIST to  search

for "Subject:".  LIST nicely highlights the found line.  To search for

the next occurrence with  LIST, hit the "A"  key.  To quickly  scan to

reach the next newsgroup, use "* * <" as the search argument, etc.

Some of your callers might be confused as to what first to do with the

news packet received.  The source of confusion stems from the use of a

characteristic file extension of "ZNS" for the compressed news packet,

rather than ZIP or ARC, etc.  Most unarchiving utilities will work  on

a valid archive when supplied the  full filename and extension.  If  a

caller  has  a  ZipNews  packet  created  with PKZIP, for example, the

unarchiving command would be:

PKUNZIP yourbbs.zns  (where "yourbbs" represents the name you have

selected for the "abbrev" parm in ZN.CFG)

Distribution  and  Restrictions

-------------------------------

ZipNews is made available to you as a fully functional package.  It is

distributed as SHAREWARE.   Should you  decide to use  ZipNews on your

system you are obliged  to register the product  for a fee of  $19.95.

Registered users are provided support via the BBS mentioned at the end

of this documentation OR via e-mail on the Internet.  You may use  the

form ZNORDER.FRM for registering your copy.

You are welcome to distribute this package to others so long as it  is

unmodified and distributed in its entirety, and it is not included  or

bundled  with  other  goods  or  services  for which a fee is charged.

Exceptions are  that ZipNews  may be  distributed by  bulletin boards,

Internet archive sites, or other information services even though they

receive  fees  to  access  their  downloadable  files,  or  by library

services so long as the fee for the diskette on which this package  is

contained is not more than $6.50.

ZipNews v0.97                      Page 10

Use  of  ZipNews  beyond  a  brief  evaluation  period by individuals,

business entities, corporations, and government agencies is prohibited

without payment of a registration fee of $19.95 per copy.

After You Register ZipNews

--------------------------

Once your registration and check have been received, you will be  sent

a  registration  number  (via  U.S.  mail  OR Internet e-mail, at your

option).  You may use this  number to convert to a registered  copy of

ZipNews which will  eliminate displays of  "unregistered", "evaluation

copy", etc., and it will insert  your name in displays and text  files

as having duly registered the product.

To  make  a  registered  copy,  invoke  ZipNews  with the command line

parameter of --REGISTER, thusly:

zn --register

You are  prompted to  enter your  name and  your registration  number.

That's all there is to it.

(IMPORTANT NOTE:   If  you are  running a  multi-node system  and have

followed the  suggestion of  setting the  file attribute  of ZN.EXE to

READ ONLY,  you will  have to  set it  back to  normal before invoking

ZN.EXE in registration mode.  Once you have completed the registration

procedure, you should set ZN.EXE back to READ ONLY mode.)

Support

-------

Support is available to registered users of ZipNews primarily  through

the main message base of the Northern Lights BBS which can be  reached

at the number given below.

Registration fees may be made payable to:

J.C. Kilday Associates

P.O. Box 1961

Portland, ME 04104

The author  may be  contacted at  the above  address, or  via Internet

e-mail (jkilday@nlbbs.com),   or  at the Northern Lights BBS  (address

messages to "Sysop")  at 207-761-4782.   The BBS operates  24 hours, 7

days.  Calls accepted at 14400 (v.32bis or VHST) or below.