(translation by Alain Bourassa VE2MTV QTH:SHAWINIGAN P.Q.)

HOW TO USE TPK WITH BAYCOM ?

*********************************

Version  931116

*************************************************************************

* This  version  void and replace all prior versions                    *

* We suggest that you destroy any prior version in your possesion       *

* some files are new or improved                                        *

* This version 931116 suppress the resident files when you leave TPK    *

* Good packet qso with TPK and Baycom !                                 *

*************************************************************************

1) Face to a rushing need of info on how to use TPK with a Baycom

modem, here's a documentation that will help you to start a new

possibility in packet radio: using TPK without a TNC.

You will have to use the 1.81 version of TPK if you want that all

TPK functions be correctly activated.

The next TPK version programmed by F1EBN and F6FBB will be available

somewhere near the end of 1993. You will have to wait some more time

if you want to enjoy the latest improvements of TPK.

But the method show  here is already working.

2) The Hardware

Attention, it seems that some PC XT and compatible are not supporting

TPK-baycom no problem where found on AT class machine...

We suppose that you already have a working Baycom modem

You will replace the TNC by your Baycom modem.

That's all!

3) Here is the list of files that of will copy in your TPK rep.

TB1.BAT et TB2.BAT   (the .bat files are new!)

TFPCX.EXE

TNCDED.EXE   (new file, replace TPKDED!)

BTPK.COM

(Those 3 files are to add in your c:\tpk)

Copy also TPK-BCOM.DOC in your doc rep like,

C:\TPK\DOC

4) Starting TPK TPK

First you will need to reinstall TPK because the driver to use is no

TPK but INIT14.

In the case where tPK is already insatlled,keep it and reinstall TPK.

in another rep

In both case validate INIT14 and continu normal installation to the end

of the process

Voici le texte d'un .BAT qui vous permettra de comprendre la sequence

permettant de demarrer TPK.

Here is an example on a commun case using a mouse on com 1 and TPK

on com 2.

ECHO Start TPK in baycom mode on com 2

Rem    Don't change the order of the files!

Rem

@ECHO ON

CD C:\TPK

TFPCX -PCOM2

TNCDED 2

BTPK

Rem    Suppression of the resident files.

TNCDED 2 /U

TFPCX -U

@ECHO OFF

CD C:\

CLS

Type TB1 <RC> if you use com 1

Type TB2 <RC> if you use com 2

The starting of the software is beginning. There is a chance that

you get an error message on DCD or COMx : Don't take care  of that.

5) The inconvenients

They are smalls!

All the particularities of TPK are working.

But since there is no more TNC there is no more digipeater !

(translator note from VE2MTV: read the read.me file to correct

that problem I found the solution myself !)

If you area is well equiped in digi stations then there is

no problem

HAVE FUN WITH TPK-BAYCOM !

�����������������������������������������������ͻ

For the doc:    � 73's QRO de Claude, F5CDC @ F6FBB.FMLR.FRA.EU �

�����������������������������������������������ͼ

�����������������������������������������������ͻ

For the diffusion:    � Amities  de Alain , F5NUF @ F6FBB.FMLR.FRA.EU �

�����������������������������������������������ͼ