SET COMBASE=nnnn

Specifies the I/O port base for the active COM port in decimal

format. COM1 is usually 1016, COM2 is usually 760.

SET COMIRQ=n

Specifies the IRQ level for the active COM port in decimal

format.  COM1 usually uses IRQ 4 and COM2 uses IRQ 3.

For com ports other than the normal COM1 and COM2, you will need insert

the following statements in your BOARD.BAT file:

SET COMBASE=nnnn    ;port base I/O address (decimal)

SET COMIRQ=n        ;port IRQ setting

Use these SET statements in conjunction with the com port setting

that's in your PCBOARD.DAT file, if your running PCBoard 14.x.