Intermodulation Analysis

WHAT IS INTERMODULATION?

Intermodulation (IM) occurs whenever one or more radio frequencies

are combined in a non-linear device. This produces sum and difference

frequencies which are known as IM products.  The non-linear device may be

a transmitter, a receiver, an isolator, a bad connector or any number of

other items.  Sometimes the condition is desirable, as in the mixer stage

of a superheterodyne receiver where signals are combined and selected to

produce the familiar "Intermediate Frequencies" (I.F.).  Most of the time

however, intermodulation is undesirable and steps must be taken to prevent

or eliminate the conditions which give rise to intermodulation.  Such

measures usually include the use of circulators and cavities to prevent

signals from entering the final amplifier stages of transmitters, and

the use of band-pass cavities at the inputs of receivers.  These factors

are of special concern at mountain top repeater sites.

The number of receivers and transmitters at most repeater sites

can vary anywhere between a dozen to a few hundred.  The number of

intermodulation products which can be generated could be in the thousands.

A site manager may be able to exert total control over the conditions

at a particular site, or only nominal influence over the activities of

other users of the site.  In either case, a site manager faced with an

intermodulation problem would be faced with the formidable problem of

tracking down the one combination out of possibly tens of thousands

which might be the source of the problem.

This program is designed to make that task easier.  Even though the

number of intermodulation products is theoretically infinite, only a

limited number of them are significant for a given set of receivers.

The products which we are mainly interested in are only those which

will fall on or near a receiver frequency, and are of low enough order

to be of significant amplitude.  What does "order" mean?  The general

scalar form of the equation for intermodulation products is:

For two transmitters, Tx(1) and Tx(2);

F = nTx(1) + mTx(2)  and...

F = nTx(1) - mTx(2)  | n = 1,2,3... where m = n+1

The order of the product is defined as m+n.  These are merely the

coefficients of the terms of the equations.

As you can see, when n is 1, m is 2 and the order is 3.  The order

of the product is always odd.  And since the frequencies involved are

really phasors, the sign of F is related to its phase and not its

amplitude so for purposes of analysis we can ignore it.*

*    A phasor is a vector which rotates about its origin with time.  The

intermod program uses absolute value to eliminate the signs.

The accompanying program to this document calculates intermodulation

products for, two and three-transmitter combinations and second and third

harmonics of single transmitters.  Two-transmitter combinations of 3rd,

5th and 7th order, and since the coefficients are all 1, 3rd order

products of three-transmitters.  The equation for three-transmitter

products is:

F = Tx(1) + Tx(2) - Tx(3)

We artificially limit the order of the products to 7th because as

the order increases, the amplitude of the product signal decreases and

therefore becomes insignificant.  The program would take forever

to finish calculating an infinite number of products.

Once the products of interest have been calculated its merely

a matter of sorting through the data to see which products fall on

which receiver frequencies and print the results.  Since the program

incorporates a sort function, we can get a list of the "hits" in

ascending frequency and finding out which transmitters might be

candidates for "most likely suspect" in our search.

ABOUT THE PROGRAM

This program was originally inspired by a simple intermod program

written in BASIC for the TRS-80 Model I by Dennis Hanley.  The source

code for INTERMOD 2.3 is QuickBasic 4.0 and with advanced features like

recursion, makes it possible to code this program in ways not possible

with ordinary BASIC.  Believe it or not, there is only one line number

in the whole program.  The program was developed over a period of several

months and was capably critiqued by Dennis Fandrich, Abel Montes, and Jon

Maudal.  Many of the features of the program were suggested and tested by

several trial runs on their various systems and data.  The program is

capable of managing up to 1000 transmitters and 1000 receivers

simultaneously.  It automatically tests for the presence of a math co-

processor, and uses it if present.  If a co-processor is not present

it uses the built-in floating-point emulation.

INTERMOD.EXE runs on any PC/XT or AT compatible running MS-DOS 3.0

or higher, with or without floating-point processor support.  However,

if you plan to use large arrays (more than 100 transmitters and receivers)

or use INTERMOD on an XT, then use of a coprocessor is strongly

recommended.

The program has only one menu and commands are all one-letter,

single keystroke commands based on the first letter of each menu item.

The program will prompt for other information as each category requires.

Hitting the ENTER key without the required data terminates the command

and returns to the main menu.  Where information appears in angled

brackets, e.g.; <N>, it specifies the default value the program will

use when ENTER is keyed at the prompt.  In the ADD and DELETE command

categories, ENTER is used to terminate data entry, and if you are in the

transmitter data sub-category the command proceeds to the receiver data

sub-category.

THE COMMANDS

A    Add data to the transmitter/receiver arrays.  Enter each

frequency one at a time.  As data is added the entire array

is displayed and the user is prompted for more input.

C    Change Color.  Toggles the display between white-on-blue and

white-on-black.  Useful if you have monochrome monitor on

a color adaptor or a LCD display and want more contrast, or

simply want a change of scenery.

D    Delete Frequencies.  Used to delete frequencies from the

transmitter and receiver arrays, transmitters first, then

receivers.  Searches the current array for a match to the

entered data, deletes it if found and re-displays the entire

array.  Enter the frequencies one at a time.  ENTER terminates

data entry in the current array.

E    Exits the program.  If data in the arrays has been altered

the program asks the user if the data should be saved.

F    Files.    Display a DOS style directory, with pauses.

Allows pathnames, wildcards etc.  Prompts before returning

to main menu.  Useful for checking for data file names

without exiting INTERMOD.

G    Gateway.  Executes COMMAND.COM as a child process.  Typical

DOS gateway.  Type EXIT at the DOS prompt to return

to INTERMOD.

L    Load file.  Loads an INTERMOD data file from disk.  Prompts

user for filename, if none entered, returns to main menu.

If file not found or other error it returns with description

of error.

N    New Data.  The usual way to get new data into the program prior

to saving or printing it.  Prompts user for the site name

and date before entering the "add data" mode.

O    Order.  Performs Quick-Sort of data in transmitter and

receiver arrays.  Really fast.  Do this before printing

the products to get consecutive output.  Sorts in ascending

order.

P    Print Products.  Prints the products to the screen, line

printer and disk if selected, the user can direct the output

to the printer or to a disk file or both.  INTERMOD calculates

two-transmitter products first, then three-transmitter, then

harmonics.  Allows user to select which products to calculate,

default state is all products.  Prompts user for the search

bandwidth, this tells INTERMOD how wide the hit "window" will be.

Default window is 15 kHz.  Prompts user for choice of form-feed

or continuous output if printer or disk is selected.

S    Save data.  Saves the INTERMOD data file to disk.  Prompts

user for filename.  Same protocol as Load.

V    View data.  Allows user to view the data in memory without

risk of altering it.  Displays 140 frequencies at a time.

Prompts at end of each page of data.

F1   Aborts a product printout in progress.

OPERATING TIPS

Keep the number of receivers in any one file as small as

possible, enter only those receivers which you are the most

interested in.  The number of products increases very rapidly as

the number of transmitters increases, multiply that by the number of

receivers and a single run can take several hours.

Send the output to a disk file.  This allows you to print

the entire output file using the DOS print command instead of

sending it to the printer as the program calculates.  If this

isn't a problem, send it to both printer and disk.  The output is

identical whether sent to the disk or to the printer.

The actual receiver frequency is included at the rightmost column

of the output.  This makes searching for a target receiver fairly easy

since the receivers are listed numerically in each category.

If the printer is not ready when the INTERMOD tries to use it

an error message is displayed.  Pressing ENTER at the prompt causes

the program to continue from the error.  Pressing F1 before ENTER tells

INTERMOD to return to the main menu.  If the printer is on-line and

printing products, pressing F1 causes INTERMOD to return to the main

menu immediately.

Use consistent filenames when naming data files and output

files.  Something like SAMPHILL.DAT for input data files and

SAMPHILL.OUT or SAMPHILL.PRO for the product files.

Group the transmitters and receivers by building.  The strongest

products will most likely be generated nearby.  It's easier trace the

products outward from the victim as well.

Remember, the presence of a product in the list is not an

indication that the product actually exists.  INTERMOD produces a

list of POTENTIAL products and the most likely sources given the

proper conditions.  Only careful interpretation and testing can

prove the correctness of the result.

LICENSE AND CONDITIONS OF USE

You are free to use, copy and distribute this product provided:

NO FEE IS CHARGED FOR USE, COPYING OR DISTRIBUTION OF

THE PRODUCT.

The program and the documentation must not be altered in any way.

This program is provided AS IS, without warranty, expressed or implied,

including but not limited to suitability and fitness for a particular use.

If you find INTERMOD to be a useful and convenient product, a

contribution would be appreciated, send a contribution of $20.00 to:

Geoffrey L. Joy

12964 Smoketree Pl.

Chino, CA 91710

Contributors will be placed on a list of "most favored users" and

will be notified promptly of any upgrades or improvements.