Users

-----

For someone to be able to use the modem port, you MUST have given them a

password.  Do an EU and use W to set the password.  The user can change

it using the NP command over the modem port.

Modem Settings

--------------

The default settings are:  'T E0 V1 X1 Q0 S7=60 S0=0'

Do not change E0, Q0, V1, X1, or S0.  All other settings are fair game.

You can make any changes automatic by putting them in the

INITIALIZE_FILE (see PARMS.DOC)

Modem switches

--------------

You should set your modem to hangup and enter command mode when DTR goes

low.  On older modems, this is a hardware switch.  On newer modems, it

is a software switch (ATSxx=y type).

Some users have reported intermittent crashes during initialization with

certain modems.  A suggested fix is to

ECHO AT&C1&D2 > COMx

before bringing up the BBS.  The above line can be put in a BAT file.

Substitute a real com number for the "x" above.  This fix will only work

with COM1 and COM2.

User settings

-------------

Recommended user setting for their terminal programs are:

- Half duplex or Echo on -- Program must supply echo of what is being

typed.

Forwarding

----------

A typical forwarding script might be:

PATH AA6 B AA4RE

CT*70,8483865

R*call sign*

SAA4RE

R*password

SMYPASS

Note that this represents a change from previous versions.  The CONNECT

message is now taken care of by the code.  The system will also change

speed based on the connect message.