PCROUTE

Version 2.210

Users Manual

September 16, 1989

by

Douglas E. Ehlers

Copyright (C) 1989

2

Table of Contents

Notice of Copyright.......................................3

Introduction..............................................4

Installation..............................................5

System Requirements..................................5

Software Setup.......................................5

Input/Output Menu.........................................6

Board Definition Menu.....................................8

Chip Definition......................................7

Chip Placement.......................................8

Interactive Router.......................................13

Autorouter...............................................16

Board Statistics.........................................17

Printer Menu.............................................18

Exiting the program......................................18

Keyboard control summaries...............................19

Brief Tutorial...........................................20

Order Form...............................................22

3

Notice of Copyright

This program  is protected  by a  copyright held  by me  and

filed with the  United States Government.  Although  this program

is being distributed  as Shareware, that  does not make it  free.

You are granted  limited use of this  program to test it  out and

see if it will  fulfill your needs.  If you  find that your needs

can be satisfied  by this program and  you decide to use  it, you

are obligated to pay  the 65 dollars to register your  copy.  You

are allowed  and  encouraged to  give  unaltered copies  to  your

friends to try out.  This same agreement applies to them.

4

Introduction

PCROUTE  is  a  computer aided  design  program  for printed

circuit boards.  It contains board layout routines that should be

quite  for  most purposes.    It  also contains  a  full featured

autorouter that averages around  a 93% completion rate.   It also

supports three of the more popular printers for its output.

This program is  to my knowledge the first of its type to be

released as shareware.  Although it lacks several of the features

of  similar  products that  are  commercially available,  it also

lacks a $1000 price tag.

Key features include:

1.   Autorouting

2.   Interactive routing

3.   Board layout functions

4.   Flexible pinouts for devices

5.   2X output for use in making PC boards

6.   Disk functions built into the software

7.   Sophisticated component and connection

entry systems

8.   General statistics

9.   Up to 300 components and 1800 connections

10.   The authors continued support

5

Installation

Contents of Archive/Disk:

PCROUTE .EXE -  The main program

PCROUTE .DOC -  The documentation for PCROUTE.EXE

HPLASER .DAT -  Printer data for the HP Laserjet II

SG10    .DAT -  Printer data for the IBM PC printer

PAN1124 .DAT -  Printer data for the EPSON LQ-2500

SMALL   .CDT -  Sample board that can be edited and routed

DIP24V6 .DEF -  24 PIN DIP chip definition-vertical .6 wide

DIP20V3 .DEF -  20 PIN DIP chip definition-vertical .3 wide

DIP40V6 .DEF -  40 PIN DIP chip definition-vertical .6 wide

PAD     .DEF -  Definition of a single pad

2PAD50V .DEF -  2 PADS 0.5 inches apart-vertical

works well for 1/4 watt resistors

2PAD15V .DEF -  2 PADS 0.15 inches apart-vertical

2PAD10V .DEF -  2 PADS 0.10 inches apart-vertical

PRINTDOC.BAT -  Prints the documentation stored on disk

System Requirements:

To run this program requires:

1. CGA or better graphics

2. 640k of memory (must all be available)

no other memory residents can exist as they may

lock up the system, you'll  have to experiment

on this part.

3. 1 floppy disk drive

4. One of the following printers:

a. IBM PC or compatible printer

b. EPSON LQ-2500 or compatible printer

c. Hewlett Packard Laserjet II or compatible

5. Hard-Disk (not essential but recommended)

A fast processor is also nice.

Software Setup:

If  you are  going to run  this program  from a  floppy disk

system, simply copy all the files that came in the archive  or on

the  distribution  disk onto  an empty  formatted floppy  disk by

using the following command:

COPY A:*.* B:

Where A: contains the  original PCROUTE disk and B:  contains the

empty formatted  floppy disk.   All  the files  that came on  the

diskette are required to  run the program except the  PCROUTE.DOC

file.  The PCROUTE.DOC file may be deleted from the run disk.

If you  will be using a hard disk to run the program, make a

directory  in  which to  place the  program.   Use  the following

6

commands to accomplish this:

1.   cd \

2.   md pcroute

3.   cd pcroute

4.   copy a:*.*

Where A: holds the original PCROUTE files.

For PCROUTE to  run reliably, you must  also have a line  in

your config.sys file setting  files=20.  If you are  running most

any major applications program this  command probably exists, but

you should check.   To execute  the program type PCROUTE from the

DOS prompt.

Input/Output Menu

1. Load Data:

This  function  is  used  to  retrieve  from  disk  the

necessary  information to  design a  printed circuit  board.

Select this function from the menu  and when prompted, enter

the file name.  No  extension is used, the program  will add

the  .CDT extension for you.  The  drive and path should not

be used  on the filename itself.   There is a  function that

will be  discussed later  to set  the drive  and path  name.

After entering the filename, press Return.  The program will

ask if this is the correct name, enter Y or N to answer.  If

the file is  not found you will have a chance to reenter the

file name.  You can also use the ESC key to jump back to the

main menu instead of entering the file name.

2. Save Data:

This function will  save a complete description  of the

design layout, component list, and connection list.  It does

not  save the information  used for  deleting traces  in the

interactive  router  which will  be  discussed later.   When

prompted, enter the filename.  Again do not specify the file

extension, drive designation,  or path  name.  PCROUTE  will

ask after entering the  file name if everything  is correct,

enter a  Y or N.  If the file exists the program will ask if

you want  to overwrite  the file.   If you  say no,  it will

prompt for a new file name.  ESC also works to return you to

the I/O menu instead of entering the file name.

3  Write Autocad file:

This function will  write a  file that can  in turn  be

read into and edited with Autocad.  The  process is one way,

you can not  read an autocad  file back into PCROUTE.   Call

this  function  from  the I/O  menu  and  give  a name  when

prompted.  Here as usual, specify no path or file extension.

7

The extension  .scr will  be added to  the file.   Once  the

program is done  writing the file,  you may run Autocad  and

use  the  command  "SCRIPT"   from  the  "Command:"  prompt.

Autocad will ask for the script file name.  Specify the same

name  (with path if  needed) as the one  you used in PCROUTE

and Autocad will read in the file.  It may take awhile so be

patient.    Autocad  will  display  three   layers:  solder,

component, and silkscreen.   You may  edit them however  you

like and save  them as  Autocad drawing files.   Doing  this

greatly increases the number of  printers you can output  to

as Autocad supports mega-hardware.

4. Erase Data:

This  routine  clears the  memory of  the program.   It

erases the  boards, stacks,  component list,  and connection

list.  The program will ask if you are sure that you want to

erase  the data in memory.  Answer Y or N.  The program will

respond with "Erasing..."  and after a few  seconds drop you

back to the I/O menu.

5. Disk Directory:

This function displays a directory of the current drive

and path.  Use the  space bar to view twenty file names at a

time.  When no more  files exist, the space bar  will return

you to the I/O menu.

6. Change drive/path:

This function changes  the current  drive and or  path.

The  ESC  key will  back  you  out of  the  function without

changing the  path.  The  function will display  the current

drive and path  and then prompt for  a new one.   Just enter

the new one and  press Return.  You will be  returned to the

I/O menu.

Define/Edit Board Menu

1. Component Edit/Entry:

This function is used  to enter components into the  system.

Once  you  have entered  this  function  you will  placed  in the

entry/editing environment.   If no  components have been  entered

you will see only two possible  commands, if there are components

in memory you  will have the option  to delete, copy and  perform

other functions.

New Component Entry:

Hitting an "A" will  allow you to enter new components.  The

program will  display the component number being entered and then

prompt  for the name.   The name is up  to 15 characters long and

serves only to  remind the designer of  what the device is.   ESC

can  be used  at this  point to return  you to  the entry/editing

environment.

8

The  next  question is  for the  designation.   This  is the

unique  name that  you will  give the  component  and use  in the

future to refer to it.   Typical names include IC1, R1, IC2,  and

A1.  The only  reserved designation is for vias. (A via  is a pad

with no device  attatched to it.  It is just  to connect a  trace

from one side of the board to the other)  Designations that start

with "V" and  then have a number  after that, are not  allowed in

the system.   So V1 would not  be accepted by VI1 would  be.  ESC

may be  used to back  up while  entering the designation  name to

reenter the component name.

Third the program will prompt for the definition file  name.

This is the name of a file that tells the program how the pins on

the device being entered are situated.  The program first shows a

list of possible definition  file names at the  lower right.   At

the end of the  list the program will prompt for a  name.  Do not

enter a file extension, the program will add it  for you.  If the

name of the file  entered exists, it will read the  file for you,

and display the  chip definition for you to see if it is correct.

If  you answer  yes that  it is  the correct definition  you will

continue to the  next question.   If the file  did not exist  you

will be  asked if  you wish  to create  the definition.   If  you

answer no,  the program  will re-prompt  for the  definition name

after showing you  the possible files.  Otherwise it makes a  new

definition.  ESC may be used to back up and reenter the component

designation.

Creating New Definition Files:

The program needs to know the exact  pin layout of

any  component  placed  on the  board.    This function

allows you to create these definitions.  Upon entry you

will see a graphic screen  with some information at the

bottom and a green square in  the middle of the screen.

In the top left of the box is your cursor.  You use the

cursor keys  to move  the cursor around  to define  pin

locations.  The pins are entered in the order from 1 to

up to 40.  Move the cursor to the position of the first

pin  and hit  return to  place it.   Then  move to  the

second position and hit return to place pin 2.  You may

also remove pins in  the reverse order you placed  them

by positioning your  cursor over  the last pin  entered

and  hitting  the backspace  key.   This key  will only

delete the  last entered pin.   The information  at the

bottom of the  screen will  tell you what  pin you  are

entering,  as  well  as its  relative  location  on the

screen.  When you  have placed 40 pins or  are finished

with the component definition, hit ESC to  finish.  The

program will ask if you want  to reedit.  Answering yes

will  clear  the screen  and you  can  start over.   By

answering no you will receive  the prompt asking if you

want to keep the definition or not.  Answering yes will

save the definition for later use, no will give you the

list of existing  definitions on the previous  screen.

9

All components are moved to the upper  left most corner

for saving.   This  is  for internal  reasons; you  can

design the chip anywhere in the  square.  You will need

to make  definition files for  all your components.   I

would  suggest  a meaningful  system  for  naming them.

Maybe something like DIP16V and DIP16H for 16  pin Dual

Inline   Package   Vertical  attitude   and  Horizontal

attitude respectively.

***Note:  Most  chip dimensions  are available in  data

books such as RCA cross reference manuals.  Radio Shack

also  puts  device  dimensions  on  the back  of  their

packages.

The next question the  program will ask is if  everything is

correct.    Answering  no  will  restart  this  process with  re-

prompting for the name.  Answering yes will get the next prompt.

The  next  prompt  is  if  you  wish to  keep  this  device.

Answering no  will skip the  next prompt and  ask if you  wish to

enter another device.  Answering yes gives you the next prompt.

This  prompt  asks if  you  wish  to place  the  device now.

Entering  no  will allow  you  to delay  this until  later.   All

devices must be placed somewhere on the board before  they can be

connected.  Placing a chip goes as follows:

Placing Chips:

When  this graphic screen comes  up you will see a

representation of  the current  circuit board with  all

other placed components  and ground planes shown.   The

chip you are entering  will be shown in the  upper left

corner of the  board.   The arrow keys  will move  this

chip  around  on  the  board.  By using  a  shift-arrow

combination the chip  can be  moved faster.   Move  the

chip to where you want it and press return.  You cannot

overlap  chips.    The  chip  is  now  placed  in  this

location.  If you change your mind you can press ESC to

exit without placing the chip.  You will be prompted if

you are sure you want to abandon chip placement or not.

Yes will go to the  next prompt, no will put you  right

back where you were before hitting ESC.

The last prompt is if you wish to enter another chip or not.

Answering yes will  put you back up  at the name prompt  to enter

another  chip.    No  will  put  you back  in  the  entry/editing

environment.

Editing Component Definitions:

Selecting E from the entry/editing environment will give you

the ability to change any part of each definition.  Simply select

the  letter of the  item to change.   For example,  to change the

name, hit A after hitting E and you will be  prompted for the new

name.  Either enter the  new name, or you can press ESC  to leave

it  unchanged.   When you  are done  editing, press the  key that

10

corresponds to exiting the edit.

Deleting Component Definitions:

This  function  allows  you  to delete  a  component.    The

component that will be deleted is  the one currently displayed on

the screen.  Press D and you will be asked if you are sure if you

want to delete this  component or not.  Answer  accordingly.  All

connections  that  contained the  deleted  function will  also be

deleted.

Copying Components:

This function will let you make  copies of a chip definition

to speed the  entry of chips.   It will copy everything  from the

displayed  definition  to   a  new  definition  except   for  the

designation  from  which it  will take  the  alpha part  and then

increment the numerical part to the next available part.  It also

does not put the chip in the same location.  You will be asked if

you wish to place the chip now or later.

Searching for a Component:

Sometimes finding the component  you wish to edit or  delete

can take some time on a large board.  This function allows you to

search for components  by the designation.   When prompted  enter

the designation, PCROUTE will  change the case to upper  for you.

If  it exists the program will display  that device's number.  If

it doesn't exist, you will continue right where you were.

Incrementing and Decrementing the displayed Component:

Using  +  and -  will  change  the displayed  component.   +

increments and - decrements.  The  display is circular in nature,

therefore hitting  + at the  last component  will put you  at the

first and hitting - at the first will put you at the last.

Exiting the Entry/Editing Environment:

Use the N key to end these functions and return to the board

definition menu.

2. Connection Edit/ Entry:

This  system  allows  for  entry  and  editing  of  all  the

connections that need  to be made  between the components in  the

system.    The  functions  are  very  similar  to  the  component

functions.

New Connection Entry:

Typing an "A" will allow you to enter new connections.   The

program will display the connection number being entered and then

prompt for the  name of  the source designation.   The name  must

exist in memory as a component.  ESC can be used at this point to

return you to the entry/editing environment.

The next question is for the source chip pin number.   Enter

the number of the pin that you want the trace to connect to.  ESC

will back you up to the previous prompt.

11

The program will now prompt for  the name of the destination

designation.  The name must exist in  memory as a component.  ESC

can be used at this point to return to the previous prompt.

The next  question if for  the destination chip  pin number.

Enter the number  of the pin that  you want the other  end of the

trace to  connect  to.   ESC will  back you  up  to the  previous

prompt.

The program will now ask if this is a priority route or not.

By answering yes, this trace will be routed before all other non-

priority  routes  no matter  how long  it may  be and  what other

routes it may block.  This might be used for making sure a memory

access bus is all done in  copper instead of using jumpers for  a

vital connection.

The next question the  program will ask is if  everything is

correct.   Answering no will restart  this process by reprompting

for the source designation.  Answering yes will bring up the next

prompt.

The next  prompt is  if you  wish to  keep this  connection.

Answering  no will skip  the next prompt  and ask if  you wish to

enter  another  connection.   Answering  yes gives  you  the next

prompt.

The  last  prompt   asks  if  you  wish   to  enter  another

connection.  Answering yes,  will put you  back up at the  source

designation  prompt to enter another connection.   No will places

you back in the entry/editing environment.

Editing Connection Definitions:

Selecting E from the entry/editing environment will give you

the ability to change any part of each connection.  Simply select

the letter  of the item  to change.   For example, to  change the

source  designation,  hit  A after  hitting  E  and  you will  be

prompted  for  the  new  designation.     Either  enter  the  new

designation, or  press ESC to  leave it unchanged.   When you are

done editing, press the key that corresponds to exiting the edit.

Deleting Connection Definitions:

This  function allows  you  to  delete  a connection.    The

connection that will be deleted is the one currently displayed on

the screen.  Press  D and you will  be asked if you are  sure you

want to delete this connection.  Answer accordingly.

Searching for a Connection:

Sometimes finding the  connection you wish to edit or delete

can take some time on a large board.  This function allows you to

search for connections by the  designation.  When prompted  enter

the designation sought.  PCROUTEwill change the case to upper for

you. PCROUTE then will search from  the current connection to the

end.  It  looks at both  the source and destination  designations

for a match.   If  it finds  the search string  the program  will

display that connection's information.  If it doesn't  exist, you

continue right where you were.

12

Incrementing and Decrementing the displayed Connection:

Using  +  and -  will change  the  displayed connection.   +

increments and - decrements.  The  display is circular in nature,

therefore hit + at the last connection  will put you at the first

and hitting - at the first will put you at the last.

Exiting the Entry/Editing Environment:

Use the N key to end these functions and return to the board

definition menu.

3. Ground Plane:    This  function  is used  to  set up  a ground

plane.  The program will display a picture of

the board.   It will prompt for  the width of

the ground plane  on all  four sides on  both

the  component side  and solder  side of  the

board.  Each digit entered corresponds to .05

actual inches.  You may  come back and change

the ground plane sizes at any time.  ESC will

also back you  out of  the first prompt  only

(top-solder side).

***   Please note  that this  function erases

any  work  done  in  the  interactive  router

(discussed later).

4. Define Board Size:    This  function  is  the  first  function

used.  It defines the  dimensions of the

board.  The maximum board size supported

by PCROUTE is  11" X 8".   When prompted

for numbers, you must  enter the numbers

as  multiples   of  5.     Each   number

represents 0.05" of actual  size.  So an

entry  of  100 would  correspond  to 5".

You may change  board sizes at any  time

during the design process by calling the

routine.  If you make the board smaller,

the chips  that  were  outside  the  new

border will  have to be  replaced.   ESC

will  back up to the previous prompt, or

jump out  of  the  routine  leaving  the

board size unchanged.

5. Spider Web: This  function is useful for checking  to see if a

chip  would  be  more logically  placed  somewhere

else.   By using the +  and - keys, you can toggle

through the chips.   As each chip  is selected, it

as  well  as  all  traces   connected  to  it  are

highlighted in red.   Once a chip  is highlighted,

you can select M to move the chip.  After pressing

M, the  arrow keys become  active and the  chip as

well as all its connections  will be rubber banded

around the board.  You can hit RETURN to place the

chip  at the current  location, or hit  ESC to not

13

place  the chip and  put it  back at  its original

location.  The  ESC or  E key will  exit you  from

this function.

*** Please note,  if you move any  chips, the work

done by the interactive router is erased.

Interactive Router

This board editing system allows you to interactively design

a  board. It can be very  useful for finding possible routes that

the autorouter was unable  to find.   Upon entry to this  section

the program  generates the borders and places the pads of all the

chips.  It will also ask you to place all the chips that have not

been placed yet.   If you intend  to route the entire  board this

way,  place your chips  and then leave them.   Moving them around

after placing  the chip will distort your  board.  PCROUTE has no

way of connecting  traces up to a chips new location.  The screen

will  show all the  chips with  their designations  centered amid

each components pins.  This system  has many keys that are active

while in this environment.  An explanation of each follows.

S - switch sides:   This function  changes the side  of the board

displayed.   When you  enter S,  you will  be

shown  the  other  side of  the  board.   The

program  supports  only 2  sides,  solder and

component.

E or ESC - end:     Pressing  E  or ESC  will  exit you  from the

interactive router.

Cursor Keys:   The cursor keys move the solid block around on the

board.   Each time you  press the  arrow key,  the

cursor will  move 1  space.  Relative  coordinates

are displayed at  the bottom of the screen.  Using

Shift-Arrow moves the  cursor 10  spaces.  If  you

come to the edge of the screen but not the edge of

the board, the  screen will  shift the board  over

for you.

Function Keys:

The function keys numbered F1-F8 do the majority of the work

in PCROUTE.   They allow you  to place  vias, run traces,  delete

traces, blank old traces, and delete vias.

F1 - Set Source:    By  positioning  the cursor  over  a pad  and

hitting F1 you select that pad as your source

location.  The bottom  status line will  tell

you exactly what chip and pin number you have

selected.

F2 - Set Dest.:     This  function   works  exactly  as   the  F1

14

function but selects the destination pin.

F3 - Run Trace:     Once  you   have   defined   a   source   and

destination,  hitting  F3  will   attempt  to

connect the two pins with a  trace.  If it is

successful, you  will see  the trace  appear.

If  it  cannot  complete  the  route it  will

notify you at  the bottom.  At this point you

might try placing a via  and routing from the

source to it on one side  and from the via to

the destination on the other.

F4 - Delete Trace:  This  function will  delete traces  that have

been  routed since the  program has been run.

If  you  have just  loaded  the board  up and

decide to  delete a trace  this function will

not work.   Saving  the board  does not  save

deletion information.   Assuming the  data is

in memory to perform the  delete, the program

will highlight the  trace to delete.   If the

highlighted  trace is the  one to  be deleted

answer  Yes when  prompted.   Hitting N  will

advance to the next trace if it is available.

Hitting ESC will drop you  back to the cursor

control mode.

F5 - Add Via:  This function key  will add a via  at the location

of the cursor.   It will  be V and  then the  next

available number.  Use these for multilevel routes

or just extra pads needed.  You cannot place a via

adjacent to  an existing pad.   You can  place one

diagonally  from  one though.    You also  may not

place a  via in the  middle of a trace.   All vias

are deleted by the autorouter when it is invoked.

F6 - Del Via:  This  function  deletes  the  via  at  the  cursor

location if one exists there.

F7 - Erase Space:   This  function will remove the trace piece at

the location of the cursor.  This function is

useful if you deleted a  series of traces and

left   something   unconnected.     Use  this

carefully.

F8 - Check Route:   This function will check to see if a selected

pair  of  pads  can be  connected.    It will

report back as  to whether  the route can  be

completed.  This is  useful for doing  multi-

layer routes  using  vias.   Checking for  an

available  route  before  laying  the  actual

trace will save  a bunch of deletes  if later

your path is blocked.

15

F9 - Manual Route:  This  function allows  you to  manually route

traces.  Place the cursor on the starting pad

or trace and press F9.   If the cursor is not

on  a  valid  starting  point,  nothing  will

happen.  Now  use the cursor control  keys to

route your trace.  Every  time you move, your

trace is lengthened.  When  you are done, you

can either stop  in and open space  and press

F9 or you can stop on  a trace or pad and hit

F9,  the  correct  connecting piece  will  be

added  to make  the connection.   Please note

that  if  you  go  over  an  existing  trace,

PCROUTE will cut  that trace in favor  of the

new path.  This works  well for putting short

jumpers on the  board.   This is very  useful

for routing traces  that would normally block

other  routes if  they were  not placed  this

way.     As  my  autorouter   algorithm  gets

"smarter" hopefully we won't need this.

16

Autorouting

Selecting this from the menu will  enable you to connect all

or most  of the connections you  desire.  The program  will first

draw borders, place  pads, and set  up the ground  planes.  If  a

component has not  been placed, you  will be asked  to place  it.

The  program requires all chips to be placed before it can route.

There are  four routing  strategies that  can be  employed.   You

select each  strategy by hitting  Y or N  as the cursor  moves to

each option.   If you hit Y for  single or double via routing you

will  also be prompted for the  maximum number of tries for each.

Just  enter  a number.    Bigger  numbers are  better  for bigger

boards, smaller number for small boards.

The  PCROUTE  system  is  totally  re-entrant.    The  first

question  you are  asked  is if  you wish  to  clear all  traces.

Answering yes clears anything  currently on the board.   No would

keep what was on  the board such as work done  by the interactive

router.  You can hit ESC during any routing strategy to exit that

routine and the  autorouter.  The program will then show the list

of all unconnected routes still existing.   If the router has not

completed one  run-through and you have routed before, traces may

have been completed by the earlier route and still be reported as

unconnected.  You  can do a design  check to see which  routes in

you connection  list have not been connected either by hand or by

autorouter  by running the autorouter and not erasing the current

traces.  Just select the double or single sided route system.  It

will report any routes that  are incomplete.  Any route that  was

completed  before will be  marked as  complete and  not reported.

The program will report whatever routes weren't done and the ones

it couldn't complete.

Single Sided Route: This function will  attempt to place  all the

routes  on the solder side of  the board.  It

is the  least efficient strategy but is great

for  small projects where  ease of making the

board itself  is a  priority.   This function

cannot be  used in conjunction  with the next

three strategies.

Double Sided Route: This  function  will do  its  routing  on two

sides  of  the  board.     It  has  a  higher

completion percentage than  the single  sided

strategy.

Single Via Route:   This function requires that the double  sided

strategy be select  first.  This  function is

used after the  double sided strategy to  try

to  complete  any  uncompleted  routes.    It

places a via and tries to do the single route

with a trace  on each side  of the board.   A

number such as 50 is  good for maximum number

of tries.

17

Double Via Route:   This  function  also  requires  double  sided

routing be  selected to  work.   It uses  two

vias to  attempt to  complete the  route.   A

good maximum  number of  tries is  30 or  so.

This function can  take a great deal  of time

so be patient.

Board Statistics

This function displays 3 statistics that may be of some interest.

Equivalent Integrated Circuit Count:    This   function   is  the

total   number  of   pins

divided by 16.

Board Density: This is the  amount of space total  that each chip

has  to occupy.   It is  calculated by  taking the

area of the  board and dividing by  the Equivalent

IC count.

Total Trace Length: The function  gives the total  length of  all

the traces  on the  board.   The smaller  the

number, the better the design.

18

Printer Menu

1. Print Component List: This function  will generate  a list  of

all the components.  It can be sorted by

name or designation.

2. Print Connections:    This  function will  generate a  list of

all the  connections in memory.   It can

be sorted  by the source  or destination

designation.

3. Print Solder Side of Board:

This function  will generate  the artwork  to make  the

printed circuit board.   The program will ask if you want to

mirror  the  board.   This  may  be necessary  if  your film

requires  printing on a  particular side.   The mirroring is

done left to right.  Next it will ask if you want a negative

made.  This  will make the  traces white and the  background

all black.   On Laser printers you  also have the  option of

making an actual size printout, or the standard 2X size that

the dot matrix printers print.  The ESC key will exit out of

the printout at any time.

4. Print Component Side of Board:

This function  is identical  to number  3 above  except

that it prints the component side of the board.

5. Print a Silkscreen Representation:

This function will generate a pseudo-silkscreen.  It is

not  the same  scale as numbers  3 and  4 but is  useful for

remembering where components  are to be placed.   Select the

printer to be used and watch it go.

Exiting the Program

Selecting 7 from  the main menu  will drop you from  PCROUTE

back to DOS.  If you have data in memory you will be asked if you

wish to save it. Answering Yes  will prompt you for a file  name.

Enter the file name as you would in the save data function in the

Input/Output menu.   You can hit  ESC to abort  the save, but  it

will not  return you to  the program.   Answering No to  the save

query, will drop you immediately to DOS.

19

Keyboard Summaries

Chip Placement Screen:

ESC                 - Option to Abandon Placement

RET                 - Place chip at current location

Cursor Keys         - Moves chip one space for each

depression

Shift Cursor Key    - Moves chip 10 spaces for each

depression

Define Chip Definition:

ESC                 - Option to Abandon Definition

RET                 - Place pin at current location

Cursor Keys         - Moves cursor one space for each stroke

BACKSPACE           - Removes last placed pin

Spider Web System:

M*                  - Set system to move highlighted chip

Cursor keys    - Move chip one space on board

RET            - Place chip at current location

ESC            - Abandon placement

+,-*                - Increment and Decrement through chips

E*                  - Exit Web System

* - Denotes that this selection is displayed on screen.

Interactive Router:

S                   - Display other side of board

E,ESC               - Exit interactive router

F1                  - Select source pad

F2                  - Select destination pad

F3                  - Attempt to connect source and

destination

F4                  - Delete traces connected since system

started

F5                  - Place via at cursor location

F6                  - Delete via at cursor location

F7                  - Remove trace piece at cursor location

F8                  - See if route is possible between

source and destination pads

F9                  - Enters and exits the manual router

20

Brief Tutorial

The following  is intended  as a  method of  getting up  and

running with the program.  It will give a general idea of how the

program works.  It is not intended as a "learn all" tutorial.

1.   Begin by  running PCROUTE.   This information  can be  found

back in the section on setting up PCROUTE for your machine.

2.   Once at the main menu, hit 3 for define/edit board, and then

select 4 for define  board dimensions.  For our  purposes we

will use  a board size of 80 x 80, which translates into a 4

inch square board.

3.   Select function 1  to enter  new components.   You will  see

that no data has been entered yet.  Hit A to enter data.  We

will put three chips on the board and two resistors.  Answer

each prompt below  with the following  data.  When asked  to

place each chip  answer Y and place the chip with the cursor

keys.  After you have entered the last item, answer N to the

"Enter another component?" prompt.

Name: 8051 uP                 Designation:  IC1

Definition file: DIP40V6      Placement: Left side of board

Name: 74ls373                 Designation:  IC2

Definition file: DIP20V3      Placement: Upper right

Name: 2716                    Designation:  IC3

Definition file: DIP24V6      Placement: Lower right

Name: 100k resistor           Designation:  R1

Definition file: 2PAD50V      Placement: Upper middle

Name: 10k resistor            Designation:  R2

Definition file: 2PAD50V      Placement: Lower middle

N exits you to the board definition menu.

4.   Next, we want to  enter some connections to be  made between

the chips.  Select 2 from  the menu to enter the  connection

entry  section.  Enter the following  connections.  Answer N

to each prompt  of "Is this  a priority connection?".   When

you have entered the last connection  answer N to the "Enter

another connection prompt?".

Source Defice       Pin       Desitination Device      Pin

IC1            2              IC3                 2

IC1            3              IC2                 16

IC3            22             IC2                 6

R1             1              IC2                 4

R2             1              IC1                 8

21

R2             2              IC3                 6

R1             2              IC1                 31

IC2            8              IC3                 17

IC1            21             IC1                 15

IC2            8              IC3                 8

IC2            9              IC3                 9

IC2            10             IC3                 10

IC1            14             IC2                 9

N returns you to the menu.

5.   Ok, lets autoroute  the board.  Go back to the main menu and

select  4  to autoroute  the board.    When prompted  to use

single sided routing or not, answer Y.   Now hit the ESC key

and  watch the  machine  route.   When it  is  done is  will

display the list  of uncompleted connections to  the printer

or screen.   Select  whichever  you wish.   We  will now  be

dropped back to the  main menu.  We can take  a look at what

we have  done by selecting number 5 from  the menu.  You can

cursor around and see  a representation of the board  on the

screen.  Hit ESC when done.

Now lets re-route  the board using double  sided techniques.

This time say  N to single sided  and Y to double  sided and

then hit ESC.  The machine will route double sided and again

give a list of incomplete connections  if any exist.  Select

printer or screen  for this list.   Lets take a look  at the

board again.  This time we may also want to use the S key to

change sides of the board that we are viewing.  When you are

done hit ESC.

6.   From the main  menu, hit  6 for statistics.   Displayed  are

three statistics that tell you how good your design is.  Hit

space when you are done with this.

7.   Last of all  we'll print this baby  out.  Hit 2  for printer

menu.  Now select silkscreen.   Select your printer and then

hit  N for mirrored  image.  Your  printer will  now print a

quick mockup of the  board.  This is useful  for remembering

what  chip goes where.   Now hit  3 to print  out the solder

side of the board.  Select your printer and answer N to each

prompt. Your printer will shortly come to life and print out

a 2X print  of the copper patern  on your printer.   You may

hit ESC  to abort the  printout if you are  tired of sitting

around.

22

Order Form

Please enclose this form when ordering.

Name:     __________________________________________

Address1: __________________________________________

Address2: __________________________________________

City:     _________________   State:    _______   Zip:_________

Phone Number:  (________) ________-______________

Computer name: _____________________________________

Processor: 8086  8088  80186  80188  80286  80386  80386SX

Printer type:  _____________________________________

Suggestions/comments/other information:

____________________________________________________

____________________________________________________

____________________________________________________

____________________________________________________

____________________________________________________

____________________________________________________

____________________________________________________

____________________________________________________

Please also  enclose the $65 check  or money order,  (do not send

cash) made payable to:

Douglas E. Ehlers

4520 S. 58th

Lincoln, NE  68516

(402) 435-8440