CALCOMP 4.2 PROTECTED MODE ADI DRIVER

The DGCALCMP.EXP file is a 4.2 protected mode ADI driver. This version

supports Autocad versions 11 and 12. This disk also contains two files called

DGCALCMP.DLL and DGCAL11.DLL for use with AutoCAD 11 and 12 for Windows. These

files need to be copied to the ACADWIN directory for use with AutoCAD for

Windows.

Example: COPY A:\DGCALCMP.DLL C:\ACADWIN

DGCALCMP.DLL - For AutoCAD Release 12 For Windows

DGCAL11.DLL  - For AutoCAD Release 11 For Windows

USING THE DIGITIZER WITH AUTOCAD FOR WINDOWS

To Install the Table mouse driver for Windows follow these steps:

1. Start Windows SETUP by typing SETUP at the DOS prompt from the WINDOWS

directory.

2. Select POINTING Device using the arrow keys and pressing ENTER.

3. Press the down arrow key to select OTHER. Press ENTER.

4. Type the path where the CalComp file reside. Example:

B:\

or

C:\CALCOMP\DRVS

5. A list of CalComp Tablets will be displayed. Select your tablet and

Press ENTER.

6. Press ENTER again to except the above changes. Start Windows.

7. Start AutoCAD for Windows.

8. Enter CONFIGURATION -  select Digitizer (#4)

9. Select "CalComp Digitizer Series ADI".

10. Select Windows Tablet.

11. Select YES to switch button to change modes on cursor or select NO t o

toggle with bottom right corner of the tablet.

12. Select a button on the cursor.

TO USE THE TABLET WITH A MOUSE DEVICE

1. Enter AutoCAD configuration to. Select Digitizer

2. Select CalComp Digitizer Series ADI.

3. Select the correct model of digitizer.

4. Select the com port that the digitizer is connected to.

5. Select NO to mole mode.

NOTE:

We have found some problems using the AutoDesk mole selection. Problems like

the system hanging or just dropping out to DOS. Also we have had symptoms of

the cross hairs on the screen moving in the opposite direction of the cursor

movement. To avoid all of these symptoms select the WINDOWS TABLET selection

in  the list of supported tablets in the CalComp ADI driver. The WINDOWS

TABLET will support all of CalComp's different tablet models.

USING THE ADI DRIVER WITH AUTOCAD FOR DOS

You will need to copy the ADI driver DGCALCMP.EXP to a sub-directory on

the hard drive called DRV in the AutoCad directory. This is the default

directory for AutoCad 12. If you are using AutoCad 11 you will need create a

sub-directory in the AUTOCAD directory called DRV. Then tell AutoCad that the

ADI driver is in the sub-directory call DRV. This is done by typing

SET DGPADI=C:\ACAD\DRV\DGCALCMP.EXP

at your DOS prompt. You may want to add this line to your AUTOEXEC.BAT

file or to another batch file used to start AUTOCAD.

If you have the digitizer connected to COM 3 be sure to select polling

mode in the AutoCad configuration. If interrupt mode is selected you may

not get any cross hairs displayed in the graphic screen.