LUBBOCK FIRE DEPARTMENT		Instructor's Guide	Lesson Plan for Ground Ladders

****************************************************************************************************

Feb. 28, 1993

I.  Introduction

1.	Introduce yourself, welcome, and make any administrative announcements needed

2.	This section consist of 32 hours of instruction in Ladders.  It is broken down into Eight ( 8 ) Four

( 4 ) hour blocks

II.  Objectives

At the conclusion of this unit, the student will be able to

1.	Identify the NFPA Standard applicable to fire service ladders

2.	Demonstrate the proper pickup and ladder carry

3.	Demonstrate ladder raisers

4.	Demonstrate / explain the care, procedures, and safety procedures of ladder operations

5.	Demonstrate / explain the use of aerial ladders and elevating platforms

III.  References

1.	International fire service training association.  Fire Service Ground Ladder Practices, 8th edition.  Stillwater, 					Oklahoma, Fire Protection Publications, Oklahoma State University, 1984

2.	National Fire Protection Association.  NFPA 1932 " Standard on Use, Maintenance, and Service Testing of Fire 				Department Ground Ladders, 1989 edition.  NFPA Publications, Batterymarch Park, Quincy, Ma. 02269

3.	National Fire Protection Association.  NFPA 1901 Pumper Fire Apparatus, 1991 edition, NFPA Publications, 					Batterymarch Park, Quincy, Ma. 02269

4.	National Fire Protection Association.  NFPA 1904 Aerial Ladder and Elevating Platform Apparatus, 1991 edition, 				NFPA Publications, Batterymarch Park, Quincy, Ma. 02269

IV.  Teaching aids / materials requirements

1.	Instructor

A.	Texas Commission on Fire Protection, Certification Curriculum Manual, Basic Fire Suppression, Fire Service 					Ladders, Revised July 14, 1993

B.	Ladder slides ,overheads and Video tapes

C.	Chalkboard, chalk, eraser, pointer, slide projector, video player, and overhead projector,

D.	Assorted ladders consisting of lengths listed in NFPA Standard 1901 Pumper Apparatus, and NFPA Standard 1904 				Aerial Ladder and Elevating Platform Fire Apparatus.  The Lengths shall be those listed for Equipment Carried on 				Pumpers and Equipment Carried on Ladder Trucks.

E.	Aerial ladder / Elevating Platform Apparatus

F.	Life lines, Safety belts, Axe, 8 foot pike pole, Stokes basket, and dummy

G.	Instructor and course evaluation sheets

H.	Videos		( Ground Ladders by Film Communicators LFD # 4.01 )

( Ladders; FireFighters I Series by Fire Service Training LFD # 4.02 )

( Essentials of FireFighting Ladders 1 by Action Training Systems LFD # ________ )

I.	LFD slides on ladder carries

2.  Student

A.	Bunker gear

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

Module 1		Give Students																	Time

I.	Definitions / Terminology

A.	Definitions		IFSTA Ground Ladders 8th edition pg xi

1.		Alloy				A substance composed of two or more metals fused together and dissolved in each other when 										molten.

2.		Anchor			That part of the ladder which is used to attach the halyard to the ladder

3.		Beam				One of the two principal structural sides of a ladder

4.		Bed				The lower section of an extension ladder

5.		Butt Spur			Points located on the bottom of a ladder to prevent slipping

6.		Checks			Cracks or breaks in wood

7.		Corrugated		Formed into ridges or grooves

8.		Dimpled			Depressed or indented, as on a metal surface to aid in gripping

9.		Eave				The lower border of a roof that projects beyond the face of a building

10.		Extension Ladder	A term to identify a ladder with two or more sections

11.		Fascia					The broad flat surface over a storefront or below a cornice

12.		Flat Roof				A roof that has a pitch not exceeding 20 degrees

13.		Fly						The upper section or top section of an extension ladder

14.		Ground Ladder		A descriptive term to designate the difference between ladders raised on the ground and 											those raised from the apparatus

15.		Growth Rings		A layer of wood ( as an annual ring ) produced during a single period of growth

16.		Guides					Wood or metal strips on an extension ladders which guide the fly section while being 												raised.  ( Sometimes in the form of slots or channels )

17.		Halyard				Rope or cable used for hoisting and lowering the fly sections of an extension ladder

18.		Heel				The act of preventing the butt end of a ladder from slipping while in use / The bottom end of a 									ladder

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

19.		Heelman				The firefighter who carries the butt end of the ladder and / or who subsequently heels or 											secures it from slipping during operations.

20.		Heat Sensor Label	A special tab or label attached to the top section or sections of ladders which have a 													predetermined heat deg setting.  When the label is exposed to heat at this setting, the label 											changes color, thus indicating that the ladder was exposed to excessive heat.

21.		Hooks					A curved, sharp, metal device ( two in number ) which fold outward from each beam at the 										top end of a roof ladder

22.		Knurled				Having a series of small ridges or beads as on a metal surface to aid in gripping

23.		Leg Lock				A method of entwining a leg around a ladder rung to free the climber's hands for working 											while insuring that the individual cannot fall from the ladder

24.		Mortise				A hole, groove, or slot cut into a wood ladder beam to receive a rung tenon

25.		NFPA 1901			Refers to National Fire Protection Association " Standard for Automotive Fire Apparatus "											1979 edition

26.		NFPA 1931			Refers to National Fire Protection Association, Standard on Design, and Design 														Verification Tests for Fire Department Ground Ladders, 1984 edition.

27.		NFPA 1932			Refers to National Fire Protection Association, Standard on Use, Maintenance, and Service 										Testing of Fire Department Ground Ladders, 1984 edition

28.		Pawl / Lock			Metal devices used to hold the fly section in place after it has been extended

29.		Pitch					There are two ( 2 ) meanings

a.	The slope of a roof

b.	Resin present in certain woods

30.		Rack					There are two ( 2 ) meanings

a.	The framework used to support ladders while being carried on fire apparatus

b.	The act of placing a ladder on the apparatus

31.		Rails					The Two ( 2 ) lengthwise members of a trussed ladder beam which are separated by truss 											blocks

32.		Rhythm				Handling and climbing ladders with a smooth motion

33.		Scuttle					An opening in the roof or ceiling providing access to the roof or attic of a building

34.		Single Ladder			Straight Ladder, Wall Ladder, Term used to identify a ladder of one section

35.		Serrated				Notched or toothed edge

36.		Spanish Windlass		An apparatus such as a stick or dowel used for post tensioning lines by twisting

37.		Splice					To join two ( 2 ) ropes or cables by weaving the strands together

38.		Spur					Metal points at the lower end of staypoles

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

39.		Staypoles				Tormentor poles, Support or brace poles attached to Pole / Bangor ladders IFSTA pg 69

40.		Stops					Wood or metal pieces which prevents the fly section from being extended too far from the 											bed

41.		Throw a ladder		Raise a ladder quickly

42.		Tie Rod				Metal rods running from one beam to the other.  Usually under every 4th rung

43.		Toggle				A device by which a staypole ( tormentor pole ) is attached to a long ladder

44.		Tongue				The rib on one edge of a ladder beam that fits into a corresponding groove or channel 												attached to the edge of another ladder beam.  Its purpose is to hold the two sections 													together while allowing the sections to move up and down.

45.		Torque				Something that produces or tends to produce rotation, a turning or twisting force

46.		Truss					An assembly of beam members so combined as to form a rigid framework to be used as a 											ladder

47.		Truss Block			Separation pieces between the rails of a trussed ladder.  Sometimes used to support the 												rungs ( IFSTA Ground Ladders pg

48.		Wind Shakes			Damage done to timber by repeated flexing in the wind

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

B.	Ladder Terminology		IFSTA Ground Ladders 8th edition pg 4

1.		Angle of inclination			Refers to the angle of a in place ground ladder in relation the horizontal

2.		Bedded Position				When the fly of an extension ladder is fully retracted it is said to be in the bedded 													position.  The position in which the ladder is carried on the apparatus

3.		Base							The lowest or widest section of a ground ladder

4.		Butt ( Heel )					The bottom end of the ladder.  The end which will be placed on the ground or other 												supporting surface when the ladder is raised.

5.		Designated Length			The length as marked within 12 inches of the butt of each side rail of a single 													ladder and the base section of extension and pole ladders, as required by NFPA 														1931.  The Marked length must be visible when the ladder is in the bedded position.

6.		Dogs / Pawls					Devices used to hold fly sections at the desired height during use.  The lock the 														sections together.

7.		Identification Number		NFPA 1931 requires that ground ladders bear a unique individual identification 														number / Serial number.  Usually located on the outside beam of the ladder.

8.		Inside Width					The distance measured from the inside of one beam to the inside of the opposite 														beam

9.		Max Extended Length		The total length of the extension, pole, or some combination ladders when all fly 													sections are fully extended and the pawls ( dogs ) are engaged.

10.		Nesting						The procedures whereby ladders of different sizes and or  types are racked partially 													within on another to reduce the space required for storage on the apparatus.

11.		Outside Width				The distance measured from the outside of one ladder beam to the outside of the 														opposite ladder beam.

12.		Pivoting						The act of turning on one axis

13.		Rungs							Cross members which provide the foothold for climbing.  The rungs extend from 													one beam to the other, except on pompier ladders where the rungs pierce the single 													beam.

14.		Stripping ladder				A device which resembles a short, beefed  up roof ladder but which actually is a 														tool to strip roofs of shingles, paper and roof boards.  It is not used for 															climbing

15.		Tip / Top						The extreme top of the ladder

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

II.		Ladder Types

A.		Objectives of Fire ground operations are				IFSTA Ground Ladders pg 1

1.	Rescue

2.	Confinement

3.	Extinguishment

B.		Ladders help accomplish these objectives in that				IFSTA Ground Ladders pg 5

1.	They allow access to upper floors

2.	They provide a vantage point for fire streams

3.	They serve other special purposes

C.		NFPA 1931 sets the minimum lengths, types and numbers of ladders to be carried on apparatus.

D.		Seven ( 7 ) Types of Ladders		IFSTA Ground Ladders pg 9

1.	Single ladders ( Wall ladder )

2.	Roof ladders

3.	Folding ladders ( Attic ladder )

4.	Extension ladders

5.	Pole ladders

6.	Combination ladders ( A Frame )

7.	Pompier ladders

E.		Single or Wall Ladders		( LFD calls a Straight ladder )			IFSTA Ground Ladder pg 9

1.	One ( 1 ) section only

2.	From 6 to 32 feet

3.	Commonly 12 to 24 feet

4.	Used on 1 or 2 story buildings

F.		Roof Ladders

1.	Single ladders

2.	With two ( 2 ) hooks on one end

a.	Hooks are nested between the beams when used on a wall

b.	Hooks swivel out to be anchored to a sloped roof

3.	From 12 to 24 feet

G.		Folding ladders		( LFD calls a attic ladder )

1.	Have hinged rungs

a.	Fold into a compact assembly

2.	Can be carried

a.	Down narrow hallways or aisles

b.	Around Corners

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

c.	Easier to place in the attic scuttle hole

1.	Inserted folded

2.	Opened in place

3.	About twelve ( 12 ) inches wide ( narrow )

4.	Eight ( 8 ) to Sixteen ( 16 ) feet long

a.	Ten ( 10 ) feet most common

5.	NFPA 1931 requires foot pads

6.	Disadvantages

a.	Limited weight loading

1.	Less than 1 / 2 a regular ladder

b.	Climbing is awkward

c.	Leg locking is impractical

H.		Extension Ladders

1.	Two ( 2 ) or three ( 3 ) sections

2.	Manually operated by

a.	Rope

b.	Cable

3.	Referred to by fully extended lengths

a.	Lengths from 12 to 35 feet

I.		Pole / Bangor ladders

1.	A extension ladder that have staypoles ( tormentor poles )

2.	Two ( 2 ) to four ( 4 ) sections

3.	Lengths from 35 to 65 feet

a.	Most common is 50 feet

4.	Use is limited to manpower available

5.	Used up to 5 stories high

J.		Combination / A frame ladders

1.	Can be used as

a.	Step ladder ( A frame )

b.	Single ladder

c.	Extension ladder

2.	Lengths from 8 to 14 feet

a.	Most popular is 10 feet

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

3.	Used inside and very useful		( LFD does not have any of this type )

K.		Pompier ladder

1.	Single beam

a.	Rungs pass thur the beam

b.	Climb with one foot on each side of the beam

2.	A large metal hook ( gooseneck ) is on top

3.	Used in windows or other openings

a.	Could be used to climb from window to window

4.	Lengths from 10 to 20 feet

5.	Mainly a training ladder

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

Module 2		Give Students																	Time

I.		Construction Materials

A.		Solid and Truss Design

B.		Wood, Metal, or Fiberglass

C.		Materials are first quality

1.	Wood		requirements		( IFSTA Ground Ladders pg 31 )

a.	Weight per cubic inch

b.	Bending strength

c.	Stiffness

d.	Hardness

e.	Resistance to shock

f.	The most desirable type of wood is

1.	Beams & Truss Blocks

a.	Clear Straight Grained Douglas Fir ( dried for 2 years )		IFSTA Ground Ladders pg 34

b.	White spruce has been used for truss blocks

2.	Rungs

a.	Second growth Hickory		( IFSTA Ground Ladders pg 36 )

3.	Annual growth rings relatively close together

4.	Sawed at an angle to the grain for max. strength

g.	Shrinkage occurs when exposed to

1.	Low Humidity

2.	Artificial heat

h.	Advantages

1.	Non - Conductor of electricity when dry

2.	Not subject to sudden failure

3.	Good insulator

I.	Disadvantages

1.	A finish is required such as varnish

2.	Deteriorates with age quickly when compared to metal

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

2.	Metal

a.	Heat treated aluminum alloy

b.	Advantages		( IFSTA Ground Ladders pg 31 - 33 )

1.	Lighter		( with few exceptions )

2.	Tougher

3.	Will not chip or crack when subject to impact

4.	No splinters

5.	Easier to care for

6.	Do no deteriorate as much with age

c.	Disadvantages

1.	Extreme Caution is Necessary whenever metal ladders are used near electrical 								power sources

2.	Subject to sudden failure when exposed to heat or flame

a.	600 deg F.

b.	Any metal ladder subject to direct flame contact or heat high enough to cause water contacting it 								to sizzle or turn to steam, or whose heat sensor label has changed color, should be 											removed from service and subjected to a hardness test.

c.	Does not regain strength after being heated

3.	Become cold in winter and hot in summer ( good conductivity )

3.	Fiber Glass		( IFSTA pg 37 )

a.	Beams are fiber glass, other parts are metal

b.	Advantages

1.	Non - Conductor of electricity when dry

2.	Will regain strength after being heated

3.	Tough

4.	No finish required

5.	Dis - colors when exposed to flame ( a visual sign you can see )

6.	Poor conductor of heat

c.	Disadvantages

1.	Heavy

2.	Will chip and crack with severe impact

3.	May suddenly fail or crack when overloaded

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

4.	Composite	( wood and metal )

a.	Wood Truss beams and metal rungs

b.	Advantages

1.	Non - Conductor of electricity

2.	Lighter than an all wood ladder

c.	Disadvantages

1.	The beams require frequent maintenance

II.	Features of Construction		( IFSTA pg 38 )

A.		NFPA 1931 is the primary standard for construction features

B.		Major components of a ladder are

1.	Beams

2.	Rungs

C.		Beams

1.	Two Basic designs

a.	Solid Beams

1.	For lengths of 24 feet or less ( wood or metal )

2.	From 25 feet to 35 feet ( metal is lightest as opposed to wood ladders of the same length  )

b.	Truss Beams

1.	Used in ladders over 35 feet

c.	Ladder Widths

1.	7 1 / 2 inches

a.	Folding / Attic ladders

b.	Combination ladders

c.	Extension ladders under 16 feet

2.	16 inches

a.	Single and Roof ladders

b.	Extension ladders over 16 feet

3.	12 inches

a.	Pompier

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

d.	Ladder loading

1.	300 pounds

a.	Folding Ladders

b.	Pompier Ladders

2.	750 Pounds

a.	Single and Roof Ladders

b.	Extension and Pole Ladders

c.	Combination Ladders

D.		Metal ladder construction

1.	C - Channel design

a.	Has a plate to hold the rung		( IFSTA Ground Ladders pg 41 )

2.	C - Channel with a modified Plate ( with a tongue attached )

a.	Called a Tongue & Groove

b.	Uses the tongue to slide in  the groove on extension ladders

3.	Tubular Beam

a.	Extruded Structural members

1.	Made by riveting two parts together to make a beam

2.	It has a U - channel plate bolted to the outside to slide the fly section in

b.	A one piece extruded tubular beam		( IFSTA Ground Ladders pg 43 )

1.	A smaller C - Channel is on top of the bed section which holds the fly

2.	The rungs protrude thur both sides of the beam channel

3.	Nylon bushings are placed on the run ends to allow for easy sliding in the channel

4.	Truss Beams

a.	Two rectangular rail for each beam			( IFSTA Ground Ladders pg 44  fig. 2.23 )

1.	A U - shaped channel is manufactured on each face

2.	Truss plates are riveted across between the U - channels

b.	Two T - channels are joined by a truss plate		( IFSTA Ground Ladders pg 44 fig. 2.26 )

1.	A U - channel is on the inside of the top rail

2.	The rungs of the fly slide in this U - channel on the top rail

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

5.	Metal Rungs

a.	NFPA 1931 requires

1.	Not less than 1 1/4 inch in diameter		( IFSTA Ground Ladders pg 45 )

2.	Spaced on 14 inch centers

a.	Plus or Minus 1 / 8 inch

3.	Slip proof rungs

a.	Heavy duty corrugated		( ridges )

b.	Serrated		( saw toothed )

c.	Knurled	( not smooth )

d.	Dimpled

e.	Coated with a skid resistant material

4.	Rungs are attached in one of three ways (IFSTA Ground Ladders pg 45 )

a.	Truss / Rung Plates

1.	The rung is inserted in the plate

a.	The rung is then expanded ( crumpled ) on each side of the plate

b.	The end is welded to the inside of the beam

2.	The rung is inserted in the plate

a.	The rung is welded to the outside truss plate

3.	A bushing is placed on the end of the rung

a.	The rung is then expanded ( crumpled ) in the bushing

5.	The rungs must maintain 75 % of their Strength at 300 Deg. F.

E.		Wood Ladders

1.	Solid Beam ( just that a solid piece of wood )

a.	Uses a metal guide to hold the fly

2.	Truss Beam

a.	Two Rails Separated by a Douglas Fir Wood Block ( Truss )

b.	Three basic rung mountings			( IFSTA Ground Ladders pg 47 and 184 )

1.	Rung placed in center of the Truss block

a.	Two Styles

1.	One piece of wood form the rung and truss block

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

2.	Separate truss blocks and rungs

a.	Three variations of this style

1.	Rung is centered in the truss block

a.	Same size parallel rails

2.	Rung mounted in the bottom center of the truss block next to the bottom rail

a.	Same size parallel rails

3.	Rung mounted in the bottom center of the truss block next to the bottom rail

a.	Same size parallel rails

b.	Top rail arches

c.	Bottom rail straight

d.	Truss blocks are of different sizes to create the arch

2.	Rung placed in the Bottom Rail

a.	Two Parallel rails

1.	Bottom rail is larger ( thicker ) to support the rungs

3.	Rung placed in the Top Rail

a.	The Top rail is largest to support the rungs

b.	Both rails arch except for roof ladders

1.	The roof ladder with rungs mounted in the top rail has

a.	A straight bottom rail ( to lie on the roof ) and an arched top rail

c.	Because wood does not have as much strength in compression as in tension

1.	NFPA 1932 requires			( IFSTA Ground Ladders pg 47 )

a.	When Rungs are mounted in Truss Blocks or Bottom Rails

1.	The fly is OUT ( bed of ladder is between building and the fly section )

b.	When Rungs are mounted in the Top Rails

2.	The fly is IN ( between the bed and the building )

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

3.	Rung Requirement for wooden ladders		( IFSTA Ground Ladders pg 50 )

a.	NFPA 1931 not NFPA 1932

b.	Must be a minimum of 1 1/4 inches in diameter

c.	Usually the same thickness the full length

1.	Can be thicker in the center

a.	Called a SWELL rung

d.	NFPA 1931 calls for placement in beams at 14 inches centers

1.	Plus or Minus 1 / 8 inch

e.	No tread surface is required on wood rungs

f.	Three methods of rung installation

1.	Blunt shoulder ends on the rungs / tenon

a.	The Tenon ( a reduction in the rung diameter ) is set in a mortise drilled in the ladder beam ( rail )

1.	Tie rods are required to prevent beam spreading

2.	To prevent the rung from turning in the mortise

a.	A metal key is inserted in the tenon or it is glued

b.	A nail or wood screw is inserted in the end of the tenon thur the beam

2.	Tapered shoulder ends on the rungs / tenon

a.	Tie rods are required to prevent beam spreading

b.	The tapered ends should prevent turning of the rungs in the beam

1.	It could be nailed, or a wood screw used and or glue

3.	Truss block installation of rungs

a.	Rungs attached by one of the above methods

1.	Tenon passes thur the truss block ( a longer tenon is required )

b.	Where one piece of wood forms the rung and truss block

1.	The Truss block is keyed ( notched ) to the upper and lower beam

F.		Fiber Glass		( IFSTA Ground Ladders pg 52 )

1.	Solid Beams ONLY

2.	C - Channel with a metal rung plate riveted across the C - channel

3.	Rungs are metal

a.	The same as for metal ladders

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

G.		Composite ( wood & metal )

1.	Truss Beam ONLY

2.	Douglas Fir is used for two parallel rails

3.	Metal truss plates

a.	Which are held together by Two beam bolts extending thru the beam at each truss plate

4.	Metal Rungs are used as for metal ladders

H.		Fire Ladders

1.	Folding Type

a.	Metal ( most common ) and wood

b.	NFPA 1931 restricts length to

1.	14 feet

2.	Foot pads

3.	Rungs hinged on both beams

4.	Rungs ARE NOT Round

c.	Three designs

1.	Tubular Rail with a U - channel on each side

a.	Two flat metal strips held by a single hinge pin are the rungs

2.	U - channel beam with a smaller U -  channel to support the rungs

a.	Rungs are square tubes

3.	C - channel beam with square tubes for rungs

a.	Rungs are between the C - channel

b.	Attached by a hinge pin on each beam

d.	Wood folding

1.	Two parallel beams

2.	No Truss Blocks

3.	The rung is square

a.	Held together by the rung hinge pin

2.	Pompier

a.	Metal construction mostly

b.	NFPA 1931 restricts them to 16 feet

c.	Minimum overall width of rungs

1.	12 inches

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

d.	Two ( one at top and one at bottom of the beam ) standoff brackets

1.	Minimum standoff distance is 7 inches

e.	Serrated goose neck shaped hook at the top end

1.	Made of harden steel

f.	Wooden Pompier

1.	Beams are of hickory

3.	Combination ( A frames )

a.	Four Variations

1.	Single / A Frame

2.	Telescoping beams single / A Frame

3.	Extension / A Frame

4.	Extending / A Frame

b.	Single / A Frame

1.	Two equal length sections

2.	Joined at ends of beams by steel hinges

3.	Hinges lock open at 180 deg for a straight ladder

c.	Telescoping beams single / A Frame

1.	Metal only

2.	Three equal length sections

3.	Joined at end of beams by steel hinges

4.	Hinges lock open at 180 deg for a straight ladder

5.	Second locking position allows for a raised platform

6.	The ladder is wider at each end

7.	Square rungs are attached to the top beam

d.	Extension / A Frame

1.	Short extension ladder

a.	No hinges

b.	Sections locks in place

2.	A Frame ladder

e.	Extending / A Frame

1.	Fly that fits between two equal length sections

2.	Modified Pawl which holds the fly at the desired height

NOTE: ONE HOUR FROM PAGE 1 TO HERE

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

III.		Hardware and Accessories		( IFSTA Ground Ladders pg 59 )

A.	Pawls ( ladder locks )

1.	Two types

a.	Automatic latching

b.	Manual latching

2.	Enclosed Automatic latching

a.	In the fly section beam itself

b.	Three main parts

1.	The hook

2.	Finger ( locks the hook in place on the rung )

3.	Torsion spring ( keeps the finger in place )

3.	Open Automatic latching		( IFSTA Ground Ladders pg 61 )

a.	Parts of this latch are

1.	Pendulum

2.	Pendulum spring

3.	Pendulum / Main spring support

4.	Pawl

5.	Main Spring

6.	Steel rod connecting the two latches

a.	Both latches work as one unit

b.	Pawls are attached

c.	Halyard anchor pulley is attached to the center of the rod

4.	Manual latching ( IFSTA Ground Ladders pg 63 )

a.	Two A shaped steel pawls are attached to the ends of a steel rod

1.	The rod runs across between the beams

a.	The rod is about 4 inches above the bottom rung of the fly section

b.	A one piece ( continuous ) halyard is attached to an inverted L shaped halyard anchor

B.	Roof Ladder Hooks

1.	Solid steel with tapered ends

2.	A coil spring holds the hooks in the desired position

a.	Open ( for use on a roof )

b.	Closed ( turned inward toward the opposite beam )

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

C.	Halyard, Anchor, and Pulley

1.	NFPA 1931 specifies the rope be

a.	3 / 8 inch in diameter

b.	Minimum breaking strength of 825 pounds

2.	On 3 and 4 section extension ladders

a.	NFPA 1931 allows the use of cable

1.	On the 2nd and 3rd fly sections

2.	Must be at least 3 / 16 inch in diameter

3.	No splices are allowed

4.	Must have means of adjustment of length

D.	Stops

1.	All extension ladders have them

2.	Usually an L shaped piece of metal

a.	Attached at the beam ends

b.	Keeps the fly from being pulled off the top end

E.	Butt Spurs / Foot Pads / Toe Rod / Mud Plate / Tie Rod / Protection plate / Levelers / Labels

1.	Butt Spurs

a.	Attached at the heel

b.	Provides gripping capabilities for the heel ( prevents slipping )

c.	Can have a MUD plate between the beams a few inches from the end of the spurs

1.	 Prevents the ladder from sinking too far into the ground

2.	A Toe Rod may be placed here as well

a.	A Toe Rod is added to the Heel of a ladder

1.	A fire fighter might place his toe there to hold the ladder

2.	Foot Pads

a.	Swinging metal plates attached to the heel beam ends

b.	Usually covered with rubber to prevent slipping

3.	Tie Rods

a.	Steel rods which help hold the beams together

b.	Usually placed below every 4 th rung from beam to beam

4.	Protection Plates

a.	strips of metal placed at chafing points on the ladder

5.	Levelers

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

a.	Attachments at the base / heel of a ladder to help level the ladder on slopes

6.	Labels		( IFSTA Ground Ladders pg 71 )

a.	NFPA 1931 requires

1.	Manufacturer's Identification label

a.	Name, Address, Series or Model number

2.	Serial Number Label

a.	NFPA 1931 requires a unique number for each ladder

b.	Embossed, stenciled, Branded, Stamped, or a Metal plate attached

3.	Certification Label

a.	Certifies the ladder meets NFPA 1931 and OSHA standards

4.	Heat Sensor Labels

a.	Placed near the top of each section of the ladder ( where the heat will be greatest )

b.	It turns a different color at a preset temperature

1.	If the color changes, a hardness test should be performed

5.	Design Verification Testing ( IFSTA Ground Ladders pgs 74 thur 77 )

a.	NFPA 1931 ( starting in 1984 ) requires a series of test on new ladder models

b.	Only applied to a new ladder model

c.	The ladder is destroyed after conducting all tests

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

IV.	Inspection and Maintenance of Ladders		( IFSTA Ground Ladders pg 77 )

A.	NFPA 1932 ( 1984 edition ) provides for ground ladder inspection and Maintenance

1.	NFPA calls for an inspection

a.	Monthly

b.	After each use

2.	Check heat sensor labels for a change of color

a.	If there is no heat sensor label then check for

1.	Bubbled or blackened varnish on wood ladders

2.	Discoloration of fiber glass ladders

3.	Heavy soot deposits

4.	Bubbled paint

3.	Check Rungs for snugness and tightness, cracks, deformations, etc

4.	All bolts and rivets for tightness

5.	All welds for cracks

6.	Beams for cracks, deformations, splinters or burs, etc

B.	Inspections for specific ladders

1.	Wood ladders and wood components of composite ladders

a.	Check for varnish chafing or scrapes

b.	Discoloration ( darkening ) of the varnish

c.	Dark streaks in wood ( indicates breaks or deep splits in the wood )

2.	Roof Ladders

a.	Check hook operation

b.	For rust

c.	For deformation of the hooks

3.	Extension and Pole ladders

a.	Proper operation of pawls

b.	Fraying or kinking halyard

c.	Halyard snug ( little slack in rope )

d.	Proper synchronization of sections

e.	Free turning of all pulleys

f.	Free movement of sections in guides

g.	Proper operation of staypole toggles

h.	General all around operation of ladder

4.	Mark damage areas with chalk		( IFSTA Ground Ladders pg 82 )

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

C.	Maintenance for specific ladders		( IFSTA Ground Ladders pg 79 )

1.	NFPA 1932 requires

a.	Moisture free

b.	Protection from elements

c.	No paint except for identification at

1.	Top 12 inches

2.	Bottom 12 inches

d.	Keep clean

1.	Wash Dirt off

a.	Always dry after washing

2.	Use a safety solvent to remove tar or oil			( IFSTA Ground Ladders pg 82  )

2.	Metal and fiber glass ladders

a.	Coat with auto paste wax

b.	Keep Fiber glass out of direct sunlight ( fiber glass will break down under sunlight )

3.	Wood Ladders

a.	No Steam applied

b.	Out of direct sunlight

c.	Out of artificially reduced humidity areas

1.	Wood will dry and lose strength

d.	Apply two ( 2 ) coats of

1.	Clear spar varnish OR

2.	Polyurethane varnish

3.	At least annually

e.	If damage is done to the varnish

1.	Remove ( sand or scrape ) peeling varnish

2.	Spot cleaned areas with varnish

3.	Sand when dry

4.	Apply at least 2 coats of varnish to ladder

f.	If all varnish is removed from the ladder with a liquid remover			( IFSTA Ground Ladders pg 83 )

1.	Apply two ( 2 ) coats of sealer

2.	Then apply two ( 2 ) coats of varnish

4.	Roof ladders

a.	Disassemble, clean and lubricate rusty hooks

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

5.	Extension ladders

a.	Pawls

1.	Lubricate as manufacturer's require

b.	Pawl torsion spring

1.	Replace every five ( 5 ) years if they are weak

c.	Ladder slides and guides

1.	Lubricate as required by manufacturer's require

2.	Clean off lubricant

a.	Use a safety solvent to do so

3.	Dry or spray lubricant is to be used on metal or fiber glass ladders

4.	Candle wax is to be used on wood extension and pole ladder guides

a.	At least every six ( 6 ) months

b.	Thin wax will not work

6.	Halyards / Halyard Pulleys

a.	Sketch halyard arrangements before removing the old halyard

1.	So you can replace it properly

b.	Lubricate pulleys

Note:	Use a Non - Water Soluble Lubricant

D.	Testing of Ground Ladders		( non - Manufacturer's test )

1.	NFPA 1932,		1984 edition

2.	Whenever testing

a.	Expect sudden failure

3.	At least annually

4.	If it is suspected of being unsafe

5.	After Overloading

6.	After impact loading

a.	Sudden weight being applied

7.	Any unusual conditions happening

8.	After heat exposure

9.	After any repairs

10.		Two sets of test

a.	All ladders except Pompier ladders

b.	For Pompier ladders

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

**************Do overheads of the test starting on page 86

11.		Strength Service Test		( IFSTA Ground Ladders pg 86 )

a.	Horizontal bending test

1.	First rung of each end is the support of the ladder

2.	Test load applied equally to a center span

a.	Covering 16 inches each side of the center inclusive

3.	Steps to the Test

a.	Metal or Fiber Glass ladders

1.	Step one

a.	No Impact loading of test weights

b.	Use a 350 pound weight

c.	Remains on the center span for 1 minute

2.	Step two

a.	Remove the 350 pounds

b.	Measure the distance between beams at

1.	Edge of top rails

2.	Edge of bottom rails

c.	Use a 500 pound weight and place in same location as the other weight

d.	Wait 5 minutes

e.	Remove the 500 pound weight

f.	Take same measurements again

g.	Calculations of measurements

1.	Difference between beams for 350 pound test and 500 pounds test

a.	Not to exceed 1/2 inch for 25 foot ladders or less

b.	Not to exceed 1 inch for ladders over 25 foot

2.	If the distance is greater that the above

a.	Destroy the ladder

b.	Wood ladders

1.	Use the 500 pound test for metal ladders ( only )

a.	No visible failure, then Ok, if failure then destroy the ladder

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

c.	Roof Ladder

1.	Same test as for metal ladder

2.	In addition

a.	Hang the ladder by the hook points

b.	Safety secure ladder

c.	Attach 1000 pounds to bottom rungs

1.	Remain in place for 1 minute

d.	Hooks spread is to be not less that 10 degrees to pass

d.	Extension and Pole Ladders

1.	Same test as for metal ladder

2.	In addition

a.	Inclined at 75 deg from a building

b.	Extended 1 rung from the bedded position

c.	1000 pounds attached to rung of fly section

1.	Remain in place for 1 minute

d.	Visual check after the test

e.	Pompier Ladder

1.	Hang Vertically by hook

2.	Safety secured

3.	1000 pounds attached at bottom

a.	No time limit given

b.	Assume 1 minute

b.	Hardness test

1.	Metal ladders only

a.	Constructed from 6061-TB Aluminum alloy

b.	Must have a hardness tester

c.	Uses a predetermined scale see chart page 91 of IFSTA Ground Ladders

c.	Eddy Current Test

1.	For aluminum ladders only

2.	Looking for heat damage

d.	Liquid Penetrant test

1.	Metal ladders only

2.	Looking for small invisible cracks in the metal or the welds

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

E.	Ladder Test Records

1.	List test that were performed

2.	Ensure timely annual testing

3.	Identify the service status ( in service or out of service )

4.	Tracks ladder abuse / damage and tested that was performed

5.	Data bank for ground ladder failures / to manufacturers

6.	Data to defend any legal action

7.	Used to show insurance companies ( ISO inspections )

8.	Data bank of evaluations for choosing one brand over another when reordering ladders

9.	Should be on a standard form from year to year

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

Module 3		Give Students																	Time

I.	Handling Ladders		( IFSTA Ground Ladders pg 99 )

A.	Placement on Apparatus

1.	No established standard for apparatus racking

B.	Pumper / Engine Ladders

1.	NFPA 1901 sets minimum lengths and types of ladders

a.	One 14 foot roof ladder

b.	One 24 foot Extension ladder

1.	One 35 foot ( 3 ) section ladder maybe carried instead of the 24 foot ladder

c.	Usually mounted vertically ( on the beam ) and ( nested ) on the right side of the engine.

d.	Could be in an overhead rack ( IFSTA Ground Ladders pg 101 )

2.	The engine can carry more that what is required

C.	Ladders carried on Aerial Apparatus

1.	NFPA 1901 sets minimum lengths and types of ladders

a.	One 10 foot folding ( attic )

b.	One 14 foot extension

c.	One 16 foot roof

d.	One 20 foot roof

e.	One 28 foot extension

f.	One 35 foot extension

g.	One 40 foot Pole ( extension )

2.	Mounting on apparatus

a.	Flat or vertical racking

1.	Rear or side removal

3.	Aerial ladders maybe wider that pumper ladders 			( IFSTA Ground Ladders pg 110 )

D.	Ladders carried on other Apparatus		( IFSTA Ground Ladders pg 105 )

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

E.	Correct ladder for the job		( IFSTA Ground Ladders pg 107 )

1.	Knowledge / Choosing the correct length of ladder for the job is the key

a.	Residential story will average

1.	8 to 10 feet

b.	Distance from floor to the window sill is

1.	3 feet

c.	Commercial story will average

1.	12 feet from floor to floor

2.	From floor to window sill will average 4 feet

2.	Rules of thumb for choosing ladders

a.	The tip of the ladder should extend 3 rungs beyond

1.	Window sill

2.	Roof edge

b.	A ladder is used on an angle ( inclined at 75 degrees )

1.	The length of a ladder

a.	IS NOT it's Reach

b.	Reach is less than the length

2.	NFPA 1931 requires the measured length to equal the designated length

a.	Extension and Pole ladders maybe

1.	Six ( 6 ) inches less than the designated length

b.	As a general rule

1.	Lengths of 35 feet or less

a.	Reach is 1 foot less than length

2.	Lengths of over 35 feet

a.	Reach is 2 foot less than length

3.	Ref: Chart IFSTA Ground Ladders pg 109

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

F.	Problem

1.	What ladder length is needed to gain access to a third story window of a boarding house

a.	Feet per story 8 to 10 ( take the larger )

b.	10 feet times 2 story

1.	20 feet

c.	Floor to window sill

1.	3 feet

d.	Extension beyond sill ( 3 ) rungs

1.	14 inch times 3 rungs

a.	42 inches = 4 feet

e.	Total length from ground to correct height above window sill

a.	27 feet

f.	Reach factor

a.	Ladder over 35 feet = 1 foot addition

g.	Length of ladder needed

a.	28 feet

2.	The 35 foot extension ladder is the most versatile ladder			( IFSTA Ground Ladder pg 110 )

G.	Removing Ladders from Apparatus

1.	Ladders are secured to the rack in some manner

a.	LFD has a spring tension rod which turns to lock or unlock, or a pin to lock the rack

2.	Ladders are carried nested

a.	More than one ladder may have to be removed from the rack to get the one needed

1.	Usually larger ladders are next to the truck and smaller ones on the outside

b.	Remaining ladders are not secured and may fall after others are removed

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

H.	Proper lifting and lowering methods 		( IFSTA Ground Ladders pg 113 )

1.	Rules of Thumb in lifting Ladders

a.	Adequate man power

b.	Back straight, Bend Knees, Lift with legs

c.	Do not lift with back or arms

d.	Lift in unison ( on Command )

e.	One person ( at the rear ) is to be in command at all times

2.	When Lowering a ladder

a.	Use the reverse of lifting

b.	Use leg muscles

c.	Keep body and feet parallel with the ladder	NOTE: FROM PAGE 17 TO HERE 1 HOUR

SHOWED TWO VIDEOS

II.	One Fire Fighter Carries

A.	Three ( 3 ) methods

1.	Low Shoulder

2.	High Shoulder

3.	Arm's Length

B.	Low Shoulder Carry

1.	From Vertical Racking / Side Removal		LFD		( IFSTA Ground Ladders pg 116 )

a.	Remove the locking device

b.	Face the center of the ladder

c.	Grasp a rung in each hand

1.	One hand on each side of the center line of the ladder

d.	Lift ladder and step back a step

e.	Release the Rung which is toward the tip of the ladder

1.	Pivot toward ( face ) the butt

2.	Insert the free arm between the rungs

3.	Place the upper beam on the shoulder of the free arm

4.	Grasp the rungs comfortable for carrying with the upper beam on the shoulder

5.	Carry with the butt end lowered

2.	From vertical racking / Rear removal

a.	Grasp bottom rung and pull ladder back

b.	Move to midpoint of ladder facing the butt end

c.	Place arm thur rungs

d.	Pull ladder completely out of rack

e.	Ladder is resting with upper beam on shoulder

f.	Grasp rungs for carrying

f.	Butt lowered

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

3.	From Flat Racking / Rear Removal

a.	Pull ladder almost all the way out of rack butt end first

b.	Lower butt to ground

c.	Grasp rungs at midpoint of ladder

d.	Bring far beam up as the body is pivoted toward the butt

e.	Arm on the ladder tip side is placed with the should against the upper beam

4.	From Flat on the Ground		( IFSTA Ground Ladder pg 119 )	TFC Standard to be performed

a.	Kneel facing the tip

b.	Grasp middle rung

1.	Palm facing forward

c.	Lift the ladder

1.	Pivot the body toward the butt with free arm between rungs

d.	Rest upper beam on shoulder

e.	Grasp rungs comfortable for carrying

f.	Carry facing the butt with butt lowered

g.	Lower shoulder carry with a roof ladder is different

1.	Carried tip forward and lowered ( NOT butt forward )

a.	Kneel and Face the butt

b.	Pivot toward the tip

5.	Texas Commission on Fire Protection, Basic Structure Fire Suppression Performance Standards 									Evaluation Form Fire Service Ladders ( # 5 )

a.	Performance Standard	Section 105-2.01b-f, and 105-3.01b-g

b.	One ( 1 ) man Low Shoulder Carry and Right Angle Raise of a 14 ft or 16 ft single ladder											( IFSTA Ground Ladder pgs 114 and 192 )

c.	Standard to be completed in less than 1 1/2 minutes ( 90 seconds )

1.	In full protective clothing for structural firefighters

a.	Helmet, Gloves, Coat, Trousers, boots, SCBA ( face piece not worn )

2.	Start 100 feet from a one story structure

3.	Lift ladder and using low shoulder carry approach building

4.	Verbalize " CLEAR OF ALL OBSTRUCTIONS " ( including overhead )

5.	Ladder the building

a.	Place butt against building, raise ladder, then move butt outward

b.	Place butt on the ground, out from building at proper distance, raise the ladder

6.	Check proper angle of inclination

7.	Verbalize " LADDER IS SUITABLE FOR CLIMBING "

NOTE : This is in two parts; First a carry ( Low Shoulder ) then a Raise ( Right Angle )

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

C.	High Shoulder Carry		( IFSTA Ground Ladder pg 121 )

1.	Suited for making beam raises

2.	Not used for carrying roof ladders

a.	When it is to be taken up another ladder to the roof

3.	From Vertical Racking / Side Removal

a.	Face the ladder at midpoint

b.	Hand nearest the ladder butt grips the top beam

c.	Lift the ladder enough to get the free hand under the bottom beam

d.	Lift the ladder free of the rack

e.	Pivot the body toward the ladder butt

f.	Place bottom beam on shoulder

g.	Grasp rungs comfortably for carrying

4.	From Vertical Racking / Rear Removal

a.	Pull ladder almost out of rack

b.	Move to midpoint of ladder

c.	Lift ladder, pivot body toward butt

d.	Place bottom beam on shoulder

e.	Grasp top beam and rung at a comfortable position

5.	From Flat Racking / Side Removal

a.	Face midpoint of ladder

b.	Pull ladder outward

c.	Place OUTSIDE RAIL ( BEAM ) on shoulder

d.	Turn inside beam up, resting bottom beam on shoulder

e.	Grasp top beam and a rung comfortable for carrying

6.	From Flat Racking /	Rear Removal

a.	Pull ladder almost out of rack

b.	Place butt on ground

c.	Move to midpoint of ladder

d	Lift ladder and turn to vertical

e.	Pivot body under bottom beam

f.	Place bottom beam on shoulder

g.	Grasp top beam and a rung for comfortable carrying

7.	From Flat on the Ground

a.	Kneel facing the tip of the ladder

b.	Grasp a middle rung palm facing toward tip

c.	Lift ladder and turn to vertical on beam

d.	Pivot body ( toward the butt ) and place shoulder under lower beam

e.	Position hands for comfortable carrying

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

D.	Arm's Length Carry		( IFSTA Ground Ladder pg 127 )

1.	Best suited for light weight ladders

2.	Control of ladder's side to side move is minimal

3.	The ladder will bump against the leg

4.	From Vertical Racking /	Side Removal

a.	Face the ladder at midpoint

b.	Lift the ladder and place on the ground on a beam ( use legs not back )

c.	Pivot body toward butt

d.	Kneel and grasps upper beam with one hand

e.	Straighten up with legs

f.	Carry with ladder hanging from one arm at arm's length

5.	From Vertical Racking / Rear Removal

a.	Pull ladder almost out of rack

b.	Place beam on ground and move to midpoint of ladder

c.	Pull ladder clear of the rack and place on beam on ground

d.	Pivot body toward the butt

e.	Kneel and grasp upper beam with one hand

f.	Lift ( with legs ) and with ladder hanging from one arm carry at arm's length

6.	From Flat Racking / Side Removal

a.	Face midpoint of ladder

b.	Pull ladder outward by a beam

c.	Hand nearest the butt grasps a rung

d.	Pull ladder completely out of the rack

e.	Allow the ladder to come to the vertical

f.	Release rung and pivot body toward the butt

g.	Carry at arm's length

7.	From Flat Racking / Rear Removal

a.	Pull ladder from rack

b.	Lower ladder to vertical

c.	Release hand nearest butt

d.	Pivot body toward the butt

e.	Carry ladder at arm's length

8.	From Flat on the ground			( IFSTA Ground Ladder pg 130 )

a.	Kneel facing the ladder at midpoint

b.	Turn the ladder on to one beam

c.	Grasp the upper beam ( lifting the ladder with the legs not the back )

d.	Pivot body toward butt

e.	The hand nearest the butt is released

f.	Ladder hangs at arm's length for carrying

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

E.	Special Ladder One Man Carries		( IFSTA Ground Ladder pg 169 )

1.	Folding / Attic ladders

a.	Ladder is in closed position

b.	Held about horizontal

c.	Butt first, slightly lowered

2.	Combination ladders

a.	At arm's length on edge

3.	Pompier ladders

a.	Hook to back

b.	Over Shoulder

c.	Butt slightly lowered

4.	Narrow Passage way carry

a.	Arm's length on edge

b.	Over the fireman's head

5.	Roof Ladders		( IFSTA Ground Ladder pg 120 )

a.	When

1.	One firefighter is to climb another ladder

a.	Roof ladder carried low shoulder

b.	Tip ( hooks ) forward

b.	From the ground

1.	Kneel facing butt ( reverse of other carries )

2.	Pivot toward the tip

c.	Carried to second ladder

1.	Another man opens the hooks OR

2.	The ladder is put down

a.	Hooks opened

b.	Picked up at midpoint of ladder

c.	Advanced up the second ladder

d.	When safe to do so, Roof ladder can be carried with hooks opened from rack to roof

1.	Hooks always turned away from the firefighter

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

III.		Two Fire Fighter Carries

A.	Three methods of carry

1.	Low Shoulder

2.	Hip / Under the arm

3.	Arm's Length

B.	Low Shoulder Carry

1.	From Vertical Racking / Side Removal

a.	Face the ladder ( one man near the tip, one man near the butt )

b.	Grasp the ladder and remove from rack

c.	Pivot body towards the butt

d.	Slide arm thur rungs and place top beam on shoulders

e.	The man in the rear ( near the tip ) is in control

2.	From Vertical Racking / Rear Removal

3.	From Flat Racking / Side Removal

4.	From Flat Racking / Rear Removal

5.	From Flat on the Ground		( IFSTA Ground Ladder pg 135 )

a.	Men on same side of  the ladder ( one near the butt and one near the tip )

b.	Kneel next to the ladder facing the tip

c.	Grasp rungs with palm forward toward the tip

d.	Butt man orders ( shoulder the ladder )

e.	Lift ladder with legs.  Bring the ladder to the vertical ( on one beam )

f.	As the ladder comes up pivot the body toward the butt and place an arm thru rungs

g.	The upper beam is placed on the shoulders

h.	The rear man ( near the tip ) is now in command

i.	Butt man places gloved hand over the butt spur of the top beam

j.	When carrying a roof ladder ( 2 man low shoulder )

1.	That is to be carried up another ladder

a.	Carry with the tip forward

b.	Hooks are closed until

1.	The second ladder is reached

2.	Stop, the tip man opens the hooks

6.	Texas Commission on Fire Protection, Basic Structure Fire Suppression Performance Standards 									Evaluation Form Fire Service Ladders ( # 5 )

a.	Performance Standard	Section 105-2.02a(1), and 105-3.02a(1)

b.	Two ( 2 ) man Low Shoulder Carry and Right Angle flat Raise of a 24 ft extension ladder

c.	Man to be evaluated will be at the ladder butt

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

d.	Standard to be completed in less than 3 minutes ( 180 seconds )

1.	In full protective clothing for structural firefighters

a.	Helmet, Gloves, Coat, Trousers, boots, SCBA ( face piece not worn )

2.	Start 100 feet from a two or more story structure.  Raise the ladder to

a.	A second story roof  OR

1.	Three (3) rungs above the roof

b.	A second story window

1.	Three (3) rungs inside the window

3.	Heel man verbalizes " Shoulder the ladder "

4.	Lift ladder and using proper low shoulder carry approach building

a.	Heel man has hand over butt spur

5.	Verbalize " CLEAR OF ALL OBSTRUCTIONS " ( including overhead )

6.	Ladder the building

a.	Heel man places butt on ground at the proper distance

7.	Heel man stands on the bottom rung of the ladder, facing the tip man

8.	Heel man kneels and grasps rung for lifting ladder and leans back

9.	Tip man walks ( hand over hand down the ladder toward the heel man ) lifting the ladder to 										the vertical

10.	Heel man grasps rungs higher and higher up as the ladder is raised until

a.	The ladder is vertical

b.	The men are facing each with the ladder between them

11.	Heel man verbalizes " FLY IS IN / OUT "

a.	Ladder is used with the Fly OUT

1.	Bed of ladder is between building and fly section

b.	If the fly is IN

1.	Pivot the ladder for fly out position

12.	Tip man grasps the outside of beams

13.	Heel man extends the fly

a.	Three (3) rungs into a window OR

b.	Three (3) rungs above a roof

14.	Heel man sets and locks the pawls and ties off the excess halyard

15.	Together they lower the ladder into position

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

16.	Tip man steps out of the way

17.	Heel man positions himself to check the angle of inclination

18.	Heel man Verbalizes " LADDER IS SUITABLE FOR CLIMBING "

C.	Hip / Under Arm Carry

1.	Not suited for three section ladders

2.	From Vertical Racking / Side Removal

a.	Face the ladder ( one man near the tip and one man near the butt )

b.	Grasp and lift the ladder from the rack

c.	Bring the ladder to chest high

d.	Release the hand near the tip

e.	Pivot the body toward the butt and reach across the tip beam

f.	Grasp a rung with the top beam placed against the body just under the armpit

g.	The rear man ( near the tip ) is in command

3.	From Vertical Racking / Rear Removal

4.	From Flat Racking / Side Removal

5.	From Flat Racking / Rear Removal

6.	From Flat on the Ground			( IFSTA Ground Ladders pg 138 )

a.	Stand on same side of the ladder, facing the butt

b.	Kneel and lift ladder to vertical ( on beam )

c.	Reach across the top beam and grasp a rung

d.	Stand ( using the legs not the back ) lifting the ladder against the body

e.	The upper beam should be just under the arm pit

f.	Butt man keeps a gloved hand over the heel spur

g.	Man in rear ( near the tip ) is in command

D.	Arm's Length on Edge Carry

1.	From Vertical Racking / Side Removal

a.	Face ladder ( one man at each end )

b.	Grasp ladder and lift from truck

c.	Lower ladder to arm length without bending body

d.	Release the hand nearest the tip

e.	Grasp the top beam with free hand

f.	Extend ladder at arm length and release hand nearest the butt

g.	Pivot body toward butt

h.	Man at butt puts gloved hand over heel spur

i.	Man at tip is in command

2.	From Vertical Racking / Rear Removal

3.	From Flat Racking / Side Removal

4.	From Flat Racking / Rear Removal

5.	From Flat on the Ground		( IFSTA Ground Ladder pg 143 )

a.	Get on same side of the ladder

b.	Tilt ladder on to one beam

c.	Squat slightly and grasp top beam

d.	Lift ladder with legs

e.	Carry at arm's length

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

E.	Multiple - Ladder Carry

1.	Remove ladders and stack ( one on top of the other ) on the ground ( largest on bottom )

2.	Men kneel facing each other ( one at each end of the ladders )

3.	Grasp rungs of bottom ladder

4.	Lift ladders ( with legs not back )

5.	Man at butt pivots so that the ladder is held behind his back

6.	Butt man leads

7.	Rear ( tip man ) is in command

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

IV.		Three Fire Fighter Carries

A.	Four ( 4 ) methods of carries

1.	Flat Shoulder

2.	Low Shoulder

3.	Flat arm's length

4.	On edge arm's length

B.	Flat Shoulder Carry

1.	From Vertical Racking / Side Removal

a.	Two fire fighters face the ladder ( one at each end )

b.	Third man stands to one end of the ladder so that he can walk behind the ladder as it is lifted off

c.	Lift ladder off rack and step back ( carefully )

d.	Third man steps between the rack and the ladder ( to mid point )

e.	Third man swings and lifts the lower beam and places it on his shoulder ( he faces the butt )

1.	The Top beam could be tilted onto the third man's shoulder

f.	The other two men pivot toward the butt and place the beam on their shoulders

2.	From Vertical Racking / Rear Removal

3.	From Flat Racking / Side Removal

4.	From Flat Racking / Rear Removal

5.	From Flat on the Ground		( IFSTA Ground Ladder pg 152 )

a.	Facing ladder tip

b.	Two men on one side of the ladder, kneel ( one at tip and one at butt )

c.	Third man kneels at midpoint on opposite side

d.	Knee closest to ladder is touching the ground

e.	Lift ladder to chest high

f.	All men pivot toward the butt

g.	Place the ladder beam on shoulders

C.	Low shoulder carry		( IFSTA Ground Ladder pg 156 )

1.	This is the same as the Two ( 2 ) man Low Shoulder carry except

a.	Third man is at the mid point on the same side

b.	REF: III. Two Fire Fighter Carries Numeral B

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

D.	Flat Arm's Length Carry		( IFSTA Ground Ladder pg 153 )

1.	From Vertical Racking / Side Removal

a.	Two fire fighters face the ladder ( one at each end )

b.	Third man stands to one end of the ladder so that he can walk behind the ladder as it is lifted off the

c.	Lift ladder off rack and step back ( carefully )

d.	Third man steps between the rack and the ladder ( to mid point )

e.	Third man grasps lower beam and pulls it upward towards himself until ladder is almost horizontal

f.	All three men ( using the hand nearest the ladder tip ) grasps a rung

g.	The men lower the ladder to arm's length

h.	Release the hand that is closest to the butt and pivot the body toward the butt

i.	Carry the ladder Flat at arm's length

2.	From Flat Racking / Side Removal

3.	From Flat Racking / Rear Removal

4.	From Flat on the Ground

a.	Two men kneel on the same side facing the butt ( one toward each end of the ladder )

b.	Third man kneels on opposite sides at the midpoint

c.	Grasp a rung and lift the ladder on command of the rear man ( nearest the tip )

d.	Carry Flat at arm's length

E.	Arm's Length on Edge Carry		( IFSTA Ground Ladders pg 157 )

1.	From Vertical Racking / Side Removal

a.	Face ladder, one man at each end, and third man between them

b.	Grasp ladder and lift from truck

c.	Lower ladder to arm length without bending body

d.	Release the hand nearest the tip

e.	Grasp the top beam with free hand

f.	Extend ladder at arm length and release hand nearest the butt

g.	Pivot body toward butt

h.	Man at butt puts gloved hand over heel spur

i.	Man at tip is in command

2.	From Vertical Racking / Rear Removal

3.	From Flat Racking / Side Removal

4.	From Flat Racking / Rear Removal

5.	From Flat on the Ground

a.	Three ( 3 ) men get on same side of the ladder, evenly spaced apart

b.	Tilt ladder on to one beam

c.	Squat slightly and grasp top beam

d.	Lift ladder with legs

e.	Carry at arm's length

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

V.		Four Fire Fighter Carries		( IFSTA Ground Ladder pg 157 )

A.	Four ( 4 ) methods of carries

1.	Flat Shoulder

2.	Low Shoulder

3.	Flat arm's length

4.	On edge arm's length

B.	Flat Shoulder Carry		( same as for three man )

1.	From Vertical Racking / Side Removal

a.	Two fire fighters face the ladder ( one at each end )

b.	Third and Fourth men stand at end of the ladder, they  walk behind the ladder as it is lifted off

c.	Lift ladder off rack and step back ( carefully )

d.	Third and Fourth men step between the rack and the ladder

e.	Third and Fourth men swing and lift the lower beam and places it on their shoulders ( face the butt )

1.	The Top beam could be tilted onto the third and fourth men's shoulders

f.	The other two men pivot toward the butt and place the beam on their shoulders

2.	From Flat Over Head Racking on a pumper / Rear Removal

3.	From Flat on the Ground

a.	Facing ladder tip

b.	Two men on one side of the ladder, kneel ( one at tip and one at butt )

c.	Third and Fourth men kneel on opposite side at same positions as first two men

d.	Knee closest to ladder is touching the ground

e.	Lift ladder to chest high

f.	All men pivot toward the butt

g.	Place the ladder beam on shoulders

C.	Low shoulder carry		( IFSTA Ground Ladder pg 163 )

1.	Not intended for use with pole ladders

2.	This is the same as the Two ( 2 ) man Low Shoulder carry except

a.	Third and Fourth men are evenly spaced ( all men are on the same side of the ladder )

b.	REF: III. Two Fire Fighter Carries Numeral B

D.	Flat Arm's Length Carry		( IFSTA Ground Ladder pg 162 )

1.	From Vertical Racking / Side Removal

a.	Two fire fighters face the ladder ( one at each end )

b.	Third and Fourth men stand to one end of the ladder, they  walk behind the ladder as it is lifted off

c.	Lift ladder off rack and step back ( carefully )

d.	Third and Fourth men step between the rack and the ladder

e.	Third and Fourth men grasps lower beam and pull it towards horizontal

f.	All four men ( using the hand nearest the ladder tip ) grasps a rung

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

g.	The men lower the ladder to arm's length

h.	Release the hand that is closest to the butt and pivot the body toward the butt

i.	Carry the ladder Flat at arm's length

3.	From Flat on the Ground

a.	Two men kneel on the same side facing the butt ( one toward each end of the ladder )

b.	Third and Fourth men kneel on the opposite side

c.	Grasp a rung and lift the ladder on command of the rear man ( nearest the tip )

d.	Carry Flat at arm's length

E.	Arm's Length on Edge Carry		( IFSTA Ground Ladders pg 163 )

1.	Not intended for pole ladders

2.	From Vertical Racking / Side Removal

a.	Face ladder one man at each end and the other two spaced evenly between them

b.	Grasp ladder and lift from truck

c.	Lower ladder to arm length without bending body

d.	Release the hand nearest the tip

e.	Grasp the top beam with free hand

f.	Extend ladder at arm length and release hand nearest the butt

g.	Pivot body toward butt

h.	Man at butt puts gloved hand over heel spur

i.	Man at tip is in command

3.	From Flat on the Ground

a.	Four ( 4 ) men get on same side of the ladder

b.	Tilt ladder on to one beam

c.	Squat slightly and grasp top beam

d.	Lift ladder with legs

e.	Carry at arm's length

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

V.		Five Fire Fighter Carries		( IFSTA Ground Ladder pg 164 )

A.	Normally required for a 40 or larger pole ladder

B.	Only Two types of carries are used

1.	Flat Shoulder

2.	Flat Arm's Length

C.	Flat Shoulder Carry

1.	From Flat Overhead Racking on a Pumper / Rear Removal		( IFSTA Ground Ladder pgs 158 & 165 )

a.	Two men stand on tailboard to pull ladder

b.	Three men stand about where the butt of the ladder will come down

1.	One man at the end of the ladder

2.	One man on each side of the ladder

c.	As the ladder comes out of the rack

1.	Three men on the ground take the butt

2.	The men on each side hand walk the ladder toward it's middle

a.	The man at the butt walks backward as the ladder comes off

3.	The men on the tailboard take the tip of the ladder and step from the truck

4.	The man at the butt moves around to one side of the ladder

5.	Alternate space three men on one side of the ladder and two on the other

6.	Pivot toward the butt and rest the ladder beams on the shoulder's

7.	Carry flat, butt forward

8.	Man at right rear ( tip end ) of ladder is in command

2.	From Flat on the Ground		( IFSTA Ground Ladder pgs 152 & 164 )

a.	Facing ladder tip

b.	Three men on one side of the ladder, kneel ( one at tip and one at butt and one at midpoint)

c.	Fourth and Fifth men kneel equal distance from the end on the opposite side

d.	Knee closest to ladder is touching the ground

e.	Lift ladder to chest high

f.	All men pivot toward the butt

g.	Place the ladder beam on shoulders

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

D.	Flat Arm's Length Carry

1.	From Vertical Racking / Side Removal		( IFSTA Ground Ladder pgs 156 & 166 )

a.	Three fire fighters face the ladder ( one at each end & one at middle )

b.	Fourth and Fifth men stand at the ends of the ladder so that they can walk behind the ladder

c.	Lift ladder off rack and step back ( carefully )

d.	Fourth and Fifth men step between the rack and the ladder ( in a staggered position to men opposite )

e.	Fourth and Fifth men grasps lower beam and pulls it upwards until ladder is almost horizontal

f.	All five ( 5 ) men ( using the hand nearest the ladder tip ) grasps a rung

g.	The men lower the ladder to arm's length

h.	Release the hand that is closest to the butt and pivot the body toward the butt

i.	Carry the ladder Flat at arm's length

2.	From Flat Racking / Side Removal

3.	From Flat Racking / Rear Removal

a.	One man faces the butt

b.	Two men stand on each side of the ladder and face each other

c.	All three pull ladder from rack

d.	As the ladder is pulled out of the rack

1.	The men on the side remain in position and hand walk the ladder out of the rack

2.	When one third ( 1 / 3 ) of the ladder is pulled out

a.	The fourth and fifth men step up to take the ladder at the tip

3.	As the ladder is pulled free of the rack

a.	The butt man steps to one side of the ladder

b.	The men stagger themselves equal distance

c.	Three men on one side two on the other

d.	All pivot to face the butt

e.	Grasp a rung palm forward

f.	right side rear ( tip of ladder ) man is in command

g.	Lower ladder to flat arm's length for carry

4.	From Flat on the Ground

a.	Three men kneel on the same side facing the tip ( one at each end one at middle )

b.	Fourth and Fifth men kneel on opposite side, staggered to men opposite

c.	Grasp a rung ( palms forward ) and lift the ladder

d.	Right rear man at the tip is in command

d.	Carry Flat at arm's length butt forward

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

VI.		Six Fire Fighter Carries		( IFSTA Ground Ladder pg 167 )

A.		Only Two types of carries are used

1.	Flat Shoulder

2.	Flat Arm's Length

B.		Flat Shoulder Carry 			AND

C.		Flat Arm's Length Carry

1.	Men are located across from one another ( 2 each at )

a.	Butt

b.	Midpoint

c.	Tip

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

Module 4		Give Students																	Time

I.	Ladder Placement Guidelines		( IFSTA Ground Ladders pg 183 )

A.	Positioning / Placement of Ladders			( IFSTA Ground Ladders pg 172 )

1.	Who is responsible

a.	Normally an officer

b.	FireFighter nearest the butt

1.	On the right side of the ladder

2.	Two Objectives in Positioning

a.	Placement for intended use

b.	Placement of butt at the proper distance from the building for

1.	Safety

2.	Easy of climbing

3.	Intended Use of the Ladder

a.	Breaking a window for ventilation

1.	To Windward

2.	Along side

3.	The tip is even with the top of the window

b.	The Ladder is used in the same manner for a narrow window as for breaking a window ( M 4-I 3 a )

1.	For climbing in or out of the window

c.	Rescue from a window

1.	Ladder Tip is

a.	Even with OR Slightly below

1.	The window sill

b.	Window sill projects out from the wall then

1.	Wedge the tip under the window sill

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

c.	Wide Window

1.	Ladder is placed in the window by

a.	Two or Three rungs

d.	Vantage point for a hose stream		( IFSTA Ground Ladder pgs 173 & 346 )

1.	NO ENTRY is to be made

a.	Raise the ladder directly in front of the window		( Ventilation says never do this )

1.	Tip of ladder on wall above the window

e.	Smoke ejector support

1.	Used in the same manner as for hose stream support

B.	Considerations before placement of ladders		( IFSTA Ground Ladder pg 175 )

1.	Over head obstructions

a.	Wires		( Electrical or others )

2.	Slopes / Inclines

3.	Ground Obstructions

a.	Vehicles

b.	Trees / Scrubs

c.	Unsafe ground

a.	Mud

4.	Areas to avoid in raising a ladder

a.	Exit or Entry paths

b.	Areas of possible Flame / Ladder contact

c.	Unstable ground

d.	Trapdoors

e.	Against unstable walls or surfaces

C.	Ladder Butt Placement		( IFSTA Ground 176 )

1.	Desired angle of inclination

a.	75 degrees

1.	Less stresses on ladder

2.	Easy of Climbing

a.	The climber is perpendicular to the ground

b.	And at arm's length from the ladder

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

b.	Butt too far from building

1.	CAN NOT carry as much weight

2.	More likely to slip

c.	Butt too close to building

1.	Stability is reduced

a.	Tip pulls away from the building

2.	Placement from building RULE OF THUMB

a.	Estimate length of ladder needed for the job		( IFSTA Ground Ladder pg 107 )

1.	Divide required length by four ( 4 )

b.	The distance the butt is to be placed from a building

1.	Is one fourth ( 1 / 4 ) the length needed to do the job

2.	Example

a.	If 20 feet of a ladder is needed

b.	20 divided by 4 equals 5 feet

c.	The butt is placed 5 feet from the building

II.	Support Skills for Ladder Raises		( IFSTA Ground Ladders pg 184 )

A.	FLY IN / FLY OUT that was the Question

1.	All ladders are used with the FLY OUT except for

a.	Wood Truss Ladder where

1.	The rungs are mounted in the top rail

2.	IFSTA says FLY OUT

3.	NFPA 1932 says FLY OUT

4.	National Bureau of Standards says FLY OUT

B.	Pivoting 		( IFSTA Ground Ladders pg 185 )

1.	The act of turning on a vertical Axis

2.	When needed

a.	Ladder raised with fly in ( between bed and building )

b.	Any ladder flat raised and parallel to the building

3.	One man pivot

a.	Used only for

1.	Single ladders

2.	roof ladders

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

4.	Single and Roof ladder Pivot procedures for ONE MAN

a.	Decide which beam to pivot on

b.	Place foot on inside butt spur of pivot beam

c.	Grasp pivot beam low

d.	Grasp Non - Pivot beam high

e.	Tilt ladder on pivot beam

f.	Pivot body and ladder

1.	90 degrees

g.	Face ladder and adjust as needed

5.	TWO MAN Pivot Procedures		( IFSTA Ground Ladder pg 186 )

6.	Pole ladder pivot from a parallel raise		( IFSTA Ground Ladder pg 225

C.	Vertical Shifting		( IFSTA Ground Ladders pg 185 )

1.	Moving while vertical

2.	One man Shift

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

3.	Two man Shift

4.	Pole Ladder Shifting

D.	Hoisting a ladder		( IFSTA Ground Ladder pg 349 )

III.	ONLY TWO Raises in relation too the building		( IFSTA Ground Ladders pg 192 )

A.	Right Angle Raise

B.	Parallel Raise

IV.	One FireFighter Raises		( IFSTA Ground Ladders pg 194 )

A.	Short single and roof ladders

1.	Fourteen ( 14 ) feet or less

B.	Long Single and Roof ladders

1.	Over fourteen ( 14 ) feet

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

C.	Extension Ladders		( IFSTA Ground Ladder pg 196 )

D.	Folding / Attic		( IFSTA Ground Ladder pg 230 )

E.	Combination Ladder		( IFSTA Ground Ladder pg 231 )

1.	Extension / A Frame

2.	Single / A Frame

3.	Telescoping Beam / A Frame

4.	Extending / A Frame

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

5.	Pompier Raise		( IFSTA Ground Ladder pg 237 )

a.	Procedure

b.	Using a Roof Ladder as a Pompier Ladder		( IFSTA Ground Ladder pg 343 )

V.		TWO FireFighter Raises		( IFSTA Ground Ladders pg 201 )

A.	Flat Method

B.	Beam Method

VI.	THREE FireFighter Raises		( IFSTA Ground Ladders pg 204 )

A.	Flat Raise

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

VII.	FOUR FireFighter Raises

A.	Flat Raise

VIII.	POLE ladder Raises		( IFSTA Ground Ladders pg 209 )

A.	Handling the Staypoles

1.	Attached

2.	Un - Attached

B.	Four FireFighter Pole ladder raises

1.	Right Angle Raises

2.	Parallel Raises

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

C.	Five and Six man pole ladder Raises

1.	Flat Right Angle Raise		( IFSTA Ground Ladder pg 219 )

2.	Flat Parallel Raise

3.	Parallel Beam Raise		( IFSTA Ground Ladder pg 227 )

IX.	Dome / Auditorium Raise		( IFSTA Ground Ladders pg 239 )

A.	Used to reach places

1.	Where there is nothing to support the ladder tip against

B.	Four ( 4 ) guy lines are arranged at the ladder tip

1.	Two ( 2 ) 125 foot life lines are used

2.	Requires a minimum of Six ( 6 ) men

a.	Eight ( 8 ) men is preferred

C.	Six ( 6 ) Man Dome Raise

1.	Ladder placement

a.	Flat

b.	Fly up

1.	Extended about five ( 5 ) rungs

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

2.	Rope Attachment		( IFSTA Ground Ladder pg 240 )

a.	Rope is looped ( doubled )

b.	Loop is placed over top beam ( at fly )

1.	Passed down between the rungs

2.	Under the bottom beam

3.	Padded up the outside beam

4.	Passed over the ropes on the top beam

5.	The loop is placed over the tip of the fly

1.	Pulled tight

3.	Raising the Dome

a.	Four ( 4 ) men grasp the ropes

1.	One ( 1 ) to each corner

2.	Hold the rope behind the rump

a.	The ladder pulls against the man's weight

b.	Raise the ladder in the usually manner

1.	For pole ladders, keep the guy lines outside the poles

2.	Extend the fly

3.	Engage the Pawls

4.	The rope and pole men brace themselves for the ladder climb

X.	Hotel / Factory Raises		( IFSTA Ground Ladders pg 242 )

XI.	Obstructed Ladder Raises		( IFSTA Ground Ladders pg 245 )

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

XII.	Securing the ladder after the raise		( IFSTA Ground Ladders pg 252 )

A.	Heeling

1.	NFPA 1932 requires it

2.	Common sense dictates it

3.	Two Methods

a.	FireFighter beneath ladder

1.	Grasps ladder beams at eye level

a.	DO NOT grasps rungs

2.	Pull slightly back on the beams

3.	Feet apart

4.	DO NOT look up

b.	FireFighter in front of ladder

1.	Stands in path of travel

2.	FireFighter faces ladder

3.	Toes against the butt spurs

4.	Grasps

a.	Beams OR

b.	Rungs

5.	DO NOT look up

B.	Tying in

1.	Top

a.	Prevents tip from pulling away from the building

2.	Bottom

a.	Prevents slipping

3.	Use a Rope hose tool or a length of rope

C.	Tying the Halyard

1.	Tying off excess halyard

a.	Prevents the fly from falling

b.	Prevents tripping of firefighter

2.	Open ended halyard tie

a.	Wrap excess around two rungs

b.	Pass free end of rope across working end and rung

c.	Tie a Clove Hitch

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

3.	Closed ended halyard tie

a.	Wrap excess around two rungs

b.	Pass loop across working end and rung

c.	Tie a Clove Hitch

4.	The Tied Halyard Positioning

a.	Remain in middle of ladder

b.	Pushed to one beam

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

Module 5		Give Students																	Time

I.	Ready to Climb		( IFSTA Ground Ladders pg 259 )

A.	Climbing Skills

1.	Angle of Inclination		( IFSTA Ground Ladder pg 14, and 259 )

a.	Face ladder

b.	Toes Touching butt spurs

c.	Arms at right angle to body

1.	Grasp a rung, palm down

a.	Palm falls on rung at shoulder height

1.	Correct angle of inclination

b.	Palm will not reach the rung

1.	Butt too far from building

c.	Palms extend thur ladder

1.	Butt too close to the building

2.	Techniques of Climbing

a.	Smooth

b.	Rhythmical

c.	Use bended knees to ease climbers weight impact on ladder

d.	No sudden or jerky movements

e.	Body erect

f.	Arms straight and 90 degree angle to body

g.	DO NOT reach above the shoulder to grasp a rung

h.	Palms down

1.	Thumbs on underside of rung

i.	Check each fly section pawls before stepping onto the fly

j.	Practice slowly

1.	Develop form

2.	NOT speed

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

k.	Carrying items on a ladder

1.	Slide free hand along under the beam

2.	Best to use a rope to hoist items

II.	Working from a ladder		( IFSTA Ground Ladder pg 259 )

A.	Locking in, Two ( 2 ) Methods

1.	Safety belt

a.	Called ladder belt

b.	Truck man's belt

c.	Procedure

1.	Place around waist

2.	Hook is placed over rung OR

3.	Strap is placed around the ladder

2.	Leg lock

a.	Climb to where you need to be

b.	Place both feet on rung

c.	Grasp a rung just below shoulder height

d.	Lift up the leg which is

1.	Opposite from which the work will be done

2.	Place the leg one rung higher

3.	Move the hips back

a.	Keep leg and arm straight

e.	Bring ladder close to ladder after leg is in place

f.	Place the foot back thur the ladder

1.	Knee is around one rung

2.	The instep of the foot can be placed over the beam

B.		To Place a Roof Ladder		( IFSTA Ground Ladder pg 270

1.	Two ( 2 ) methods for ONE firefighter

2.	Three ( 3 ) methods for TWO firefighters

3.	ONE firefighter method

a.	Low Shoulder Carry

1.	Carry ladder to the second ladder

2.	Check for over head obstructions

3.	Place the Roof ladder on the ground

4.	Open hooks

5.	Again pick up the Roof ladder by the LOW SHOULDER method

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

6.	Mount the second ladder

a.	Turn the body slightly away from the building

b.	Grasp the beam with the free hand

7.	Swing the Roof ladder up to clear the roof of the building

8.	Lock in

a.	Safety Belt

b.	Leg Lock

9.	With the Roof ladder resting on the beam

a.	Remove the arm from the rungs

10.	Slide the Roof ladder ( still on the beam )

a.	Up the roof to the ladder's balance point ( midpoint )

11.	Lay the Roof ladder flat

a.	Hooks down

12.	Slide the ladder all the way up the building roof

13.	The Hooks should drop over the ridge peak of the roof

14.	Pull back on the Roof ladder to properly set the hooks to grasp the peak

b.	Modified Shoulder Method for ONE firefighter

1.	Ladder is carried to second ladder

2.	Check for overhead Obstructions

3.	Place the ladder on the ground

4.	Open the hooks

5.	Pick the ladder up vertically

6.	Place ( lean ) the roof ladder against the beam of the second ladder

7.	Mount the second ladder

8.	Place an arm thur the rungs of the Roof ladder

a.	One or Two rungs above midpoint of the Roof ladder

9.	Swing the Roof ladder up to clear the roof of the building

10.	Lock in

a.	Safety Belt

b.	Leg Lock

11.	With the Roof ladder resting on the beam

a.	Remove the arm from the rungs

12.	Slide the Roof ladder ( still on the beam )

a.	Up the roof to the ladder's balance point ( midpoint )

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

13.	Lay the Roof ladder flat

a.	Hooks down

14.	Slide the ladder all the way up the building roof

15.	The Hooks should drop over the ridge peak of the roof

16.	Pull back on the Roof ladder to properly set the hooks to grasp the peak

4.	Two FireFighter method		( IFSTA Ground Ladder pg 275 )

a.	Hook first method

1.	Roof ladder is carried using Low Shoulder method to second ladder

2.	FireFighter at tip opens hooks away from the body

3.	Check for overhead obstructions

4.	Mount the second ladder

5.	The tip man locks in

a.	Safety Belt

b.	Leg lock

6.	Remove ladder from shoulder

a.	Place ladder on beam

b.	Both men push the ladder onto the roof to balance point of the ladder

7.	With the Roof ladder resting on the beam

a.	Remove the arm from the rungs

8.	Slide the Roof ladder ( still on the beam )

a.	Up the roof to the ladder's balance point ( midpoint )

9.	Lay the Roof ladder flat

a.	Hooks down

10.	Slide the ladder all the way up the building roof

11.	The Hooks should drop over the ridge peak of the roof

12.	Pull back on the Roof ladder to properly set the hooks to grasp the peak

b.	Butt First Method

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

c.	Modified Butt First Method

C.	Climbing a Pompier Ladder		( IFSTA Ground Ladder pg 280 )

III.	Rescues by Ladder		( IFSTA Ground Ladder pg 292 )

A.	Two ( 2 ) Basic Types of Rescues

1.	Conscious 	/	 Physically Able

2.	Unconscious 	/	 Non - Physically Able

B.	Conscious / Physically Able

1.	Best to use

a.	Stairway

b.	Thur connecting buildings

c.	Over connecting roofs

d.	Elevating Platforms

e.	Aerial Ladder

f.	Ground Ladder

2.	General Rescue Procedures

a.	One or more firefighters enter the building to help victim

b.	One Firefighter at top of ladder

c.	Assistant firefighters spaced on ladder

d.	Place victim on ladder in a normal climbing position

e.	Firefighter at tip

1.	Places his arms under the arm pits of victim

2.	Grasp the rung in front of the victim

f.	Firefighter places one knee between the legs of the victim

g.	Firefighter explains to victim what is happening, what is going to happen, and calms victim

h.	Descend ladder together, at victim's pace

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

C.	Unconscious / Non - Physically able

1.	General Rescue Procedures

a.	Two or more firefighters enter building

b.	One firefighter to tip of ladder

1.	Position Knee at a 90 degree bend

2.	Horizontal to ground

c.	The victim is passed to firefighter at the ladder tip

1.	Sit victim on bent knee

2.	Victim's back is to the firefighter

3.	Victim's feet are placed on the outside of the beams

4.	Firefighter's arms are under the victim's arm pits

a.	Grasp a rung in front of the victim

5.	If victim revives

a.	He can climb down

b.	Firefighter can secure him if he comes to and panics

d.	Victim is held to ladder as the Firefighter's knee is moved down one rung

e.	Victim is lowered onto the knee again

f.	Continue down the ladder in like manner

g.	Upon reaching the ground with an Unconscious / Non - Physically able victim

1.	Step off ladder

2.	Lock hands in from of victim's chest

3.	Pull victim clear of ladder

4.	Remove victim to safe area

a.	Best to use two firefighters for carry

2.	Face To Face - Alternate Method

a.	Same as General Procedures except

1.	Victim is face to face with Firefighter

a.	Victim's arms are over Firefighter's shoulders away from rungs

2.	Firefighter is more vulnerable to

a.	Choking

b.	Hitting

c.	Kicking

d.	Screaming

e.	Bad Breath

f.	Being distracted by big tits

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

3.	Removing an Unconscious FireFighter, Leg Locked in		( IFSTA Ground Ladder pg 298 )

a.	Procedures for freeing leg

1.	One Firefighter up ladder

a.	Give first aid as needed

b.	Secures unconscious Firefighter

1.	Arms under victims arms

2.	Grasp rung in front of victim

3.	Lean against victim

2.	Raise second ladder

a.	Same side as victim's leg lock

b.	For large victims, a third ladder will be needed on opposite side

3.	Firefighter up second ladder to level of victim's leg lock

a.	Free victim's foot

1.	Foot should hang over one rung between building and ladder

b.	Remove Boot

c.	Firefighter climbs above victim and leg locks in

4.	Both FireFighters lift victim to free leg completely

5.	Precede as with unconscious victim ( Module 5 III C )

b.	Alternate method fro freeing leg

1.	Person on second ladder cuts rung

a.	Limited time

b.	Heavy victim

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

Module 6		Give Students																	Time

I.	Special Uses for Ladders		( IFSTA Ground Ladder pg 301 )

A.	Common Special Uses

1.	Below Grade use of ladders

2.	Bridging with Ladders

a.	Four ( 4 ) methods

b.	Bridging over Fences		( IFSTA Ground Ladder pg 355 )

c.	briding for a hoseline across roadway		( IFSTA Ground Ladder pg 358 )

3.	Ladders as a slide for stretchers

4.	Ventilating with a ladder tip

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

5.	Supporting Smoke Ejectors		( IFSTA Ground Ladder pg 361 )

6.	Water Chute / Catch Basin		( IFSTA Ground Ladder pgs 364 & 374 )

B.	Uncommon Special Uses

1.	Ladders as Fulcrums		( IFSTA Ground Ladder pg 315 )

2.	Ladder as a Sling Support

3.	Ladders as stretchers

4.	Ladders used in ice Rescue

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

5.	Ladders as a Pry tool

6.	Ladders used in Shoring

7.	Ladders used as a Gin pole		( IFSTA Ground Ladder pg 334 )

8.	Extending the reach of an aerial platform

9.	As a Battering Ram		( IFSTA Ground Ladder pg 366 )

10.	Improvised Cellar Pipe

11.	Daming a small stream

LUBBOCK FIRE DEPARTMENT		Instructor's Guide			Lesson Plan for Ground Ladders

****************************************************************************************************

II.	Ladder Safety Review		( IFSTA Ground Ladder pg 283 )

A.	Precautions for ladder operations

1.	There is no substitute for the use of good common sense during all operations ( ladders or otherwise )

2.	Full protective gear at all times

3.	Adequate man power for all operations at all times

4.	Use leg muscles, NOT BACK or ARMS, to lift

5.	ALWAYS, ALWAYS check for OBSTRUCTIONS ( Overhead, Beside, In Front OF, etc. )

6.	When heeling a ladder, DO NOT LOOK UP.  Especially when heeling from under the ladder

7.	ALWAYS before climbing a ladder check for proper placement ( Angle of Inclination, Stability, etc )

8.	ALWAYS check the Pawls of each Fly section before climbing onto it.

9.	ALWAYS tie off the excess halyard

10.	Secure the ladder top if possible

11.	When climbing a ladder do so Smoothly and Rhythmically

12.	When working from a ladder ALWAYS tie in ( leg lock or safety belt )

13.	DO NOT MOVE a ladder a firefighter has used for making entry ( he may need to get out )

14.	ALWAYS watch the tip of a ladder when raising or lowering the fly section

15.	Inspect Periodically and AFTER EACH USE

16.	When in doubt, Check it out.  Heat and other test as needed