IFSTA Ground Ladders -- 8th Edition

Ladder Test Complete Book

January 27, 1994

1.  Lad - pg xii

______ is a hole, groove, or slot cut into a wood ladder beam to receive a rung tenon.

A.  Mortise		B.  Knurled		C.  Checks		D.  Dimpled

2.  Lad  - pg xii & FE pg xiii

______ is a notch, hole, or space cut into a door to receive a lock case, which contains the lock mechanism

A.  Mortise		B.  Knurled		C.  Checks		D.  Dimpled

3.  Lad - pg 1

The major objectives of fireground operations are

A.  Rescue, confinement, and extinguishment		B.  Fire prevention, confinement, and extinguishment

C.  Fire prevention, rescue, and extinguishment		D.  Fire prevention, confinement, and extinguishment

4.  Lad - pg 4

NFPA ____ - "FireFighters Professional Qualifications" sections 3-9, 4-9, and 5-5 of this standard are requirements for working with ground ladders

A.  1011		B.  1901		C.  1932		D.  1001

5.  Lad - pg 5

The minimum lengths, types, and numbers of ground ladders to be carried on apparatus and specified by NFPA Standard ______ : Standard on Design and Design Verifications Tests for Fire Department Ground Ladders

A.  1931		B.  1932		C.  1001		D.  1001

6.  Lad - pg 5

The  use, care, and service testing of ground ladders are specified by NFPA Standard ______ : Standard on the Use, Maintenance, and Service Testing of Fire Department Ground Ladders.

A.  1931		B.  1932		C.  1001		D.  1011

7.  Lad - pg 9

Single ladders vary in length from 6 to _________ feet with more common lengths ranging from 12 to 24 feet

A.  28		B.  32		C.  40		D.  45

8.  Lad - pg 9

"A ladder that is non adjustable in length and consists of but one section."  The statement best describes which one of the following types of ladder, according to the terms used on the Lubbock Fire Dept.

a)	combination		b)	extension

c)	straight			d)	folding

9.  Lad - pg 9

A ladder that has one section and is of fixed length is a ______ ladder.

a.  pole		b.  combination		c.  single	d.  attic

10.  Lad - pg 10

Roof ladders are single ladders with a fixed gooseneck shaped hook attached to the tip.

a.  true		b.  false

11.  Lad - pg 10

Which one of the following statements most relates to a "roof ladder":

a.  a mechanical unit that is mounted upon a specially built chassis.

b.  equipped with folding hooks at the top end.

c.  consists of two or more sections which travel in guides to permit length adjustment.

d.  suitable for use as a stepladder.

12.  Lad - pg 11

A ladder designed for use in narrow hallways or aisles is a ________ ladder.

a.  folding		b.  pompier		c.  roof			d.  straight

13.  Lad - pg 12

A ladder that can be used as either a single ladder or an A-frame ladder is one type of ________ ladder.

a.  jacobs		b.  combination		c.  pompier		d.  attic

14.  Lad - pg 12

Pole ladders are long extension ladders that have staypoles attached.

a.  true		b.  false

15.  Lad - pg 12 & 69

The poles which are attached to long extension ladders are called tormentor or ____________

a.  outriggers		b.  pompier 		c.  truss			d.  staypoles

16.  Lad - pg 12

Which one of the following statement best describes an extension ladder:

a.  consists of two or more sections which travel in guides or brackets to permit length adjustment.

b.  a ladder equipped with folding hooks at the top end.

c.  adjustable in length and having a means for locking the sections together so that the two sections can form

equal angles with the ground as a stepladder.

d.  non adjustable in length and consisting of but one section.

17.  Lad - pg 12

Pole ladders are extension ladders that have stay poles added for stability.  This ladder is also called _______ ladders

A.  Bangor		B.  Pompier		C.  Tormentor		D.  Three Sectional Extension

18.  Lad - pg 14

The two principal structural sides of a ladder are called:

a.  rungs		b.  plates		c.  beams		d.  truss blocks

19.  Lad - pg 14

A ladder having a single beam with rungs protruding from both side of it is a _____ ladder.

a.  folding		b.  jacobs		c.  pompier		d.  combination

20.  Lad - pg 14

________ Is the term that refers to the way a ladder is placed in relation to a horizontal plane.

a.  horizontal angle		b.  horizontal inclination

c.  angle of inclination		d.  angle of horizontal

21.  Lad -  pg 15

The lower, 2 beams, section of an extension ladder is known as the:

a.  fly		b.  bed		c.  beam		d.  heel

22.  Lad -  pg 15

The lowest end piece of the bed section of an extension ladder is known as the _______.

a.  bed		b.  fly		c.  heel		d.  toe rail

23.  Lad - pg 16

The upper section of an extension ladder is known as the _______.

a.  fly		b.  beam			c.  bed		d.  rail

24.  Lad - pg 16

The rope used for hoisting the fly sections of a ladder is known as a:

a.  hand line		b.  block		c.  safety line		d.  halyard

25.  Lad - pg 16

The cable used for hoisting the fly section of a ladder is known as a ______.

a.  handline		b.  safety line		c.  halyard		d.  cable line

26.  Lad - pg 16

NFPA 1931 requires the ladder length marker to be placed within _______ inches of the butt of each side rail of a single ladder and the base section of extension and pole ladders.

A.   6		B.  12		C.  15		D.  18

27.  Lad - pg 17

NFPA 1931 requires that ground ladders bear a unique individual identification number (serial number)

A.  true		B.  False

28.  Lad - pg 18

The term "Ladder Nesting" refers to the construction feature of extension ladders where the fly section fits between the beams of the bed section.

a.  true		b.  false

29.  Lad - pg 20

The cross members on a ladder (usually round) between the beams on which people climb are known as the:

a.  beams		b.  rungs		c.  rails			d.  struts

30.  Lad - pg 20 & 21

A beefed up roof ladder, which is NOT TO BE USED FOR CLIMBING, is called a _________ ladder

A.  Roof		B.  Stripping		C.  Roof Stripping	D.  None of the above

31.  Lad - pg 27

There are ____ types of ground ladders

a.   5		b.   6		c.   7		d.  10

32.  Lad - pg 30

NFPA 1931 requires all manufacturers to provide certification attesting that the particular ladder was constructed in accordance with the standard's requirements.  This certification is in the form of a _____ which is affixed to the ladder

a.  label			b.  identification number		c.  heat sensor label	d.  none of the above

33.  Lad - pg 31

Metal fire service ladders are usually built from ______.

a.  steel alloy		b.  magnesium		c.  aluminum alloy	d.  chrome alloy

34.  Lad - pg 31

The beams of wooden ladders are generally made of:

a.  white spruce		b.  Douglas fir		c.  white ash		d.  Hickory

35.  Lad - pg 31

Fiberglas is heavier than corresponding metal and wood ladders

A.  True		B.  False

36.  Lad - pg 33

If a metal ladder loses its strength during exposure to heat, it _____ regain its original strength when it cools.

a.  will						b.  will not

c.  depends on how well made the ladder is		d.  will warp but

37.  Lad - pg 33

A major disadvantage of a metal fire service ladder is ____.

a.  weight				b.  strength

c.  conductivity (electrical)		d.  durability

38.  Lad - pg 33

Metal ladders are subject to sudden failure when exposed to heat or flame temperatures of _____ deg. F. even for  short periods of time

a.  300		b.  400		c.  600		d.  800

39.  Lad - pg 34

_____ is the damage due to wind flexing the growing trees.

a.  checks			b.  wind shakes		c.  pitch pockets			d.  shim splits

40.  Lad - pg 35

Ladder side rail stock sawed so that the grain is ANGULAR will provide strength but not as much as when the stock is sawed with the grain FLAT.

A.  True		B.  False

41.  Lad - pg 36

The rungs for wooden ladders are usually made from:

a.  second-growth hickory		b.  wide-grained Douglas fir

c.  close-grained white oak	d.  first-growth white spruce

42.  Lad - pg 37

What indication, other than a change in the heat sensor label, may be present if a fiber glass ladder is exposed to excessive heat?

a.  discolor		b.  crack		c.  swelling		d.  blister

43.  Lad - pg 38

The specified widths of ladders are based on the type of ladder except in one case where it is based on the length of the ladder.  This would be for extension ladders over ______ feet and pole ladders.

A.  10		B.  14		C.  16		D.  24

44.  Lad - pg 39 table 2.3

What is the load rating for single, roof, extension and pole type ladders?

a.  250 LBS		b.  300 LBS		c.  750 LBS		d.  500 LBS

45.  Lad - pg 39 table 2.3

The maximum ladder load weight for folding and pompier ladders is ______, for all other ladders it is ______.

a.  300, 750		b.  400, 800		c.  400, 850		d.  400, 750

46.  Lad pg 39

The weight of a charged hose line being used off of a ground ladder is not counted when figuring maximum ladder loading.

a.  true		b.  false

47.  Lad - pg 39

The new NFPA 1931 now requires all ladders except pompier ladders and extension ladders under 16 feet to be the same minimum width of ___ inches

a.  12		b.  16		c.  18		d.  16 to 20

48.  Lad - pg 39 table 2.3

Three fire fighters are climbing a properly positioned 20 foot single ladder.  All are on ladder at one time.  One weigh 150 pounds, another 175 pounds and the other weighs 190 pounds.  Each is wearing protective clothing weighing 40 pounds and each has on an SCBA weighing 30 pounds.  The ladder is over loaded, according to maximum weight limitations specified in NFPA 1932.

a.  true		b.  false

49.  Lad - pg 40

Usually metal ladders less than _____ feet are solid beams because they are lighter, ladders longer than this are truss beams because they are lighter.

a.  24		b.  35		c.  16		d.  40

50.  Lad - pg 40

Solid beam ladders in lengths of 24 feet and under are lighter than comparable truss beam ladders.

a.  true		b.  false

51.  Lad - pg 40

There are two basic beam designs used for single, roof, and pole ladders which are solid and ______ beams

a.  truss		b.  side rail		c.  rail		d.  bed

52.  Lad - pg 42

Generally extension ladders make use of tongue and groove construction, but ladders with guides are also still in  use.  If the ladder has guides, one is riveted to the fly section beam near the ______________ and another guide is riveted to the bed section beam near the __________.

A.  Bottom; Top		B.  Top; Bottom		C.  Top; Top		D.  Bottom; Bottom

53.  Lad - pg 45 & 50

Rungs for both solid and truss beam ladders must be a minimum of _____ inches in diameter

a.  1 1/2		b.  1 1/8		c.  1 1/4		d.  1 1/16

54.  Lad - pg 45

NFPA 1931 requires rungs, except for those of ___________ and Pompier ladders, to be not less than 1 1/4 inches in diameter.

A.  Straight ladders less than 10 feet		B.  Extension ladders less than 24 feet

C.  Folding ladders				D.  Stripping ladders

55.  Lad - pg 45

Rungs on ladders are spaced on _________ inch centers plus or minus 1/8 inch.

A.  12		B.  14		C.  16		D.  18

56.  Lad - pg 45

A 24 foot ladder will have how many rungs

A.  18		B.  19		C.  20		D.  21

57.  Lad - pg 46

All structural components of metal ladders are designed to maintain at least 75 percent of their designated design strength at a minimum of _____ deg. F.

a.  200		b.  300		c .  600		d.  400

58.  Lad - pg 47 & E of FF pg 99

Wood does not have as much strength in compression as in tension

a.   true		b.  false

59.  Lad - pg 47

The IFSTA recommends that when raising a wooden extension ladder with the rungs in the bottom rail, the fly section should be placed

a.  between the bed ladder and the building

b.  so as to become the base of the ladder.

c.  with the bed ladder between the fly and the building.

d.  below the window or roof being laddered.

60.  Lad - pg 50

NFPA 1931 calls for rungs to be spaced _____  inches on center.

a.  14		b.  15		c.  16		d.  18

61.  Lad - pg 53

NFPA restricts the length of folding ladders to 14 feet.

a.  true		b.  false

62.  Lad - pg 55

Stand off brackets set the Pompier ladder away from the building a minimum distance of _____ inches

a.   6		b.   7		c.   8		d.  10

63.  Lad - pg 60 & 81

The three main parts of the enclosed automatic latching pawl assembly are hook, finger and ______ which should be replaced every 5 years.

a.  pawl spring		b.  torque spring		c.  torsion spring		d.  mortise spring

64.  Lad - pg 65

Halyard rope has to be a minimum of _____ inch in diameter with a minimum breaking strength of _____.

a.  1/2, 600		b.  3/8, 825		c.     1, 900		d.  3/4, 600

65.  Lad - pg 67

All extension ladders have ____ to keep the fly sections from traveling off the top end

a.  stops		b.  dogs		c.  guides		d.  pawls

66.  Lad - pg 69 E of FF pg 74 & overhead

The poles which are attached to long extension ladders are called:

a.  outriggers		b.  pompier poles		c.  truss poles 		d.  tormentor poles

67.  Lad - pg 76

The average force taken to raise the fly section of a ladder can not exceed _____ times the weight of one ladder fly section.

a.  2		b.  3		c.  4		d.  5

68.  Lad - pg 79

When an extension ladder using wire rope (cable) for the second and third fly is in the fully retracted position the cable should be slack.

a.  true		b.  false

69.  Lad - pg 80

_________ should be applied to metal and fiber glass ladders to enhance their finish.

a.  car wax		b.  varnish		c.  spar varnish		d.  polyurethane varnish

70.  Lad - pg 80

Liquid varnish remover is an approved method, when conditions warrant, of removing old varnish from wood ladders.

a.  true		b.  false

71.  Lad - pg 81

NFPA 1932 recommends that _____ be used to remove old lubricant from ladders.

a.  safety solvent		b.  paint thinner		c.  turpentine		d.  steam

72.  Lad - pg 81

_____ or ______ lubricant is used on metal and fiber glass ladder slide areas when recommendations of NFPA 1932 are followed.

a.  dry, spray			b.  machine oil, spray

c.  Bees wax, candle wax		d.  dry soap, machine oil

73.  Lad - pg 100

N.F.P.A. specifications require fire department pumpers to be equipped with an extension ladder NOT less than ____ feet long.

a.  24		b.  35		c.  16		d.  32

74.  Lad - pg 100

N.F.P.A. specifications require fire department pumpers to be equipped with a roof ladder NOT LESS than ____ feet long.

a.  10		b.  12		c.  14		d.  24

75.  Lad - pg 107 chart pg 110

Generally, the distance from the window sill to the floor in most commercial buildings is about _____ feet.

A.  6		B.  5		C.  4		D.  3

76.  Lad - pg 107

Roughly speaking, a residential building story will average ___ feet from floor to floor.

a.   8 - 10		b.  12 - 14		c.   9 - 10		d.  10 - 12

77.  Lad - pg 110

The ____ extension ladder is probably the most versatile of all extension ladders.

a.  20		b.  24		c.  35		d.  50

78.  Lad  pg 115

When one man carries a straight ladder, he should be positioned _____.

a.  four rungs from the center of the ladder			b.  one or two rungs from the center

c.  at the balance point of the ladder			d.  in the center of the ladder

79.  Lad pg 120

When carrying a roof ladder, that will be used on the roof the end with the hooks should be carried to the _________

a.  front and lowered slightly		b.  front and raised slightly

c.  rear and lowered slightly		d.  rear and raised slightly

80.  Lad - pg 172 & 173

It is desirable and a good practice to have the ladder extend approximately ____ the window sill for a firefighter entering a building through a wide window.

A.  One foot below		B.  Three rungs above

C.  One rung above		D.  One foot above

82.  Lad  pg 172

When work on a upper story window from a ladder must be done, placement of the ladder tip in relation to the window is important.

1.  When breaking out a window for ventilation,  place the tip alongside the window to the windward side about even with

the top of the window.

2.  When the ladder is to be used by a fireman to climb into or out of a narrow window, the ladder tip should be placed

slightly below the sill

3.  For rescue from a window the ladder usually should be placed even with or slightly below the sill.  If the sill projects

out from the wall, the tip can be wedged  up under the sill

4.  If the window opening is wide enough, the ladder tip should be placed so  that two or three rungs extend above the sill

into the window to facilitate rescue.

a.  only 1, 2 and 4 are correct		b.  only 1, 2 and 3 are correct

c.  only 1, 3, and 4 are correct		d.  all statements are correct

83.  Lad - pg 173 & 174

When a ladder is to be used as a vantage point from which to direct a hose stream into a upper window opening and no entry is to be made, it is placed

a.  just below the window sill

b.  2 or 3 rungs into the window

c.  directly in front of the window with the tip on the wall above the window

d.  to the windward side, with the tip even with the top of the window.

81.  Lad - pg 176

The distance at which the heel of the ladder is placed from the building determines the climbing position and establishes the angle between the ground.  This angle should be about ___ degrees.

a.  90		b.  75		c.  45		d.  85

84.  Lad  - pg 176

The easy way to determine the distance that the heel of a ladder should be placed from a building is to:

a.  divide the total length of the ladder by 5		b.  divide the amount of the ladder used by 4

c.  divide the total length of the ladder by 4		d.  divide the amount of the ladder used by 5

85.  Lad - pg 176

If a firefighter uses a total of 28 feet of a 35 foot ladder to ladder a second story window, how far should the heel of the ladder be placed from the building?

a.  9 feet	b.  7 feet		c.  6 feet		d.  5 feet

86.  Lad - pg 172 & 204

When three men are raising a ladder, it is the responsibility of the _____ to select the location for the raise, space the heel from the building and determine if the ladder will be raised parallel or at right angles to the building.

a.  company officer		c.  heel man

b.  beam man			d.  senior man holding the ladder

87.  Lad - pg 263

When climbing a ladder, the climber's hands should grasp the:

a.  rungs with the palms down		b.  beams with the palms in

c.  rungs with the palms in		d.  beams with the palms out

88.  Lad - pg 264

When training with ladders, practice climbing should be done so as to develop speed rather than form.  The proper ladder climbing techniques will be developed as speed is achieved.

a.  True		b.  False

89.  Lad  - pg 265

When carrying a fire axe up or down a ladder, the firefighter should place one hand on the ____ of the ladder as he climbs carrying the axe.

a.  underside of the beam

b.  top beam

c.  rungs

d.  never carry an axe up a ladder, use a rope to pull it up to you

90.  Lad - pg

In considering safety measures for ladder climbing, a good general rule-of-thumb to remember is to space one man for every _____ feet of ladder:

a.  2		b.  5		c.  10		d.  15