SENDCODE:  Send Hex codes to any DOS character device (e.g., printer).

Usage:   SENDCODE xx xx xx ... > PRN

Each xx in the above sequence is a Hex code to be sent to the printer.

Multiple codes are separated by blanks.

Some codes for Epson and Panasonic printers:

Code        Function       Page in Panasonic KX-P1080i manual

-------------------------------------------------------------

08         Backspace                   4-3

0D         Carriage return             4-4

0C         Form feed                   4-6

0A         Line feed                   4-7

0F         Compressed mode             4-8

1B 50      Pica pitch                  4-11

1B 4D      Elite pitch                 4-12

1B 57 01   Double width                4-14

1B 6E      NLQ pica                    4-30

1B 6F      NLQ elite                   4-31

1B 61 01   Centering                   4-38

1B 6C n    Left margin n (in Hex)      4-62

1B 40      Reset printer               4-72