******************************************************************

DESIGN CENTER - 6.0 RELEASE

******************************************************************

This file describes enhancements and modifications made to the 6.0

release of the Design Center programs.

Note that this file provides information for all platforms supported by

the Design Center programs.  For this reason, some of the information

may not apply to your system.

******************************************************************

CONTENTS

******************************************************************

Part 1.  New Features	2

Part 2.  General	3

2.1.  File Name Changes	3

2.2.  Environment Variable Name Change	3

2.3.  Version Compatibility -- Schematics	3

2.4.  Command Line Option: -i	3

2.5.  Using Extended DOS Versions Under Microsoft Windows	4

Part 3.  Installation Considerations	4

3.1.  Extended DOS Versions	4

Part 4.  PSpice and PLogic	4

4.1.  New PLogic Digital Simulator	4

4.2.  Using FILE Specification in PWL Forms	5

4.3.  Noise Immunity Behavior Modeling for Digital Devices	5

4.4.  Digital Library Updates	6

Part 5.  Probe	6

5.1.  Probe Accelerator Keys (Windows and OpenWindows only)	6

5.2.  Marching Waveforms (Windows and OpenWindows)	6

5.3.  Timing Violations and Error Tracking (Windows only)	6

Part 6.  Schematics	7

6.1.  User Interface Changes (Windows and OpenWindows)	7

6.1.1.  Menu Bar Changes	7

6.1.2.  Setup Block and View Functionality	7

6.1.3.  Edit/Model Changes	8

6.2.  PLogic Digital Simulator Integration	8

6.3.  The VIEWPOINT Symbol	8

6.4.  Creating Symbols/Netlisting--Illegal Characters	8

6.5.  App Settings Dialog for Application Settings	8

6.6.  Windows Stimulus Editor Integration (Windows only)	8

6.7.  Reconfiguring Symbol and Package Libraries	8

6.8.  Symbols	8

6.8.1.  Breakout Symbol Name Change - "DBreak3"	8

6.8.2.  Analog Devices with Visible Pin Numbers	9

6.8.3.  Obsolete Symbols	9

6.9.  Hierarchical Symbols and Blocks	9

6.10.  Pin Name Translation Changes	9

6.11.  The "msim.ini" File	9

6.11.1.  New [Schematics Colors] Entries	9

6.11.2.  New [MicroSim] Entries	9

6.11.3.  New [Schematics] Entries	10

6.11.4.  "Marker.slb"	10

6.12.  Font-Related Issues	10

6.12.1.  Using the RASTERFONT=ON/OFF Switch	10

6.12.2.  Using Adobe Type Manager with Schematics	11

Part 7.  Control Shell for Extended DOS Versions	11

7.1.  New Features	11

7.2.  Features Not Supported in the 6.0 Control Shell	11

7.3.  Known Anomalies	12

Part 8.  Stimulus Editor	12

8.1.  Stimulus Editor for Windows	12

8.2.  New Stimulus Editor Circuit File Commands	12

8.2.1.  Stimulus Library Files (.STIMLIB)	12

8.2.2.  Stimulus Definition(.STIMULUS)	13

Part 9.  Polaris Signal Integrity	13

Part 10.  Intel IBIS Model Support	13

Part 11.  New Libraries and Devices	13

11.1.  New Transmission Line Models	13

11.2.  New Comlinear Model	14

11.3.  Library Name Changes - "nom.lib"	14

Part 12.  Parts Program	14

12.1.  New Voltage Reference Template	14

12.2.  Miscellaneous Changes	14

12.3.  Mouse Support	14

12.4.  Preservation of the MDT file in case of error	14

Part 13.  MODTOMDT Utility	15

Part 14.  Filter Designer	16

******************************************************************

Part 1.  New Features

******************************************************************

DESIGN CENTER

* Mentor integration for HP and Sun systems

* Expanded libraries now with over 9000 devices, including over 1,700

Japanese devices

* New PLogic digital-only circuit simulator

* PSpice availability for analog-only and mixed-signal circuit

simulations

SCHEMATICS (Microsoft Windows and Sun OpenWindows platforms)

* Extended support for the SCICARDS external board layout package and

EDIF 2 0 0 netlist format

* Stand-alone Schematics evaluation package

* Global and local configuration of symbol and package library files

* Support for the PLSyn Programmable Logic Synthesis package

* Improved pan and zoom functions

* Attribute editing on a global or multiple-part basis

* Integrated stimulus editing within the Schematics environment (Windows

only)

* New symbols for external input, output, and bi-directional ports

to identify peripheral circuit connections, and facilitate persistent

hazard identification and tracking, which is a new feature of Probe

(Windows only)

* Menu naming conventions consistent with Probe and PSpice

PSPICE/PLOGIC

* Simulation status display showing the three fastest transmission lines

* New .EXTERNAL command which provides a means of identifying and

distinguishing those nets representing the outermost (or peripheral)

connections to the circuit

* The .STMLIB command which makes stimulus library files available to

PSpice and PLogic

* The .STIMULUS command which allows you to define the source devices

(V, I, or STIM).  The .STIMULUS definition encompasses only the

Transient specification portion of what is allowed in the V, I, or

STIM device syntax

* Improved noise immunity modeling for digital devices

CONTROL SHELL

* New Control Shell user interface for Extended DOS platforms

* Integrated access to PSpice, PLogic, Probe, Parts, and StmEd

* Improved printing functions

* Interactive analysis specification through dialogs and prompts

PROBE

* Append function for combining trace data from multiple data files

* Marching waveforms on the OpenWindows platform

* Improved pan and zoom functions

* Timing Violations and Error Tracking (Windows only)

STIMULUS EDITOR

* New native Windows version

PARTS

* New Voltage Reference Template

* Improvements to the MOSFET template

POLARIS

* Support for TangoPro board layout package

******************************************************************

Part 2.  General

******************************************************************

2.1.  File Name Changes

The file "pspice.iff" used in previous versions has been replaced by the

file "msim.iff" in this release.

The Windows PSpice executable file name has been changed from

"wpspice.exe" to "pspice.exe."

2.2.  Environment Variable Name Change

The environment variable PSPICELIB used in previous versions has been

replaced by the environment variable SIMLIBPATH in this release.

2.3.  Version Compatibility -- Schematics

Warning:  Schematic and/or Symbol Library files which are created and/or

modified using the 6.0 version of Schematics may not be backward

compatible with previous versions of Schematics.  That is, if you have a

5.3 schematic that you modify using the 6.0 version of Schematics, you

may not be able to read that schematic back into 5.3 or 5.4 Schematics.

Similarly, if you create new symbols using the 6.0 Symbol Editor, these

symbols may not be compatible with previous versions of Schematics.

2.4.  Command Line Option: -i

PSpice, Probe, the Stimulus Editor (Windows version) and Schematics

support a -i command line option which allows you to specify a

configuration file other than "msim.ini."

To use this option, you can change the command line associated with the

program's icon to be:

<program name>  -i <new_file_name>.ini

If no path is specified for the new ".ini" file, it is assumed to be in

your Windows directory (on the PC) or in your home directory (on the

Sun).

From Schematics, an initialization file can be configured through the

Options/Editor Configuration/App Settings dialog.  The initialization

file configured here will only be used by programs invoked from within

Schematics.  (The file specified by the Options/Editor Configuration/App

Settings dialog is written to the "inifile" specification within the

[Schematics] section of the "msim.ini" file.)

2.5.  Using Extended DOS Versions Under Microsoft Windows

The Probe "Mid-Analysis Snoop" feature will not work if PSpice (PLogic)

is being run from within a DOS "Window" (under Microsoft Windows).  The

symptom is an error message,

"DOS/16M error: [32] DPMI host error (possibly insufficient memory)"

To avoid this problem, run PSpice (or PLogic) directly from the Control

Shell or the DOS command line, not under Windows.

******************************************************************

Part 3.  Installation Considerations

******************************************************************

3.1.  Extended DOS Versions

The environment variable PSPICELIB used in previous releases has been

replaced by the environment variable SIMLIBPATH in this release.  This

variable can be set from the DOS command line, in your "autoexec.bat"

file, or from within the Control Shell using the Analysis/Analysis

Options dialog.

******************************************************************

Part 4.  PSpice and PLogic

******************************************************************

4.1.  New PLogic Digital Simulator

PLogic is available for the Windows and OpenWindows schematic capture

environments, and for the Extended DOS with Control Shell environment.

PLogic is a digital-only circuit simulator that analyzes timing behavior

during a transient analysis.  Detailed analysis is conducted subject to

the timing constraints (e.g., setup/hold times and min/max propagation

delay ambiguities) specified for individual devices.

PLogic operates identically to PSpice.  In the Design Center environment

with Schematics, a schematic is prepared for simulation from which

Schematics generates a circuit file set.  In circuit file entry

environments, the circuit file is created directly through the Control

Shell or a text editor.  The circuit file set (or circuit file)

containing the circuit netlist and analysis commands is read by PLogic

for simulation.  PLogic calculates the states of the nodes connected to

digital devices.  Any timing violations or hazards are reported by

PLogic in the form of messages written to the simulation output file and

to the Probe data file.

With the Design Center for Windows, the results are formulated into

meaningful graphical traces in Probe which can be marked for display

directly from your schematic.  Probe's messaging feature (Windows only)

allows you to automatically display digital signals and buses along with

detailed message text associated with a given timing violation or

hazard;  trace segments with diagnostics are visually tagged.

PLogic analyses can be classified as standard and simple multi-run.  For

more information on running a particular analysis as well as specifying

analysis parameters and output, refer to the Circuit Analysis User's

Guide provided with your system.

4.2.  Using FILE Specification in PWL Forms

When using the FILE keyword within a PWL (Piece-Wise Linear)

specification, be aware that anything following the file name is

ignored.  For example, if you are using the FILE form with a

REPEAT...ENDREPEAT loop, the ENDREPEAT must appear on a continuation

line if it follows the file name.

The following examples illustrate correct and incorrect forms:

Correct:

I3   5,6   PWL   REPEAT FOR 5 FILE DATA1.TAB

+ ENDREPEAT

Incorrect:

I3   5,6   PWL   REPEAT FOR 5 FILE DATA1.TAB ENDREPEAT

Correct:

I4   7 8   PWL   TIME_SCALE_FACTOR=0.1

+   REPEAT FOREVER

+   REPEAT FOR 5 (1,0) (2,1) (3,0) ENDREPEAT

+   REPEAT FOR 5 FILE DATA1.TAB

+   ENDREPEAT

+   ENDREPEAT

Incorrect:

I4   7 8   PWL   TIME_SCALE_FACTOR=0.1

+   REPEAT FOREVER

+   REPEAT FOR 5 (1,0) (2,1) (3,0) ENDREPEAT

+   REPEAT FOR 5 FILE DATA1.TAB ENDREPEAT

+   ENDREPEAT

NOTE:  The REPEAT ... ENDREPEAT specifications may be nested to any

depth.

4.3.  Noise Immunity Behavior Modeling for Digital Devices

PSpice A/D and PLogic now model the noise immunity behavior in digital

devices.  By setting the Pulse Width Rejection Threshold (PPWRT)

parameter in a digital device's I/O Model, the simulator will

appropriately generate a glitch when the pulse width is greater than or

equal to TPWRT and the pulse width is less than the propagation delay.

Refer to the Circuit Analysis User's Guide for additional information.

4.4.  Digital Library Updates

The Digital component libraries, DIG_1.LIB, DIG_2.LIB, DIG_3.LIB,

DIG_4.LIB and DIG_5.LIB, have been modified to support the new "Embedded

Constraint" form of PINDLY device syntax (in those models that use both

PINDLY and CONSTRAINT devices).  These changes do not affect the logic

operation of the models, but they support the new Probe "Error Tracking"

functionality (offered with Windows-based packages at this time).

The new syntax, as of release 6.0, represents the preferred modeling

style for devices of this type; the old syntax (i.e., "separate"

PINDLY/CONSTRAINT devices) will still work (logically), but will

generally not provide correct results with respect to "Error Tracking"

behavior.

******************************************************************

Part 5.  Probe

******************************************************************

5.1.  Probe Accelerator Keys (Windows and OpenWindows only)

The following accelerator keys have been added to Probe:

<CTRL X>    EDIT/CUT

<CTRL C>    EDIT/COPY

<CTRL V>    EDIT/PASTE

5.2.  Marching Waveforms (Windows and OpenWindows)

Marching Waveforms was supported for Microsoft Windows in release 5.4,

and remains unchanged for 6.0.  Support for OpenWindows is now included

in 6.0 as well.

5.3.  Timing Violations and Error Tracking (Windows only)

When there are problems with your design, such as setup/hold violations,

pulse-width violations, or worst-case timing hazards, PSpice and PLogic

will log messages to the simulation output file and/or Probe data file.

Using Probe (Microsoft Windows only), messages can be selected causing

associated waveforms and detailed message text to be automatically

displayed.

In the digital domain, the types of problems identified by the simulator

are generally classified as either timing violations or timing hazards.

Timing violations include SETUP, HOLD, and minimum pulse WIDTH

violations of component specifications.  The occurrence of such a

violation may result in a change in the state behavior of the design,

and potentially the answer.  However, the effects of many of these

errors are short-lived and don't influence the final circuit results.

PSpice and PLogic now identify any digital net that may carry a possibly

invalid state value due to a timing hazard; Probe identifies digital

traces of these nets by highlighting them with a different color.  The

color difference exists for the duration of the potential logic

difference due to the timing hazard.  Changes in a trace's color may or

may not coincide with actual state changes, depending on the nature of

the timing violation.  In Probe, when a digital trace is highlighted in

this manner, double-click on the actual highlighted portion of the trace

to see a Simulation Message pop-up dialog; clicking on "OK" will then

bring up a new window containing both the message text and a plot of the

signals involved in the hazard.

Timing hazards are most easily identified by simulating a design in

worst-case timing mode, usually close to its critical timing limits.

Under such conditions, the simulator will report things such as

AMBIGUITY CONVERGENCE hazards.  Again, these may or may not pose a

problem to the operation of the design.  However, there are identifiable

cases that cause major problems.

The types of error conditions reported by the simulator are summarized

in the "Simulation Condition Messages" section of the "PSpice Analyses"

chapter in the Circuit Analysis User's Guide.

******************************************************************

Part 6.  Schematics

******************************************************************

6.1.  User Interface Changes (Windows and OpenWindows)

6.1.1.  Menu Bar Changes

To maintain consistency with the other Design Center programs, some

minor changes have been made to the commands in the Menu Bar:

Old Command            New Command

--------------------------------------------

Zoom                   View

Zoom/Normal            View/Fit(*)

Configure              Options

Edit/Search            Edit/Find

Analysis/Run PSpice    Analysis/Simulate(**)

(*)The View/Fit command replaces the Zoom/Normal command, although the

functionality is somewhat different.  Refer to the Schematic Capture

User's Guide for details.

(**)The simulator (PSpice or PLogic) is selected through the

Options/Editor Configuration/App Settings dialog.

Opening a schematic (either via File/Open or Navigate/Push) now does an

automatic View/Fit to adjust the zoom factor so that all wires, parts,

etc. on the schematic are visible.

The View/Area command now deselects the viewed area after zooming.

6.1.2.  Setup Block and View Functionality

The Edit/Setup Block command was removed and functionality combined with

the Edit/Views command.

Thus, for 6.0, to change the schematic associated with a block, the

Edit/Views command should be used.  The value of the DEFAULT view is the

default schematic name.  The command can also be used to add, delete, or

display all views and schematics associated with a block.

If a hierarchical symbol is chosen (as opposed to a block), Edit/Views

will display the list of views but not allow editing.  Views for

hierarchical symbols can only be changed in the Symbol Editor.

6.1.3.  Edit/Model Changes

When editing a model definition, <ENTER> should now be used instead of

<CTRL-ENTER> to go to the next line.

6.2.  PLogic Digital Simulator Integration

The Analysis/Setup command will bring up a different dialog depending on

whether PSpice or PLogic is configured as the simulator.  For PSpice

(analog and mixed A/D), the Analysis Setup dialog provides a new Digital

Setup button.  For PLogic, the Analysis/Setup command displays only the

digital analysis options dialog.

6.3.  The VIEWPOINT Symbol

If you are using PLogic, the VIEWPOINT symbol will not show any

information.

6.4.  Creating Symbols/Netlisting--Illegal Characters

Do not use the "%" character in names that will appear in the template

for PSpice.  Also do not use "%" in the value of an attribute which must

be substituted in the template by the netlister (e.g., DEVTOL=@DEVTOL

where DEVTOL=10%).  This will cause the netlister to emit erroneous "No

such pin" errors.

6.5.  App Settings Dialog for Application Settings

The Options/Editor Configuration dialog has been changed to include

another button, labeled "App Settings...".  This button displays a

dialog which allows the user to specify the simulator (PSpice or PLogic)

and the command lines used to invoke the simulator, Probe, and StmEd.

(The SUN version of Schematics will not have the StmEd command line

field).

6.6.  Windows Stimulus Editor Integration (Windows only)

The Analysis/Library and Includes dialog now includes Stimulus Library

files.

6.7.  Reconfiguring Symbol and Package Libraries

Prior to 6.0, if the list of libraries was changed via Options/Editor

Configuration, then all libraries in the list would be re-read.  For 6.0

this has been sped up by recognizing which libraries were previously

loaded and only reading new libraries.  This change means that you

should never replace symbol and package libraries manually (using the

DOS copy command, for example) while Schematics is running.

6.8.  Symbols

Local symbol library support has been added.  You can now associate

symbol libraries with a particular schematic.  These symbol libraries

will be loaded when that particular schematic is loaded.  This is done

by selecting the Options/Editor Configuration/Library settings and

selecting the new Add Local button.  Local libraries are always searched

prior to global libraries (those loaded for all schematics).  For a

hierarchical schematic, only the local libraries associated with the

topmost schematic are used.

6.8.1.  Breakout Symbol Name Change - "DBreak3"

The name of D3Break symbol has been changed to DBreak3.

6.8.2.  Analog Devices with Visible Pin Numbers

A symbol library file, "analog_p.slb," is shipped for those who wish to

have visible pin numbers on the analog devices R, L, C, R_VAR, and

C_VAR.  The "analog_p.slb" file must be configured by adding it to the

list of configured symbol library files before or in place of the file

"analog.slb."

6.8.3.  Obsolete Symbols

The "dig_old.slb" symbol library file is included for compatibility with

earlier releases. The symbols in this file are considered obsolete and

have been replaced by new symbols in other library files.  These symbols

are provided for compatibility, only, and should not be used for new

schematics.

6.9.  Hierarchical Symbols and Blocks

Hierarchical symbols and blocks now use a new default reference

designator (REFDES) to avoid conflicts with packaging.

The default REFDES for hierarchical blocks is now HB?.

The default REFDES for hierarchical symbols is now HS?.

Pin numbers are now P1, P2, etc. instead of 1, 2, etc.

6.10.  Pin Name Translation Changes

Within pin names, the characters "<," ">," and "=" are now replaced by

"GT", LT", and "EQ," respectively.  (In 5.4 and prior, these characters

were replaced by "G", "L", and "E" respectively.)

6.11.  The "msim.ini" File

Note: You cannot use tabs within the "msim.ini" file.

6.11.1.  New [Schematics Colors] Entries

ENTRY     DESCRIPTION

-------------------------------------------------------------------

plsyn     specifies the color of PLSyn parts and blocks

(IMPL=PLSYN or blocks with a DSL view)

stimulus  specifies the color of StmEd stimulus parts

(those with a STIMULUS attribute)

6.11.2.  New [MicroSim] Entries

ENTRY       DESCRIPTION                         DEFAULT

-------------------------------------------------------------------

plsyncmd    Command to invoke PLSyn.           plsyn

stmedcmd    Command to invoke StmEd.           stmed

polariscmd  Command to invoke Polaris.         polaris.exe /FT /RC

plogiccmd   Command to invoke PLogic.          plogic

backup      Previously in [Schematics] section

6.11.3.  New [Schematics] Entries

ENTRY              DESCRIPTION                             DEFAULT

-------------------------------------------------------------------

viewfitpercent     See description of View Fit below

lastfile1-4        Most-recently used Schematic files

lastslb1-4         Most-recently used Symbol Editor

files

plsyn              ON adds Tools/PLSyn command to           OFF

menu, and shows PLSyn Setup button

on the Analysis Setup dialog.

polaris            ON adds Tools/Polaris to menu.           OFF

simulator          PSPICE or PLOGIC.  Set using Options/

Editor Configuration/App Settings...

inifile            Name of initialization file to use when invoking

other Design Center Apps (PSpice, PLogic, StmEd,

Probe).  If this is left blank, the default

"msim.ini" file is used.  Can be changed through

the Options/Editor Configuration/App Settings.

A file specified through one of the program-

specific commands in the [MicroSim] section

(such as pspicecmd, probecmd, stmedcmd, etc.)

will take precedence over the file specified

with the inifile command setting.

usescicardsp2      Flag to govern use of P2 record when       ON

netlisting in SCICARDS format.

6.11.4.  "Marker.slb"

"Marker.slb" should not be included in the list of libraries.  If it is,

you will get a warning when you start Schematics:

"marker libraries should not be in configured list."

This is just a warning; you can continue to use Schematics.

6.12.  Font-Related Issues

6.12.1.  Using the RASTERFONT=ON/OFF Switch

If the line RASTERFONT=ON is included in your "msim.ini" file, it causes

the Helvetica font to be used instead of the modern font for display in

Schematics.  This will cause rotated text to appear vertically versus

being rotated on its side.  If your symbols and schematics use rotated

text, setting this item to ON may cause problems with positioning text.

For this reason, we suggest using the RASTERFONT=OFF switch setting.

6.12.2.  Using Adobe Type Manager with Schematics

If you have Adobe Type Manager installed and are using Schematics, you

may encounter problems with how rotated text is displayed.  We recommend

that you edit the "ATM.ini" file and comment out the line

Modern=Helvetica

in the [ALIASES] section.  If this is not done, rotated text may be

positioned differently on the display versus on hard copies.  We also do

not recommend using the FONTTYPE=ADOBE setting in your "msim.ini" file

at this time as it has the same problem.

******************************************************************

Part 7.  Control Shell for Extended DOS Versions

******************************************************************

7.1.  New Features

The 6.0 Control Shell provides the following features:

*  A menu-driven environment for setting up your netlists, circuit

files, and the analysis parameters

*  32-bit implementation

*  Integrated Help, with entries for the commands supported

by PSpice and PLogic

*  Command entry through function keys, pull-down menus, or buttons

for the most frequently used functions

*  Availability of multiple editing and browse windows

*  Context-sensitive help for menus and dialogs

*  Syntax checker for circuit files

*  Printing functions, including multiple-copy output and page

customization options

*  File size is no longer limited to 32KB (All of the available free

RAM and disk space may be used.)

The above features are described in detail in the Circuit Analysis

User's Guide and the Circuit Analysis Reference Manual.

7.2.  Features Not Supported in the 6.0 Control Shell

1.  The External Editor command (e.g. for calling OrCAD's SDT) is not

currently supported.  Use the File/DOS Shell menu command to "call"

other DOS-based programs.

2.  The use of the ";" character for user comments on a command line is

not supported.  Comments must appear on a separate line which begins

with the "*" character.

3.  Command pairs such as .ac/.noise and .tran/.four will appear

together when set up or altered using the Control Shell Analysis Setup

dialogs.  These command pairs cannot be separated by comment lines.

4.  As with previous versions of the Control Shell, you cannot have more

than one circuit description in a circuit file.

5.  Global parameters (.param) cannot be edited through dialogs.  You

must manually define .param parameters in the edit window.

7.3.  Known Anomalies

The following problems and/or restrictions are inherent in the software

tools used to create the Control Shell, and are currently beyond our

control.

1.  Positioning of the text within the edit window is spurious when some

of the dialogs are invoked, used, or closed.

2.  Cursor positioning on a line in the edit window drifts to the right

when the mouse is used.  The farther to the right on the line that the

mouse pointer is clicked, the more the cursor position drifts.

3.  If a line in the circuit file contains a space character in the

first column, spurious "garbage" characters may be inserted when new

characters are entered in column one. These characters may be manually

deleted by the user.

4.  During a Replace operation, the target text is not highlighted.

Instead, a solid block character (similar to a cursor) is placed in

front of the selected text.

5.  Various anomalies have been observed when using the Control Shell in

a DOS window under Microsoft Windows.  For best performance under

Windows, use the ".pif" file provided with the Control Shell.  If you

encounter problems, try running the Control Shell directly from DOS.

******************************************************************

Part 8.  Stimulus Editor

******************************************************************

8.1.  Stimulus Editor for Windows

The Stimulus Editor (StmEd) program has been rewritten as a native

Windows-based program for the Microsoft Windows platform.  The 6.0

version is tightly integrated with Schematics through new voltage,

current, and digital stimulus symbols.

Refer to the Circuit Analysis User's Guide for complete details.

8.2.  New Stimulus Editor Circuit File Commands

8.2.1.  Stimulus Library Files (.STIMLIB)

General Form:     .STMLIB <file name[.stl]>

Examples

.STMLIB mylib.stl

.STMLIB volts.stl

.STMLIB dgpulse

Stimulus library files created by StmEd are made available to PSpice and

PLogic via the .STMLIB command.  The <file name> specification

identifies a file containing .STIMULUS statements.  The file extension

is not defaulted to ".stl."  If you specify a file name, you must

include its extension.

8.2.2.  Stimulus Definition(.STIMULUS)

General Form:

.STIMULUS <stimulus name> <type> <type-specific parameters>*

Examples:

.STIMULUS  InputPulse PULSE  (-1mv 1mv 2ns 2ns 50ns 100ns

.STIMULUS  DigitalPulse  STIM  (1,1)

+  0S    1

+  10NS   0

+  20NS   1

.STIMULUS 50KHZSIN  SIN  (0 5 50KHX 0 0 0 )

.STIMULUS statements generally appear within stimulus libraries created

by StmEd.  These define <stimulus name>s which are referred to by the

source devices (V or I), or by the digital STIM device.  The .STIMULUS

definition encompasses only the Transient specification portion of what

is allowed in the V, I, or STIM device syntax.

******************************************************************

Part 9.  Polaris Signal Integrity

******************************************************************

Polaris now supports Version 2.0 of the TangoPro PCB layout package with

the ASCII Database Translation option.

******************************************************************

Part 10.  Intel IBIS Model Support

******************************************************************

A library of Intel's Pentium (TM) 82430 (Mercury) PCIset IBIS models is

now available.  These highly accurate driver/receiver buffer models

include effects such as switching times, nonlinear transfer

characteristics, and package parasitics.  These allow you to perform

accurate signal integrity simulation of I/O interconnects on printed

circuit boards.  The models are located in "buffer.lib."

******************************************************************

Part 11.  New Libraries and Devices

******************************************************************

This section lists the new libraries and devices added to the 6.0

release which are not documented in the documentation set.

11.1.  New Transmission Line Models

Four new twisted pair transmission line subcircuits have been added to

"tline.lib." The names are as follows:

TP19AWG     TP22AWG     TP24AWG    and TP26AWG

These models use a .SUBCKT named ABMlossyTline which implements a

general lossy transmission line with R, L, C, and G, as functions of

frequency.

The symbol library "tline.slb" has also been updated to reflect these

changes.  A base part, TWSTPAIR, has also been added.  The coax symbols

now automatically choose between the .MODEL and .SUBCKT implementations,

depending on whether the user specified an operating frequency with the

FRQ parameter.

11.2.  New Comlinear Model

One new model from Comlinear Corporation has been added to the

"comlinr.lib" library since publication of the manuals.  It is a CLC532

2:1 analog multiplexer.

11.3.  Library Name Changes - "nom.lib"

The file "nom.lib" now references different files depending on the

simulator that was installed.  If, for example, you are using the PLogic

digital simulator, the file "nom.lib" will reference the files in the

Digital Library.  If you have the PSpice mixed-signal simulator, the

file "nom.lib" will reference files in both the Analog and the Digital

library.  In the latter case, this is done through a reference to the

file "nom_dig.lib" which in turn references the Digital Library files.

******************************************************************

Part 12.  Parts Program

******************************************************************

12.1.  New Voltage Reference Template

A new voltage reference template has been added for 6.0, with 5 screens.

A subckt-based model will be generated.  It is added as menu item 9 in

the Parts program main menu.  The voltage reference template is

described in the Circuit Analysis Reference Manual.

12.2.  Miscellaneous Changes

The power MOSFET template in the Rev. Drain Current screen has been

changed and the extraction algorithm was modified to yield a better fit.

Minor changes to the BJT template for some model parameter ranges were

made.

12.3.  Mouse Support

The use of the mouse is not supported for commands in Device_curve

submenus (such as Exit, Add, Change, or Delete).

12.4.  Preservation of the MDT file in case of error

Under the following error conditions, the original MDT file will not be

over-written (as would happen with release 5.4 and previous releases):

i)    A corrupted MDT file is read - the MDT file is corrupted.

ii)   A non-compatible MDT file for BJT and MOSFET is read - the

existing MDT file was created using Parts 5.3 or before.

iii)  A wrong device type MDT file is read - the existing MDT file was

created with a different device type (e.g., user trying to run

Parts for a diode but using a BJT MDT file).

An error message is displayed and Parts will return to the Main menu.

******************************************************************

Part 13.  MODTOMDT Utility

******************************************************************

MODTOMDT is a utility program to convert PSpice model files or library

files containing PSpice models to MDT files in Parts format.  The MDT

files generated will be compatible with other MDT files generated by

Parts.

Currently, the following device types are supported:

DIODE        JFET

BJT          MOSFET (LEVEL 3)

The following lists features and usage of this utility:

1.  Allow users to load a model file into Parts and see the performance

curves of the model.

2.  Allow users to perform a "what-if" analysis when they change the

model parameters in Parts.

3.  Allow our users to modify/update the model by entering screen data

into the Parts program and re-fit some model parameters

Usage:      modtomdt [options] <infile>

An input file must be provided.  This could be a single model file, or

it could be a .LIB file that contains many .MODEL statements.

Command line options:

-h   provide a summary of usage

-n   force all answers to program queries to 'No'

-y   force all answers to program queries to 'Yes'

Each .MODEL statement in a file will result in an individual MDT file.

If the output MDT file already exists, the user is prompted to confirm

overwriting the existing file.

An example:

A user wants to use a device, "A," for PSpice simulation, but cannot

find the model in the library.  However, a similar device, "B," is in

the library.

Here is the procedure for creating the A.MDT file using Parts and the B

device:

1.  Copy the model for device B from the appropriate library file to a

file <infile>.  (The model begins with a .MODEL statement.)

2.  Run MODTOMDT <infile> to create B.MDT.

3.  Rename B.MDT to A.MDT.

4.  Run Parts, and load A.MDT.

5.  Modify/Add data using the Parts program, then generate the file

A.MOD.

******************************************************************

Part 14.  Filter Designer

******************************************************************

Filter Designer is no longer supported on the Macintosh Platform.