v15.3

Additional optimization to squeeze the last ounce of speed

from the application. It should now be ready to handle

vFAST modems effectively.

v15.2

Additional optimization to speed up the program.

v15.1

Switched to 386 instructions to speed up the program.

V15.0

Changed the startup timers and added some stuff to take into

account RIPterm's very SLOW and buggy Ymodem Batch and Ymodem-G.

V14.2

-T<D,W,O>

This  governs  the TYPE of time slicing the  protocol  driver

will use. Leaving off the -T switch means no time slicing.

D= Desqview or Topview

W= Windows

O= OS-2

E.g. -TD, -TW, -TO

While time slicing does reduce the drain on your CPU it  may,

depending  on  the multitasker (OS-2,  Windows,  Topview,  or

Desqview)  and your hardware, greatly reduce transfer  rates.

Despite  the term "Multitasking", multitasking is  impossible

using  a  single  CPU.  The CPU can  only  process  a  single

instruction (2 integer instructions if the new Intel P5  chip

is  used) at once. Multitasking with a single CPU is  nothing

more than quickly swapping in and out programs giving each  a

short  period of time to run before swapping in another  one.

For instances, when Windows swaps away it freezes  everything

in  that  windows  and this includes ANY  device  drivers  or

interrupt  service routines. If you use the  internal  serial

driver,  it to is frozen and cannot send data that may be  in

the  transmit  buffer.

While all multitaskers do this, some are more efficient  than

others.  Some  will give interrupt services  higher  priority

than  other tasks and allow them to use much more  CPU  time.

Even if the application only gets access to the CPU once ever

oh  .  .  . 9 clock tics, interrupt  routines  used  by  that

application  may  get access to the CPU every 2  clock  tics.

This is sometimes referred to as multi-threading.

There   are  conditions  when  it  doesn't  matter  how   the

multitasker works. For instances, intelligent serial cards.

If  you have an intelligent serial card, it  handles  sending

the  data and the multitasker does not stop it from doing  so

by  swapping out the window. In this case, always  use  -Time

slicing.

Fossil  drivers  can sometimes get around this  if  they  are

loaded  BEFORE  the multitasker. However,  some  multitaskers

will  not allow dual access, from different windows,  to  the

fossil.

To determine if your system works better with or without time

slicing, testing is the only way.

Do uploads and downloads using time slicing and without using

time slicing. If you run a single node, compare the  transfer

rates.

If  you  run  multiple nodes, it gets  more  involved.  After

getting  the first set of figures, repeat the entire  process

but  make  sure  you  have  someone  doing  the  uploads  and

downloads on ALL nodes at once. THEN compare ALL the transfer

rates.  Doing  multiple  transfers at once  DOES  effect  the

outcome.  You  may  find  that  while  transfer  rates   drop

drastically  when  a single download is going  on,  they  may

actually  go  up when you have multiple  transfers  going  at

once.

When  you  are doing the tests, keep in  mind  that  transfer

rates  WILL  vary, but only a little bit. Look  for  the  big

differences and don't worry about 1 to 3 percent.