*****************************

HDDINFO 1.0

A DISK IDENTIFICATION UTILITY

*****************************

This program has been written to help identify MFM, RLL, ESDI  and  IDE  hard

drives. If you know Murphy's Law, then you should be aware of the fact,  that

when you'll need the technical data of your harddisk, you wont find  it.  You

say, that don't happen? Just think about what you would have to do  when  you

power-on your pc and all that happens looks like:

CMOS checksum invalid

Press F1 to run setup

Or maybe you're hardware distributor, tired of  searching  endless  lists  of

drive data when you setup your machines. Then you should try this program.

[1] Hardware requirements:

--------------------------

* at least 384 kByte of dos memory

* MDA, CGA, EGA or VGA video display

* optional mouse (supported if dos mouse driver installed)

* MSDOS 3.0 or above

[2] Tips for using this program:

--------------------------------

* Create a self booting floppy disk, include your mouse driver if  you  like,

and copy HDDINFO onto that disk. HDDINFO is useless if copied to a harddisk

which parameters have been lost!

* NEVER use HDDINFO for checking SCSI harddisks! This may cause unpredictable

results.

* You may check almost any MFM, RLL, ESDI and IDE harddisks. Please be  aware

that normally

- IDE drives show up translation mode parameters

- MFM and RLL drives show up physical parameters

- ESDI drives show physical / translation parameters depending on

controller setup (normally a jumper)

* Harddisks may be soft-sectored or hard-sectored. Soft-sectored disks always

show a sector size of 512 bytes. Hard-sectored disks  often  show a  sector

size including all header and sync bytes  (unformatted  capacity).  HDDINFO

always uses a 512 byte sector size when calculating the disk capacity.

* Don't worry about differences in disk  capacity  between  HDDINFO  and  the

manufacturer of your disk. Many disk  manufacturers  calculate  1 MByte  as

1000 kBytes, but HDDINFO calculates 1 MByte as  1024 kBytes.  (The  CMOS of

your PC also will do it this way).

* HDDINFO will not run under MS-Windows, IBM OS/2 dos box  and  Desqview.  To

get the information about the harddisks, HDDINFO must directly  communicate

with the harddisk controller of your PC. This is normally impossible  under

these environments.

[3] Using HDDINFO:

------------------

HDDINFO is a software designed to the SAA standard. If you have a mouse, just

click on the menues and button you like to activate. If you  control  HDDINFO

with your keyboard, use the ALT-key together with a letter to activate a menu

or button.

After starting the program, you will see an empty desktop with a  menu  which

consists of "HddInfo" and "Drives". Pressing Alt-H will open the  first  menu

where you can select the "About" and "Exit" commands. Pressing Alt-D will set

up the second menu which allows you to select "Drive C" or "Drive D".  Please

remember that these drives  letters  correspond  to  your  first  and  second

physical harddisks (like your PC CMOS setup does).

When you select a drive and HDDINFO is able to find that drive,  your  screen

might look like this:

-----------------------------------------------------------------------------

Drive information for [C]:

==========================

Media Type...........: Fixed harddisk, hard sectored, not MFM-coded

Cylinders............: 1024

Heads................: 8

Sectors per Head.....: 35

Bytes per Sector.....: 595

Bytes per Head.......: 20832

Sectors Total........: 286720

Capacity (MByte).....: 140

Controller Type......: Dual ported multiple sector read-ahead buffer

Controller Model.....: WD1007

Controller Revision..: REV. Y2

Serial Number........: 00000000000000000000

Buffer (Sectors).....: 32

Number of ECC-Bytes..: 7

Transfer speed.......: > 5 MBit/sec, � 10 MBit/sec

-----------------------------------------------------------------------------

This screen shows the data of an 140 MByte Micropolis ESDI drive connected to

a Western Digital WD1007 controller.

You may now select "OK" to close this screen, or "Protocol",  to  write  this

information to a disk file, or "Into CMOS", if you have a registered  version

of HDDINFO. And this is best of HDDINFO: If your computer is equipped with an

AMI - Bios, HDDINFO is able to setup your hard drives into the CMOS setup. No

further user interaction is required to get the harddisk running.

[4] Trouble shooting:

---------------------

* HDDINFO cannot find any disk drive:

-----------------------------------

Usually this occurs when there is a hardware problem or the CMOS  setup  is

incorrect (e.g. bus clock too high for harddisk controller);

* HDDINFO recognizes drive C as drive D and vica versa:

-----------------------------------------------------

Normally this is a result of incorrect cabling or  jumper  setting  on disk

drive;

* HDDINFO finds an IDE drive as both "C" and "D":

-----------------------------------------------

Your IDE drive has incorrect jumper settings;

[5] Legal terms and conditions:

-------------------------------

* HDDINFO is distributed as shareware. That means, you are encouraged to test

this program for a period of 30 days. After that  period  you  should  stop

using the program if you think it does not suit your needs. But I hope that

you will like HDDINFO and register the program.  The  following  conditions

apply to registering:

"Single User Copy": This license is  suitable  for  anybody.  The  user  is

allowed to use this program on as many machines as he likes, but he is  not

allowed to duplicate the program except for backup reasons. So if  you're a

hardware dealer,  you  also  can  use  this  license  for  setting up  your

customers' computers.

"Distributor Copy": This licence is for dealers only. It allows a dealer to

spend a copy of this program together with each harddisk he is selling. The

distributor copy is not for commercial reselling of HDDINFO. If  HDDINFO is

sold together with hard drives and is installed  on  floppy,  no more  than

US $5.00 may be charged for copy costs.

* Pricing:

--------

Shareware Copy ...................... free

Single user Copy .................... US $ 10.00

Distributor Copy .................... US $100.00

If the shareware copy is copied by commercial shareware  dealers,  a charge

of no more than US $5.00 may be taken for copying costs.

These prices include shipment via CompuServe mail.  If  you  prefer  that I

send you a diskette, please add US $5.00. This seems to be  very  much  but

after looking at my address you will understand:

Send your orders by:

* mail to:      Markus Klama

Carl-Maria-von-Weber-Weg 8

82538 Geretsried

Germany

*email to:      CompuServe 100115,2167    Markus Klama

Please use international money orders or  checks  made  payable  to  Markus

Klama, 82538 Geretsried. If you register HDDINFO, you will receive:

+ the latest version of HDDINFO with the CMOS setup feature included

+ an enhanced version of this documentation file

+ a coupon for another free update of HDDINFO