Presented by:

Toby joined Microsoft in January, 1993. He was the principal architect at Microsoft of the Windows Telephony API, and has continuing responsibility for coordinating the design and development of extensions, including migrating it to other platforms.

During the preceding nine years, Toby was Principal Engineer at Hayes Microcomputer Products, Inc., and represented Hayes on US and international standards committees on modems, facsimile, and communication protocols. He chaired US and international committees on modem command languages, and had a significant role in the development of several standards including V.42 (error control), V.42 bis (data compression), and the Class 1 and Class 2.0 fax modem command sets. He has worked in the data communication field for 17 years, and was co-inventor of two patents related to computer-modem interfaces.

Toby and his wife Irene have five children. A true rebel, he appears to be the single employee in the Microsoft Systems Division to wear a tie to work every day.

This document is to be presented at Microsoft Tech•Ed ’94, March 28-31 in New Orleans.

## Introduction

### Why Windows Telephony?

Initially, the desktop computer was a personal productivity tool – first through the use of spreadsheets and word processing programs – and later with additional tools such as databases, project managers, presentation packages and personal information managers.  Then local area networks (LANs) connected desktop computers within workgroups and departments, allowing people to share information and resources.   In addition, LANs allowed people within a workgroup to communicate electronically, further expanding the role of desktop computers to communicate business information.

The next step in the evolution of the desktop computer is to combine it with one of the most basic business tools – the telephone.  Intel Corporation and Microsoft Corporation, in cooperation with many major telecommunications, PC and software companies, have created a standard interface – the WindowsÔ Telephony Application Programming Interface (TAPI) – for the integration of telephones and personal computers running the MicrosoftÒ WindowsÔ operating system.

The API brings the utility and reach of the telephone to the desktop computer.  In fact, with the availability of cellular telephone technology, the capabilities are not even limited to the desktop.

### Enabling a Broad Range of Applications

The possibilities are exciting for users of both telephones and computers.  Products based on the Windows Telephony API will improve existing capabilities, such as:

•	Visual Interface to Telephone Features.  Users can easily access telephone features through the Windows graphical user interface.  Telephone providers have been adding features to telephones at a quick pace – call forwarding, call parking, speed dialing, and conference calling.  Unfortunately, most people typically use only one or two of the most basic features simply because they can’t remember the more sophisticated features – let alone how to use them.  Through telephony software applications, users will be able to access even the most complex features with ease.  For example, setting up a conference call may be as easy as “clicking” on the names of the conferees.

•	Personal Communication Management.  With a graphical user interface, people can have their PCs handle incoming telephone calls, automatically controlling which calls reach them.  For example, people will be able to ensure that they receive important calls by requesting that certain calls be forwarded automatically to locations at which they expect to be working.

•	Telephone Network Access.  Connecting existing applications directly to the telephone network will greatly expand the usefulness of these applications and increase users’ productivity.  For example, a personal information manager can be used not only to look up telephone number, but also to actually place calls to those numbers.

Additionally, several interesting new applications will be enabled by combining computers and telephones:

•	Integrated Messaging.  With direct connection to the telephone, integrated messaging becomes a reality.  Today, when people arrive at the office, they must check three separate “in-boxes”:  voice mail, electronic mail and the fax machine.  With the desktop computer connected to the phone network, users can combine these three forms of messaging into a single in-box.  They can review and manage all of their messages – electronic mail, fax and voice mail – from a single place.  In addition, voice-mail messages can be accessed randomly, which is far more efficient than the serial access provided on most telephone-based voice-mail systems.

•	Integrated Meetings.  One of the most attractive capabilities of the computer is that it can store, communicate and present information that spans the entire spectrum of media – text, data, graphics, voice and video – in any combination.  By itself, the telephone can communicate and present voice information only.  By combining the functions of the computer and the telephone, people in geographically separate locations can participate in interactive meetings and share visual as well as audio information.  That means they can hold meetings over the telephone network that are nearly as rich in information content as in-person meetings.

These two applications merely scratch the surface of possibilities presented by the integration of the personal computer and the telephone.  More will emerge as products are developed on the Windows Telephony API specification.

### The Windows Telephony API Approach

One of the major problems in connecting the computer to the telephone is the enormous complexity and diversity found in the underlying telephone network. There is a wide variety of telephone services and equipment, and many incompatible protocols.  The analog Public Switched Telephone Network (PSTN), proprietary PBXs, key systems, cellular systems, and Integrated Services Digital Network (ISDN) facilities vary across service networks and switches.  The Windows Telephony API provides a standard application programming interface that allows users and application developers to take advantage of the many capabilities and services of the telephone.  At the same time, it isolates users and application developers from the complexity and variability of the underlying telephone network.

The Windows Telephony API allows application programs to control telephony functions.  This includes such basic functions as establishing, answering and terminating a call.  It also includes supplementary functions, such as hold, transfer, conference and call park found in PBXs and other phone systems.  The API also provides access to features that are specific to certain service providers, with built-in extensibility to accommodate future telephony features and networks as they become available.

The Windows Telephony API operates independently of the underlying telephone network and equipment.  It isolates the application from the network complexity, greatly simplifying the application development task.  In addition, the API is independent of the method of connection between the computer and the telephone.  This gives maximum flexibility to integrate the PC with the telephone system.  For example, the PC can be connected to the telephone network as shown in Figure 1. Connect PC to Network.  The PC can be connected through a telephone as shown in Error: Reference source not found.  Multiple PCs can share telephone resources through a voice server, as shown in Figure 3. Connect PC to Voice Server.  Or a server can be connected through a switch-to-host link, as shown in Figure 4.  Different connection models offer different capabilities to applications.  For example, conferencing applications require a connection as shown in either Figure 1. Connect PC to Network or Figure 3. Connect PC to Voice Server, in order for the PC to get access to the information transmitted over the phone line.

Figure 1. Connect PC to Network

Figure 2. Connect PC to Telephone

Figure 3. Connect PC to Voice Server

µ §

Figure 4. Connect PC via Switch-to-host Link

The Windows Telephony API is part of the Microsoft Windows Open Services Architecture (WOSA), which provides a single set of open-ended interfaces to enterprise computing services.  WOSA encompasses a number of APIs, providing applications and corporate developers with an open set of interfaces to which applications can be written and accessed.  WOSA also includes services for data access, messaging, software licensing, connectivity and financial services.

WOSA services such as the Windows Telephony API consist of two interfaces.  Developers write to an applications programming interface (API).  The other interface, referred to as the service provider interface (SPI), is used to establish the connection to the specific telephone network.  This model is similar to the computer industry model whereby printer manufacturers provide printer drivers for Windows-based applications. Figure 5. Windows Seamlessly Integrates Applications and Telephone Networks shows the relationship between the “front-end” Windows Telephony API and the “back-end” Windows Telephony SPI.

µ §

Figure 5. Windows Seamlessly Integrates Applications and Telephone Networks

Applications can combine the Windows Telephony API with other capabilities of Windows to provide a combination of services.  For example, an application can use the Windows Telephony API to establish a connection and then use the Windows audio functionality to record and play back voice information over the connection.

### Picking Up Where Others Have Fallen Short

Windows Telephony is not the first or only telephony application programming interface in the industry. However, most of those that have gone before have faced serious roadblocks to success. These included:

•	Limited Platforms. Some attempts to integrated telephony into personal computers have used specially-designed PC hardware. In many cases, this hardware was not expandable, insufficiently powerful for general computing needs, and introduced incompatibilities with standard operating systems software and applications. This experience has shown that, to be successful, integration must be based on standard PC hardware and use standard software.

•	Call Center Focus. Most telephony application programming interfaces concentrate on large, call-center operations in which equipment such as an automatic call distributor is closely coupled with a PBX and mainframe computer to control an entire department. The most basic functions in these APIs are complex due to their “third-party call control” nature.

•	Limited Media Stream Access; Single Application Focus. At the same time that these legacy APIs are too complex, the overall APIs are insufficient for use in a desktop environment.  They lack functions to control media stream input and output on the PC, and are unable to deal with multiple PC applications sharing telephony resources.

•	Restricted to Single Switching Platform. Virtually all of the telephony APIs in the industry today have been promulgated by an individual switch manufacturer, and require use not only of their PBX but also of interface hardware or server software sold by the same manufacturer. APIs designed by standards committee, while available on a variety of hardware platforms, have nevertheless been generally restricted to a single networking environment, such as PBXes or ISDN, and not portable to other environments such as POTS. This has meant that the market for any particular application written in conformance to these APIs has been small, since different versions (often with quite different user interfaces) were required for each switch or network.

•	Lack of Extensibility. Those APIs which have been available on multiple platforms (such as those developed by standards bodies) have been narrowly focused. In general, they have not included facilities for manufacturers to expose special, additional features to differentiate their products and make those features available to software developers.

•	Unfamiliar Programming Model. Many of the existing telephony APIs are based on arcane languages (such as the Hayes® “AT” command set) or involve exchange mechanisms, data structures, and messaging models which are unfamiliar to experienced Windows programmers.

Windows Telephony, on the other hand, addresses all of these issues:

•	General-purpose Computing Platforms. Building on the Microsoft Windows foundation, Windows Telephony uses standard PC hardware and software, preserving users investments and allowing the flexibility to which they have become accustomed.

•	Desktop Focus. Windows Telephony focuses on the “desktop” – generally, a single PC and telephone. The desktop is assumed, in the parameter structure of API functions, to be one endpoint of each call, thereby simplify the API and making it easier for programmers to learn and use. However, the capability of performing “third-party” call control (on behalf of other desktops) is preserved where needed, and future extensions for call center operation are planned. The underlying mechanism of communicating with the network can be either first-party or third-party.

•	Media Stream Access. Windows Telephony provides linkage between telephony call control operations and existing Windows functions for data communications, fax, digitized audio input and output, and other media streams, thereby facilitating the creation of interactive voice resource, fax-back, and other sophisticated applications. Touch-tone input and output is supported through the API, as well as detection and generation of other tones. The API supports media type detection and reporting and automatic routing of calls to the correct application to handle the detected media mode.

•	Multiple Application Support. Windows Telephony provides the capability for multiple applications to share telephony devices and to cooperatively control telephone calls. Monitoring and logging applications are supported. Multiple applications can be waiting for incoming calls while outgoing calls are placed on the same device. Control of calls can be handed off between applications.

•	Network- and Hardware-Independent. Windows Telephony is designed to be a superset of telephone network capabilities, allowing all networks to be modeled: POTS, ISDN, PBX, Centrex, key systems, cellular, wireless, etc. Applications can use the functions of the API without regard to the underlying hardware configuration, be it PC-based (add-in card), phone-based (smartphone or external module), or server-based (over a LAN). Differences in capabilities are reflected in data structures available to applications so that user interfaces can properly reflect available operations at any point in time. This permits a single application to be portable across a large number of platforms, so that, for example, a multinational corporation can use the same telephony application at all sites even though different PBXes, phones, and networks are used, cutting support and development costs.

•	Fully Extensible. Windows Telephony service providers can expose any number of special features to applications. Special device-specific functions and messages are provided, and most data structures and data types include device-specific extensibility. Facilities are provided to detect the extensions supported and negotiate versions of those extensions.

•	Familiar Programming Model. The Windows Telephony API is based on C-language functions and libraries, data structures, and messages, familiar to Windows programmers. Telephony applications can be developed using standard Windows development and testing tools.

Windows Telephony addresses the major issues which have confronted previous telephony APIs, and provides the basis for the widespread integration of telephony and personal computers.

### Benefits of Windows Telephony

Windows Telephony brings benefits to all segments of the telephony and computing industries.

•	Application Developers. A standard API, providing telephony network- and hardware-independence on the most popular operating system platform, gives the critical market mass to warrant an investment in development of full-featured telephony applications, for both commercial and corporate developers. Developers focus on the value-added, not the plumbing, and need not be concerned with being tied to a particular hardware platform.

•	Telephony network, switch, and hardware vendors can focus on the area of their expertise – hardware and switching – without needing to come up to speed on Windows user interface programming. Rather than having to sell a limited range of internally-developed applications, a full-range of broad- and vertical-market applications will be available from third parties, greatly expanding the market for telephony hardware for PC platforms and PCs with integrated telephony capabilities.

•	End Users. Investments in applications software and telephony hardware are protected by the interchangability of components. Competition in telephony application software means that a wide variety will be available at competitive prices, and new features will be continually rolled-out by commercial developers to meet growing user needs.

### Broad Industry Support

Over 40 major companies announced support for Windows Telephony when it was made public in 1993. These included:

Acer

Active Voice Corporation

Alcatel Business Systems Group

Ameritech

Analog Devices

Articulate Systems

AT&T Global Business Communications Systems

Bell Atlantic

Centigram Communications Corporation

Compaq Computer Corporation

Contact Software International

Cypress Research Corporation

Delrina

Dialogic

Digital Equipment Corporation

DSP Group, Inc.

Ericsson Corporation

Floreat, Inc.

Fujitsu Business Communications Systems

Harris Corporation

InteCom, Inc.

ISOCOR

Jensen-Jones, Inc.

Lotus Development

Mitel

Motorola Digital Signal Processor Operation

National Semiconductor, Inc.

Natural MicroSystems

NEC Corporation

Northern Telecom

Octel Communications Corporation

OCTuS, Inc.

Polaris Software

Rockwell International Corporation

Siemens Private Communication Systems Group/ROLM

Sequent Computer Systems, Inc.

Smart Technologies

Spectron

Stylus Innovation

TeleInt GmbH

Unifi Communications Corporation

US West Communications, Inc.

VMX, Inc.

Voice Technologies Group, Inc.

These companies, and many others, participated in the open design and review process which resulted in the Windows Telephony API and SPI specifications. Over 10,000 copies of the preliminary specification were downloaded from electronic sources and shipped in hardcopy form, and many more copies of the released specification have already been downloaded. Most major participants in every segment of the telephony  industry – switches, desktop equipment, voice processing, network providers, chipsets – as well as personal computers and software, support Windows Telephony, and are actively developing hardware, service providers, and applications.

### Summary

The Windows Telephony API brings a whole new range of functionality to business users by enabling the integration of the desktop computer with the telephone.  Because the API isolates applications from the complexity of the underlying network, application developers can generate network-independent applications.  Also, the API is independent of the type of PC-to-telephone connection, giving application developers and users configuration flexibility.  Removing these barriers is critical to the development of PC applications that exploit the telephone network.

The availability of Windows Telephony support (via service providers) will soon become a key factor in the selection of PBXes and telephony hardware. Many service providers will become available during the first half of 1994. Now is the time for developers to begin working with Windows Telephony, considering new applications to improve services and productivity through computer and telephony integration.

## The Windows Telephony Software Development Kit

### What’s In It

The Windows Telephony 1.0 Software Development Kit (SDK) includes:

·	TAPI.DLL, which routes TAPI functions between applications and service providers.

·	TAPI.LIB, which provides entry points to the functions of TAPI.DLL.

·	TAPIEXE.EXE, which is an application that is automatically loaded by TAPI.DLL and provides an application context for handling telephony events.

·	An applet called the Dialer, which is a sample call-manager application that provides the most basic telephonic functionality, such as processing requests from applications to make voice calls. It also has its own user interface that lets users make calls directly. Full source code for the Dialer is provided, and this source code is the starting point from which most programmers will begin programming their own telephony applications.

·	A Sample Service Provider (ATSP.C) that can establish and control a simple call using a HayesÒ-compatible modem. Its main functions are to help test Windows telephony applications and to provide sample source code for service-provider writers.

·	The Telephony Control Panel, a control panel applet (TELEPHON.CPL) which is used through the Windows Control Panel to install, remove, and configure Telephony service providers and to set location and calling-card information used in address translation.

·	A number-translation module (TAPIADDR.DLL) that translates addresses from canonical to dialable format.

·	Header Files, including TAPI.H for API functions, which must be included by all Windows Telephony applications.

·	Electronic Documentation, which includes the Telephony Application Programmer’s Guide and the Telephony Service Provider Programmer’s Guide. The documentation is provided electronically in Microsoft Word for Windows version 2.0 format, and can be printed on almost any laser printer. No hardcopy documentation is provided with the Windows Telephony SDK.

·	Help files, for both application programmers and service provider programmers. These Windows Help files contain complete on-line function references of all Windows Telephony API and SPI functions.

The components necessary to run Windows Telephony applications may be redistributed by developers at no cost (no license fees to Microsoft or Intel).

### Availability

The Software Development Kit is currently available from several sources:

•	CompuServe: Either GO MSL (Microsoft Software Library) and search for the keyword “S14400”, or GO WINEXT (the Windows Extensions Forum) and download the file TAPI10.ZIP from Library 3.

•	Internet: Use anonymous FTP to ftp.microsoft.com (198.105.232.1) and look in the directory “/devtools/tapi” for the file TAPI10.ZIP.

•	Microsoft Download Service BBS. Dial +1 (206) 936-6735, log in, and look for the file “TAPISDK.EXE”.

•	Microsoft Windows Development Platform Level II CD-ROM: This set of four CD-ROMs contains all Microsoft SDKs, DDKs and operating systems. This quarterly offering for developers accompanies the MSDN Level I Development Library, which contains all documentation for the development platform (but no code).  For more information, call 1-800-759-5474 in the US or Canada, 05.90.59.04 in France, (0130) 810211 in Germany, (060) 222480 in the Netherlands, (0800) 960279 in the UK.  For any other country in Europe call +31 (10) 258 8864.  For other countries outside of Europe or the US and Canada, call +1 (402) 691 0173.

•	Tech·Ed ’94 CD-ROM: All attendees of Tech·Ed ’94 receive a complete copy of the Windows Telephony 1.0 Software Development Kit for Windows 3.1 on their CD-ROM.

There is no charge for obtaining and using the SDK from network sources except for the cost of connect time for downloading (approximately 3.4 megabytes, compressed). However, do note that the SDK will be available for download for only a limited time, and thereafter will be available only by subscription to the Level II Development Platform CD-ROMs.

### Future Versions of Windows

Windows Telephony will be a standard feature of future versions of Microsoft Windows and Microsoft Windows NT. The Windows Telephony Software Development Kit will become a part of the SDK for these future versions as well. A beta release version of the Windows Telephony Software Development Kit for Windows NT is expected to be available in 1994.

### Support for Application Developers

Windows Telephony discussions can be found on Section 3 of the Windows Extensions Forum on CompuServe (GO WINEXT). This is the preferred venue for technical support questions, discussion of proposed future features and extensions, contacting other developers, etc. The author of this paper checks the forum daily and personally answers questions, and other members of the Windows Telephony team also make regular visits.

Private questions or feedback about Windows Telephony can be sent via the Internet to “telephon@microsoft.com” (on CompuServe, send to “>INTERNET:telephon@microsoft.com”), or via fax to “Windows Telephony Coordinator” at +1 (206) 936-7329. Further information on technical support can be found in the README.TXT file in the Windows Telephony SDK.

## An Overview of Windows Telephony Concepts

### Telephony Services

Telephony services are divided into Assisted Telephony services and the services provided by the full Telephony API. In general, the full Telephony API is used to implement powerful telephonic applications and Assisted Telephony is used to add minimal but useful telephonic functionality to non-telephony applications. Telephony’s services are divided into the groups shown in Error: Reference source not found.

µ §

Figure 6. Overview of Telephony Services

### Assisted Telephony

Assisted Telephony provides telephony-enabled applications with an easy-to-use mechanism for making phone calls without requiring the developer to become fully telephony literate. These functions provide very basic telephony functionality to primarily non-telephonic applications. If your application needs extensive telephonic control or is meant to handle incoming calls, you will need to use the full Telephony API.

Error: Reference source not found shows how requests from telephony-enabled applications are passed through TAPI.DLL to a full-featured call manager application for processing.

µ §

Figure 7. Path of Assisted Telephony Requests

### Basic Services

Basic Services are a minimal subset of the Windows Telephony specification. The functionality contained in Basic Telephony roughly corresponds to the features of POTS. Since all service providers must support the functions of Basic Services, applications that use only these functions will work with any TAPI service provider; minor variations in behavior are indicated in device capabilities data structures that can be obtained from the service provider.

The following functions are included in Basic Services:

•	Fundamental API interactions

•	Initialization and shutdown of the telephony API

•	Negotiation of API version

•	Filtering of status messages to be received

•	Opening and closing line devices

•	Capabilities and Status

•	Querying line device and address capabilities, associated media stream devices, icons, etc.

•	Querying line device, address, and call status

•	Querying and setting media device configurations

•	Operations

•	Translating addresses and setting associated parameters

•	Dialing and answering calls

•	Dropping calls

•	Controlling call privileges and handing off call control

### Supplementary Services

Supplementary Services are the collection of all the services defined by the API other than those included in the Basic Telephony subset. It includes features found on modern PBXes, such as hold, transfer, conference, park, and so on.

All supplementary features are considered optional; applications cannot assume that every service provider will support any particular supplementary service. An application can query a line or phone device for the set of supplementary services it provides using functions such as lineGetDevCaps or lineGetAddressCaps. Note that a single supplementary service may consist of multiple function calls and messages. It is important to point out that the Telephony API, and not the service provider developer, defines the behavior of each of these supplementary features. A service provider indicates that it provides a Supplementary Telephony service only if it can implement the exact meaning as defined by the API.

Today, many programmers will use only the services provided by Basic Telephony. But others, such as those writing code for PBX phone systems, will need the functions of Supplementary Telephony. Soon, the demand for ISDN and other network services, along with advancements in telephone equipment, will drive even greater usage of Supplementary Telephony.

The following is a summary of the functions included in Supplementary Services:

•	Holding and unholding calls

•	Transfering calls, both blind and with consultation

•	Conferencing

•	Forwarding calls

•	Parking and unparking calls

•	Picking up calls that are ringing elsewhere

•	Camp-on and other automatic call completion

•	Accepting, rejecting, and redirecting incoming calls

•	Securing calls from interruptions

•	Generating in-band dial digits and tones

•	Monitoring media mode, received DTMF digits, and tones

•	Controlling the routing of media stream information

•	Sending user-user information (ISDN)

•	Changing call parameters on-the-fly

•	Controlling the physical phone terminal (speakers, microphones, ringers, display, lamps, buttons,  etc.)

### Extended Services

The Telephony API contains a mechanism that allows service-provider vendors to extend the API using device-specific extensions. Extended Services (or Device-Specific Services) include all extensions to the API defined by a particular service provider. Since the API defines the extension mechanism only, the definition of the Extended-Service behavior must be completely specified by the service provider.

TAPI’s extension mechanism allows service-provider vendors to define new values for some enumeration types and bit flags and to add fields to most data structures. The interpretation of extensions is keyed off the service provider’s Extension ID, an identifier for the specification of the set of extensions supported, which may cross several manufacturers. Special functions and messages such as lineDevSpecific and phoneDevSpecific are provided in the API to allow an application to directly communicate with a service provider. The parameters for each function are also defined by the service provider.

### Telephony Devices

The Windows Telephony API defines two device classes; the line device class and the phone device class.

The line device class is a device-independent abstraction of a physical line device, such as a modem. It can contain one or more identical communications channels (used for signaling and/or information) between the application and the switch or network. Since channels belonging to a single line have identical capabilities, they are interchangeable. In many cases (as with POTS), a service provider will model a line as having only one channel. Other technologies, like ISDN, offer more channels, and the service provider treats them accordingly.

Just as a line device class is an abstraction of a physical line device, the phone device class represents a device-independent abstraction of a telephone set. TAPI treats line and phone devices as devices that are independent of each other. In other words, you can use a phone (device) without using an associated line, and you can use a line (device) without using a phone.

Service providers that fully implement this independence can offer uses for these devices not defined by traditional telephony protocols. For example, a person can use the handset of the desktop’s phone as a waveform audio device for voice recording or playback, perhaps without the switch’s knowledge that the phone is in use. In such an implementation, lifting the local phone handset need not automatically send an offhook signal to the switch.

This independence also allows an application to ring the local telephone in a manner that is independent of inbound calls. The capabilities of service providers is limited by the capabilities of the hardware and software used to interconnect the switch, the phone, and the computer.

### Media Streams

TAPI by itself provides control only for line and phone devices and does not give access to the information exchanged over a call. To manage this information (known as the “media stream”), developers need to integrate functions from other APIs into telephony applications. In this manner, TAPI works in conjunction with other Windows-based services such as Windows multimedia wave audio, Media Control Interface (MCI), or data APIs. This separation of control provides interoperability with existing audio or data applications.

For example, an application that provides an interface for managing fax or data transmission would use TAPI functions to control and monitor the line over which bits are sent, but the actual transmission of data is accomplished with functions from another API, such as the communications functions (known collectively as the Comm API) of the Windows SDK. See the Windows SDK documentation for more information about controlling data communication devices. Generally, an API (such as the Comm API) is associated with a particular media stream. For example, the Wave API can handle the voice stream that is transmitted over a TAPI-controlled line.

The media stream in an “interactive voice” call is produced and controlled not by TAPI, but by one human talking to another. However, the line on which that call is established and monitored, and the call itself, remain in control of the TAPI application. The media stream on a “stored voice” call, on the other hand, is routed either to or from the PC (or both), generally using the Wave API.

µ §

Figure 8. Other APIs complement the functionality provided by TAPI to control voice and data communications

### Address Formats and Address Translation

Many users choose to dial people, fax machines, bulletin boards, and other entities by selecting their names from an address book. The actual number that is dialed depends on the geographic location from which a call is to be placed.  For example, a user may make calls from a portable or handheld computer and want to use a single, static address book even when calling from different locations or telephony environments. Or, a corporation may keep a master address book which is available to all users, who might be in different area codes or even different countries. In the past, it was necessary to have separate address book entries, or separate address books, for each location from which a number might be dialed, which is a maintenance nightmare. TAPI’s address translation capabilities let the user inform the computer of the current location and the desired line device for the call. TAPI then handles any dialing differences, requiring no changes to the user’s address book. An application uses the lineTranslateAddress function to convert an address from the canonical address format to the dialable address format.

The canonical address format is intended to be a universally constant directory number. For this reason, numbers in address books are best stored using canonical format. The canonical address format used in TAPI is very similar to the international format for printed telephone numbers defined in ITU-T (CCITT) Recommendation E.123. It includes a “+” character, the country code, the city or area code surrounded by parentheses, and the local number. For example, the main number for Microsoft Corporation would be represented in canonical format as “+1 (206) 882-8080”. Various other types of information, such as ISDN subaddresses, can be appended to the end of the canonical address.

The dialable address format describes a number that can be dialed on a given line. A dialable address contains part addressing information and is part navigational in nature. In addition to the actual digits to be dialed, the dialable address may include additional characters for performance of special functions during dialing, such as: hookflash; switching between pulse and tone dialing; pausing for a fixed time; waiting for a second dial tone, quiet answer, or calling card prompt tone; instructing the application to prompt the user to continue dialing; and dividing a phone number into separate dialing instructions. Although an application can use dialable addresses returned by lineTranslateAddress, it is not limited to them and can compose its own dialable numbers.

Information related to the current location, such as the country code, area code, and outside line access codes, is entered by the user through the Telephony applet in the Windows Control Panel. The Telephony applet can also store information on telephone calling cards, including card names, numbers, and dialing procedures. Applications can obtain and present to the user a list of available locations and calling cards, and update the Telephony environment based on the user’s settings. The selected location and calling card information is then used by the lineTranslateAddress function to properly convert numbers in canonical format into the dialable format.

### Call Handles and Call Privileges

A call represents a connection between two (or more) addresses. The originating address (the caller) is the address from which the call originates, and the destination address (the called) identifies the remote end point or station with which the originator wishes to communicate.

TAPI identifies a specific call by means of the call’s handle, and TAPI.DLL assigns call handles as required. One call handle exists for every call known to an application, and an application can obtain call handles in a number of well-defined ways. Certain TAPI functions create new calls. As they do so, they return any new call’s handle to the application. Sometimes, call handles are provided unsolicited in callbacks made to the application from TAPI.DLL, as is the case with inbound calls or calls being handed off by other applications.

Associated with each call handle is the application’s privilege on the call – either owner or monitor. At any given time, one or more applications can own a call. While an application has ownership of a call, it is allowed to manipulate the call in ways that affect the state of the call. An application that does not own a call (but has a handle to it) is a monitor of the call and is prevented from manipulating it; it can only perform status- and information-query operations on that call. While one or more applications are owners of a call, still other applications can be monitoring the call.

An application can become an owner of a call in several ways. For example, an application that makes an outgoing call is the initial sole owner of that call. Ownership of an incoming call is assigned by TAPI.DLL to a single application based on the media mode of the call as indicated by the service provider and according to a user-settable priority scheme. Applications which have a monitoring handle to a call can also request to become an owner of the call.

An application that is currently an owner of a call can pass (“hand off”) ownership to another application that has the call’s line open. Such a handoff can be to a specific application (by name), or to the highest priority application for a particular media mode.  Handing off a call between applications never affects the state of the physical call as perceived by the switch or the service provider. A call can have several owners simultaneously, although the usual situation is for only one application to be the owner of a call.

## Programming Examples

In this section, basic techniques for programming applications to use the Windows Telephony API are presented. The topics covered include:

•	Using Assisted Telephony from Visual Basic

•	Initializing the API

•	Receiving and Processing Messages

•	Version Negotiation

•	Finding a Line Device and Processing Variable-Sized Data Structures

•	Opening a Line Device

•	Translating Addresses

•	Placing a Call

•	Tracking Call States

•	Finishing a Call

•	Closing the Open Line

•	Shutting Down the API

Except for the first example, the source code fragments used in all of these examples is taken from the Dialer application which is included with the Windows Telephony Software Development Kit. The readers is assumed to be familiar with programming Microsoft Windows applications in the C programming language.

### Using Assisted Telephony from Visual Basic

Assisted Telephony allows any application to be easily “telephony-enabled”, or used to initiate telephone calls, even from macro languages and other environments which do not (easily) support functionality such as callbacks and parsing of variable-sized data structures needed to implement an application that uses the full Telephony API. See the description of Assisted Telephony on page Error: Reference source not found for more information on how Assisted Telephony works and when to use it instead of the full Telephony API.

The simplest Assisted Telephony operation is tapiRequestMakeCall. This function allows an application to pass a telephone number and related information to the user’s preferred call manager application (Dialer, by default) for handling. This programming example shows how this function could be used in a simple Visual Basic application. This sample application accepts a phone number and an optional name string (separated by a slash) from the DOS command line. This allows you to create Program Manager icons to dial frequently-used numbers by placing the appropriate parameters in the Command Line field of the Preferences dialog associated with the icon.

First, it is necessary to declare the function and any constants used. This is done in the general declarations section of the form:

Declare Function tapiRequestMakeCall& Lib "TAPI.DLL" (ByVal DestAddress$, ByVal AppName$, ByVal CalledParty$, ByVal Comment$)

Global Const TAPIERR_NOREQUESTRECIPIENT = -2&

Global Const TAPIERR_REQUESTQUEUEFULL = -3&

Global Const TAPIERR_INVALDESTADDRESS = -4&

Here is the complete code for the application. First, it fetches the contents of the command line used to launch the program:

Sub Form_Load ()

a$ = Command$

Next, it searches for a “/” (which would indicate a name to go in the Call Log follows), and if one is present, parses the command line into the address to dial (a$) and the name (n$):

n$ = ""

i = InStr(a$, "/")

If i <> 0 Then

n$ = Trim$(Mid$(a$, i + 1))

a$ = Trim$(Left$(a$, i - 1))

End If

It then passes these fields to tapiRequestMakeCall. If the result is 0, that means that the request has been accepted; it is up to the call manager application to do any further work.

retval& = tapiRequestMakeCall&(a$, "", n$, "")

If retval& = 0& Then End

If an error occurs (non-zero result), a dialog box can be displayed to explain what happened. This code builds up an error message string and places it in a dialog box:

Beep

b$ = "Your request to dial " + Chr$(34) + a$ + Chr$(34)

b$ = b$ + " could not be fulfilled because "

Select Case retval&

Case TAPIERR_NOREQUESTRECIPIENT

b$ = b$ + "no Windows Telephony dialing application is running and none could be started."

Case TAPIERR_REQUESTQUEUEFULL

b$ = b$ + "the queue of pending Windows Telephony dialing requests is full."

Case TAPIERR_INVALDESTADDRESS

b$ = b$ + "the number you specified is invalid."

Case Else

b$ = b$ + "of an unspecified error.  Sorry!"

End Select

label1.Caption = b$

' Position form on screen

Left = (Screen.Width - Width) / 2

Top = (Screen.Height - Height) / 2

End Sub

The form (shown in Error: Reference source not found) consists of simply the label area to contain the error message, an icon (for aesthetics, not function), and an OK button that simply ends the program.

Figure 9. Sample Assisted Telephony Application Error Dialog

The remaining programming examples in this section apply to applications using the full Telephony API rather than Assisted Telephony.

### Initializing the API

Before an application can call the functions of Windows Telephony for using a line device (other than the Assisted Telephony client functions), it must initialize the TAPI environment and enable function callbacks with the lineInitialize function. Invoking this function also informs the application of the number of line devices available. In the Dialer application, this initialization takes place in the ErrStartTapi routine.

After declaring local variables, ErrStartTapi calls lineInitialize. This is done as part of a while loop, because the function might need to be repeated if TAPI were in the process of reinitializing and waiting for other applications to shut down their use of the API. If the lineInitialize function succeeds (the result will be 0) or if some other error is returned, the body of the loop is skipped.

The app passes into TAPI.DLL a pointer to a DWORD where TAPI will return an “application handle” which the app must save and use in other TAPI function calls. It also passes a pointer to its callback procedure, which will be called whenever TAPI has a message for the application (more on that later). A pointer to the application name string is also passed in, as is a pointer to a DWORD where TAPI will return the number of telephony line devices in the system.

while ((lResult = lineInitialize(&(vdts.hApp),hInst, (LINECALLBACK)MakeProcInstance((FARPROC)LineCallBackProc, hInst), szAppName,&(vdts.cLines))) == LINEERR_REINIT)

{

MSG msg;

if (fFirstTime)

{

EnableWindow(HwndDialerMain(),FALSE);

DisableDialerDesktop(TRUE);

} /* if */

The loop calls PeekMessage to give other applications an opportunity to execute while it is waiting to TAPI to reinitialize, if that is necessary.

if (PeekMessage(&msg,0,0,0,PM_REMOVE)) {

{

TranslateMessage(&msg);

DispatchMessage(&msg);

} /* if */

Dialer waits for the reinitialization for up to five seconds. If that time expires, the user is given an opportunity to cancel the Dialer startup or continue to wait.

if (GetTickCount() - tc >= 5000 || fFirstTime)

{

char szTitle[cchSzMax], szText[cchSzMax];

LoadString(HInstDialer(),ikszWarningTitle, szTitle, sizeof(szTitle));

LoadString(HInstDialer(),ikszWarningTapiReInit, szText, sizeof(szText));

if (MessageBox(HwndDialerMain(),szText,szTitle,

MB_RETRYCANCEL) == IDCANCEL)

break;

tc = GetTickCount();

fFirstTime = FALSE;

} /* if */

} /* while */

if (!fFirstTime)

{

DisableDialerDesktop(FALSE);

EnableWindow(HwndDialerMain(),TRUE);

} /* if */

If an error occurs in lineInitialize, the function returns the error code (which is processed in another part of the code and an appropriate error dialog is displayed). Otherwise, it sets a flag show that lineInitialize succeeded, but if no lines are set up in the telephony system, an error is synthesized.

if (lResult != errNone)

return lResult;

vdts.fLineInited = TRUE;

if (vdts.cLines == 0)

return errNoVoiceLine;

### Receiving and Processing Messages

An application receives information in two ways: solicited and unsolicited. Solicited information is requested by the application through a function call such as lineGetDevCaps or lineGetAddressCaps. Unsolicited information arrives in the form of messages – most importantly call-state messages. Often, the two mechanisms are used together, as when an application receives a LINE_CALLSTATE message, after which it checks the information contained in the LINECALLINFO structure by calling lineGetCallInfo.

When a TAPI application initializes its connection to the line service of TAPI.DLL with the function lineInitialize, it gives a pointer to the callback function it uses for that line service to TAPI.DLL. This pointer applies to all lines subsequently opened with the same application handle. The callback function obtains a large amount of information for the application, including asynchronous replies, indications of Assisted Telephony requests, or messages resulting from ongoing monitoring of a line. These messages include indications that the status of a line, address, or call has changed; indication that media, tone, or digit monitoring has detected the requested event; and indication that information about a call has changed.

The callback function used in the Dialer application is called LineCallBackProc. It consists of a switch statement based on the message type parameter passed into the function by TAPI.DLL. The other parameters of the callback function are the line or call handle (which is passed depends on the message type), an instance value (which the application can optionally provide with the lineOpen function), and others which are dependent on the message type.

Note that note all parts of the switch statement are shown in this document.

void FAR PASCAL _export LineCallBackProc(DWORD hDevice,DWORD dwMessage,DWORD dwInstance,DWORD dwParam1,DWORD dwParam2,DWORD dwParam3)

{

switch (dwMessage)

{

case LINE_LINEDEVSTATE:

{

if (dwParam1 == LINEDEVSTATE_REINIT)

vdts.fReInitTapi = TRUE;

break;

}

To keep the size of Dialer’s callback function manageable, the processing of call-state messages is performed in a separate function, newCallState, which responds to all call-state messages received by the application. It is in this function that call-state transitions (such as from connected to idle) are made known to Dialer as notification of the actual call states received from TAPI.DLL and/or the service provider.

/* process state transition */

case LINE_CALLSTATE:

{

NewCallState(dwParam1, dwInstance, dwParam3, (HCALL)hDevice, dwParam2);

break;

}

case LINE_CLOSE:

...

/* handle simple tapi request. */

case LINE_REQUEST:

...

The LINE_REPLY message is sent to an application upon successful or unsuccessful completion of an asynchronous function. Dialer uses only three async functions: lineMakeCall, lineDial, and lineDrop. Only one of these will be outstanding at any given time, so the processing is as simple as taking note of which was done last and acting on the result. This code appears somewhat convoluted because of the intermixing of processing of calls on behalf of other applications using tapiRequestMediaCall (Dialer is an Assisted Telephony server application) and those dialed by numbers entered by the user manually or through tapiRequestMakeCall (which are processed as though entered by the user).

/* handle the assync completion of TAPI functions

lineMakeCall/lineDrop */

case LINE_REPLY:

{

if (vdcr.requestID != dwParam1)

break;

If the program state is making a call or abandoning the making of a call, the result is processed as being from lineMakeCall or lineDial.

if (vdcr.mcs == mcsMaking || vdcr.mcs == mcsStopMaking)

{/* reply message for lineMakeCall */

if (dwParam2 != errNone)

{

If the call was being made as a result of a tapiRequestMediaCall request, a failure notification is sent to the client application.

if (vdcr.fIsProxy)

SendProxyMessage(&vdcr, TAPIERR_REQUESTFAILED);

Dialer always closes the line handle it used for making an outgoing call after each call. By keeping it closed when it is not actively using the line, it allows non-TAPI applications and DOS applications to use the resources. It then hides the call status dialog it had displayed while the call was active.

DoLineClose(&vdcr);

HideCallStatusDlg();

If the call failed because no call appearances were available, the user is informed via a dialog that the line is in use. If any other error occurs, a generic error processing function is invoked.

if (!vdcr.fIsProxy)

if (dwParam2 == LINEERR_CALLUNAVAIL)

DisplayLineInUseDlg();

else

DialerErrMessageBox (IkszFromErrCode(dwParam2));

The internal program state is then set to show that no call is active.

vdcr.mcs = mcsIdle;

} /* if */

else

If the call was established, the program state is updated. If the call was being abandoned, it is dropped (the call to lineDrop was deferred until after lineMakeCall asynchronously completed).

{

MCS mcsOld = vdcr.mcs;

vdcr.mcs = mcsMade;

if (mcsOld == mcsStopMaking)

FDropCall(&vdcr);

} /* else */

} /* if */

If the message is an asynchronous completion of lineDrop, then the line is closed.

else if (vdcr.mcs == mcsDropping)

DoLineClose(&vdcr);

break;

}

Several messages are not supported; these are immediately followed by a break statement. Developers can enhance Dialer’s functionality by enabling it to handle these currently unsupported call-state messages.

/* other messages that can be processed */

case LINE_ADDRESSSTATE:

break;

...

} /* switch */

} /* LineCallBackProc */

### Version Negotiation

A Telephony application must negotiate the version of the API to be used during interactions with TAPI.DLL. This negotiation takes place for each line to be used. Dialer does this for all lines in the system, since it monitors activities on all of them for call logging purposes. The function used is lineNegotiateAPIVersion.

The application passes as parameters to this function the application handle returned by lineInitialize, the line device number for which negotiation is to be performed (which must be between 0 and one less than the number of line devices indicated by lineInitialize), the minimum and maximum API versions supported by the application, and a pointer to a DWORD in which TAPI will return the version selected for use with that line. It also passes a pointer to a DWORD where TAPI will return the identification code for device-specific (proprietary) extensions supported by the driver (the application can ignore this value if it doesn’t use any proprietary extensions).

#define tapiVersionCur		0x00010003

lResult = lineNegotiateAPIVersion(vdts.hApp, iLine, tapiVersionCur, tapiVersionCur, &(vdts.lprgLineInfo[iLine].dwAPIVersion), &lineExtensionID);

if (lResult != errNone)

goto Ldone;

If the function succeeds, it returns 0; a negative value is returned in case of an error. The most likely cause of an error would be if the range of API versions supported by the application did not overlap the range supported by the service provider for the specific line device – but this itself is actually quite unlikely to occur.

### Finding a Line Device and Processing Variable-Sized Data Structures

The Dialer application opens all of the lines in the system for monitoring. It also provides a drop-down list control in the Dialing Options dialog to allow the user to select the line device Dialer is to use to place voice calls. As part of the ErrStartTapi function that is used to initialize the TAPI interface, it also scans through all of the line devices, opens them for monitoring, and uses the lineGetDevCaps function to identify those with voice dialing capability. Those that have voice dialing capability are included in the drop-down list.

First, Dialer allocates memory to hold the information it keeps internally about each line, and fails if insufficient memory is available. It sets the allocated memory to all 0.

vdts.lprgLineInfo = (LPLINEINFO) _fmalloc(sizeof(LINEINFO)*(int)vdts.cLines);

if (vdts.lprgLineInfo == NULL)

return LINEERR_NOMEM;

_fmemset(vdts.lprgLineInfo, 0, sizeof(LINEINFO)*(int)vdts.cLines);

Next, Dialer allows a buffer to store the data returned by lineGetDevCaps. An initial size is assumed, which is large enough to hold the information returned by a vast majority of service providers (lcbLineDevCapsInitial is declared as the size of the fixed portion of the LINEDEVCAPS structure plus 1000 additional bytes). If it is successful, the dwTotalSize field in the structure is set equal to the amount of memory allocated, to inform TAPI.DLL of the space available for returning information.

if ((lpDevCaps = (LPLINEDEVCAPS) _fmalloc(lcbLineDevCapsInitial)) == NULL)

return LINEERR_NOMEM;

lpDevCaps->dwTotalSize = lcbLineDevCapsInitial;

Dialer then loops through the available lines. It negotiates the API version to use for each line (shown above), and opens each line for monitoring (not shown). It then uses lineGetDevCaps to get information on the capabilities of the line device. If errors occur, the startup is abandoned and the function returns the error to the mainline code.

for (iLine = 0; iLine < vdts.cLines; ++iLine)

{

LINEEXTENSIONID lineExtensionID;

char szLineName[cchLineNameMac];

LPSTR lpszLineName;

lResult = lineGetDevCaps(vdts.hApp, iLine, vdts.lprgLineInfo[iLine].dwAPIVersion, 0, lpDevCaps);

if (lResult != errNone)

goto LDone;

If the function completes successfully, Dialer must still check to see if TAPI was able to return all available information about the line device. It does this by checking to see if the value returned in the dwNeededSize field in the structure exceeds the amount of memory allocated and indicated in dwTotalSize. This is done in a while loop because it may need to be repeated if the variable information changes between calls to the function. (In reality, it is unlikely that the data returned by lineGetDevCaps will change between functions, because it returns mostly static information. However, it is quite possible for functions that return structures containing status information to require more memory on consecutive calls. For the sake of consistency, Dialer uses a while loop like this one everywhere it needs to do a “check and reallocate” function.)

while (lpDevCaps->dwNeededSize > lpDevCaps->dwTotalSize)

{

DWORD lcbNeeded = lpDevCaps->dwNeededSize;

The previously-allocated memory is freed, and the new memory is allocated sufficient to hold the number of bytes indicated by dwNeededSize. The function fails if that amount of memory is not available.

_ffree(lpDevCaps);

if ((lpDevCaps = (LPLINEDEVCAPS) _fmalloc((size_t)lcbNeeded)) == NULL)

{

lResult = LINEERR_NOMEM;

goto LDone;

} /* if */

If the memory reallocation succeeds, the new amount of memory available is once again written to dwTotalSize, and the function is called again. If it succeeds, the loop cycles to check to see if all of the information was obtained.

lpDevCaps->dwTotalSize = lcbNeeded;

/* try it one more time */

lResult = lineGetDevCaps(vdts.hApp, iLine, vdts.lprgLineInfo[iLine].dwAPIVersion, 0, lpDevCaps);

if (lResult != errNone)

goto LDone;

} /* while */

Once all of the information is obtained, Dialer copies into its local storage the parts that it needs to retain – in this case, the number of addresses supported on the line, whether or not the line supports INTERACTIVEVOICE calls, and the name of the line. Note that if no name is indicated, Dialer uses a default name. Note also the mechanism that Dialer uses to obtain the line name from the variable portion of the structure, and that the offset is from the beginning of the entire structure, not from the beginning of the variable portion.

vdts.lprgLineInfo[iLine].cAddr = lpDevCaps->dwNumAddresses;

vdts.lprgLineInfo[iLine].fIsVoiceLine = ((lpDevCaps->dwMediaModes & LINEMEDIAMODE_INTERACTIVEVOICE) != 0);

if (lpDevCaps->dwLineNameSize > 0)

lpszLineName = (LPSTR)(lpDevCaps)+lpDevCaps->dwLineNameOffset;

else

{/* use default name */

...

The remainder of code in the function is where Dialer selects which line should be initially used for placing voice calls.

### Opening a Line Device

As was previously noted, Dialer opens all lines in the system for monitoring so that it can collect information to go into the Call Log. Dialer requests only monitoring privilege on calls on those line handles. When the user asks to dial a call (either through the Dialer user interface, or by a tapiRequestMakeCall function from another application), Dialer uses the lineOpen function to obtain an additional line handle on the line the user has specific to use for dialing. This occurs in the function fLocalInitiateCall.

This function first copies some information stored in Dialer global memory concerning the call to be dialed (not shown here). It then opens the specified line device with LINECALLPRIVILEGE_NONE, which means that Dialer is not interested in receiving incoming calls through this line handle, nor in receiving any status information about calls other than those it places itself.

The other parameters passed to lineOpen include the application handle originally given to Dialer by lineInitialize, the line device number desired, a pointer to a memory location to which TAPI will write the handle to the line, the API version previously negotiated on the line, the version of device-specific extensions desired (Dialer always uses 0, since it does not use any extensions), a callback instance which is passed back to Dialer in all messages pertaining to the line, the media modes for incoming calls the application desires to process (0, since no incoming calls are desired), and a pointer to a LINECALLPARAMS structure (which is used only when the LINE_MAPPER function is being used to scan for lines by their capabilities, which Dialer does not do, so NULL is passed).

If an error occurs, either an appropriate dialog message is displayed, or a failure message is returned, depending on whether the call was placed by the user or requested using tapiRequestMediaCall.

lResult = lineOpen(vdts.hApp, lpdcr->iLine, &(lpdcr->hLine), vdts.lprgLineInfo[lpdcr->iLine].dwAPIVersion, 0, (DWORD)(lpdcr), LINECALLPRIVILEGE_NONE, 0, NULL);

if (lResult != errNone)

/* display error or send message */

### Translating Addresses

Whenever Dialer is going to use place a call, it uses the TAPI lineTranslateAddress function to convert the number to dial from the canonical (international) format to a dialable string that can be passed to lineMakeCall. It does this in the Dialer function TranslateAddress.

The parameters to lineTranslateAddress include the application handle that was returned by lineInitialize, the line device ID on which the call is intended to be dialed, the negotiated TAPI version, the original number entered by the user, the ID of the calling card selected by the user and a parameter forcing TAPI to use that card number, and a pointer to memory to contain a variably-sized LINETRANSLATEOUTPUT structure.

Dialer uses the same “check and reallocate” process for the LINETRANSLATEOUTPUT structure that we discussed with regard to the LINEDEVCAPS structure and lineGetDevCaps, so the code to reallocate the structure is not repeated here. Dialer also performs some manipulations on the phone number to allow non-canonical phone numbers to be expanded to full canonical format if calling from North America; that code is also not shown here.

if ((lResult = lineTranslateAddress(vdts.hApp, lpdcr->iLine, vdts.lprgLineInfo[0].dwAPIVersion, lpdcr->szDialString, vdts.dwCardCur, LINETRANSLATEOPTION_CARDOVERRIDE, lpLineTransOutput)) != errNone)

goto LTranslateError;

lineTranslateAddress returns both a “dialable string” which is passed to lineMakeCall, and a “displayable string” which can be safely shown in call status dialog boxes without fear of disclosing the user’s calling card numbers to “shoulder surfers”. Dialer copies these, and the bits showing the result of the translation to local memory for later use.

fstrncpyDialer(lpdcr->szDialString, (LPSTR)(lpLineTransOutput) + lpLineTransOutput->dwDialableStringOffset, sizeof(lpdcr->szDialString), lpLineTransOutput->dwDialableStringSize);

_fstrncpyDialer(lpdcr->dialStringDisplayable, (LPSTR)(lpLineTransOutput) + lpLineTransOutput->dwDisplayableStringOffset, sizeof(lpdcr->dialStringDisplayable), lpLineTransOutput->dwDisplayableStringSize);

lpdcr->dwTranslateResults = lpLineTransOutput->dwTranslateResults;

Dialer next checks to see if the call is being placed in North America. If it is, it checks to see if the leading three digits are “911”, and if so it asks the user if they’re sure they want to dial 911. This code is not shown in this example.

### Placing a Call

Dialer invokes its internal function DwDialerLineMakeCall when it is ready to actually dial a call. It passes in pointers to structures containing the phone number to dial (previously translated from canonical to dialable form using lineTranslateAddress).

The most important function performed in DwDialerLineMakeCall is the processing of “dial pause” characters that might appear in the dialable string but which are not directly supported by the service provider handling the line device on which the call is being placed. Some telephony hardware cannot detect the call progress tones needed to properly handle these pause requests, but the need for the pauses cannot be ignored. The solution is for the application to prompt the user to indicate when the necessary tone has occurred and to cause the application to continue dialing.

First, the function sets up and initializes some local state variables, and saves off the initial dialable string.

/* %%Function:DwDialerLineMakeCall */

static LONG DwDialerLineMakeCall(LPDCR lpdcr,LPLINECALLPARAMS lpParams)

{

LONG lResult;

BOOL fCallMade, fCallAborted;

LPSTR lpchDialStart;

char szCtrlCh[2] = {0,0};

fCallMade = FALSE;

lpchDialStart = lpdcr->szDialString;

Next, the function enters into a loop (which will be eventually exited by a return statement).

while (TRUE)

{

LONG dwLineErrCode;

int ichCtrl;

char chSave;

During the first pass through the loop, the lineMakeCall function is used. Once the call is established, if the number has not been completely dialed due to the presence of dial pause characters which are not supported by the line device, lineDial will be used to dial the remaining portion of the number, after the user is prompted to tell Dialer when to continue.

/* try to make the call */

if (!fCallMade)

lResult = lineMakeCall(lpdcr->hLine, &(lpdcr->hCall), lpchDialStart, 0, lpParams);

else

lResult = lineDial(lpdcr->hCall, lpchDialStart,0);

If the lineMakeCall or lineDial function returned a positive value, it means that the dialable string was accepted, and Dialer must wait for a LINE_REPLY message to determine the final outcome of the attempt to establish the call. This function exits, and further processing would occur in the Dialer LineCallBackProc.

if (lResult > 0) /* async */

{/* we succeeded */

lpdcr->requestID = lResult;

return lResult;

} /* if */

If the dialing attempt failed for any reason other than the presence of a dialing pause character not supported by the line device, the function returns so that the Dialer user interface code can display an appropriate message to the user.

if (lResult != LINEERR_DIALBILLING && lResult != LINEERR_DIALDIALTONE && lResult != LINEERR_DIALPROMPT && lResult != LINEERR_DIALQUIET)

return lResult;

If the dialing failed because of an unsupported dial pause character, Dialer must identify the offending character. This switch statement converts the error code into a string containing the offending character.

dwLineErrCode = lResult;

switch (lResult)

{

case LINEERR_DIALBILLING:

{

szCtrlCh[0] = '$';

break;

}

case LINEERR_DIALDIALTONE:

{

szCtrlCh[0] = 'W';

break;

}

case LINEERR_DIALQUIET:

{

szCtrlCh[0] = '@';

break;

}

case LINEERR_DIALPROMPT:

{

szCtrlCh[0] = '?';

break;

}

} /* switch */

The saved dialable string is scanned for the offending character. The character is replaced by a semicolon, which will cause the line device to dial up to that point and remain in LINECALLSTATE_DIALING. The immediately following character is saved and then replaced with a null to terminate the dialable string.

ichCtrl = _fstrcspn(lpchDialStart,szCtrlCh);

lpchDialStart[ichCtrl] = ';';

chSave = lpchDialStart[ichCtrl + 1];

lpchDialStart[ichCtrl + 1] = 0;

Now that the offending character has been removed and the dialable string truncated at that point, the call is attempted again. If the original lineMakeCall failed, then lineMakeCall is used again, otherwise lineDial is used.

/* try to make the call again */

if (!fCallMade)

lResult = lineMakeCall(lpdcr->hLine, &(lpdcr->hCall), lpchDialStart, 0, lpParams);

else

lResult = lineDial(lpdcr->hCall,lpchDialStart,0);

Since the offending character was removed, if the function fails again, the problem is something that can’t be handled by this function, so control is returned to the mainline code. Otherwise, Dialer sets up to process the asynchronous completion in LineCallBackProc.

if (lResult < 0)

{/* we failed again but shouldn't */

return lResult;

} /* if */

if (lResult > 0) /* async */

lpdcr->requestID = lResult;

Now that dialing up to the point of the desired pause is in progress, Dialer displays a dialog to ask the user whether Dialer should hang up or continue with the rest of the dialing. It passes to the dialog function the original error code indicating the type of pause desired, so that it can display a message indicating what the user should listen for. The dialog function returns 0 if the user indicated to continue dialing, and another value if the call is to be aborted.

fCallAborted = (DisplayDialingPromptDlg(dwLineErrCode) != 0);

Dialer then sets up the dial the remainder of the dialable string. It restores the character that it had originally replaced with a null, and sets its pointer for the beginning of the string to that position in the string. It also indicates that lineMakeCall has already returned, so that lineDial will be used to dial subsequent portions of the string.

lpchDialStart[ichCtrl] = szCtrlCh[0];

lpchDialStart[ichCtrl + 1] = chSave;

lpchDialStart += ichCtrl + 1;

fCallMade = TRUE;

If the user said the abandon the call rather than continue, Dialer drops the call; otherwise, it cycles to dial the next portion of the dialable string.

if (fCallAborted)

{

FDropCurrentCall();

return 0;

} /* if */

} /* while */

} /* DwDialerLineMakeCall */

### Tracking Call States

Once a call is placed using lineMakeCall, LINE_CALLSTATE messages are sent to the application through the callback function it provided to lineInitialize to indicate the state of the call. When Dialer receives such a message, it uses its function NewCallState to parse and process the message.

static VOID NewCallState(DWORD newState,DWORD callbackInstance, DWORD privilege, HCALL hcall,DWORD dwParam2)

The first portion of this function involves the processing of messages for calls which Dialer is monitoring for the purpose of call logging. This processing will not be discussed in this paper.

If the LINE_CALLSTATE message is for a call that Dialer initiated, it first checks to see if it is a duplicate of the preceding state, and ignores it if so.

...

lpdcr = (LPDCR)callbackInstance;

if (lpdcr->lcs == newState)

return;

Otherwise, it saves the state, and then performs specific actions based on the new state.

lpdcr->lcs = newState;

switch (newState)

{

If the new call state is LINECALLSTATE_CONNECTED, Dialer invokes its UpdateCallStatusDlg function to show the connected state to the user, or it invokes its SendProxyMessage function to send a message to the application which called tapiRequestMediaCall to indicate that the call is connected (note that both of those functions include code to check whether or not the call was made as a result of tapiRequestMediaCall, so that code is not included here).

case LINECALLSTATE_CONNECTED:

{

UpdateCallStatusDlg(TRUE, lpdcr->szCalledParty, lpdcr->szDialStringOriginal);

SendProxyMessage(lpdcr,TAPIERR_CONNECTED);

break;

}

If the new call state is LINECALLSTATE_DISCONNECTED, Dialer updates the call status dialog (not shown), and then drops the call.

case LINECALLSTATE_DISCONNECTED:

{

...

FDropCall(lpdcr);

...

}

Dialer does not have the capability to automatically redial busy calls, so it treats LINECALLSTATE _ BUSY and LINECALLSTATE_SPECIALINFO the same as LINECALLSTATE_DISCONNECTED. It does this by recursively calling the NewCallState function to set the LINECALLSTATE_DISCONNECTED state, indicating the substates for BUSY or UNREACHABLE.

case LINECALLSTATE_BUSY:

{

NewCallState(LINECALLSTATE_DISCONNECTED, callbackInstance, privilege, hcall, LINEDISCONNECTMODE_BUSY);

break;

}

case LINECALLSTATE_SPECIALINFO:

{

NewCallState(LINECALLSTATE_DISCONNECTED, callbackInstance, privilege, hcall, LINEDISCONNECTMODE_UNREACHABLE);

break;

}

The last state a call enters after it has been dropped in LINECALLSTATE_IDLE. When this message is received, Dialer hides the call status dialog or sends a message to the application that called tapiRequestMediaCall (not shown here). Most importantly, it then deallocates its call handle to free resources in TAPI.DLL used to track the call. It then resets its internal call process state to show that it has no outgoing call in progress, and re-enables the controls on its main window, so that the mainline code will once again be ready to process another call request from the user or an Assisted Telephony application.

case LINECALLSTATE_IDLE:

{

...

lineDeallocateCall(lpdcr->hCall);

lpdcr->fIsProxy = FALSE;

lpdcr->mcs = mcsIdle;

DisableDialerDesktop(FALSE);

break;

}

} /* switch */

} /* NewCallState */

### Finishing a Call

When the user clicks the Hang Up button in the call status dialog, Dialer calls its FDropCall function. If this is done before lineMakeCall has asynchronously completed (i.e., Dialer’s internal state is not “mcsMade”), the drop is deferred until lineMakeCall completes.

static BOOL FDropCall(LPDCR lpdcr)

{

DWORD lResult;

if (lpdcr->mcs != mcsMade)

{

if (lpdcr->mcs == mcsMaking)

lpdcr->mcs = mcsStopMaking;

return TRUE;

} /* if */

If an active call is in progress and lineMakeCall has completed, Dialer calls the TAPI lineDrop function. It does not pass any user-user information (which is indicated by the NULL and 0 parameters). If lineDrop returns a positive value indicating that it will complete asynchronous, Dialer sets up to process the matching LINE_REPLY message in its LineCallBackProc, and disables the controls on its main window while waiting. If the lineDrop function fails, Dialer indicates this to the mainline code by returning FALSE as the result of this function, and setting its internal state to IDLE.

lResult = lineDrop(lpdcr->hCall,NULL,0);

if (lResult > 0) /* async */

{

lpdcr->mcs = mcsDropping;

lpdcr->requestID = lResult;

DisableDialerDesktop(TRUE);

return TRUE;

} /* if */

lpdcr->mcs = mcsIdle;

return FALSE;

} /* FDropCall */

### Closing the Open Line

When Dialer is done using a line for an outgoing call, to keep from monopolizing resources that might be needed by non-TAPI-aware and DOS applications (actually, whether or not any such resources, such as serial ports, depends on the underlying implementation of the telephony hardware and service provider, but Dialer nevertheless does its best to be a “good citizen”). This is done in the function DoLineClose. In this function, Dialer simply makes sure that its internal states show that it is idle, and then it invokes the TAPI lineClose function. It ignores any result returned by the function, since there’s not much it could do about it anyway (whether or not this is ideal programming practice is open to discussion).

static VOID DoLineClose(LPDCR lpdcr)

{

lpdcr->mcs = mcsIdle;

lpdcr->lcs = LINECALLSTATE_IDLE;

lineClose(lpdcr->hLine);

} /* DoLineClose */

Finally,

### Shutting Down the API

When the user exits Dialer, it calls its function TapiDone to disconnect from the Telephony API and release all of its resources. It deregisters as a recipient of Assisted Telephony requests (not shown), closes all of the lines that it opened for monitoring (not shown), and then calls lineShutdown to disconnect from TAPI.

If the initial call to lineInitialize had failed, then this function returns immediately.

void TapiDone()

{

if (!vdts.fLineInited)

return;

...

/* disconnect from TAPI */

lineShutdown(vdts.hApp);

/* free the rest of the memory allocated */

if (vdts.lprgdmr)

_ffree(vdts.lprgdmr);

} /* TapiDone */