TrainSIM 1.0

by

Ron Balewski

Please note:  TrainSIM is being distributed as Shareware.  You are

permitted to try TrainSIM before purchasing it.  If you continue to use

TrainSIM beyond the 30-day trial period, you are expected to become a

registered user by sending the $20 registration fee to the author.

Whether you register or not, however, you are encouraged to pass copies

of TrainSIM along for others to try.

Index

What is TrainSIM ....................... 4

About This Manual ...................... 4

Hardware Requirements .................. 4

Let's Run a Train ...................... 5

Detailed Instructions .................. 6

Diesel Locomotive Theory ............. 6

Running a Train ...................... 8

Initial Set-Up WIndow ................ 8

Engines section .................... 8

Cars section ....................... 8

Line FIle section .................. 8

Start button ....................... 8

Build button ....................... 8

Primary Control window ............... 8

File menu ................. ........ 8

Speed menu ......................... 9

Window menu ........................ 9

Help menu .......................... 9

Brake Pressure gauge ............... 9

Reserve Pressure gauge ............. 9

Air Flow gauge ..................... 10

Current gauge ...................... 10

Train Brake section ................ 10

Engine Brake section ............... 10

Throttle section ................... 11

Direction section .................. 11

Power button ....................... 11

Time section ....................... 11

Horn button ........................ 11

Bell button ........................ 12

Telemetry section .................. 12

Sand button ........................ 12

Slip light ......................... 12

Temp light ......................... 12

Oil light .......................... 12

Load light ......................... 12

OL light ........................... 12

Stall light ........................ 13

No Sound button .................... 13

Emergency button ................... 13

Speedometer window ................... 13

Speedometer gauge .................. 13

Mileage section .................... 13

Page 2

Clear section ...................... 13

Engine Satus window (Analog) ......... 14

Fuel gauge ......................... 14

Current gauge ...................... 14

Oil Pressure gauge ................. 14

Temperature - Engine gauge ......... 14

Temperature - Traction gauge ....... 14

On Line button ..................... 14

Traction Motor Cutout section ...... 14

1-10 Locomotive Selector buttons ... 14

"D" Digital button ................. 15

Engine Status Window (Digital) ....... 15

Gauges ............................. 15

Traction Motor Cutout section ...... 15

1-10 Locomotive Selector buttons ... 15

"A" Analog button .................. 15

Path window .......................... 15

Scenery WIndow ....................... 17

Building a Track ....................... 18

Overview ............................. 18

Initial Set-up Window ................ 18

Build Window ......................... 18

File menu .......................... 18

Start section ...................... 18

Control section .................... 19

Auto section ....................... 20

Vert Inc display ................... 20

Currents -- Horiz and Vert sections  20

SL section ......................... 20

MP section ......................... 20

Goodies section .................... 21

Length display ..................... 21

Checkpoint display ................. 21

Append button ...................... 21

Delete button ...................... 22

Goodies button ..................... 22

Map window ........................... 22

Helpful Hints .......................... 22

How to Register ........................ 23

What comes with Registration ........... 23

Add-Ons ................................ 24

Future Enhancements .................... 24

Permission to Distribute ............... 25

Attention Shareware Vendors ............ 25

Appendix A:  Track File Information .... 26

Appendix b: System Resources ........... 26

Page 3

What Is TrainSIM

================

TrainSIM is a simulation of operating a freight train along a

point-to-point route.  I've tried to mathematically model the "physics"

(acceleration and braking on hills and curves based on train weight,

etc.) and the controls of a train as accurately as possible.

Please remember that this is a "simulation" and not a "game."  It

can indeed get boring sometimes (like when you've got to pull up a

steep grade at ten miles an hour because you don't have enough power).

However, nobody ever said that reality must always be exciting -- and

with this version, I'm trying for reality!!

About This Manual

=================

This manual is divided into three parts.  The first will give you

just enough information to get a train moving.  You should read

this part of the manual while running TrainSIM, since it'll include a

lot of "push this button now" simplicity just to get you going.

The second part will go into great detail about each control on each

window visible while operating the train.  It's laid out on a

window-by-window basis, so it'll be a handy reference as you learn

TrainSIM.

The third section will take you through building your own track with

the track layout section included in TrainSIM.

Hardware Requirements

=====================

Being a Windows application, TrainSIM requires Windows -- version

3.1, to be exact.  You MUST also have SHARE.EXE loaded.  If you get an

Init error the first time you run TrainSIM, it's probably because you

dodm't load SHARE!

TrainSIM will work with virtually any graphics display that Windows

will support.  However, there are a lot of windows available in

TrainSIM.  To keep them all open with a minimum amount of overlap, you

should have a high resolution (1024x768 or better) display system.  For

this version, the sixteen-color setting will be adequate.  However, a

future version may prefer 256-color mode when I improve the scenery

window.

Page 4

I'd also recommend as fast a processor as possible.  The current

version of TrainSIM works fine in real-time on a 25 MHz '386, which is

as slow as I'd recommend.  A slower machine, although it will work,

will run the simulation slower than real-time.

Let's Run a Train

=================

OK, let's put on that striped engineer's cap and get our eastbound

freight moving!  We've got a schedule to keep!

To start TrainSIM, just double-click its icon from the program

manager.  If you're not registered, you'll see a shareware notice box.

For now, just press the "I'm still testing!" button.

You're now at the Train Set-up panel.  You can add or remove engines

or cars by clicking the up/down spinners.  You can also choose another

line file by pressing the F button in the Line File section and

selecting a new line file.  For now, let's use the defaults of 4

engines, 80 cars, and MAINLINE.DB.  So, to get underway, press the

Start button.

You'll now see two windows, the Primary Control window and the Speed

window.  There are others available, but they're not displayed yet to

reduce clutter.  You'll also notice some activity in the Brake

Pressure, Reserve Pressure, Air Flow, and Telemetry indicators.  This

is good -- it means your air tanks are charging up.  I'll give you more

details on this in section 2.

To get moving, you'll need the Direction and Throttle levers.  The

Direction lever is represented by three buttons - Reverse, Neutral, and

Forward.  In Version 1.0, Reverse isn't supported yet -- going into

reverse is the same as going into forward.  The Throttle lever is

represented by nine buttons.  An engine has eight power settings and an

Idle setting.  The left-most button is Idle (it's now red to show that

it's selected).  The right-most button is throttle position 8 -- full

throttle.  The buttons in between, of course, represent throttle 1 thru

throttle 7.  The window to the right of the buttons will display the

current power setting (since we're in Idle, IDLE is currently

displayed there).  Press the F button and then the Throttle position 1

button.

Page 5

OK, we're moving!  You'll notice the Current meter go up, indicating

that electric current is being delivered to the traction motors.

You'll also notice the speed gradually climb to about 9 MPH if you used

the default engine/car settings.  Let's speed up a bit.  Take 'er up to

throttle 4.  You'll see that we're delivering more current and we're

accelerating.  We should level out at about 68 MPH.  Notice how few

engines and how low a throttle setting is needed to move so many cars

at a good speed on the straight-and-level.

What does the track look like?  Good question!  Select Map under the

Windows menu on the Primary Control panel.  A Map window will open up.

The black/green/red line indicates the track.  Black is level, red is

an up-hill grade, and green is a down-hill grade.  The yellow grid is

in square-miles.  The blue dot is the front of your train.  The white

dot is the back of your train.  Section 2 will go into more detail on

what you can do with this map window.

What's ahead of us?  Another good question.  Very few engineers

operate their trains while blind-folded.  They might miss some rather

important information -- like speed limit markers!  Select Scenery from

the Windows menu and let's have a look around.  All we'll usually see

is track.  Keep an eye out for mileposts on the right-hand side --

there's one every mile.  We should see a speed-limit sign whenever we

come into a curve, too.  It'll also be on the right-hand side.  Pay

attention to those speed-limit signs.  They tell us the maximum safe

speed around the curve.  If we go too fast, we may just derail -- and

I'd hate to explain that one to the Trainmaster!

What about fuel?  Temperature?  And other engine controls?  I'm glad

you asked!  Select Engine from the Windows menu.  This window will show

you the "vital signs" of any engine in your consist.  You'll also

be able to take an engine off-line or bypass groups of traction motors.

Page 6

Detailed Instructions

=====================

Diesel Locomotive Theory

------------------------

Most modern diesel locomotives are really Diesel-Electrics.  The

diesel engine drives an electrical generator which produces 600 volts

of DC electrical power.  The electricity is then passed to a set of

traction motors - one per axle - which provide power to the wheels.

Locomotives today generally produce 3000 - 4000 horsepower.  Most

are either four-axle or six-axle.  They can be assembled into any

necessary multiple-unit (MU) combinations all under the control of one

engineer.

Because each axle is connected to an electric traction motor,

today's locomotives can provide "dynamic braking" which can assist

stops to reduce friction on the brake shoes.  When a locomotive is put

into dynamic braking mode, the traction motors are disconnected from

the generator and connected to a resistance grid.  The traction motors,

driven by the wheels, act as generators which take their power from the

moving train.  The resulting power is dissipated as heat in a

resistance grid, thus slowing the train.

Train braking is accomplished with air brakes.  However, a reverse

action fail-safe method based on George Westinghouse's 1872 invention

is used.  The brake pipe is held at high pressure when the brakes are

released.  Applying the brakes will reduce the brake pipe pressure.

Each car contains its own air reserve cannister and pressure monitoring

valve.  When the pressure monitoring valve sees the pipe pressure go

down, it routes air from the local reserve cannister to the brake

cylinders.  WHen the pipe pressure comes back up, the brakes are

released and the reserve cannister is re-charged.  With this method, if

an air hose were to break, the brake pipe pressure would quickly drop

and the brakes would apply.

Before leaving, the brake system is charged with air to between 70

(lower legal limit) and 110 pounds per square inch (PSI) of pressure.

Federal law further mandates that the train cannot leave unless the

pressure difference between the front and back of the train is less

than 15 PSI and air flow into the brake pipe from the locomotive's

Page 7

compressor is less than 60 cubic feet per minute (CFM).  Many trains

today use an end-of train (EOT) telemetry device.  The EOT continuously

radios the brake pipe pressure at the back of the train to a receiver

at the engineer's console.  This information can also aid the engineer

in confirming that brake applications and releases happened properly.

Running A Train

----------------

Initial Set-up Window:

Engines section

The up/down spinner allows you to choose how many engines you wish

to have in your traib.  You need at least one.  The maximum number

is ten.

Cars section

The up/down spinner allows you to choose how many cars you want in

your train.  The maximum number is 200.

Line File section

The text window displays the currently selected line file.  The

default is MAINLINE.DB.  To choose another line file, press the F

button and choose a line file from the list.

Start button

Starts the simulation with the chosen line file, engine, and car

values.

Build button

Puts you into the Build Line mode.  See the Building a Track

section for complete details.

Primary Control Window:

File menu / Save simulation

Saves the current simulation to a disk file.

File menu / Load simulation

Loads a saved simulation back into the simulator.

File menu / Restart

Re-starts the current simulation from the beginning.

File menu / Exit

Exits TrainSIM.

Page 8

Speed menu / Pause

Pauses the simulation.

Speed menu / Slow

Sets TrainSIM to run at half of real-time speed.

Speed menu / Real-Time

Sets TrainSIM to run at real-time speed where one simulated second

is one real second.

Speed menu / Fast

Sets TraiNSIM to run at four times faster than real-time -- or as

fast as your computer will allow it to.  It takes a very fast PC

to get up to four-times speed.

Window menu / Speed

Hides or displays the Speedometer window

Window menu / Engine

Hides or displays the Engine Status window.

Window menu / Scenery

Hides or displays the Scenery window.

Window menu / Map

Hides or displays the Map window.

Window menu / Cascade

Arranges all open TrainSIM neatly one atop the other in the upper

left corner of your screen.  Very useful if you "lose" one of your

windows underneath another or on a virtual screen.

Help menu / About TrainSIM

Displays a TrainSIM About dialog box.

Help menu / About TrainDLL

Displays the version information of the TrainDLL train simulation

DLL used by TrainSIM.

Brake Pressure gauge

Indicates the current brake pipe pressure at the engine.  Federal

law mandates that this be at least 70 PSI with brakess released.

Reserve Pressure Gauge

Pressure in the locomotive's air compressor reserve tank.

Page 9

Air Flow gauge

Amount of air flowing from the compressor's reserve tank into the

brake pipe.  Federal law mandates that this must be less than 60

CFM before a train is allowed to leave.

Current gauge

This gauge indicates the amount of current flowing through the

locomotive's traction motors.  When dynamic braking is applied,

the meter will deflect downward into the black region.  When power

is being applied, the meter will move up into the green or red

sections.  The more current flowing into the traction motors, the

more heat they'll accumulate.  You can safely operate in the green

range indefinitely.  However, if you go into the red section, you

should monitor your traction motor temperature and cut down power

if they start to heat up too much.

Note that this gauge only indicates the current flow for the head

engine.  Unless engines are off-line, all currents can be assumed

to be equal.  However, you can double-check things with the Engine

Status window.

Train Brake section

The up/down spinner arrows will apply and release the train

brakes.  The gray bar will turn red as the brakes are applied with

the Up spinner and will indicate the percentage of braking.

Clicking the Down spinner arrow will release the brakes and return

the bar to gray.

As you apply brakes, you'll notice the Brake Pressure drop.  You

should also soon notice the pressure coming back from the

Telemetry device drop.  Releasing the brakes will cause both

pressures to go back up.

Engine Brake section

The Engine Brake controls work just like the Train Brake controls.

However, the Engine Brake only applies friction braking to the

engines and NOT to the entire train.  Brake pipe pressure is NOT

affected by applying the engine brakes.

Page 10

Throttle section

All locomotives have a nine-poition throttle lever with an idle

position and eight power positions.  The nine push buttons

represent the nine possible throttle settings.  The button on the

left side of the row is for the Idle positon.  THe right-most

button is for the throttle 8 position (full throttle).  The

buttons in between represent (of course) throttle positions 1 thru

7.  The display window to the right of the buttons will indicate

which throttle setting is active.  Likewise, the active throttle

button will be red.

On this particular locomotive, the throttle lever also doubles as

the dynamic brake lever.  When in Braking mode, Throttle positions

1 thru 8 are braking levels 1 thru 8 where 8 is the most braking.

The Idle position causes no braking at all.

This lever will control the throttle positions of all locomotives

in the train.  All will supply power equally except for those that

are off-line (they supply no power) or have tracton motor banks

by-passed (they supply reduced power).

Direction section

The locomotive direction is set by a three-position lever.  The

three buttons in the Direction section simulate this lever.  F is

forward, N is neutral, and R is reverse.  Currently, Reverse is

inoperative.  Shifting into Reverse is the same as shifting into

Forward.  The selected direction button will be red.

Power button

This button shifts you between Power and Dynamic Braking

mode.  Pressing the button will cause it to toggle between

Power and Brake.  You should NOT press this button unless the

throttle lever is set to Idle!!

This button will control the power mode for all locomotives in the

consist.  Off-line locomotives will NOT contribute to braking.

Time section

Displays the current time.

Horn mutton

Blows the locomotive's horn!

Page 11

Bell button

Rings the locomotive's bell!

Telemetry section

Brake pipe pressure at the back of the train received from the EOT

telemetry device.

Sand button

The Sand button will toggle the sander on & off.  If your wheels

start to slip due to a steep grade and a large load, you may be

able to alleviate the problem by turning on the sander.  Sand is

dropped onto the track in frontof your wheels to increase

traction.

Slip light

When ANY locomotive in the train experiences "wheel slip", this

light comes on and the alarm sounds.  Wheel slip means that the

drive wheels of the locomotive are slipping on the track rather

than providing power.  To alleviate slippage, you can turn on the

sander and / or reduce your throttle setting until the Slip light

goes out.

Temp light

When the engine or traction motor temperature of any locomotive in

the consist approaches a dangerous level, this light will come on

and the alarm will sound.  The light will go off when all

temperatures drop below the danger level.

Oil light

Whenever the oil pressure of any locomotive engine drops below the

danger point, this light will come on and the alarm will sound.

Load light

Whenever any locomotive isn't "loading" properly, this light will

come on and the alarm will sound.  The term "loading" means that

electrical power is getting from the generator to the traction

motors.

OL light

Whenever any engine is off-line -- regardless of whether you took

it off-line or it went off-line because of a problem -- this light

will come on and the alert will sound.

Page 12

Stall light

Whenever the train's speed drops below more than 3 MPH for more

than a few seconds with power being applied, this warning light

will come on and the alarm will sound.  It means that you are very

likely to stall (stop moving, but still have power applied) soon

unless action is taken.  The most common cause for stalling is

climbing a hill with insufficient power for the load.

No Sound button

Whenever an alert light is turned on, an alarm sound will also

start.  The alarm sound will continue until it's reset with this

button.

Emergency button

Puts the train into "emergency" by setting the throttle to Idle

and setting the brakes to absolute maximum.

Speedometer Window

Speedometer Gauge

This is about what you'd expect -- a speedometer gauge in miles

per hour (MPH).  The small digital window on the bottom of the

gauge will display your speed digitally.

Mileage section

This is a re-settable odometer.  As you move, it will count

upward.  Pressing the R will reset it to 0.

Clear section

This handy device is used to let you know when your train is clear

of a point.  Normally, it displays "Ready ---".  When you press

the Ready button, the Ready changes to RUN and the window displays

the length of your train in feet.  As you move, the display window

counts down.  When it hits zero, an alarm sounds and the control

returns to its "Ready ---" state.

I often use this device to tell me when I'm totally clear of a

speed restriction.  You CANNOT speed up until your entire train is

clear of the speed restricted area.  As soon as I pass the higher

speed marker, I hit the Ready button.  When the alarm sounds, I

know its safe to speed up.

Page 13

Engine Status Window (Analog)

Fuel gauge

Displays how much fuel is left in the selected locomotive, of

course.

Current gauge

Displays the current going to the traction motors in the selected

locomotive.

Oil Pressure gauge

Displays the oil pressure in the engine of the selected

locomotive.

Temperature - Engine gauge

Displays the engine coolant temperature of the selected

locomotive.

Temperature - Traction gauge

Displays the traction motor temperature of the selected

locomotive.  This gauge is really three gauges in one.  The Top

section is for traction motor pair 1 and 6, the center section is

for traction motor pair 2 and 4, and the bottom section is for

traction motor pair 3 and 5.

On Line button

The ON Line button indicates whether or not the current locomotive

is on line and will toggle between on line and off line pressed.

If the locomotive was taken off line automatically due to high

temperature, etc.  this button will reflect the change.  To put it

back on line, you must manually press this button.

Traction Motor Cutout section

The six traction motors are arranged in three groups of two.  Any

one group can be taken out of service due to problems without

having to take the entire engine off line.  Of course, when you

cut out a traction motor pair, the locomotive's horsepower output

is reduced by one third.

1-10 Locomotive Selector buttons

These buttons will select which lococomotive's status is being

displayed.  There'll be only as many numbered buttons as you have

locomotives.  All others will be greyed.

Page 14

"D" Digital button

Press the D button to change the Engine Status window gauges from

analog to digital.

Engine Status WIndow (Digital)

Fuel gauge

Same as on analog gauge panel.

Traction Current gauge

Same as on analog gauge panel

Oil Pressure gauge

Same as on analog gauge panel

Engine Temperature gauge

Same as on analog gauge panel

Traction Temperature gauge

Same as on analog gauge panel

Traction Motor Cutout section

Same as on analog gauge panel

1-10 Locomotive Selector buttons

Same as on analog gauge panel

"A" Analog button

Press the A button to change the Engine Status window gauges from

digital to analog.

Path Window

Zoom In

Zooms the view in 50%.  The window scroll positions are

automatically changed to put the front of the train as close to

the center of the window s possible.

Zoom Out

Zooms the view out 50%.  The window scroll positions are

automatically changed to put the front of the train as close to

the center of the window s possible.

Page 15

Zoom Full Out

Zooms the view completely out to see teh entire track.

Zoom Reposition

Re-positions the scroll settings so the front of the train is at

the center of the window like Zoom in/out functions do, but

doesn't affect the magnification.

Zoom Redraw

Re-draws the current screen without changing the magnification or

scroll settings.

Scenery Window

There's nothing to do in the Scenery window -- it's for display

only.  However, it does provide some useful information.  The cab view

section occupies most of the window.  You'll see the track, speed

markers, checkpoint markers, and various other track-related objects.

In a later version, this window will also provide real, full-color

scenery -- trees, stations, buildings, bridges, and whatever else I can

think of putting in).

The Track Information section displays the current grade and curve

of the track, along with the last milepost and checkpoint detected.

Note that the detectors aren't perfect.  If you're going too fast, you

could go right by a milepost or checkpoint without the detector

updating the window.  Don't rely on them when making important

decisions!  Watch the track yourself!!

The two gray bars on the bottom of the Scenery window are "danger

detectors."  Curves and possibly rundown sections of straight track

have maximum safe speeds.  The faster you go beyond the safe limit, the

more chance of a derailment.  Likewise, the more cars within the

train that are exceeding the speed limit, the greater the chance of a

derailment.  The detector on the right shows the overspeed danger level

of the worst car in the train by turning red from left to right.  The

detector on the left shows your train's overall overspeed danger level,

again by turning red from left to right.

Page 16

A couple of examples may help this explanation.  First, let's assume

that you'e on a long straightaway with only one car-length section of

track restricted to 30 MPH and your 80-car train is travelling at 50

MPH.  As soon as you hit that section, the right-hand danger detector

will turn partially red because at least one car is exceeding the speed

limit by 20 MPH.  The detector on the left, though will, at worst, only

display a slight bit of red -- and probably none at all.  This is

because at any time, only one of your cars is in danger of derailing.

The rest are completely safe.  You could probably get through this

small problem area safely at 50 MPH because the overall danger is so

low -- but there is the chance that a car will derail.

Next, let's assume that you're travelling at 50 MPH into a curve

rated at 30 MPH. As soon as your first locomotive hits the curve, the

right detector will turn red.  The left one will stay grey.  As more

locomotives and cars enter the curve, though, the left detector will

gradually fill with red.  In this case, since more of your train is at

risk at all times, something will probably derail when you get a ways

into the curve unless you slow down.

Yes, I realize that real trains don't have anything like this to

warn the engineers of impending disaster.  I added these while working

on the derailment algorithms to see what was going on -- and liked

them.  So I left them in!  If enough purists complain about them,

though, I'll include a way to hide them in a future release!

Page 17

Building a Track

----------------

Overview

TrainSIM track sections are 66 feet each (the same length that

TrainSIM uses as an average car length -- how about that!!).  Each

section can have a grade and curve value associated with it, along with

some number of track or scenery objects.  The track must be single

track point-to-point.  Switches and sidings are not allowed in this

version.

Initial Set-Up Window

Press the Build button to enter the track builder.  The line file

selection in the set-up window is NOT used.  When you press Build,

you'll be asked for the name of the line you wish to build.  If that

line already exests, you'll be so informed and asked if you wish to

continue editing it.  If you select YES, the line will be loaded and

you'll be at the end ready to add more track.  If you select NO, the

old line will be deleted and you'll start from the beginning.

The track editor is, admittedly, quite complicated to use.  I tried

to simplify it as much as possible (in fact, I'm still trying), but

haven't been as successful as I'd like.  There's so much versatility

available in the track format that whenever I simplify the designer I

find that it limits what I can do to the track.  For this release, I

decided to keep the power and put up with the complexity

Build window

File menu / Exit

Exit will quit TrainSIM.  There's no need to save your new line

file before selecting Exit.  TrainSIM builds and maintains the

line file on disk as you work.

Start section

When you start a new track file, this array of buttons determines

where the track will start on the Map screen.  Initially, the top

button will be red.  This means the track will start from the top

and proceed downward.  If you press the left butotn, the track

will start from the left side of the map and proceed to the right.

You can select these buttons freely until the first piece of track

is laid.  As soon as you put down the first section, all the Start

buttons become black and cannot be pressed

Page 18

Control section

This is where most of the action takes place.  These buttons

control the slope, direction, and advance of the track.  The

center up and down arrows raise and lower the Vertical Increment

by .01 percent grade (NOTE: This does not translate directly into

grade -- I'll get back to it in a moment).  The left and right

arrows control curve in 1 degree increments from left 15 to right

15.  The gradiant up arrow in the center (I hope it looks more

like "forward" than "up") lays one or more sections of track with

the current horizontal curve and vertical slope attributes (info

in the Currents section).

The spinner and number in the lower left corner of the Control box

will let you lay from 1 to 30 sections of track with each press of

the Forward button.  The number indicates the current lay size.

This is very useful for laying long straightaways.

The INC button in the lower right corner is where the confusion

will start.  Remember I said the up and down arrows control the

Vertical Increment and NOT the current vertical grade directly?

What happens is this -- Whenever you press the Forward button, the

Vertical Increment is added to the Current Vertical Grade before

the first section of track is laid.  This lets you easily create

nice, smooth grades that graduallly build up to the maximum grade,

then gradually taper off to level.  Ihe INC button lets you decide

if you want the increment to take place only once for each press

of the Forward button (INC button will be black) or for each track

section laid (INC button will be red).  For instance, if you set

that lower-left sections indicator to 30, set the Vertical

Increment to .01, turn on the INC (Red), and press Forward, you'll

get 30 sections of track.  The first will have a .01% grade, the

second a .02% grade, the third a .03% grade, etc.  After the

operation, the Current Vertical grade indicator will indicate a

.3% grade.  I'll admit it's confusing -- but it's easier than

laying one section of track at a time and having to manually

increase the grade each time!

Page 19

Auto section

Automatically places certain goodies along the track as follows:

Speed:

When Auto Speed is turned on, changing the curve will post a

speed limit sign containing the recommended speed for the curve

installed.  Use of this feature is highly recommended!

Drager / Wayside:

A dragging equipment detectors and / or waysides are placed a

fixed distance apart along the length of the track.  Each time

either of these is turned on, a dialog box comes up containing

a mileage display and a spinner.  Use the spinner to set the

distance between items and press OK.

Vert Inc display

Displays the current vertical increment value as described above.

Currents - Horiz and Vert section

Displays the current settings for horizontal curve and vertical

grade for new track.

SL section

The SL section works together with the Speed Marker object in the

Goodies section.  When you place a Speed Marker, this box sets the

speed limit value to be placed in the marker.  The display window

shows the current setting.  The up/down spinner will change it in

5mph increments.  Note that turning on Autp Speed marker and

changing the curve with the Control left/right arrows will also

change this setting to what's correct for the selected curve

degree.

MP section

Whenever a track is built, it'll always have milepost markings.

This section controls the FIRST milepost marker on the line.  By

default it's 100 -- which is fine most of the time.  If you'd like

to start at some other value, though, change it here with the

up/down spinner.  Note that this MUST be changed BEFORE any track

is laid!  Once a track is laid, the milepost designations for the

line are finalized and CANNOT be changed!

Page 20

Goodies section

To manually place a piece of trackside scenery, choose the object

to be placed before clicking the Forward control button.  On a

multiple-section forward command, the object is placed on the

FIRST section laid in the group.  Each Forward command will also

return the selected Goodie to None!

Most Goodies are self-contained.  Only the Speed Marker and

Checkpoint need extra information.  The Speed Marker will take its

maximum speed display from the current SL (Speed Limit) section

setting.  The Checkpoint object will get the name of the

checkpoint from the current Checkpoint section value.

Length display

This is simply a running total current line length in miles.

Checkpoint display

This section is used along with the Checkpoint object in the

Goodies section.  When you place a Checkpoint, you must gove it a

name.  Simply click in the white edit box and type in the

checkpoint name.

Append button

Whenever you start a new line or edit an existing line, you're

initially in the append mode -- placing track on the end of the

current line.  To insert some new track in the middle of the line,

click on where you'd like the insert to start on the Map window.

The Append will change to a red Insert to let you know your

insertion point was successfully found.  Now, as you lay track,

it'll look like you're laying a branch line -- the track will

start from your insertion point.  THIS IS NOT A BRANCH LINE!!!

THe rest of the track will be added to the end of what you've

inserted when you're done inserting!!!

When you're done inserting, press the red Insert button.  The line

will be re-built and re-displayed with the rest of the track

tacked onto the end of the new insert, the red Insert button will

return to Append, and you're back in Append mode where additions

are put at the end of the track.

Page 21

Delete button

To delete a section of track, first click the starting point in

the Map window.  The Append button will change to Insert, as

described above.  Next, press the Delete button.  Both the

Append/Insert and Delete buttons will display a red "Delete".

Now, click on the end point of the deleted section in the Map

window.  The track will be re-built and re-displayed without the

deleted section and both the Append and Delete buttons will return

to normal.

Goodies button

The Goodies button will let you place an object (speed sign, etc.)

at any point along an already-completed track.  First, press teh

Goodies button.  It will turn red.  THen, click anywhere on the

track in the Map window.  If your click is close enough to the

track to be meaningful, a dialog box will appear.  Select the

object you wish to place and press OK.  The object will be placed

and the Goodies button will return to black.  Note that the Speed

Marker and Checkpoint objects require that you also fill in either

the SL seting or Checkpoint name respectively.

Map window

The Zoom menu works just like it does while running a train.

Clicking in the Map window during Build mode will put you in Insert

mode if you click close enough to the existing track.  All other

clicks are ignored.

Helpful Hints

=============

Probably the best advice I can give on running the train is to take

it nice and easy!  Like other electric motors, the locomotive's

traction motors can only put out so much torque.  After a certain

point, more energy is dissipated as heat than is used to turn the

motor.  This of course causes the traction motors to heat up

considerably, while not increasing your speed too much.  If the

traction motor current is in the red, try dropping the throttle down a

notch.  You may be able to save fuel and take a large heat load off the

traction motors with very little loss of speed.

Page 22

Plan ahead.  If you've got a long hill coming up, keep things nice

and cool before starting up it -- long hills really take their toll on

engine and traction motor temperature.  Starting out cool will let

you apply over-power for a longer time to make it up the hill.  If

you've got no problem areas coming up,  pushing the engines for some

extra speed on the level will be safe.

How do I register?

==================

To register TrainSIM, just press the Let's Prepare the Registration

Form button on the opening screen and fill in the blanks.  Then, print

the form and mail it with the $20 registration fee to the address on

the form.  You'll soon recieve a registration code number.  Type the

number EXACTLY as given into the Registration Number window on the

opening screen and press the Here's My Code button.  If all's OK,

you'll never see the opening non-registered screen again.  If the

code's invalid, you'll be so told.

What do I get when I register?

==============================

Registered users are entitled to updates for a greatly reduced cost

-- usually just a few dollars to cover my duplication & shipping costs.

At some point, the registered version of TrainSIM may be more advanced

than the Shareware version.  I often keep the registered version a

version or two ahead of the shareware one.  More importantly, you'll

receive a code to get rid of that annoying UNREGISTERED opening

screen!!

Registered users of any of my products are also allowed to register

other products at a discount -- usually 30 percent off the normal

registration fee.

Please note that I DO NOT regularly send upgrade notices to all

registered users whenever I release an upgrade.  Doing so would be

quite expensive.  I'd have to either raise my registration fee or

charge more for upgrades to cover the expense -- or simply not release

upgrades as often as I'd like.  If you're a registered user interested

in upgrading, though, you can feel free to inquire about the current

version at any time.  If you disagree with this policy and would rather

pay more for each upgrade and recieve a notification card, let me know.

If enough people are intersted in a notice and enough don't complain

about a higher upgrade fee, I'll consider starting the service.

Page 23

What else is available?

=======================

I hope to create and collect track lines and make them available on

high-density disks for registered users.  If you build a fun rail line,

please send me a copy.  I'll add it to the library and send you a disk

or two (depending on what I have) of new rail lines.  I'll probably

charge $3 per disk to cover my time and expenses.

I also hope to put together a printed and bound version of this

manual and make it available to registered users.  I don't have any

idea what the price would be -- it would depend entirely on how much

the book would cost to produce.

I also have two versions of Mah Jongg available -- Mah Jongg VGA and

Mah Jongg for Windows.  Sample disks are available to registered

TrainSIM usere for $3 per disk.  All others should be able to find

these games on a local BBS or a national service like Compuserve or

America Online.

Future Enhancements

===================

This version of TrainSIM is definitely not the last release.  There

are a LOT of things I'd like to add.  I just wanted to get something

reasonable out the door to generate some feedback on what can be done

to make the simulator "feel" more realistic.  I'm hoping that people in

the railroad industry especially will be intrigued enough by TrainSIM

to send me their thoughts, observations, and suggestions.

So, here's what I see in the future so far:  A much improved scenery

window that includes "real" scenery (trees, buildings, whatever else)

and working signal lights.  All in 256 colors! Talking dragging

equipment detectors!  Simulated dispatch operations, complete with Form

D's from the dispatcher!  Sections of double-track with traffic coming

in the other direction!  More realistic-looking controls (for example,

a throttle lever that looks like a lever)!  A choice of engines!  A

choice of freight cars, both loaded and unloaded!   Occasional

breakdowns of both the train and the track equipment (signal lights,

etc.)!  The ability to get "helper service" on certain grades!

If you have any ideas to add to this list, I'd love to hear them!

Page 24

Permission to Distribute

========================

Individuals, BBS's, clubs, and shareware distributors are permitted

to copy and distribute TrainSIM provided that they charge no more than

$5 for their disk copying service, charge nothing for the software

itself, and make all recipients aware that they are receiving

un-registered Shareware which will require a registration fee if they

use the product for more than 30 days.

Attention Shareware Vendors

===========================

You have my permission to distribute this copy of TrainSIM.  If

you'd like a copy of the latest shareware version directly from me,

please be sure to include a stamped, self-addressed return mailer and a

pre-formatted high-density disk. A $4 check to cover my time and

expenses can be substituted if more convenient (please specify whether

you'd like 5.25" or 3.5" disk format).  Your request must also be on

your company letterhead.  If you fail to comply with these

requirements, your request WILL NOT BE FULFILLED!  I ABSOLUTELY WILL

NOT send copies to shareware vendors at my expense!!!!

Page 25

Appendix A: Track File Information

==================================

The track line file is a Paradox database file.  If you need the

exact data specifications for the fields, please write and request a

current TrainSIM line database spec sheet.

Note that I expect the database to change as TrainSIM evolves.  I'll

have to add a LOT of new objects and scenery object positions as the

scenery window develops, for instance.  If enough people build track

lines for TrainSIM, I'll write conversion routines as necessary.

Whether or not a conversion routine is available, though, the format

will NOT change so drastically that the track line files can't be

quickly fixed up in Paradox by adding the appropriate fields.  I intend

to keep the exact format of the line files public to both allow for

this type of modification and to allow other developers to produce

add-on tools and utilities for use with TrainSIM track files.

Appendix B:  System resources

=============================

TrainSIM requires a large amount of system resources.  Basically,

the system resource memory is used by Windows to keep track of windows,

tools, and controls on the screen.  SInce TrainSIM uses a lot of

windows and controls, it requires Windows to use a lot if its resource

storage section.  If TrainSIM behaves strangely (controls re-draw very

slowly, screens incompletely re-draw, etc.), try closing down all other

applications to free up some resources.  You should also try exiting

and re-starting Windows, because Windows resources are often gradually

"used up" by software that doesn't release them.  Re-starting Windows

will free the unused resources.

Page 26