COMPORTS

VERSION 1.0

COMPORTS allows DOS to use the communications ports COM3 &

COM4 (if present).

DOS is set up to allow up to 4 serial communications ports

(COM1, 2, 3, & 4).  COM ports are mapped into the IBM PC's I/O

address space.  Unfortunately, only 2 address locations are

"standard" (at least on the original IBM PC).  This means that

DOS can only use 2 of the 4, because it doesn't know where the

other 2 are.  (How can you send a letter to someone if you don't

know their address, even though you have blank entries in your

address book?)

Over the last few years, there has arisen a "standard" for

the port addresses of COM3 & COM4.  These are locations 03E8h &

02E8h, respectively.  If your modem or serial card uses these

addresses, then COMPORTS can tell DOS about them, and allow DOS

to use them.

Some MODEM control software (such as BOYAN and PROCOMM) get

around this problem by directly banging the MODEM registers at

the standard addresses of COM3 or COM4.  In this way, DOS is

ignored.  This generally works for the MODEM software, but still

leaves us unable to execute DOS commands such as "MODE

LPT1:=COM3:".

All we have to do to get DOS to use COM3 & COM4 is to tell

DOS where they are (assuming they are present in the computer).

DOS maintains a table of COM port addresses starting at memory

location 0040:0000.  COMPORTS examines the standard I/O locations

for serial cards, and if present, adds their addresses to the COM

port address table that DOS maintains.  Alternately, COMPORTS can

put an entry into the table whether the card exists or not, or

simply report what it finds at each of the relevant addresses.

COMPORTS does not change any entry for COM1 or COM2, since

most PCs seem to deal adequately with these two ports.  COMPORTS

does, however, check to see whether any or all of the 4 possible

serial cards are present in your machine.  It does this by

attempting to change the value of the Modem Line Control

Registers for each of the 4 possible serial cards.  If an I/O

location corresponding to the Line Control Register can be

changed, it is assumed that a serial card is present.  Otherwise,

the card is assumed to be missing.  The previous values in these

registers are restored once they have been checked.

COMPORTS allows the following command line switches:

COMPORTS          No Switch.      Lists the allowable

commands.

COMPORTS S        Status.         Indicates which COM ports

are currently active,

their addresses, and

whether they appear to

actually be present in

the system.

COMPORTS A        Auto Detect.    Checks for the presence

of serial ports and adds

them to the DOS table if

they are present.

COMPORTS 3        Add COM3.       Add COM3 to the DOS

table, WHETHER IT IS

PRESENT OR NOT.

COMPORTS 4        Add COM4.       Add COM4 to the DOS

table, WHETHER IT IS

PRESENT OR NOT.

COMPORTS B        Add COM3 & 4.   Add both COM3 & COM4 to

the DOS table, WHETHER

THEY ARE PRESENT OR NOT.

NOTE:    Telling DOS that a COM

port exists, when in fact

it does not, may have

unpredictable

consequences.

LEGAL STUFF

COMPORTS is copyrighted freeware.  It may be freely

distributed, while the author retains all copyrights.

There is NO WARRANTY, EITHER EXPRESSED OR IMPLIED.  In no

way will the author be held liable for damages arising from the

use of this program.

I believe this program to be harmless, but it does alter

(and reset) specific hardware locations (the standard "Modem Line

Control Register" addresses), and writes info to DOS's COM port

table and COM port count byte.  Both of these actions are

potentially dangerous.  Use COMPORTS at your own risk.

I hope that you find this program useful.

J. R. Casey Bralla

a.k.a. NerdWorld

408 Halifax Drive

Martinez, Georgia 30907

(404) 860-1228 (Voice only)