___________________________________________

C O M S E T

Modem / Uart Setup Program

Version 5.2..release date 5/7/89

from Hank Volpe Computers (c) 1989

___________________________________________

Introduction

Comset is a utility that is designed to setup a communications port

and a AT (Hayes) Compatible smartmodem. Many modems, especially

internals, do not have a method to reset the modem when it is

activated, nor does an adequate method exist to setup the serial port

for use.

Comset will setup your serial port to any speed from 300 to 19200 bps

and then check to see if your modem is on-line. If it isn't, Comset

will perform a hardware reset to "wake" it up. Next a software reset

is performed by sending the command ATZ. If the reply OK is received,

you are informed that the modem is fine. If OK is not received, Comset

will attempt 3 times to reset the modem before informing you of an

error. If you have an external modem and it is off, Comset will abort

after setting up the serial port to the desired baud rate.

Some of the features of Comset include the following ;

Level 1 options...Shareware Version

** User Selected baud rates from 300 to 19200 bps.

** User Selected Comm port assignments.

** Modem Status register testing.

** Redirectable output..can be sent to a file for logging.

or used over any serial link.

** RTS/CTS DTR/DSR Handshake testing.

Registered Users Have the following advanced features;

** User Selection of Comm ports 1 2 3 4.

** Log in of Comm ports 3 & 4 to DOS for use by other programs

such as high level languages or applications.

** Batch file parameter passing and error reporting.

1

Comset....Smart modem utility version 5.2

I. USING COMSET

----------------

The usage syntax for Comset is simple.

COMSET [speed] [port]

or

COMSET h

[speed] and [port] are user options

h invokes runtime help for the user

Comset is easy to use. Typing COMSET from the dos prompt will

automatically attempt to set up serial port COM1 to 2400 bps and

attempt to wake up its modem. Selecting a different speed involves

only adding the speed after the word Comset...ie COMSET 1200 sets

serial port COM1 to 1200 bps instead of the default 2400. Trailing

zeros are not necessary, so you could also type COMSET 12 to set up

serial port COM1 to 1200 bps. If you wish to use a different serial

port, add that after the speed...ie COMSET 24 2 will set serial port

COM2 to 2400 bps. If you accidentally enter an invalid baud rate, the

default of 2400 bps will be used. If you accidentally enter an invalid

range of comm ports, the default of COM1 will be used. Defaults are

always used if the user does not supply speed and port.

Typing COMSET h will bring up a runtime summary of features and other

helpful information.

Ideally, Comset should be run out of your autoexec.bat file when you

first turn on your computer. Internal modem users can appreciate the

fact that probably 2 or 3 times out of a 100 their internal modem does

not "wake-up" properly when the computer is first turned on, and this

can hang-up some communications programs. Also, BBS sysops sometimes

experience problems with modems when stopping the BBS and trying to

use the modem to call out.  This is why Comset was written!

II. SERIAL PORT INFORMATION

---------------------------

Comset supports all the industry standard Serial Port address

assignments for modems. The shareware version supports COM1 and COM2,

while the registered version adds COM3 and COM4 support also. Almost

all PC-modems allow you to use COM1-4, however only specific software

can use COM3 or COM4 on most machines.

2

Comset....Smart modem utility version 5.2

The full featured version of Comset not only will use COM3 or COM4 for

itself, but it will log those ports into DOS for use by other programs

such as high-level languages and applications (something that under

normal conditions can't be done with an IBM-PC or most clones without

special software).

III ERROR REPORTING INFORMATION

-------------------------------

Comset checks your serial port in three distinct steps;

1) Set the Serial Port computer UART to the desired baud rate

and raise the DTR and RTS lines. If a serial port is not

detected, Comset aborts and displays the message that there

is no serial port with that COMM designation in your system.

2) Check that DSR and CTS are returning from the modem, if not

report an error and stop testing.

3a)If steps 1 & 2 are successful, wake up the modem by sending

the command string "ATZ". If OK is received, report all is

Normal and leave the RTS and DTR lines active.

b)If the modem did not wake up, issue a more advanced hardware

and software command string to reset and wake up the modem.

Three attempts are made..if it fails, the RTS and DTR lines

are turned off. This could happen if you tell Comset to

test a serial port that does not have a compatible modem

attached. In that case, a "failure" of this kind would be

"normal" for your system.

The Shareware version will report status to the screen. You can

redirect the output of Comset to a file if you wish to maintain a log

of each initialization. To do this, just use the command string

COMSET [speed] [port] >> comset.log

Speed and port are optional. The >> symbol tells dos to make a file

called comset.log and append it each time Comset is run.

3

Comset....Smart modem utility version 5.2

IV FULL FEATURE USE AND ERROR REPORTING

---------------------------------------

Registered Users of Comset receive some advanced features, some of

which were described above in previous sections. An additional feature

of the registered version is that of error-code reporting. Error code

reporting simply stated passes "result-codes" from Comset to its

calling program. This allows you to write some advanced batch-file

procedures or to take specific actions.

Running the registered copy, additional information is displayed

informing you of the Error code received. Also that error code can be

interpreted by Dos Batch files using the ERRORLEVEL function. Under

Normal conditions, a result code of 0 is returned to the calling

program. If something wrong is detected, an error code of 99 is

returned to the calling program.

A simple use of this can be to start a communications program if the

modem is ok, but if not, default to some other action...an example

batch-file program looks like this;

echo off

comset 24 1

IF ERRORLEVEL 99 goto stop

echo passed

goto exit

:stop

echo failed!

:exit

Other uses are left up to your imagination...you could flash up help

screens to the user like "HEY DUMMY...TURN ON THE MODEM!!!" or

something similar. Combined with redirecting the output to a logging

file, you could trace a history of failures or other actions.

4

Comset....Smart modem utility version 5.2

V. REGISTRATION INFORMATION

--------------------------

You can freely make copies and distribute Comset over BBS systems or

public domain libraries as long as no fee for Comset is charged

(except duplication costs) and as long as Comset.exe is distributed

along with this file Comset.doc. These may be archived together for

use on a bbs system.

Comset is copyrighted by Hank Volpe and is not public domain software.

Shareware version users are granted a limited license for its use. No

warranties for Comset are stated or implied.

Shareware authors continually provide computer users with low priced

software compared to the traditional retail approach. If you register

Comset, you will be able to use all the features ( COM 1-4 support,

auto-log in of COM3 and 4 for Dos applications, advanced Error

reporting and batch file support).

Registration is simple...and price is low. Comset is only $10, which

includes the latest version and free version updates. To register,

send $10 along with your name and address to;

Hank Volpe

108 Broadmoor Dr.

Tonawanda NY 14150

VI. ERROR MESSAGES

THERE IS NO COMx IN YOUR SYSTEM -

Comset was instructed by the user to test a non-existent comm port.

This could happen automatically if you don't have a COM1 in

your system. If this is the case, use the line

COMSET [speed] [port] to select your specific needs.

5

Comset....Smart modem utility version 5.2

MODEM NOT RESPONDING PROPERLY -

The modem under test did not set up even after 3 extensive hardware

and software attempts to do so. Either you have a non-compatible

or non-intelligent modem, or you are using the wrong opening

baud rate. Comset's defaults are 2400 bps and COM1. If you only

have a 1200 or 300 bps modem, use the line

COMSET [speed] [port] and use a lower speed. Valid speeds are

300 bps,  1200 bps , 2400 bps [default], 4800 bps, 9600 bps

and 19200 bps. Any other values default automatically to 2400 bps.

MODEM APPEARS TO BE OFF OR DISCONNECTED -

The hardware handshaking pairs RTS/CTS or DTR/DSR are not

communicating properly. This should NEVER happen on internal modems

unless the CTS signal is not enabled. Internal Modems usually keep

this signal at a logic 1, so if you see this error, check your

internal modem for proper strapping of the CTS and DSR signals.

If you have an external modem,and the power for the modem is on, you

will see this error message if you do not have the CTS signal at a

logic 1 or you have a defective or inadequate Serial Cable. Most

external modems have Dip-switches on the back for setting the CTS

signal. Normally the CTS follows the Serial Ports RTS and the modem

DSR (sometimes called MR) follows the Serial Ports DTR.

In the case of an external modem you will see this error message

everytime the modem has not been turned on. This is telling you

that your serial port hardware is fine, just that your modem is

off.

VII COMSET PROGRAM HISTORY

--------------------------

Version 1 - Simple Assy program fixed at 300 bps. Two versions

COM1 and COM2.

Version 2 - Baud rate raised to 1200 bps.

Version 3 - Baud rate raised to 2400 bps.

Version 4 - COM1 / COM2 support in one program.

Version 5 - Major rewrite, added interrupt support, hardware

and software testing, COM 1-4 support, Multiple

baud rate support, error checking and reporting,

and support for high speed modems to 19200 bps.

Current released version is 5.2...released May 7, 1989

6