..pgno01

..foot63AD-##

..head02L

..head04L

..head03LKeyboard Codes

KEYBOARD CODES

This appendix lists the keyboard codes that are returned by the

computer system when a key or key combination is pressed.

The column headings AH and AL refer to the hi and lo registers within

the AX register.

These are the actual values returned by the BIOS interrupt 16H in

assemlby language.

Key                         Key

Pressed    AH    AL         Pressed    AH    AL

--------- ----- -----       --------- ----- -----

(space)     57    32        C           46    67

!            2    33        D           32    68

"           40    34        E           18    69

#            4    35        F           33    70

$            5    36        G           34    71

%            6    37        H           35    72

&            8    38        I           23    73

'           40    39        J           36    74

(           10    40        K           37    75

)           11    41        L           38    76

*            9    42        M           50    77

+           13    43        N           49    78

,           51    44        O           24    79

-           12    45        P           25    80

.           52    46        Q           16    81

/           53    47        R           19    82

0           11    48        S           31    83

1            2    49        T           20    84

2            3    50        U           22    85

3            4    51        V           47    86

4            5    52        W           17    87

5            6    53        X           48    88

6            7    54        Y           21    89

7            8    55        Z           44    90

8            9    56        [           26    91

9           10    57        \           43    92

:           39    58        ]           27    93

;           39    59        ^            7    94

<           51    60        _           12    95

=           13    61        `           41    96

>           52    62        a           30    97

?           53    63        b           48    98

@            3    64        c           46    99

A           30    65        d           32   100

B           48    66        e           18   101

..page

..head02

..head03

..head04

Key                         Key

Pressed    AH    AL         Pressed    AH    AL

--------- ----- -----       --------- ----- -----

f           33   102        Alt-J       36     0

g           34   103        Alt-K       37     0

h           35   104        Alt-L       38     0

i           23   105        Alt-Z       44     0

j           36   106        Alt-X       45     0

k           37   107        Alt-C       46     0

l           38   108        Alt-V       47     0

m           50   109        Alt-B       48     0

n           49   110        Alt-N       49     0

o           24   111        Alt-M       50     0

p           25   112        F1          59     0

q           16   113        F2          60     0

r           19   114        F3          61     0

s           31   115        F4          62     0

t           20   116        F5          63     0

u           22   117        F6          64     0

v           47   118        F7          65     0

w           17   119        F8          66     0

x           48   120        F9          67     0

y           21   121        F10         68     0

z           44   122        Home        71     0

{           26   123        UArr        72     0

|           43   124        PgUp        73     0

}           27   125        LArr        75     0

~           41   126        RArr        77     0

Ctrl-2       3     0        End         79     0

BackSp      14     8        DArr        80     0

Tab         15     9        PgDn        81     0

Ctrl-Enter  28    10        Ins         82     0

Enter       28    13        Del         83     0

Shft-Tab    15     0        Shft-F1     84     0

Alt-Q       16     0        Shft-F2     85     0

Alt-W       17     0        Shft-F3     86     0

Alt-E       18     0        Shft-F4     87     0

Alt-R       19     0        Shft-F5     88     0

Alt-T       20     0        Shft-F6     89     0

Alt-Y       21     0        Shft-F7     90     0

Alt-U       22     0        Shft-F8     91     0

Alt-I       23     0        Shft-F9     92     0

Alt-O       24     0        Shft-F10    93     0

Alt-P       25     0        Ctrl-F1     94     0

Esc          1    27        Ctrl-F2     95     0

Alt-A       30     0        Ctrl-F3     96     0

Alt-S       31     0        Ctrl-F4     97     0

Alt-D       32     0        Ctrl-F5     98     0

Alt-F       33     0        Ctrl-F6     99     0

Alt-G       34     0        Ctrl-F7    100     0

Alt-H       35     0        Ctrl-F8    101     0

..page

Key                         Key

Pressed    AH    AL         Pressed    AH    AL

--------- ----- -----       --------- ----- -----

Ctrl-F9    102     0        Ctrl-S      31    19

Ctrl-F10   103     0        Ctrl-T      20    20

Alt-F1     104     0        Ctrl-U      22    21

Alt-F2     105     0        Ctrl-V      47    22

Alt-F3     106     0        Ctrl-W      17    23

Alt-F4     107     0        Ctrl-X      45    24

Alt-F5     108     0        Ctrl-Y      21    25

Alt-F6     109     0        Ctrl-Z      44    26

Alt-F7     110     0        Ctrl-[      26    27

Alt-F8     111     0        Ctrl-\      43    28

Alt-F9     112     0        Ctrl-]      27    29

Alt-F10    113     0        Ctrl-6       7    30

Ctrl-LArr  115     0        Ctrl--      12    31

Ctrl-RArr  116     0        Ctrl-BS     14   127

Ctrl-End   117     0        Ctrl-Break   0     0

Ctrl-PgDn  118     0

Ctrl-Home  119     0

Alt-1      120     0

Alt-2      121     0

Alt-3      122     0

Alt-4      123     0

Alt-5      124     0

Alt-6      125     0

Alt-7      126     0

Alt-8      127     0

Alt-9      128     0

Alt-0      129     0

Alt--      130     0

Alt-=      131     0

Ctrl-PgUp  132     0

Ctrl-A      30     1

Ctrl-B      48     2

Ctrl-C      46     3

Ctrl-D      32     4

Ctrl-E      18     5

Ctrl-F      33     6

Ctrl-G      34     7

Ctrl-H      35     8

Ctrl-I      23     9

Ctrl-J      36    10

Ctrl-K      37    11

Ctrl-L      38    12

Ctrl-M      50    13

Ctrl-N      49    14

Ctrl-O      24    15

Ctrl-P      25    16

Ctrl-Q      16    17

Ctrl-R      19    18

..page