#.-

GRAFX PROGRAMMER NOTES

(C) Copyright 1988, 1989 by Jim Farrell      All Rights Reserved.

GRAFX is a FORTRAN extension library of over 50 routines which gives

the FORTRAN user access to the PC's graphic video display and a

selection of DOS and BIOS services. GRAFX can save you time and money

in solving problems and communicating results.  Several complete

examples are included.

The following notes provide information on how to call the GRAFX

library subroutines and functions. Most usage and conventions can be

determined from studying the example programs included on the the

distribution diskette.

-----------------------------------------------------------------------

TOP LEVEL ROUTINE NOTES

-----------------------------------------------------------------------

The following routine uses the lower level routines to produce a x-y

plot display.

-----------------------------------------------------------------------

call xyplt(x,y,n,iv,ip)		!  produce x-y plots

real x(*),y(*)

A quick look x-y plot routine (scatter plot). Up to three plots may be

displayed on the screen by calling with IV=1..3. If 1 > IV > 3 then a

full screen plot is displayed. IP determines the display type IP=0,

plot a point; IP=1, plot a histogram like line from y=minimum to y(i);

IP=2, plot a histogram like line from y=0 to y(i); IP=3, plot from

(0,0) to (x,y); IP=4, plot a connected line.  A WARNING is issued if

the data range is inappropriate for the selected IP.

-----------------------------------------------------------------------

The following routine uses the lower level routines to display equally

spaced samples (e.g. time series) display.

-----------------------------------------------------------------------

call tplot(xmn,xmx,y,n,iv,ip)		!  produce y=f(t) plots

real y(*)

A quick look y=f(t) plot routine for equally spaced data.  Up to three

plots may be displayed on the screen by calling with IV=1..3; Any other

value of IV yields a full screen plot. IP determines the display type

IP=0, plot a point; IP=1, plot a histogram like line from y=minimum to

y(i); IP=2, plot a histogram like line from y=0 to y(i); IP=3, plot

from (0,0) to (x,y); IP=4, plot a connected line. A WARNING is issued

if the data range is inappropriate for the selected IP. See example

program for a demonstration of line drawing types.

-----------------------------------------------------------------------

The following routine uses the lower level routines to display

intrinsic or external functions.

-----------------------------------------------------------------------

call fcnplt(xmn,xmx,fcn,n,iv,ip)		!  produce y=fcn(x)

external fcn

or

intrinsic fcn

A quick look y=fcn(x) plot routine for n points over a range of x.  Up

to three plots may be displayed on the screen by calling with IV=1..3;

Any other value of IV yields a full screen plot. IP determines the

display type IP=0, plot a point; IP=1, plot a histogram like line from

y=minimum to y(i); IP=2, plot a histogram like line from y=0 to y(i);

IP=3, plot from (0,0) to (x,y); IP=4, plot a connected line. A WARNING

is issued if the data range is inappropriate for the selected IP. See

example program for a demonstration of line drawing types.

-----------------------------------------------------------------------

The following routine uses the lower level routines to produce multiple

plot display.

-----------------------------------------------------------------------

subroutine mulplt0(xmn,ymn,xmx,ymx)

...initialize limits for multiple x-y plots

A WARNING is issued if the data range is inappropriate for the selected

IP.

subroutine mulplt(x,y,n,ip)		!  produce multiple x-y plots

real x(*),y(*)

A multiple x-y plot routine. IP determines the display type IP=0, plot

a point; IP=1, plot a histogram like line from y=minimum to y(i); IP=2,

plot a histogram like line from y=0 to y(i); IP=3, plot from (0,0) to

(x,y); IP=4, plot a connected line.

-----------------------------------------------------------------------

The following routine uses the lower level routines to support

histogram display.

-----------------------------------------------------------------------

call hist(x,n)				!  plot histogram

real x(*)

...routine to plot a histogram of the distribution of x. The n

occurrences of x are counted in 100 equally spaced bins. The results are

then displayed graphically.

-----------------------------------------------------------------------

MID LEVEL ROUTINE NOTES

-----------------------------------------------------------------------

GRAFX uses two include files internally:

"physcon.icl" - physical constants

"grafcom.icl" - graphic common

These are included on the distribution diskette.

Physcon.icl contains the constant PI, conversions from radians to

degrees, and earth physical constants.

Grafcom.icl contains the named COMMON /GRAFCOM/ which contains the

information to map between display space and problem space.

-----------------------------------------------------------------------

Unless otherwise noted, GRAFX assumes that the default implicit FORTRAN

conventions are:

integer*4 and real*4.

-----------------------------------------------------------------------

A BLOCK DATA module GRAFINIT initializes the problem space window to

0.0 to 1.0 in both x and y. It initializes the display space view to 0

to 639 in the x direction and 0 to 199 in the y direction. Invocation

of either WINDOW or VIEW force the loading of GRAFINIT from the

library. If you choose to use the default problem and display space

parameters without invoking either WINDOW or VIEW, the BLOCK DATA

"GRAFINIT" must be declared "external" in your program to force loading

it from the library.

-----------------------------------------------------------------------

call window(xmn,ymn,xmx,ymx)		!  set problem space window

Set problem space window (left, bottom, right, top) in named COMMON

/GRAFCOM/. Origin is lower left. Next call view with display space

view. The defaults are 0.0 and 1.0 for bottom, left and top, right

respectfully.

-----------------------------------------------------------------------

call view(xmn,ymn,xmx,ymx)		!  set display space view

integer xmn,ymn,xmx,ymx

Set display space window view (left, bottom, right, top) in named

COMMON /GRAFCOM/.  The default is 0 to 639 for x and 0 to 199 for y.

Arguments are screen coordinates (pixels).  Note that the GRAFX

convention is for the origin in the lower left corner of the screen.

ymn is the lower coordinate and ymx is the upper.

-----------------------------------------------------------------------

-----------------------------------------------------------------------

The following routines are the mid-level problem space point, line, and

shape generators.  To compensate for the CGA non-linearities, shapes

are generated in problem space. The (optional) use of a 8087 numeric

coprocessor makes the conversion to display space efficient (about 20X

faster than with no 8087). Further line drawing speed is achieved

through the use of assembly language routines to perform vector to

raster conversion and direct manipulation display memory instead of

using BIOS & DOS INT services. Limit checking of points and clipping of

lines is not done. It is assumed that the user has defined appropriate

problem space limits. A clipping routine is provided in the library for

applications requiring clipping.

-----------------------------------------------------------------------

call point(xp,yp)		! light a pixel in problem space

Converts problem coordinates to display coordinates and calls pset to

light the pixel.

-----------------------------------------------------------------------

call color('BLACK')

Determine the color of lines. The default is 'WHITE' or on. In CGA

hi-res mode, there are only two possibilities. 'BLACK' turns off pixels

that are lit and can be used to erase lines.

-----------------------------------------------------------------------

call line(xp1,yp1,xp2,yp2)		!  draw a problem space line (P1 ->

P2)

Converts problem coordinates to display coordinates and calls pline to

draw the line.

-----------------------------------------------------------------------

call plot(x,y,ip)			!  move to problem space point P

Converts the problem coordinate to a display coordinate and moves from

the current position to x,y. If ip = 2 a line is drawn. This routine is

included to provide compatibility with CALCOMP style calls. Tests have

shown no speed advantage over subroutine line.

-----------------------------------------------------------------------

call box(x1,y1,x2,y2)		!  draw a problem space box

Call line to draw the box. P1 is lower left. P2 is upper right.

-----------------------------------------------------------------------

call circle(x,y,r)		!  draw a problem space circle

Uses recursive trigonometric techniques to draw a 64 line segment

circle in problem space. Only four multiplies and six additions are

inside the computational loop. X and y are the center point. R is the

radius.

-----------------------------------------------------------------------

call arc(x,y,r,ab,ae)		!  draw a problem space arc

Uses a technique similar to the circle routine to draw an arc.

Parameter ab is the arc begin angle and ae is arc end angle (in

radians). Angles are measured counter-clockwise from the positive x

axis. X and y are the center point. R is the radius.

-----------------------------------------------------------------------

call ellipse(x,y,a,b,theta)		!  draw an ellipse in problem space

Draw a a 64 line segment ellipse centered at x,y: the semi-major axis

is aligned delta radians CCW from the positive x axis.

a - semi-major axis;  b - semi-minor axis

Uses a formulation which eliminates the use of trigonometric

trancendental functions inside the loop.

-----------------------------------------------------------------------

call cursor(irow,icol)		!  position the cursor

Set A/N cursor position at (row,column). In FORTRAN, a write always

starts in column one. Cursor can be used to set the row.

-----------------------------------------------------------------------

call putstr(x,y,string)		!  output a string at (x,y) on screen

integer x,y

character string*(*)

-----------------------------------------------------------------------

call pause('message')		!  put pause message at bottom of the screen

call error('message')		!  put error message at bottom of the screen

call warn('message')		!  put warning message at bottom of the screen

call info('message')		!  put advisory message at bottom of the screen

---------------------------------------------------------------------

LEVEL THREE NOTES

---------------------------------------------------------------------

GRAFX level three contains FORTRAN 77 callable primitive graphics

functions in assembler for CGA HI-RES MONO mode. The routines ASET,

GSET, ACLS, GCLS are called directly from FORTRAN applications. The

routines PSET, PLINE, and PCURSO can be call directly, but are best

used by calling the companion FORTRAN subroutines POINT, LINE, and

CURSOR.

---------------------------------------------------------------------

call gset					!  set graphic; mode = 6

call gcls					!  clear graphic display

call aset					!  set alpha; mode = 2

call acls					!  clear alpha display

---------------------------------------------------------------------

integer*2 row,column

call pset(row,column)		!  turn on a pixel at screen coordinate

Directly accesses the PCs display memory and uses a variety of rather

obscure techniques (e.g. synthetic multiplication) to compute the

address of the required byte access, form the needed bit mask, and turn

on the pixel as fast and efficiently as possible.

---------------------------------------------------------------------

integer*2 row,column

call pclr(row,column)		!  turn off a pixel at screen coordinate

Turn off the pixel.

---------------------------------------------------------------------

integer*2 ptst,row,column,k

k=ptst(row,column)		!  test a pixel at screen coordinate

Test the PCs display memory to see if a pixel is currently on. A none

zero value indicates that the pixel is on.

---------------------------------------------------------------------

integer*2 y1,x1,y2,x2		!  coordinate p1 to p2

call pline(y1,x1,y2,x2)		!  draw a line from screen coordinate p1 to p2

Implements Jack Bresenham's Fast Line Algorithm, which uses only

integer addition and subtraction operations, in 8086 assembler. This

results in the fastest possible line drawing speeds. Line color is

selected by calling COLOR or PCOLOR. The default color is white or

"on".

---------------------------------------------------------------------

integer*2 color

call pcolor(color)

Sets line color 0 = black; otherwise white. This affects all level two

routines except point and can be used to erase lines and shapes.

---------------------------------------------------------------------

FORTRAN 77 callable 16 bit logical and shift functions

--------------------------------------------------------------------

logical and function			k=land(i,j)

logical or function 			k=lor(i,j)

logical exclusive or function		k=leor(i,j)

logical not function 			j=lnot(i)

logical right shift function		j=lshr(i,nbits)

logical left shift function		j=lshl(i,nbits)

Declare these functions integer*2 in calling program. I, J, K, and

NBITS must also be integer*2

integer*2 land,lor,leor,lnot,lshr,lshl

integer*2 i,j,k,nbits

---------------------------------------------------------------------

MISCELLANEOUS ROUTINE NOTES

---------------------------------------------------------------------

character ch

integer*2 ich

ich=ichar(ch)

call putch(ich)			!  put a character at current cursor position

---------------------------------------------------------------------

integer*2 row,column

call pcurso(row,column)		!  position the cursor at row, column

---------------------------------------------------------------------

call beep					!  beep the speaker

call reply				!  wait until a key is pressed

call lclr					!  clear to end of current line

---------------------------------------------------------------------

integer*2 ich

call ckey(ich)				!  check to see if a key has been pressed

If a key has been pressed and a character is in the buffer, return with

ich = ASCII code, otherwise ich = 0.

---------------------------------------------------------------------

call fdate(id,im,iy)		!  day month year

integer id,im,iy

call fsecs(seconds)			!  return time in seconds & fraction

call delay(delt)			!  delay for "delt" seconds and return

call ftime(hh,mm,ss,fss)		!  time in hours, minutes, seconds

integer hh,mm,ss,fss		!  and hundreths of seconds

integer*2 dd,mm,yy

call adate(dd,mm,yy)		!  assembly routine to get date

integer counts

call atime(counts)			!  assembly routine to get time

COUNTS can be used to seed the supplied random number generator RAND to

obtain a different list of pseudo-random numbers each time the

application is used. Time of day in seconds is equal to counts/18.2.

---------------------------------------------------------------------

subroutine clip(x1,y1,x2,y2,nc)

Clip a vector to problem space. The vector is from (x1,y1) to (x2,y2).

variable "nc" returns -1 if unplottable or the number of clips

performed (0-4). CLIP is supplied for optional use by the programmer

in applications where vectors may extend beyond the problem space window.

---------------------------------------------------------------------

call peek(iaddr,ival)			!  get a two byte quantity fm iaddr

integer*4 iaddr				!  physical address

integer*2 ival

---------------------------------------------------------------------

call poke(iaddr, ival)			!  put a two byte quantity at iaddr

integer*4 iaddr

integer*2 ival

------------------------------------------------------------

function rnd(irk)			!  pseudo-random number generator

function rand(irk)			!  pseudo-random number generator

Prime modulus linear congruential generator (Lehmer generator) of the

form f(z)=a*z mod m; where a ~=sqrt(m).  This version uses a=7**5,

m=2**31-1. Seed with an integer variable "irk" containing 1.  Meets the

minimal Lehmer standard!  See Park & Miller; "Random Number Generators:

Good Ones are Hard to Find"; Communications of the ACM; V31 N10 (1988).

WARNING: The function name RND() is used by Lahey F77L FORTRAN as a

non-standard intrinsic. When using RND with F77L declare RND external

to avoid using the non-standard intrinsic RND.

external rnd

The function RAND is included to provide the exact same service without

such a conflict.  If a different sequence of random numbers is desired

each time you run a program use the atime routine to obtain a seed

based on the the time of day counter.

-----------------------------------------------------------------------

Portability

-----------------------------------------------------------------------

GRAFX was designed to be portable in three dimensions: one, portable to

other FORTRAN compilers; two, portable to other operating systems; and

three, portable to other computers and graphics hardware.

GRAFX is written in US ANSI FORTRAN 77. The only non-standard feature

used is trailing comments, which will be a FORTRAN 8X standard. No

vendor specific non-standard features are used. Any FORTRAN extensions

required are implemented as primitive subroutines or functions (such as

bit manipulation) as part of the GRAFX library.

Only a minimal set of DOS/BIOS services are used. Access to these

system services is through calls to primitive assembly language

routines which isolate specific functions.

All GRAFX graphic operations are reduced to a few primitive functions,

which are common to all graphic displays.Rather than use a lot of

display hardware dependent shape generation algorithms, all shapes are

reduced to vector operations. The graphic display hardware specific

features are isolated in one small (about 1 KB object code) set of 8086

assembly language routines.

Because portability is "designed in" applications which use GRAFX can

be moved to newer PC technology, mini/mainframe, or even to a super

computer environment easily. In fact, the design of GRAFX reflects

years of experience in designing, building and maintaining applications

which must run in IBM 30XX, DEC VAX, and INTEL 8086 environments

inclusively.

---------------------------------------------------------------------

Compatibility

---------------------------------------------------------------------

The current distribution diskette contains libraries for Lahey FORTRAN

F77L V3.0 and Lahey Personal FORTRAN 77 (LP77).  The libraries were

created using the Lahey library manager (LM), supplied with F77L and

LP77.  You can use the Lahey Library Manager (LM) add to the GRAFX

library or add the GRAFX routines to your current libraries.

The primitives are compatible with MS DOS Version 3.10 and later and

CGA video mode 6.

Call for information on other versions and/or source code availability.

---------------------------------------------------------------------

Suggestions for additions and improvements are greatly appreciated.

Enhancements in final testing include a scalable vector (soft)

character set which can be positioned to the nearest pixel and

additional top level routines. Color support for 640 X 480 VGA mode is

under preparation.  Our priorities for introduction are a direct

reflection of your requests.

Additional library routines are available which support geographic

displays, date conversions, numerical analysis, numerical simulation,

orbital mechanics, radar systems analysis, and other scientific and

engineering applications.  For further details and custom requirements,

contact:

Jim Farrell

Ridge Technology

1867 Park Road, N.W.

Washington, D.C. 20010

(202) 232-1441

#.+#