November 1, 1986

The  program MORSE.EXE is  quite  easy  to  run.   It  has

several menu's that are  self  explanatory.   The program is

intended to help someone  learn the morse code and hopefully

get  into  the  hobby  of  amateur radio.  The program  will

normally just send tones to the speaker, but it also has the

option to key a  transmitter with a simple interface that is

described in the Amateur  Radio  Handbook.  To  do this, you

have the menu option to select keying.  In this mode it keys

the DTR line  for  an  output  that can be used for external

keying.  At startup, the default is  with  the  keying off.

There  is  a  256  char  type ahead buffer to enable you  to

outrun the speed  of  the code.  You also have the option of

making 10 buffers that can be called with the function keys,

each holding a maximum of 254 characters.  These buffers can

be injected anywhere while using the keyboard to type code.

You also have the option to read an  ascii file and send its

contents.   The  code  has  separate  timing for spaces  and

chars.  At slow speeds  the spaces get proportionally larger

than  the  chars  to   keep  the  characters  sounding  like

characters  while  giving  you  more  time to think  between

them.  There  also  is  a  3000 char memo that can remain in

memory,  this  was  added  to have a place to keep notes and

while copying from a radio.

The program is written  in  assembly,  with mostly dos 1.x

functions except that the  ascii  reading  uses  2.XX  +  to

enable path descriptions.

I had originally intended  to  try  sell  this  program to

Hams, on a disk for $20 but decided  against  it  because of

the cost  of marketing it.  Instead, I'm making it available

for trial use.  If you like it and  decide  to keep it, I am

asking for a donation of $5. Please share  the  program with

anyone interested or to any BBS that does not have it.

If you are running  a machine that is faster than 4.77 mhz

like an AT, you can slow the program down by plugging a flag

in with debug in the following manner

debug morse.com e103 ff (CR) w (CR)

This changes the flag at cs:103 to 0FFh from 00h. To speed

it back up just enter the 00 back into it.

I've included the source code.  Though  it is without many

comments, it may be of some value.

Note: I ACCEPT NO  LIABILITY  FOR THE USE OF THIS PROGRAM.

As released it works without known problems.   Since  I have

no control over what happens to it after its release, USE AT

- 1 -

YOUR OWN RISK.

Hope you enjoy the program and maybe Ham Radio

Gregg Anderson WB8BIL 247 Hill St. Ishpeming, MI 49849

- 2 -