As a service to the satellite user community, the following description of the

NORAD two-line orbital element set format is uploaded to sci.space and

rec.radio.amateur.misc on a monthly basis.  The most current orbital elements

from the NORAD two-line element sets are carried on the Celestial BBS, (513)

427-0674, and are updated several times weekly.  Documentation and tracking

software are also available on this system.  The Celestial BBS may be accessed

24 hours/day at 300, 1200, 2400, 4800, or 9600 bps using 8 data bits, 1 stop

bit, no parity.  In addition, element sets (updated weekly) and some

documentation and software are also available via anonymous ftp from

blackbird.afit.af.mil in the directory pub/space.

==============================================================================

Data for each satellite consists of three lines in the following format:

AAAAAAAAAAA

1 NNNNNU NNNNNAAA NNNNN.NNNNNNNN +.NNNNNNNN +NNNNN-N +NNNNN-N N NNNNN

2 NNNNN NNN.NNNN NNN.NNNN NNNNNNN NNN.NNNN NNN.NNNN NN.NNNNNNNNNNNNNN

Line 0 is a eleven-character name.

Lines 1 and 2 are the standard Two-Line Orbital Element Set Format identical

to that used by NORAD and NASA.  The format description is:

Line 1

Column     Description

01-01     Line Number of Element Data

03-07     Satellite Number

10-11     International Designator (Last two digits of launch year)

12-14     International Designator (Launch number of the year)

15-17     International Designator (Piece of launch)

19-20     Epoch Year (Last two digits of year)

21-32     Epoch (Julian Day and fractional portion of the day)

34-43     First Time Derivative of the Mean Motion

or Ballistic Coefficient (Depending on ephemeris type)

45-52     Second Time Derivative of Mean Motion (decimal point assumed;

blank if N/A)

54-61     BSTAR drag term if GP4 general perturbation theory was used.

Otherwise, radiation pressure coefficient.  (Decimal point assumed)

63-63     Ephemeris type

65-68     Element number

69-69     Check Sum (Modulo 10)

(Letters, blanks, periods = 0; minus sign = 1)

Line 2

Column     Description

01-01     Line Number of Element Data

03-07     Satellite Number

09-16     Inclination [Degrees]

18-25     Right Ascension of the Ascending Node [Degrees]

27-33     Eccentricity (decimal point assumed)

35-42     Argument of Perigee [Degrees]

44-51     Mean Anomaly [Degrees]

53-63     Mean Motion [Revs per day]

64-68     Revolution number at epoch [Revs]

69-69     Check Sum (Modulo 10)

All other columns are blank or fixed.

Example:

NOAA 6

1 11416U          86 50.28438588 0.00000140           67960-4 0  5293

2 11416  98.5105  69.3305 0012788  63.2828 296.9658 14.24899292346978