Parallel port & printer support functions

#include <prt.h>

int prt_busy( port );

int port;

int prt_err( port );

int port;

int prt_putc( port, c );

int port; char c;

int prt_init( port );

int port;

prt_screen( );

int prt_status( port );

int port;

Note: port refers to a parellel port and can be a 0, 1, or 2.

o Descriptions

prt_busy checks to see if the printer is busy and returns a 1 if busy, 0

if it is not.

prt_err checks the printer status to see if the device is off-line, timed

out, paper out, or an other i/o error has occurred.  Returns a 1 if an error

has occurred, 0 if no error has occurred.

prt_putc prints a character to the given parallel port.  Returns the printer

status as described below.

prt_init initializes the parallel port and returns the device status described

below.

prt_screen prints the screen as if the PrtSc key was pressed.

prt_status returns the device status code for the given port.

Device status codes

bit	 Mask	Meaning

0  0x0001	Time-out has occurred

1  0x0002	not used

2  0x0004	not used

3  0x0008	I/O error

4  0x0010	Device on line

5  0x0020	Out of paper

6  0x0040	Character acknowledge (received)

7  0x0080	Device busy or in an error state