THE LAN TUTORIAL SERIES   -- by Aaron Brenner

PART 1: Buying a LAN

A Definition.......................1

Why Buy a LAN?.....................1

LAN Components.....................2

Application Specific...............3

PART 2: PROTOCOLS

Definition.........................5

Protocols, Really..................5

The OSI Model......................6

Popular Physical Protocols.........7

Data Link Protocols................9

Transport Protocols...............10

TCP/IP............................10

Many More.........................11

PART 3: (PROTOCOLS: continued)

Data Link Protocols...............13

Transport Protocols...............14

Many More.........................15

PART 4: LAN Access Methods

Definition........................17

Ethernet..........................17

Arcnet............................18

Token Ring........................19

PART 5: LAN Interface Cards

Definition........................20

Preparing for Transmission........21

TRANSMISSION......................23

Encoding/decoding.................23

Cable access. ....................24

Handshaking. .....................24

Transmission/reception............24

PICK A CARD...................... 24

PART 1: Buying a LAN

This short article kicks off LAN Magazine's new series of

"clip-and-save" tutorials about LANs. Each month we will print an

easy-to-read tutorial -- aimed at users new to networking --

covering one aspect of LAN purchase, installation and management.

This first tutorial is a very basic introduction to the issues

involved in buying a LAN. Along the way is an overview of the

components of a LAN and a list of the next 12 topics to be

covered.

A year from now, if you clip carefully, you should have a short,

easy-to- understand introductory pamphlet about the principles of

local area networking.

A Definition

A LAN is a data communications network spanning a limited

geographical area, a few miles at most. It allows users to share

information and computer resources, including mass data storage,

backup facilities, software, printers, plotters and processors.

Typically, a LAN is made up of network interface cards (circuit

boards) that fit inside the connected computers, cable to connect

these computers, protocol software to move data from computer to

computer, user interface software to connect user and network,

and operating system software to actually service users' needs

for things like files and printers.

Why Buy a LAN?

LANs require a certain mind set, something different from

traditional MIS (Management of Information Services) thinking.

Once a LAN is installed, things like initiative, democracy,

participation, communication and independence take over.

Hierarchy, dependence, regulation and isolation are thrown out

the window.

If you have the right mind set, the four best reasons to buy a

LAN are:

* Communication. A LAN connects the people in your company. Once

connected, every possible form of discourse is possible, from

electronic "yellow sticky things" to formal legal briefs. People

like to communicate.

*  Democracy. A LAN distributes your company's computer resources

to everyone connected. Once the LAN is installed, everyone from

mail clerk to CEO will want, and should have, access.

*  Productivity. A LAN's ability to share computer resources and

information easily helps people do their jobs quickly,efficiently

and with less hassle. The LAN will quickly become the heart of your

business. It's the heart of ours.

Page 1

*  Savings. A LAN saves money by allowing users to share expensive

computer resources -- printers, plotters, hard disks, WORM drives,

CPUs, software, etc.

If you don't have the right mind set, the four best reasons not

to buy a LAN are:

*  Communication. Connecting all the people in your company might

let them talk to each other. Who knows, they might plot your

overthrow.

*  Democracy. Distributing resources will give everyone in the

company a measure of power. Who knows, they might not do what you

tell them.

*  Productivity. Doing the job in new and better ways might lead to

the elimination of dull, tedious work. Who knows, it might mean

the elimination of your job.

*  Savings. Sharing expensive computer resources saves money. Who

knows, you might have to save money all the time.

LAN Components

Buying and installing a LAN is not simple. There are many things

to think about. Here are 12 that cover the basics of LAN purchase

and installation.

1. The OSI Model , which stands for the Open Systems Interconnection

model of the International Standards Organization, is a useful

categorization of the different parts of a LAN. It is an overview of

how a network works.

2. The Access Method is the way the network arbitrates which device

may use the cable and for how long. It is necessary since devices

can't talk at the same time. Different access methods provide

different network performance and reliability.

3. The Interface Card is the device that connects the computer to

the cable. These vary by type, size, speed and much more.

Performance is a key issue.

4. The Cabling is the physical connection between networked devices.

Fiber optic, coaxial and twisted-pair are the main choices. Each has

advantages and drawbacks.

5. LAN Protocols are software that run in the computer and on the

network interface card. They provide the means for shipping data

between devices. Certain sets of protocols are good for certain

applications. Which you choose depends upon what you use your

network for.

Page 2

6. The LAN Operating System is the software that resides in the

computer. It provides the interface between the user or application

and the network. The key issues here are performance, compatibility

and ease of use.

7. The File Server stores and distributes program and data files to

be shared by users on the network. It is a hardware/software

combination heavily dependent on the LAN operating system and the

type of work you are doing.

8. Network Printing allows many users to share one or more printing

devices. Some LAN operating systems do it better than others.

Sometimes you'll need special network print utilities.

9. Tape backup, done regularly, maintains data integrity on a LAN

by recording data on tape instead of disk. Key issues include

capacity, speed, compatibility and ease of use.

10. LAN Security covers the methods used to protect data from

corruption by unknowing users, accidents and intruders. These include

physical security, encryption and passwords. But the type of security

you use depends mostly on the type of work you are doing on your

network.

11. Bridges and gateways connect networks. Each uses different methods

with different results. Bridges connect networks at a lower level than

gateways, making them more versatile. On the other hand, gateways

connect networks that bridges can't. Performance and compatiblity are

the key issues.

12. LAN Management is the name given to the best job in the world:

taking care of the network. Different LANs provide different levels of

management to make the job easier. Your level of skill and confidence

will be crucial here.

Application Specific

The type of LAN you buy depends primarily on the work it will do.

Before evaluating different vendor options, assess your company's

computer needs and resources, present and future. As much as possible,

conserve your present computing power, even if you plan to upgrade.

Will the network be used mostly to share peripherals like printers

and hard disks? If so, access methods and performance are less

important than reliability and ease of use. Will the network be used

mostly for large database access? If so, performance is paramount.

Will the network be used mostly for communications and electronic

mail? If so, wide-range standards compatibility may be the most

important issue in your decision.

Usually, you want the network to do everything: start out peripheral

sharing, add databases then connect to mainframes. Thus, growth

potential and standards are very important to your decision, since

you're laying a foundation upon which you will build.

Page 3

Other overall considerations include: the education of users

(beginners and experts); the types of computers you are connecting

(PCs, minicomputers and/or mainframes); and the amount of money you

have (lots or a little.)

Unfortunately, no LAN does everything. Vendors make compromises,

sacrificing ease of use for performance, performance for compatibility

or vice versa. Since this is the case, get to know exactly what you

want before you buy.

Page 4

PART 2: PROTOCOLS

Definition

The LAN Magazine "Glossary of LAN Terms" defines a protocol this

way: A set of rules for communicating between computers.  Protocols

govern format, timing, sequencing and error control.  Without these

rules, the computer will not make sense of the stream of incoming

bits.

But there is more. Communicating data from computer to computer

takes many steps. For example, suppose you are sending a file from one

computer to another. The file has to be broken into pieces. The pieces

have to be grouped in certain fashion. Information must be added to

tell the receiver where each group belongs in relation to others.

Timing information must be added. Error correcting information must be

added, and so on.

Because of this complexity, computer communication is usually broken

down into steps. Each step has its own rules of operation, its own

protocol. These steps must be executed in a certain order, usually

from the top down on transmission and from the bottom up on reception.

Because of this hierarchical arrangement, the term protocol stack is

used to describe the different steps of computer communication. A

protocol stack is simply a set of rules for communication, only it can

be broken down into sets of rules for each step in the sequence.

Protocols, Really

What is a protocol, really? It is software that resides either in

a computer's memory or in the memory of a transmission device like a

network interface card. When data is ready for transmission, this

software is executed. It prepares data for transmission and sets it in

motion. At the receiving end, it takes the data off the wire and

prepares it for the computer, taking off all the information added by

the transmitting end. So, protocols are just software that performs

data transmission.

But there is more. Confusion is caused by the fact that there are

many protocols, many different ways of getting data from one place to

another. Novell does it one way. 3Com does it another.  DEC does it a

third way. And since the transmitter and the receiver have to "speak"

the same protocol, these three can't talk directly to each other.

That's where the term protocol standard and the OSI Model fit in.

A protocol standard is a set of rules for computer communication

that has been widely agreed upon and implemented by many vendors,

users and standards bodies. Ideally, a protocol standard should,

when implemented, allow people to talk to each other, even if they are

using equipment from different vendors.

Of course, you don't have to have a "standard" protocol to communicate.

You can make up your own. The only problem is that you are limited to

talking to yourself.

Page 5

Let's look at some of the protocol standards that exist and see if we

can't get a feel for how protocols work. As you will see, there are

many standards -- none of which can be called universal.

The OSI Model

The OSI Model is the best place to start because it is a full protocol

stack. It is a set of protocols that attempt to define and standardize

the entire process of data communications (some protocol standards

only define part of the process). The OSI Model -- which stands for

the Open Systems Interconnection Model of the International Standards

Organization (ISO) -- has the support of most major computer and

network vendors, along with many large customers and the U.S.

government.

The OSI Model is really nothing more than a concept, describing how

data communications should take place. It divides the process into

seven layers. Into these layers fit protocol standards developed by

the ISO and by other standards bodies. At each layer, there are

numerous protocols. That is, the OSI is not a single definition of

how data communications actually takes place in the real world. It

just says, "This is the way things should be divided and these are

the protocols that you can use at each layer."  As long as a network

vendor chooses one of the protocols at each layer, the network should

work with other vendors' offerings.

Nobody really believes the hype that the OSI Model will lead to

complete, transparent intercommunication between all computers.  We

are just hoping it is a step in the right direction.  Each successive

layer of the OSI Model works with the one below it. Remember, protocol

stacks are not democratic; they are rigidly hierarchical. Each layer

of the OSI Model is modular.  That is, you may (theoretically)

substitute one protocol for another at the same layer without affecting

the operation of layers above or below. For example, you should be

able to use a Token Ring board or an Ethernet board and still use all

the other pieces of your network, including network operating system,

transport protocols, internetwork protocols, applications interfaces,

etc. Of course, vendors must create these products to the OSI Model

specifications for this to work.

The OSI Model's modularity should become clear as we describe the

major protocols that conform to it. First a look at what each layer is

supposed to do.

1. Physical Layer. The first, or Physical layer, of the OSI Model

conveys the bits that move along the cable. It is responsible for

making sure that the raw bits get from one place to another, no matter

what shape they are in. It deals with the mechanical and electrical

characteristics of the cable.

Page 6

2. Data Link Layer. The second, or Data Link, layer of the OSI Model

is responsible for getting data packaged and onto the network cable.

It manages the physical transfer, providing the blocks of data, their

synchronization, error control and flow control. The Data Link layer

is often divided into two parts -- Logical Link Control (LLC) and

Medium Access Control (MAC) -- depending on the implementation.

3. Network Layer. The third, or Network, layer of the OSI Model

establishes, maintains and terminates connections. It is responsible

for translating logical addresses, or names, into physical addresses.

4. Transport Layer. The fourth, or Tranport, layer of the OSI Model

ensures data is sent successfully between the two computers. If data

is sent incorrectly, this layer has the responsibility to ask for

retransmission.

5. Session Layer. The fifth, or Session, layer of the OSI Model

decides when to turn communication on and off between two computers.

It coordinates the interaction between them. Unlike the network layer,

it is dealing with the programs running in each machine to establish

conversations between them.

6. Presentation Layer. The sixth, or Presentation, layer of the OSI

Model does code conversion and data reformatting. It is the translator

of the network, making sure the computer is talking in the right

language for the network.

7. Application Layer. The seventh and final, or Application, layer of

the OSI Model is the interface between the software running in the

computer and the network. It supplies functions to the software in

the computer, like electronic mail or file transfer.

Unfortunately, protocols in the real world do not conform precisely

to these neat definitions. Some network products combine layers.

Others leave out layers. Still others break apart layers. But no matter

what, all working network products achieve the same result, getting

data from here to there. The question is, do they do it in a way

compatible with the rest of the world's networks?  More important, do

they care?

Popular Physical Protocols

Hopefully, all of this will become clearer if we look at some real

protocols and compare them to the OSI Model. The best known physical

layer standards of the OSI Model (there are a few), are those from the

IEEE, the Institute of Electrical and Electronic Engineers. That is,

the ISO adopted some of the IEEE's physical network standards as part

of its OSI Model. These are IEEE 802.3, or Ethernet, IEEE 802.4, or

token- passing bus and IEEE 802.5, or Token Ring.

Page 7

These three standards define the physical characteristics of the

network and how to get raw data from one place to another. Each is a

Layer 1 standard. They also define how people can use the network at

the same time without bumping into each other.  Technically, this last

part is a job for the Data Link layer, Layer 2.  We will deal with

this below. For now, let's see just what these standards mean.

IEEE 802.3 defines a physical network that has a bus (straight line)

layout. Data is broadcast throughout the network in no particular

physical direction. All machines receive every broadcast, but only

those meant to receive the data respond with an acknowledgement.

Network access is determined by a protocol called Carrier Sense

Multiple Access With Collision Detection, or CSMA/CD. It lets everyone

send whenever they want. If they bump into each other, they back off,

wait, and send again until they get through. Thus, the more users, the

more crowded and slower the network -- like the freeway. (More on

network access next month).

IEEE 802.4 defines a physical network that has a bus layout. It is

also a broadcast network. All machines receive all data but do not

respond unless data is addressed to them.

Network access is determined by a token that moves around the network

in a logical fashion. It is broadcast to every machine but only the

machine that is next for the token gets it. Once a machine has the

token, and not before or after, it may transmit data. The MAP/TOP

(Manufacturing Automation Protocol/Technical Office Protocol) standard

uses this protocol.

IEEE 802.5 defines a physical network that has a ring layout. Data

moves around the ring from station to station. Each station regenerates

the signal from the previous station. In this way it is not a broadcast

network.

The network access protocol is token-passing. The difference is that

the token moves about in a ring, rather than over a bus. IBM, Texas

Instruments and Ungermann-Bass are the only vendors of the chips needed

to make Token Ring network interface cards.  Nevertheless, it is fast

becoming one of the most popular network hardware options.

There are other Physical and Data Link layer standards, some that

conform to the OSI Model and others that don't. The most famous that

does not is Arcnet. It uses a token-passing bus access method, but not

the same one as IEEE 802.4. A new physical standard called Fiber

Distributed Data Interface (FDDI) is a 100M-bits-per-second physical

protocol using token ring over fiber optic cable. It will probably be

OSI-compatible.

Page 8

Data Link Protocols

As we said, the IEEE protocol standards are not confined to the

Physical layer but also work at the Data Link layer. We also said

that the Data Link layer is often divided into two parts. The upper

part is called Logical Link Control (LLC) and the lower part is called

Medium Access Control (MAC). As it turns out, the IEEE standards

define the lower, or MAC, half of the Data Link layer -- the part that

determines how network users keep from bumping into each other.

Medium Access Control is just what it sounds like. It is the protocol

that determines which computer gets to use the network cable when many

computers are trying. We saw that IEEE 802.3 lets everyone simply bump

into each other and keep trying until they get through. IEEE 802.4 and

802.5 are more ordered, limiting conversation to the computer with the

token. Remember, all of this is done in fractions of a second. So even

when the network is crowded, no one really waits very long for access

on any of the three types of networks.

The other half of the Data Link layer, LLC, provides reliable data

transfer over the physical link. In essence, it manages the physical

link.

There are two reasons why the IEEE split the Data Link layer in half

(and why the ISO accepted it). First of all, the Data Link layer has

two jobs to do. The first is to coordinate the physical transfer of

data. The second is to manage access to the physical medium. Splitting

the job allows for more modularity, and therefore flexibility.

The second reason also has to do with modularity, but in a different

way. The type of Medium Access Control has more to do with the physical

requirements of the network than actually managing the transfer of

data. In other words, the MAC layer is "closer" to the physical layer

than the LLC layer. By splitting the two, it is possible to create a

number of MAC layers (corresponding to physical layers) and just one

LLC layer that can handle them all. This increases the flexibility of

the standard. It also gives LLC an important role in providing an

interface between the various MAC layers and the higher-layer protocols.

By the way, Logical Link Control is the more common name of the IEEE's

802.2 specification. The numbers give it away. 802.2 works with 802.3,

802.4 and 802.5. It should also work with emerging standards, like FDDI.

There are other protocols that perform the LLC functions.  High-level

Data Link Control (HDLC) is the protocol from the ISO.  Like LLC, it

conforms to the OSI model. IBM's SDLC (Synchronous Data Link Control)

is a Data Link layer standard that does not conform to the OSI Model

but does perform similar functions. IBM has many products that do not

follow the OSI Model or its hierarchical setup. IBM has pledged support

of OSI, however.

Page 9

Transport Protocols

The ISO is in the process of establishing protocol standards for the

middle layers of the OSI Model. As of yet, none of these have been

implemented on a widespread basis, nor has the complete OSI protocol

stack been established. To make matters more confusing, most of the

middle-layer protocols on the market today do not conform neatly to

the OSI Model's network, transport and session layers. They were

created before the ISO started work on the model.

The good news is many existing protocols are being incorporated into

the OSI Model. Where existing protocols are not incorporated, interfaces

between them and the OSI Model are being implemented. This is the case

for TCP/IP, NetBIOS and APPC, the major middle-layer protocols available

today.

In the PC LAN environment, NetBIOS is the most important protocol. It

stands for Network Basic Input/Output System. IBM developed it as a

BIOS for networks. It is essentially a Session layer (Layer 5) protocol

that acts as an applications interface to the network. It provides the

tools for a program to establish a session with another program over

the network. Hundreds of programs have been written to this interface,

making it the most widespread protocol in the PC network arena.

NetBIOS does not obey the rules of the OSI Model in that it does not

talk only to the layers above and below it. As we said, programs can

talk directly to NetBIOS, skipping the application and presentation

layers. This doesn't keep NetBIOS from doing its job. It just makes it

incompatible with the OSI Model, which is not the end of the world.

Someone will write an interface between the two, soon.

NetBIOS is limited to working on one network. Therefore, some network

vendors have established an interface between NetBIOS and TCP/IP, a

protocol from the Department of Defense for use over large combinations

of networks (internetworks).

TCP/IP

TCP/IP stands for Transmission Control Protocol/Internet Protocol.

TCP is a Transport protocol (Layer 4), corresponding to the definition

we gave above. Its job is to get data from one place to another without

errors. It forms an interface between the protocols above and below

-- shielding the upper layers from concern about the connection and

the lower layers from concern about transmission content.

The IP protocol is for getting data from one network to another.

Its main concern is bridging the differences between networks so

they don't have to be modified to talk to each other. It does

this by providing rules for the breakdown of data to conform with

a given network. Gateways, which are the physical translators

between networks, use IP's rules to take data from one network,

modify it and route it correctly over another network.

Page 10

TCP/IP enjoys enormous support in government, scientific and

academic internetworks. These computers use UNIX and other

large-computer operating systems. In the past few years, business

internetworks have begun to approach the size of those in

government and universities. This has driven these businesses to

look for internetwork protocol standards. They have found TCP/IP

useful and it has become a de facto standard. Many see it as an

interim solution until the OSI transport and internetwork

protocols are finished. TCP/IP products for DOS-based networked

PCs are also available.

Often when TCP/IP is discussed, acronyms like SMTP, FTP and

TELNET are tossed around. These are applications that have been

written for TCP/IP and are widely used. They work at the

Applications layer (Layer 7). SMTP stands for Simple Mail

Transfer Protocol. FTP stands for File Transfer Protocol. TELNET

is the name for a terminal emulation protocol. These protocols,

written for TCP/IP, do exactly what they say they do.

Advanced Program-to-Program Communications, or APPC, is another

protocol for large networks. It comes from IBM and is part of Big

Blue's Systems Network Architecture (SNA). It is similar to

NetBIOS in that it provides an interface to the network for

programs so they may communicate, but it is not limited to one

network as is NetBIOS. APPC is geared toward mainframe computers,

though IBM is offering it as part of its OS/2 Extended Edition.

Using APPC, all computers communicate as peers, even PCs.

Previously in the IBM world, PCs were forced to emulate terminals

when communicating with mainframes. A number of other vendors,

mini and micro, also offer APPC.

APPC has received much publicity. Unfortunately, there are not

many applications for APPC in the PC network arena. There are

more in the minicomputer and mainframe network market.

Nevertheless, IBM and others are promoting APPC as a protocol

standard for the future. Its robustness, flexibility and

reliability make it worth the extra development effort.

There are other middle-layer protocols. XNS, IPX and NetBUEI are

all transport protocols. XNS is short for Xerox Network System.

It was one of the first local area network protocols used on a

wide basis, mainly for Ethernet (802.3) networks. 3Com and many

others use it. IPX is Novell's implementation of XNS. It is not

completely compatible with the original, but very widely used.

NetBUEI is IBM's transport protocol for its PC networking

products. All of these protocols perform similar tasks.

Many More

If it seems like the number of protocols is idiotic, it is and it

isn't. Different protocols have different advantages in different

environments. No single protocol stack will work better than

every other in every setting. NetBIOS seems to work fantastically

in small PC networks but is practically useless for communicating

with mainframes. APPC works well in mainframe environments.

TCP/IP excels in large internetworks.

Page 11

On the other hand, much more is made about the differences in

protocols than is actually warranted. Proprietary protocols are

perfect solutions in many cases. Besides, if the proprietary

protocols are widespread enough, they become standards, and

gateways between them and other standards are built. This is

happening with some of the major protocols we have not covered.

These protocols include many de facto standards in minicomputer

and scientific workstation communications. They include DEC's

entire protocol suite, Sun Microsystems' NFS, AT&T's protocols

and many others. We have also left out Apple's AppleTalk and AFP.

While these enjoy widespread use, that use is based on the

computers these companies are selling and not the proliferation

of the protocols throughout the networking industry.

Unfortunately, whether proprietary or standard, users are still

faced with the dilemma of choice. This choice is made slightly

easier by the shakeout and standardization that has occurred over

the past few years at the lower Physical and Data Link layers.

There are three choices, Token Ring, Ethernet or Arcnet. Right

now, the same is happening at the higher layers. Can you guess

which way things will go?

Page 12

PART 3: (PROTOCOLS: continued)

Data Link Protocols

As we said last month, the IEEE protocol standards are not

confined to the Physical layer but also work at the Data Link

layer. We also said that the Data Link layer is often divided

into two parts. The upper part is called Logical Link Control

(LLC) and the lower part is called Medium Access Control (MAC).

As it turns out, the IEEE standards define the lower, or MAC,

half of the Data Link layer -- the part that determines how

network users keep from bumping into each other.

Medium Access Control is just what it sounds like. It is the

protocol that determines which computer gets to use the network

cable when many computers are trying. We saw that IEEE 802.3 lets

everyone simply bump into each other and keep trying until they

get through. IEEE 802.4 and 802.5 are more ordered, limiting

conversation to the computer with the token. Remember, all of

this is done in fractions of a second. So even when the network

is crowded, no one really waits very long for access on any of

the three types of networks.

The other half of the Data Link layer, LLC, provides reliable

data transfer over the physical link. In essence, it manages the

physical link.

There are two reasons why the IEEE split the Data Link layer in

half (and why the ISO accepted it). First of all, the Data Link

layer has two jobs to do. The first is to coordinate the physical

transfer of data. The second is to manage access to the physical

medium. Splitting the job allows for more modularity, and

therefore flexibility.

The second reason also has to do with modularity, but in a

different way. The type of Medium Access Control has more to do

with the physical requirements of the network than actually

managing the transfer of data. In other words, the MAC layer is

"closer" to the physical layer than the LLC layer. By splitting

the two, it is possible to create a number of MAC layers

(corresponding to physical layers) and just one LLC layer that

can handle them all. This increases the flexibility of the

standard. It also gives LLC an important role in providing an

interface between the various MAC layers and the higher-layer

protocols.

By the way, Logical Link Control is the more common name of the

IEEE's 802.2 specification. The numbers give it away. 802.2 works

with 802.3, 802.4 and 802.5. It should also work with emerging

standards, like FDDI.

There are other protocols that perform the LLC functions.

High-level Data Link Control (HDLC) is the protocol from the ISO.

Like LLC, it conforms to the OSI model. IBM's SDLC (Synchronous

Data Link Control) is a Data Link layer standard that does not

conform to the OSI Model but does perform similar functions. IBM

has many products that do not follow the OSI Model or its

hierarchical setup. IBM has pledged support of OSI, however.

Page 13

Transport Protocols

The ISO is in the process of establishing protocol standards for

the middle layers of the OSI Model. As of yet, none of these have

been implemented on a widespread basis, nor has the complete OSI

protocol stack been established. To make matters more confusing,

most of the middle-layer protocols on the market today do not

conform neatly to the OSI Model's network, transport and session

layers. They were created before the ISO started work on the

model.

The good news is many existing protocols are being incorporated

into the OSI Model. Where existing protocols are not

incorporated, interfaces between them and the OSI Model are being

implemented. This is the case for TCP/IP, NetBIOS and APPC, the

major middle-layer protocols available today.

In the PC LAN environment, NetBIOS is the most important

protocol. It stands for Network Basic Input/Output System. IBM

developed it as a BIOS for networks. It is essentially a Session

layer (Layer 5) protocol that acts as an applications interface

to the network. It provides the tools for a program to establish

a session with another program over the network. Hundreds of

programs have been written to this interface, making it the most

widespread protocol in the PC network arena.

NetBIOS does not obey the rules of the OSI Model in that it does

not talk only to the layers above and below it. As we said,

programs can talk directly to NetBIOS, skipping the application

and presentation layers. This doesn't keep NetBIOS from doing its

job. It just makes it incompatible with the OSI Model, which is

not the end of the world. Someone will write an interface between

the two, soon.

NetBIOS is limited to working on one network. Therefore, some

network vendors have established an interface between NetBIOS and

TCP/IP, a protocol from the Department of Defense for use over

large combinations of networks (internetworks).

TCP/IP stands for Transmission Control Protocol/Internet

Protocol. TCP is a Transport protocol (Layer 4), corresponding to

the definition we gave above. Its job is to get data from one

place to another without errors. It forms an interface between

the protocols above and below -- shielding the upper layers from

concern about the connection and the lower layers from concern

about transmission content.

The IP protocol is for getting data from one network to another.

Its main concern is bridging the differences between networks so

they don't have to be modified to talk to each other. It does

this by providing rules for the breakdown of data to conform with

a given network. Gateways, which are the physical translators

between networks, use IP's rules to take data from one network,

modify it and route it correctly over another network.

TCP/IP enjoys enormous support in government, scientific and

academic internetworks. These computers use UNIX and other

Page 14

large-computer operating systems. In the past few years, business

internetworks have begun to approach the size of those in

government and universities. This has driven these businesses to

look for internetwork protocol standards. They have found TCP/IP

useful and it has become a de facto standard. Many see it as an

interim solution until the OSI transport and internetwork

protocols are finished. TCP/IP products for DOS-based networked

PCs are also available.

Often when TCP/IP is discussed, acronyms like SMTP, FTP and

TELNET are tossed around. These are applications that have been

written for TCP/IP and are widely used. They work at the

Applications layer (Layer 7). SMTP stands for Simple Mail

Transfer Protocol. FTP stands for File Transfer Protocol. TELNET

is the name for a terminal emulation protocol. These protocols,

written for TCP/IP, do exactly what they say they do.

Advanced Program-to-Program Communications, or APPC, is another

protocol for large networks. It comes from IBM and is part of Big

Blue's Systems Network Architecture (SNA). It is similar to

NetBIOS in that it provides an interface to the network for

programs so they may communicate, but it is not limited to one

network as is NetBIOS. APPC is geared toward mainframe computers,

though IBM is offering it as part of its OS/2 Extended Edition.

Using APPC, all computers communicate as peers, even PCs.

Previously in the IBM world, PCs were forced to emulate terminals

when communicating with mainframes. A number of other vendors,

mini and micro, also offer APPC.

APPC has received much publicity. Unfortunately, there are not

many applications for APPC in the PC network arena. There are

more in the minicomputer and mainframe network market.

Nevertheless, IBM and others are promoting APPC as a protocol

standard for the future. Its robustness, flexibility and

reliability make it worth the extra development effort.

There are other middle-layer protocols. XNS, IPX and NetBUEI are

all transport protocols. XNS is short for Xerox Network System.

It was one of the first local area network protocols used on a

wide basis, mainly for Ethernet (802.3) networks. 3Com and many

others use it. IPX is Novell's implementation of XNS. It is not

completely compatible with the original, but very widely used.

NetBUEI is IBM's transport protocol for its PC networking

products. All of these protocols perform similar tasks.

Many More

If it seems like the number of protocols is idiotic, it is and it

isn't. Different protocols have different advantages in different

environments. No single protocol stack will work better than

every other in every setting. NetBIOS seems to work fantastically

in small PC networks but is practically useless for communicating

with mainframes. APPC works well in mainframe environments.

TCP/IP excels in large internetworks.

Page 15

On the other hand, much more is made about the differences in

protocols than is actually warranted. Proprietary protocols are

perfect solutions in many cases. Besides, if the proprietary

protocols are widespread enough, they become standards, and

gateways between them and other standards are built. This is

happening with some of the major protocols we have not covered.

These protocols include many de facto standards in minicomputer

and scientific workstation communications. They include DEC's

entire protocol suite, Sun Microsystems' NFS, AT&T's protocols

and many others. We have also left out Apple's AppleTalk and AFP.

While these enjoy widespread use, that use is based on the

computers these companies are selling and not the proliferation

of the protocols throughout the networking industry.

Unfortunately, whether proprietary or standard, users are still

faced with the dilemma of choice. This choice is made slightly

easier by the shakeout and standardization that has occurred over

the past few years at the lower Physical and Data Link layers.

There are three choices, Token Ring, Ethernet or Arcnet. Right

now, the same is happening at the higher layers. Can you guess

which way things will go?

Page 16

PART 4: LAN Access Methods

Definition

Access method is the term given to the set of rules by which

networks arbitrate their use. It is the way the LAN keeps people

from crashing into each other as they use the network. Think of

the access method as traffic law. The network cable is the

street. Traffic law (access method) regulates the use of the

street (cable), saying who can drive (send data) where and at

what time.

Access method deals on the Physical layer of the network, the

lowest level of the OSI model. That's because it is worried about

the use of the cable that connects users. The access method

doesn't care what is being sent over the network, just like the

traffic law doesn't stipulate what you can carry. It just says

you have to drive on the right and obey the lights and signs.

Networks need access methods for the same reason streets need

traffic lights - - to keep people from hitting each other. On a

network, if two or more people try to send data at exactly the

same time, their signals will interfere with each other, ruining

the data being transmitted. The access method prevents this.

There are three major access methods in use today, though many

more exist. They are Ethernet, Arcnet and Token Ring. Actually,

these are wider-ranging standards that use particular access

methods. They also define other features of network transmission

besides the access method, like the electrical characteristics of

signals, the size of data packets sent, etc. Nevertheless, these

three standards are best known (and best distinguished) for the

access methods they employ.

Ethernet

Ethernet is the most common network access method. It was

developed by Xerox Corporation at its Palo Alto Research Center

facility in the mid-1970s. It is supported by Xerox, Digital

Equipment, Intel (the three of whom made it a standard) and many

other network vendors. At least half of the installed base of

network nodes (PCs, engineering workstations, minicomputers) use

Ethernet.

The Ethernet access method is Carrier Sense Multiple Access with

Collision Detection, or CSMA/CD. This is a broadcast access

method. That means every computer "hears" every transmission.

However, not every computer "listens" to every transmission.

Here's how it works.

When a computer wants to send a message it does, no questions

asked. The signal it sends moves up and down the cable in every

direction, passing every other computer on the network. Every

computer "hears" the message, but ignores it. Only the computer

to which the message is addressed recognizes the message and

Page 17

sends an acknowledgement. The message is recognized because it

contains the address of the destination computer. The

acknowledgement can be correctly addressed because the original

message also contained the message of the sending computer.

What happens if two computers send at the same time? A collision.

This doesn't make any noise, but it does keep the messages from

going through. When it does happen, each of the colliding

computers backs off for a random amount of time and tries again.

This happens until they get through. Of course, the whole process

takes a small fraction of a second.

Computers can tell if a collision has occurred because they don't

"hear" their own message in a given amount of time, determined by

the "propagation delay" of the network (the time it takes for a

signal to go to the end of the network and back). Remember,

messages move up and down the network in all directions. Every

computer hears every message, even its own messages. That is the

Carrier Sense Multiple Access with Collision Detection access

method.

It would seem Ethernet is an inefficient access method, prone to

collisions. But while collisions do happen often, they don't mean

very much in most cases. Since the whole

transmission/collision/retransmission process takes place so

quickly, the delay a collision causes is minuscule. Of course, if

you have lots of traffic, from lots of computers, the number of

collisions can mount and the network can slow down. This happens

with some large-scale imaging applications or on Ethernet network

segments with more than 50 to 75 nodes. Few Ethernet networks,

however, have a traffic load of more than 10 to 20 per cent,

which means delay caused by collisions is unnoticeable.

Arcnet

Arcnet was developed by Datapoint Corporation (San Antonio, TX)

in the early 1970s. The main Arcnet hardware vendors in the PC

network arena today are Datapoint, Standard Microsystems

(Happauge, NY) and Pure Data (Markham, Ontario). After Ethernet,

Arcnet is the most installed network access method, supported by

most network software vendors.

Arcnet is a token passing access method that works on a star-bus

topology. That means the network cable is laid out as a series of

stars, with each computer attached to a "hub" as the center of

the star and the hubs connected in a bus, or line. Hubs can

connect four, eight, 16 or 32 computers.

When a computer wants to send on an Arcnet network, it must have

the "token." The token is simply a series of data bits created by

one of the computers on the network. (There is a whole process

for token creation that we need not go into). It moves around the

network in a given pattern, a logical ring. All computers on the

network are numbered with an address -- from 0 to 255, so the

maximum number of computers on an Arcnet segment is 256. The

token moves from computer to computer in numerical order, even if

adjacent numbers (e.g. 14 and 15) are at opposite ends of the

Page 18

network. When the token reaches the highest number on the network

it moves to the lowest, thus creating a logical ring.

Once a computer has the token it can send one packet of data --

up to 512 bytes. It does so by attaching the destination address,

its own address, up to 508 bytes of data and some other

information to the token. This combination becomes the packet.

The entire packet then moves from node to node in sequential

order until it reaches the destination node. There the data is

removed and the token released to the next node in order.

Since one packet is often not enough for an entire message, the

token may need to make several rounds of the network to complete

a message.

The advantage of token passing is predictability. Because the

token moves through the network in a determined path, it is

possible to calculate how long it will take for it to move around

the network. Since the token will only carry up to 508 bytes at a

time, it is possible to calculate how long different sized

transmissions will take. This makes network performance very

predictable. It also means introduction of new network nodes will

have a predictable effect. This differs from Ethernet, where the

addition of new nodes may or may not seriously effect

performance.

The disadvantage of the token passing access method is the fact

that each node acts as a repeater, accepting and regenerating the

token as it passes around the network in a specific pattern. If

there is a malfunctioning node, the token may be destroyed or

simply lost, bringing down the whole network. There are, however,

provisions for token regeneration so that a lost or destroyed

token is not gone forever. The star topology also helps.

Token Ring

The Token Ring network was introduced by IBM in 1984. It is not

the first ring network, but it has had the most impact on the LAN

industry. It has evolved into IBM's ultimate connectivity

solution for all its computers -- personal, mini and mainframe.

IBM's specifications follow those of the IEEE's (Institute of

Electrical and Electronic Engineers) 802.5 standard. The other

major Token Ring hardware vendors are Proteon (Natick, MA), 3Com

(Santa Clara, CA) and Ungermann-Bass (Santa Clara, CA). The

network software vendors that support Token Ring hardware include

3Com, Novell (Orem, UT) and Univation (Milpitas, CA). The

installed base of Token Ring should surpass that of Ethernet and

Arcnet soon.

Like Arcnet, Token Ring networks use token passing. The

difference is computers are arranged in a physical ring. The

token moves around the ring, giving successive computers the

right to transmit. If a computer receives an empty token it may

fill it with a message of any length as long as the time to send

does not exceed the token-holding timer. This message moves

around the network with each computer regenerating it. Only the

Page 19

receiving computer will copy the message into its memory, then

marking the message as received. It does not remove the message

from the ring. The sending computer does that when the message

comes back around.

Because each computer looks at the message and may act on it,

each computer can perform certain tests to make sure the message

is getting through correctly. Also, since the frame is copied and

marked rather than purged, the sending computer can see if the

destination computer exists and if the message was received when

the message comes back around.

Token Ring networks have a priority mechanism whereby certain

computers can get the token faster than others. They can also

hold it for longer.

Token Ring's advantages include reliability and ease of

maintenance. It uses a star-wired ring topology in which all

computers are directly wired to a multi-station access unit, or

hub, or in IBM terms Multi-Access Unit (MAU).

These are connected in a ring. The multi-station access unit

allows malfunctioning computers to be disconnected from the

network. This overcomes the disadvantage of token passing, namely

the way in which one malfunctioning computer can bring down the

network since all computers are active in regenerating the token

and passing signals around the ring. Malfunctioning computers are

simply disconnected by unplugging them from the multi-station

access unit.

Page 20

PART 5: LAN Interface Cards

Definition

The network interface card (NIC) is the piece of hardware that

fits inside your computer to provide the physical connection to

the network. Every computer attached to a LAN uses one sort of

network interface card or another. In most cases, the card fits

directly into the expansion bus of the computer. In some cases,

the card will be part of a separate unit to which the computer

attaches through a serial or parallel connection.

The interface card takes data from your PC, puts it into the

appropriate format and sends it over the cable to another LAN

interface card. This card receives the data, puts it into a form

the PC understands and sends it to the PC. Simple, right?

Wrong. To get one byte of data from here to there hundreds of

things must happen. Buffers must be checked. Requests must be

acknowledged. Sessions must be established. Tokens must be sent.

Collisions must be detected. The list can seem endless.

Luckily, the work of the interface card can be broken down into

eight tasks: host-card communications; buffering; packet

formation; parallel-serial conversion; encoding/decoding; cable

access; handshaking and transmission/reception. These are the

steps taken to get data from the memory of one computer to the

memory of another computer.

Preparing for Transmission

PC-NIC Communications. There are three ways to move data from the

PC's memory to the network card and back again: DMA, I/O mapping

and shared memory.

Shared memory is just that. Part of host memory is shared by the

network interface card's processor. This is a very fast method of

transfer, since no buffering on the card is required. Both the

card and the PC do their work on the data in the same place, so

no transfer is necessary.

DMA is short for Direct Memory Access. All Intel-based computers

come with something called a DMA controller. It takes care of the

transfer of data from an input/output device to the PC's main

memory so the PC's main microprocessor, or CPU, does not have to.

For DMA transfer, the controller or processor on the interface

card sends a signal to the CPU indicating it needs DMA. The CPU

then relinquishes control of the PC bus to the DMA controller.

(The bus is the piece of the computer that connects other parts,

e.g. memory and processor).

Once the DMA controller has command of the bus, it begins to take

the data from the card and place it directly in memory. It can do

this because it has been informed by the CPU of the appropriate

memory address at which to begin putting data in memory. After

all the data is in memory, the DMA controller returns control of

Page 21

the bus to the CPU and tells it how much data has been put in

memory. Of course, the whole process takes fractions of a second.

There are two types of I/O transfers depending on the PC and

peripheral. The two most important are memory-mapped I/O and

program I/O. In a memory-mapped I/O transfer, the host CPU

assigns some of its memory space to the I/O device, in this case

the network interface card. So out of the possible 640K bytes

available, some amount, say 12K bytes, are given to the network

card. This memory is then treated as if it were main memory. No

special instructions in the CPU are needed to get data from the

card since it is like taking data from one part of main memory to

another.

With program I/O, the CPU is given a set of special instructions

to handle the input/output functions. These instructions can be

built into the chip or come with software. To send data, a

request is sent from the network interface card to the CPU. The

CPU then moves the data from the card over the bus to main

memory.

Shared memory is the fastest method of moving data between the

network interface card and the PC, but it not often used for

expense and execution reasons. The advantage of DMA is that it

removes work from the CPU, so it can perform other functions

while data transfer is taking place. The disadvantage is the CPU

cannot access memory while the DMA controller is working. I/O

mapping doesn't remove work from the CPU and it also takes up

memory, but it can be faster than DMA.

Different NICs use different types of NIC-PC communications.

Yours will probably use DMA or program I/O. Experts disagree on

which is better. If you really need to tell the difference,

you'll have to experiment with your own applications.

Buffering. Most NICs use a buffer. The buffer is a storage place

that holds data as it is moving into and out of the NIC. The

purpose of the buffer is simple: to make up for inherent delays

in transmission. To do this, a buffer temporarily holds data,

either for transmission onto the network cable or for transfer

into the PC. While in the buffer, data may be acted on,

"packetized" or "depacketized," or it may simply sit while the

NIC handles other things.

A buffer is needed because some parts of data transfer are slower

than others. Data usually comes into the card faster than it can

be converted from serial or parallel, depacketized, read and

sent. This is true in both directions.

Some NICs do not have buffers. Instead they use PC RAM. This can

be less expensive, but usually slower. It also takes up precious

memory.

Packet formation. This is the most important job of the network

and it is almost always done by the NIC. Packets are the units of

transmission used on most LANs. Files and messages for

Page 22

transmission are broken up into packets as they are sent. At the

other end the packets are put together to reform the original

file or message.

A packet has three sections: header, data and trailer. The header

includes an alert to signal "packet on the way," the packet's

source address, destination address and clock information to

synchronize transmission. In some networks, headers also have

preambles used for various purposes, like setting up parameters

for transmission. They can also have a control field to direct

the packet through the network, a byte count and a message type

field.

The data section is just that, the data being sent, for example,

the numbers in a spreadsheet or words in a document. On some

networks, the data section of a packet is as large as 12K bytes.

On Ethernet, it is 4K bytes. Most networks fall between 1K and 4K

bytes.

The trailer has an error checking part called a cyclical

redundancy check (CRC). It is a number which is the result of a

mathematical calculation done on the packet by the sending NIC.

When the packet arrives at its destination, the same mathematical

calculation is repeated. If the result is the same, no errors

occurred in transmission -- all the ones and zeros were in the

right place. If the numbers don't match, an error occurred and

the packet is sent again. The trailer, like the header, can hold

other information.

TRANSMISSION

Parallel-serial conversion. Data comes from the PC in parallel

form, eight bits at a time. It must travel over the cable in

serial form, one bit at a time. And vice versa. Thus, the network

interface card must perform the conversion between the two forms.

Usually this is done by a controller on the card.

Encoding/decoding.

Once a packet is formed and changed from parallel to serial, it

is ready for sending over the line. To do this it must be

encoded; it must be converted into a series of electrical pulses

that convey information.

Most network interface cards use Manchester encoding. Serial data

is divided into things called bit periods. Each of these periods

(actual fractions of seconds) is divided in half. The two halves

together represent a bit. From the first half to the second half

of each bit period there is a change in the polarity of the

signal, from positive to negative, or vice versa. There must be a

change during each bit period because the change represents the

data. A change from negative to positive represents a 1. A change

from positive to negative represents a 0. Or vice versa depending

on the network.

Of course, it is these 1's and 0's that represent data. That's

how digital data is actually sent using electrical impulses.

Page 23

Cable access.

Before data can be sent, however, the network interface card must

have access to the cable. Not all cards can send at once. If they

do, their transmissions will collide and be lost.

The various access methods used were discussed last month. Token

Ring and Arcnet LANs use an electronic token to grant network

access. Ethernet lets any workstation transmit at will and then

look for collisions to see if it must transmit again. The entire

protocol for the access method, all the circuitry and firmware

(software written into hardware), resides on the network access

card. This is its main job, getting data onto the cable in

reliable form.

Handshaking.

After getting data from the PC, formatting it, encoding it and

getting cable access, the network interface card has just one

more task before it can send data: handshaking. In order to send

data successfully, there must be a second NIC card waiting to

receive it. To make sure, there is a short period of

communication between two cards before data is sent. During this

period, the parameters for the upcoming communication are decided

upon through negotiation.

During negotiation, the transmitting card sends the parameters it

wants to use. The receiving card answers with its parameters. The

card with the slower, smaller, less complicated parameters always

wins. That's because more sophisticated cards can "lower"

themselves while less sophisticated cards can't "raise"

themselves.

Negotiation sets the maximum packet size to be sent, how many

packets before an answer, timer values, acknowledge time outs

(how long to wait for an answer), buffer sizes, etc.

Transmission/reception.

Finally, everything is set. The only thing left is for the

transceiver to put the data on the cable. The transceiver gives

the data power to make it down the line. It actually puts the

electrical signal out over the cable, making sure the data can

get to the next card, the next repeater, amplifier or bridge.

At the other end, a transceiver is waiting to accept the signal

and begin the whole process in reverse, from modulated signal

through decoding, serial- parallel conversion and depacketizing

to PC-readable data.

PICK A CARD

Most people look only at performance when they choose a network

interface card. Some will also consider the access method and

topology used. This is fine when choosing the type of network

interface card. Ethernet tends to be better for bursty networks.

Its topology makes sense in scientific environments. Token Ring

and Arcnet do better with constant traffic. Their topologies are

better in offices. But there is more to choosing a network

Page 24

interface card than performance and access method.

The most important consideration in buying a network interface is

reliability.  It doesn't matter how fast your network card is if

it doesn't work, causes errors, loses packets, drops the line,

etc. There is nothing more frustrating than having to isolate

network hardware problems. Moreover, once found, replacing a

network interface card means opening up a PC, setting dip

switches and possibly reconfiguring software. Look for a network

interface card that will work forever. This means talking to

users and installers.

Other considerations when buying a network interface card center

on your application. Figure out what kind of traffic your network

will be sending. Different topologies, different access methods

and different cabling schemes will guide your choice of network

interface card. So will the network operating system you plan on

using. But we'll get to that hairy subject in next month's

column.

-- by Aaron Brenner

Page 25

#