THE LAN TUTORIAL SERIES   -- by Jim Carr

NETWORK MANAGEMENT

Definition.........................1

From Simple to Complex ............1

Five Functional Areas..............2

Built-In NOS Management ...........3

Programmable Managers..............3

Management Stamdards ..............4

Host-Based Systems ................5

DOS to MAC Connectivity

Definition ........................6

The OSI Model......................6

Popular Physical Protocols.........7

Incompatible File Systems .........6

A Path Through the Maze............7

On To Networks ....................7

The File Server's Role.............8

NETWORK MANAGEMENT

Definition

Although the physical location of the personal computers on a

local area network seldom changes, networks are still dynamic

entities.  That is, the logical makeup of any network fluctuates

from moment to moment.

For example, the number of data and application files in use or

stored away, the amount of available disk storage space, the number

of users logged in to the LAN, and the volume of traffic passing

through the network cabling all change continually.  Moreover, a

network offers users a distributed-processing environment, with some

processing performed by a centrally located server, some done at

users' workstations, adding even more activity to the LAN.

Keeping this conglomeration of network hardware, software, cables,

and the people using them working efficiently comes under the rather

ambiguous term of network management.  It's ambiguous in that managing

a network can range from the simple to the complex, from a moment's

quick fix of plugging in a misplaced network cable to a day-long

search for an obscure disk problem.

Network management can be as simple as creating a boot diskette for a

new user and making sure that user has proper access to the network

resources. (Although in truth these jobs may not be all that simple

in some widely distributed networks.)

Or managing a network can include daily disk-maintenance duties --

backing up network files or defragmenting disk directories.  Or it may

mean troubleshooting the LAN, trying to discover why some users are

experencing slow network response.  Or it may include reconfiguring a

remote internetwork device to improve overall system performance.

In short, network management incorporates an almost unlimited list of

duties -- basically, doing whatever it takes to keep the LAN running

smoothly and efficiently, with minimal (if any) downtime.

This job has grown even more difficult as networks have become larger

and more complex.  The evolution from small workgroups of often-identical

PCs to large internets made up of dissimiliar machines -- IBM PS/2s,

Macintoshes, PC clones, printers, communications gateways, and bridges

and routers -- has brought more power to the desktop while adding immense

complexity to the network manager's job.

Fortunately, vendors are developing more and better tools -- some

software based, others complete systems that provide onscreen maps of

network resources -- to help in the endless task of managing a network.

From Simple to Complex

Network management tools, whether they are as application-specific as

a performance monitor or as comprehensive as IBM's mainframe-based

Netview product, help bring some order to the potentially chaotic

network management environment.  They give network managers information

and capabilities they can use in the battle to keep their LANs running

trouble-free.

Page 1

Whether they are intended to merely find cable breaks or to pinpoint

the cause of a network slowdown, network management tools are vital to

the network manager's day-to-day life.  They can help ensure uptime

and network reliability, maintain predetermined performance levels,

manage LAN resources optimally, plan for expansion, maintain company

security, keep track of network use, and provide a basis for charging

customers for network time.

For example, knowing how many network users regularly access a laser

printer -- and how long they have to wait for their printed material

to appear-- can help a company decide when it's time to add a second

printer.  Knowing which workstations generate the heaviest traffic

lets a network administrator predict possible bottlenecks--bottlenecks

that can be avoided by adding internetwork devices such as bridges and

routers.

Five Functional Areas

At the very basic level, network management requirements generally fall

into five functional areas: configuration management, fault management,

security management, performance management, and accounting management.

1. Configuration management applications deal with installing, intializing,

"boot" loading, modifying, and tracking the configuration parameters or

options of network hardware and software.  For instance, 3Com's Network

Control Servers can monitor and reconfigure boot-up parameters and change

port settings on the company's communications servers and change inter-

network bridge filter options.

2. Fault management tools provide an audit trail, or historical overview,

of a network's error and alarm characteristics.  These types of tools

show a LAN manager the number, types, times, and locations of network

errors.  These errors might be dropped packets and retransmissions

(on an Ethernet) or "lost tokens" (on a Token Ring).

3. Security management tools allow the network manager to restrict access

to various resources, from the applications and files to the entire

network itself; these generally offer password-protection schemes that

give users different levels of access to different resources.  For

instance, a user in marketing could be allowed to view, or "read", a

data file in accounting but not permitted to change, or "write" to it.

Security management is also important in managing the network itself--

for instance, only certain individuals (network administrators) should

be permitted to change configuration settings on a server or other key

network device.

4. Performance management tools produce real-time and historical statistical

information about the network's operation: how many packets are being

transmitted at any given moment, the number of users logged into a

specific server, and utilization of internetwork lines.  As already

noted, this type of information  can help network administrators pinpoint

areas or network segments that pose potential problems.

Page 2

Performance management tools generally allow polling individual network

nodes-- a server or bridge, for example-- for component-specific inform-

ation.  A communications server might provide information on throughput

for each serial port, while a file server might report the number of

users logged in, what applications they are using, and the number of

active files.  This information can then be studied to determine which

gateways, servers, or routers are being used heavily and may need added

capabilities in the future.

5. Accounting management applications help their users allocate the costs

of various network resources-- a public data network gateway, access to

a mainframe session, or printer time-- to those using them.  These

applications provide information about session start up/stop, user

logins and resource use, then use this information to bill departments

internally  or customers for computer and/or network time.

Built-In NOS Management

Most network operating systems (NOSs) provide some level of network

management capabilities; in particular, almost all the leading NOSs

offer password-protection schemes that limit users' access to network

resources.  Novell, for instance, implements its NetWare management

scheme through user profiles, which define not only the user's access

rights, but the user's classifications (supervisor, workgroup manager,

console operator, or user), which also determine the resources they

can access.

In this scheme, a supervisor has access rights that allow reconfiguring

and upgrading the entire system.  The workgroup manager, available only

with NetWare 386, controls only the resources of a single user or user

group.

This concept allows a supervisor to distribute some of the responsibility

for maintaining the network to others around a large network.

A user with console operator access rights can run NetWare's FCONSOLE

utility, which allows monitoring and controlling a variety of network

performance criteria, such as print queues.  The user can access only

those resources allowed by the supervisor (or workgroup manager with

NetWare 386).

Although users can access the NetWare management utilities, their rights

to actually perform management functions is severely limited.

Although other NOSs access schemes may differ in specific features from

NetWare's, they all offer similiar resource-restriction capabilities that

give the network managers control over their LANs.

Programmable Managers

Many other network-product vendors also offer specific network management

products that address more-detailed needs.  These include 3Com's NCS

products, Sun Microsystems's SunNet Manager, SynOptics' LattisNet network

management system (NMS), and Hughes LAN Systems' 9100 Network Management

Center.

SynOptics' LattisNet NMS, for instance, can be programmed to automatic-

ally disconnect stations that exceed network manager-defined performance

or error thresholds.

Page 3

Sun designed its SunNet Manager to work with other vendors' "agent"

applications that add specific functionality to a system.  For example,

various agents can perform monitoring and controlling capabilities on

gateways and routers.

Other products, however, deliver only partial solutions.  These devices

include protocol analyzers, which provide configuration and performance

data but no accounting management capabilities.

Management Stamdards

As networks have grown larger and become increasingly heterogeneous in

nature, so has the need for industry-standard network management

protocols (and products) that operate across a wide range of vendor

offerings.  The first of these protocols, the Simple Network Management

Protocol (SNMP), was developed by the Internet Activities Board in 1988.

(The Internet is an international wide area network that links thousands

of users, vendors, and universities.)  SNMP relies on the Transmission

Control Protocol/Internet Protocol (TCP/IP) as the underlying mechanism

for transferring data between different types of systems (i.e. PC LANs

and mainframes) and networks.

Briefly, SNMP is a protocol that defines the communication between a

network management station and an object to be managed.  SNMP's three

layer architecture (network management stations, network elements, and

a common set of protocols that binds them together) operates with a

management information base (MIB) and a structure of management

information (SMI).  The MIB and SMI are network management concepts

that allow defining each network element so these elements can be

monitored, then controlled by the management stations.

Though widely accepted, SNMP has several limitations.  For one, it is

considered too simplistic for managing the large, global-style networks

evolving today, and its manager-to-agent architecture leaves it incapable

of managing true enterprisewide LANs, which require manager-to-manager

and manager-to-agent systems.

Eventually, the evolving Common Management Information Protocol (CMIP),

the network management standard for Open Systems Interconnection (OSI)

networks, will overshadow TCP/IP.

Another would-be network management standard, called CMOT--for CMIP over

TCP/IP-- was proposed and developed in 1988 and 1989, but failed to gain

broad support from vendors or end users.  Basically, CMOT allows

OSI-based CMIP functions to run over TCP/IP networks.  It was designed

as a migratory protocol that would allow smooth transition from SNMP to

OSI, but never became popular.

Because products based on it are widely available-- more than 40 vendors,

including Sun and SynOptics, make compatible products-- SNMP remains the

network management protocol of choice for most PC-based network managers.

Other vendors providing SNMP-compatible products include Cabletron

Systems, which is adding SNMP capabilities to all its products, Network

Research, Wellfleet, and FiberCom.

Page 4

Host-Based Systems

Four mainframe-based network management protocols with wide industry

support are IBM's NetView, AT&T's Unified Network Management Archi-

tecture (UNMA), Hewlett-Packard's (HP) OpenView, and Digital Equipment

Corp.'s (DEC) Enterprise Management Architecture (EMA).

Although proprietary in nature, these protocols enjoy broad end-user

support because of their associated vendors' large installed bases of

computers.  With the protocols already available, many users incorporate

their primary vendor's network management products into their LANs as a

matter of course.

IBM's NetView permits non-IBM networks to access the NetView host via

its NetView/PC gateway product.  IBM has announced that it will provide

an OSI-based CMIP interface to NetView in the future.  This inteface

will allow the NetView host to manage heterogeneous networks, a task

now made almost impossible by the complex nature of NetView.

DEC, HP, and AT&T have all said they plan to utilize CMIP with their

respective protocols.  Like most other vendors, however, they are

waiting for a more stable version of the OSI protocol.

Page 5

DOS to MAC Connectivity

Definition

It wasn't until after the IBM Personal Computer and its XT/AT successors

moved en mass into the office computing environment that local area

networks became popular.  And because MS/PC-DOS-based IBM PCs, XTs, ATs,

and compatibles were so prevalent, it was only natural that the majority

of the office LANs operated in all-DOS environments.

In these homogenous networks, few users needed connectivity to other

operating systems (OS), such as UNIX or the Macintosh.  A few longtime

computer users--generally, those who had used mainframes-- may have

wished they could access host-based data from their desktop PCs.  But

the greater majority of PC users had little prior experience with

computers, so they didn't even perceive the need for inter-OS

connectivity.

Apple's introduction of the Macintosh in 1984 changed user's perception.

They found they could use the Mac to produce online charts, graphs, and

other illustrations-- a capability then lacking with MS-DOS-based PCs.

Inevitably other applications found their way into the easy-to-learn-

and-use Mac environment.  With a Mac, users could combine spreadsheet

and/or database files with word processing documents to create

attractive, professional-looking reports, white papers, and other

corporate documents without going to an outside graphics house.

This not only saved their company time and money, it also pointed out

the necessity of giving Mac and PC users access to data on the "other"

operating systems.  This was doubly important because most companies

using personal computers for bookkeeping, accounting, and customer

-profile information do so with DOS-based PCs.  It was precisely this

data that could make those attractive Mac-generated reports worth

reading.

Unfortunately, the MS-DOS and Macintosh file-storage and disk operating

systems are incompatible with each other, making it impossible to

exchange data directly between them without additional hardware and

software.  Providing connectivity between PC and Mac systems thus has

been a key issue for LAN vendors, especially those selling network

operating systems (NOS) and application software.

Incompatible File Systems

The foremost issue they faced was operating system incompatiblities.

A glance at the two OSs' user interfaces shows the obvious differences

between the Mac and MS-DOS environments.  The Mac offers a graphical

user interface (GUI), with pictures, or icons, representing files,

printers, applications, etc.  Mac users "point and click" with a mouse

to start an operation or select a file.

PC users interact with DOS through a "command-line interface"-- they

must type commands.  (Recent developments, particularly the release of

Microsoft's Windows 3.0, have brought a GUI to the DOS world, too, but

most DOS-based PC users still type commands.)

Page 6

The differences between the two operating systems run much deeper than

user interfaces, though.  The Mac and PC manipulate and store data in

different physical and logical formats-- that is, their files systems

are totally different.  That means that even if a diskette from a Mac

could fit into a PC's disk drive, MS-DOS wouldn't "understand" the data

on it, and vice versa. (The Mac's 3.5 inch diskettes aren't physically

interchangeable with the 5.25 inch diskettes used by the original PCs;

physical incompatibility is rapidly becoming a non-issue, because many

MS-DOS-based PCs now support 3.5 inch diskettes.)

Users wanting to share Mac and PC files faced yet another obstacle: Many

DOS-based applications store data in proprietary format.  Often, these

formats are unreadable by other applications.  Files from many DOS-based

database programs, for example, cannot be "imported" into a Wordstar

document file.

A Path Through the Maze

The challenges seem daunting, don't they?  Fortunately, the path to

"transparent" Mac-PC connectivity really isn't as impossible as the

above scenario sounds, and many vendors offer several solutions.

These range from the fairly straightforward that require external

floppy disk drives or modem-serial communications packages to

full-scale, server-based LANs that offer a wide variety of network

services.  Users must simply understand what level of Mac-PC

connectivity they need, and plan accordingly.

For simple file transfer between a standalone PC and Mac, one solution

is to use a product such as DataViz's (Trumbull, Conn.) MacLinkPlus.

MacLinkPlus not only provides the mechanism to transmit data between

the two computers, but it also offers "translators" for more than 45

applications, ranging from spread-sheets to database programs.  These

translators convert incompatible DOS files to the Mac format, while

preserving the original file's formatting commands.

MacLinkPlus allows two ways to exchange files: via a connection directly

through the two computers' serial ports or via a modem hookup.

Another easy-to-implement solution comes from Dayna Communications'

(Salt Lake City) DaynaFile external disk drive.  DaynaFile allows a Mac

to read files from or write files to an MS-DOS diskette; it supports

the Mac file system, so DOS files and disks appear on the Mac screen

as standard Mac icons.  And like MacLinkPlus, DaynaFile also offers a

translation function to convert DOS files to the Mac format.

On To Networks

These products are inadequate for exchanging files between groups of

Macs and MS-DOS computers; that's where networking applications such

as Apple's AppleShare and AppleShare/PC, 3Com's 3+ and 3+ for Mac,

Sitka's TOPS, and Novell's NetWare begin to make sense.

Naturally, networks require more complicated hardware and softwre

arrangements. Networks need cabling to connect the various computers,

network interface cards (NICs) inside each networked computer, a

Page 7

network operating system (NOS), and a file server.  For Mac-PC

connectivity, the file server must be capable of communicating with

both Macs and PCs.

A typical mixed Mac-PC network environment could include a LocalTalk

network (comprised of Macs linked to an Ethernet via a dedicated file

server.  Because LocalTalk connections are built into every Macintosh,

the Macs on this segment do not require add-in NICs.

That's not the case for the file server and the computers on the other

Ethernet segment.  Because the server requires connection to each type

of cabling, it must contain two NICs-- one for Ethernet and one for

LocalTalk.  These adaptors provide the physical connection to the two

workgroups.  In the case of Ethernet the connection could be via coax

cable, twisted-pair wiring, or fiber-optic cable; in LocalTalk, the

connections could be over Farallon's unshielded twisted-pair (TP)

wiring scheme or Apple's proprietary shielded TP.  The other computers,

including any Macs on the Ethernet workgroup, must also contain an

Ethernet NIC.

Each nonserver computer on the network must also be operating under

"client" software, which provides the PC or Mac with the commands it

needs to "look" to the network and exchange files with the server.

The File Server's Role

In addition, to providing the physical interface between the two

different cabling schemes, the file server in this example network also

would perform a variety of other tasks.  First of all, it must provide

all the common file-server services, particularly data-and-application

storage and retrieval. These capabilities are controlled and manipulated

by a NOS.

Because this is a mixed PC-Mac environment, this NOS must be extra

"smart": It must "understand" both the Mac and MS-DOS file systems.

It must also be able to provide data-translation functions that permit

users of Macintoshes to access files created and stored by the user of

an IBM PC or compatible, and vice versa.

Without these capabilities, Mac-PC connectivity would be severly limited.

Networked Macintoshes and PCs could share server disk space and perhaps

might even be able to read some data files created by the other.

But the level of functionality available would not be the true

transparent Macintosh-PC connectivity required by users of today's

increasingly complex heterogenous networks.

Page 8

#