LAWN Control Program for the PC v1.10

Copyright (c) 1991 by Adrienne Cousins

All Rights Reserved

VersaWare

30 Jennifer Street, Littleton, MA 01460

508-486-9351 (Voice 9am-4pm Eastern)

508-486-8507 (24 hour Support BBS)

Lawn stands for Local Area Wireless Network.  It is manufactured by

O'Neill Communications, Inc. (OCI) in Princeton, New Jersey.

It is a small white box, 4" x 2" x 7" in size, that is attached to a

serial port with an included cable.  It uses Spread Spectrum Radio

frequency, to transmit data from Lawn to Lawn at distances of 100 to

500 feet away from each other.

Lawns come with software that performs electronic mail functions, file

send operations, and modem and printer sharing.

They can also be used to run remote communications programs such as

pcAnywhere, Carbon Copy, Remote2, Close-up, Co/Session and PC-Tools/Commute.

They also work well with all communications programs, and many stand-alone

protocol drivers such as DSZ and BIMODEM.

This program provides a means of controlling a Lawn from the DOS prompt

without requiring the LAWNTSR program from OCI.  It also provides access

to many of the Lawn internal commands and functions that are not accessable

with the software from OCI.

This is useful in batch files, when it may be desirable to connect two

Lawns for a file transfer or any other operation, and then disconnect

the Lawns when the operation is finished.  This will leave the target

Lawn available for connects from other systems.

It also comes in handy if you want to attach a Lawn to an X10 Computer

Interface CP290.  A PC system can attach to the X10/Lawn, send commands to

program timed events and then disconnect, leaving both Lawns (and your

serial port) available for other operations.

This is the reason that I wrote LAWN, and the remainder of the documentation

will address the issues involved in setting up your X10 on a Lawn.

-----------------------------------------

LAWN command syntax & functions:

LAWN <cmd>[,<cmd>][,<cmd>] [>>outfile]

Where <cmd> can be any valid switches or Lawn command.

All commands are executed in the order that they appear in the command line.

Valid switches are as follows:

?      Print a short help text to the screen

-A     Echo all Lawn I/O to the screen in ascii

Causes all data sent to or received from the Lawn to be echoed to

the screen in ascii.  This can also be redirected to any standard

output device or file.

LAWN -A,-I,MYCALL,MHEARD >TEST.LOG

-Bn    Set baud rate of serial port (300-19200)

Causes the serial port to be set to the specified speed.  This

is useful following an ABAUD command to the Lawn:

LAWN -I,ABAUD 19200,-B19200

-C     Put Lawn into command mode (cmd:)

Puts the Lawn into command mode so that it prints the 'cmd:' prompt

and is ready to accept commands.

LAWN -C,DISCON

-H     Echo all Lawn I/O to the screen in hexadecimal

Causes all data sent to or received from the Lawn to be echoed to

the screen as hexadecimal bytes.  This data can also be redirected

to any standard output device or file.

LAWN -I,-H,-W >TEST.LOG

-I     Initialize serial port to same speed as Lawn

Checks for the existence of a Lawn on the specified serial port

at all of the possible baud rates supported by the Lawn.  Once

a Lawn is detected, the serial port is set to the matching baud rate.

After the -I switch, the Lawn will be left in command mode (cmd:).

LAWN -I

-Pn    Serial port that Lawn is connected to (1-4)

Use this switch to specify a serial port other the default of COM1.

Multiple -P switches may appear on the line, to send commands to

multiple serial ports in a single command.

LAWN -A,-P1,-I,MYCALL,-P2,-I,MYCALL

-Tf:l  Change timeout counters (first:last)

The value called FIRST, is the timeout in ticks until the first

byte is received from the Lawn.  The value called LAST, is the

timeout in ticks after the last byte is received from the Lawn.

FIRST defaults to 36 (2 secs), and LAST defaults to 18 (1 sec).

-W     Watch for any data received from the Lawn

This monitors the serial port for any data received from the Lawn.

It is only useful in conjunction with the -A and -H echo switches.

This mode is terminated on any keystroke.

LAWN -A,-W

Valid Lawn LAWN commands are as follows, only the uppercased letters of

each command are required:

ABaud n - Baud rate (n=300,600,1200,2400,4800,9600,19200)

ABOrt - abort any active process & disconnect

ANtenna n - Mode of antenna operation (0=switching,1=ant0,2=ant1)

AUtolf on/off - controls auto line-feed

AWlen 7/8 - data word length for serial port

Beacon x n - set beacon frequency (x=A,E n=0-250 1=3.3s)

BLink n - Lawn power light blink count per 5 seconds (n=0-8)

BReak - send a BREAK to far side Lawn

BREAKLen n - set length of break (0-250 1=33.3ms)

CHAn n - select Lawn channel (n=1-4)

CMdtime n - seconds delay required before ^Cs for cmd mode

COnnect - show connect state

COnnect name - direct connect to another Lawn

COnnect name [VIA n1 n2] - connect via lawn N1 and N2

CONOk on/off - control connect from other Lawns

CONVers - enable converse mode

CStatus - show channel status

DAytime - show current date & time

DAytime YYMMDDHHMM - set current date & time

DFc on/off - default connect on data from serial port

DFCN name - default connect Lawn name

DIGipean on/off - control use as a VIA Lawn

Disconnect - disconnect from other Lawn

DISP - display all internal LAWN variables

Echo on/off - control command line echo

EMail on/off - control email mode

EMSgs - check email status

MHClear - clear active LAWN list

MHeard - show active LAWNS in current system

MYcall - show name of local Lawn

MYcall name - set local Lawns name

NEwmode on/off - ON enters TRANS mode on CONNECT

OFF stays in COMMAND mode

Ocimode on/off - ON stays in TRANS mode on CONNECT

OFF enters TRANS mode on CONNECT

Paclen n - packet length (n=0-256)

PACTime x n - packet transmision frequency (x=A,E n=0-250 1=33ms)

PARity n - parity (0=none 1=odd 2=none 3=even)

REAd - read next received email message

REConne - reinitialize connection to current LAWN

RESet - reset all Lawn variables & counters to factory defaults

RESTart - reset counters & displays ROM version if MYCALL<>DFCN

REtry n - maximum number for TRIES counter (n=0-31)

SElftest - perform selftest & RESTART command

STatus - show number of outstanding transmissions

SYsnum n - set security code (n=0-65535)

TImeout n - disconnect time for DFC ON (n=0-250 1=5s)

Trans - enable TRANSPARENT mode

TRFlow on/off - control XON/XOFF handshake on RX line

TXFlow on/off - control XON/XOFF handshake on TX line

Xflow on/off - master switch for XON/XOFF handshake

-----------------------------------------

Some special notes for connecting an X10 to a Lawn:

Since both the Lawn and the X10 are DCE devices, you must cross pins

2 and 3 between the Lawn and the X10, using either a null-modem cable

or a break-out box.

The Lawn that is to be connected to an X10 should initially be connected

to a PC and configured with the following command (you may replace the

two occurrances of X10 with any preferred 6 character name):

LAWN -a,-i,di,reset,-i,ab 600,-b600,-c,my x10,dfcn x10,pact e 1

This sets the Lawn to the factory defaults, then sets the baud rate to

600 baud which is required by the X10 hardware.  It then sets the Lawn

name and default-connect-to name to X10 (which you may change), and then

sets the PACTIME to EVERY 1 which is necessary so that any X10 software

won't timeout and think that the X10 isn't responding properly.

This also leaves the Lawn in NEWMODE ON mode which causes any CONNECT

command to immediately enter TRANSPARENT mode instead of staying at

command mode (cmd:).

-----------------------------------------

Here are some examples for using LAWN to control an X10 via a Lawn:

LAWN -a,-i,di,ab 600,-b600,co x10 >x10.log

XA f=reset.x10 >>x10.log

LAWN -a,-c,di,ab 19200,-b19200 >>x10.log

-a      echo all I/O to the screen in ascii

-i      initialize the serial port to match the Lawn

di      disconnect any prior active connection

ab 600  set the Lawn baud rate to 600 baud

-b600   set the serial port to 600 baud

co x10  connect to the Lawn called X10

XA      runs a shareware program called XA, which forces the

serial port to 600 baud.  You may run any X10 software

you prefer at this point.

-a      echo all I/O to the screen in ascii

-c      put Lawn in command mode (cmd:)

di      disconnect from the X10 Lawn

ab 19200 sets the Lawn speed to 19200 baud

-b19200 sets the serial port to 19200 baud

-----------------------------------------

Another alternative, is to use XL.EXE which is a copy of XA that I patched

so that it doesn't reset the speed of the serial port.  This allows you

to send commands to your X10 without having to reset the speed of the

sending Lawn to 600 baud and then reset it to it's normal speed.

LAWN -a,-i,co x10 >x10.log

XL f=reset.x10 >>x10.log

LAWN -a,-c,di >>x10.log