Memory Examining Program

Written by Brian Gordon

03-21-89  2:30 am

On entering this program it will prompt you for a memory location in the form

of 0000:0000.  Code Segment:Offset within Code segment.  Once examining

memory, you can scroll up and down within memory.  If the offset is FFFF

and you attemp to go farther down, it will reset to 0000 without incrementing

the Code Segment, this will be updated in a later version, along with the

ability to attemp to edit memory on the fly.  You may ask what this is good

for, plain and simply not much.  It was a fun project though.  Listed below

is a memory map of things to be found in memory:

Memory Adress

0000:0000 - 0000:0080  Traps - Interrupt vectors 0-7     "INT 00 - 07"

8259 INT controller vectors (8-F) "INT 08 - 0F"

BIOS vectors (mostly) (10-1F)     "INT 10 - 1F"

0000:0080 - 0000:0100  PC-DOS vectors(20-3F)             "INT 20 - 3F"

0000:0200 - 0000:0400  Assignable vectors (40-FF)        "INT 40 - FF"

(Some of these vectors have been assigned

to non-Intel software and hardware products)

0000:0400 - 0000:0417  Hardware enviorment information

(Printer and RS-232 device adresses,

memory size, etc.)

0000:0417 - 0000:043E  Keyboard buffer and status

0000:043E - 0000:0442  Diskette data area

0000:0442 - 0000:0467  Video display area

(Current mode, color palette; cursor positions

of up to eight pages; active page number, etc.)

0000:0467 - 0000:0472  Cassette, timer, and miscellaneous data

0000:0472 - 0000:048F  Hard disk and additional keyboard status

0000:048F - 0000:04F0  Reserved  (I dont have docs on whats here)

0000:04F0 - 0000:0500  Interapplication communications area

0000:0500 - 0000:0600  DOS data area

0000:0600 - 0000:0800  Programs  (DOS programs, RESERVED sorta)

0000:0800 - 000A:0000  Note their is plenty (at least 98,000 HEX or

608K or more than 622,592 bytes) of room

for your programs (if you can afford the

memory and slot space)

A000:0000 - C000:0000  Graphic card memory, Monochrome starts at

B000:0000 and uses 4K after it, CGA starts at

B000:0000 and uses 64K after it.

C000:0000 - C000:C800  Not used by anything, but could be used for

by add on cards such as the Copy 2 PC board.

C800:0000 - C800:2000  Hard disk controller card space (normally)

C800:2000 - F000:0000  LARGE hunk of nothing that could be used by

an add on board such as a Copy 2 PC board.

F000:0000 - F000:8000  Reserved for RAM expansion

F000:8000 - F000:E000  1st Bios chip on 286 only

F000:E000 - F000:FFFF  2nd Bios chip on 286 and BIOS on XT (I think)

Your Bios was created at the date at memory location

F000:FFF5 - F000:FFFC.  Should look like 01/01/80

or something to that effect.

F100:0000 - FFFF:FFFF  Cassette basic and other assorted items.

I hope you have had a use for this program, sincerely Brian Gordon.