TABLETMG.COM is a UIOF format tablet driver which can emulate Microsoft(R)'s

Mouse.com driver.  Use with Microgrid, Summagraphics LCL, or 16-button

cursor SummaSketch II tablets.  TABLETMG.SYS is a system file that can be used

in your CONFIG.SYS file.  TABLETMG.COM is a TSR (Terminate and Stay Resident)

program that can be run from the DOS command prompt or from a batch file,

including the autoexec.bat.

TABLETMG.SYS and TABLETMG.COM should  NOT  be loaded at the same time.

Usage:  DEVICE=TABLETMG.SYS [Com Port] [Pointing Device] [Options]

or

TABLETMG.COM [Com Port] [Pointing Device] [Options]

Where:  Com Port = /1        for Com Port 1  <DEFAULT>

/2        for Com Port 2

/3        for Com Port 3, PS/2 models only

/4        for Com Port 4, PS/2 models only

/OFF      to disable the driver

Pointing

Device =   /C4       for four-button cursor

/CP       for stylus   <DEFAULT>

OPTIONS:           /A        Absolute Tracking

/R        Relative Tracking  <DEFAULT>

(Following options work in relative mode only)

/S##      Sets both horizontal and vertical sensitivity -

Range = 0 - 99, Default = 50

/H##      Sets only the horizontal sensitivity -

Range = 0 - 99, Default = 50

/V##      Sets only the vertical sensitivity -

Range = 0 - 99, Default = 50

/BAxxxx   Selects serial port with base address xxxx (in HEX)

/Ix       Selects serial port with IRQ line number  x

(allowable IRQ - 2 through 7)

TABLETMG.COM normally detects the size of the tablet and sets the active area

to the full tablet size.  Therefore, it is usually unnecessary to use this

option.  The /T# option allows you to override the full size and specify

a smaller active area if you desire.  The sizes available are:

/T0 for 12" by 12" active area (SummaSketch II w/16 b cursor)

/T1 for 12" by 18" active area (SummaSketch II Pro. w/16 b cursor)

/T2 for 16" by 20" active area (Microgrid)

/T3 for 17" by 24" active area (Microgrid)

/T4 for 20" by 20" active area (Microgrid)

/T5 for 24" by 36" active area (Microgrid or Summagraphics LCL)

/T6 for 36" by 48" active area (Microgrid or Summagraphics LCL)

/T7 for 42" by 60" active area (Microgrid)

The active area set by the /T argument cannot be made larger than the actual

physical dimensions of your tablet.  If either of the Y or X dimensions

specified by /T# exceeds the actual size of your tablet, the argument will

be ignored.

NOTES:  - When selecting a serial port other than 1,2,3,or 4 the /BA

and /I options must be used together.

- When using the /BA option, all four digits must be specified

(example:  for Com 1 - /BA03F8)

- Use the /BA and /I options only when a non standard serial address

is used in the computer.

Serial Port Address Table

Command Line	Serial	   Serial Port	 System

Switch		Port	   Address

------------------------------------------------------------------------------

/1		1	   03F8 Hex	 All (PC, XT, AT, 386, and any PS/2)

/2		2	   02F8 Hex	 All (PC, XT, AT, 386, and any PS/2)

/3		3	   3220 Hex	 Any PS/2

/4		4	   3228 Hex	 Any PS/2

------------------------------------------------------------------------------