KBFIX.DOC    18-Apr-1990

A problem involving incorrect keyboard data has been reported by

several LANtastic user's. When using enhanced (101-key) keyboards

with NUM-lock on, a "Left-shift on" state occasionally becomes active

when no shift key is depressed. The state can be cleared by pressing

and releasing the left shift key. Also, on some machines, an enhanced

key (Arrows, Insert, Delete, Home, End, PageUp, PageDown) are

occassionally decoded as their numeric keypad NUMlocked equivalents,

e.g. "2", "4", "6", etc.

The problem seems to stem from the fact that the enhanced keyboards

send a string of scan codes (4 scan codes when NUMlock is on) for

each press or release of the key. An E0 scan code precedes each of

the others, to indicate that the following scan code is to be interpreted

as "enhanced".

These four scan code are sent in rapid succession to the PC motherboard.

As each is received, an IRQ1 is activated, which invokes the ISR for

INT 09H. This ISR is initially set to the BIOS keyboard handler,

although it is later re-hooked by resident drivers such as REDIR.

One of the first things the AT-BIOS handler does is to inhibit the

keyboard from sending further scan codes until processing of the one

causing the interrupt is complete.

Unfortunately, before it does this, AT-BIOS executes an STI intruction,

allowing other interrupts (e.g., timer ticks) to interrupt the keyboard

ISR.

Provided the interrupting timer tick ISR executes quickly, and control

returns to the keyboard handler before the scan code is replaced by

a new one, no harm is done. However, TSRs (such as REDIR) often hook

into the timer (08H) interrupt and may prolong its service time by

a substantial amount. If two scan codes come in quick succession

(as with the enhanced keys, which send E0 followed by the keycode),

then the first one (the E0 in this case) may be overwritten by the

second and be lost. Thus, the next scan code is interpreted as

a non-enhanced keystroke (a shift key or a numeric key).

This problem would never occur if the BIOS would inhibit the keyboard

BEFORE it reenables interrupts. Unfortunately it doesn't, and the

BIOS cannot be changed.

Three solutions to this problem have been written. The first, and the

preferable one, is KBFIX.EXE. It should be run in AUTOEXEC.BAT

before any TSRs which hook INT 9 are loaded. It searches through

the MS-DOS INT 9 handler for DOS's hook into the BIOS keyboard handler.

When it is found, the hook is modified to point just AFTER the

offending STI instruction, thereby eliminating the possibility

of interrupt preemption until after the keyboard is disabled.

Unfortunately, it is necessary for a program of this nature to

take advantage of specific characteristics of DOS and BIOS. It has

been tried successfully on several 386 and 286 AT-BIOS implementations

with no problems. The program is smart enough NOT to make the

modification if a discrepancy is noted. Since the modification

is made within DOS, the program does not need to TSR, and no additional

memory is consumed.

If KBFIX.EXE cannot be used because INT9 is already hooked by a device

driver in CONFIG.SYS or the MSDOS is not compatible, the same effect

can be achieved by including KBDFIX.SYS as a device driver (ahead of

any other device drivers which hook INT 9) in CONFIG.SYS. The patch

will be applied directly to the BIOS INT 9 hook, and the driver

will not remain resident (no memory will be consumed).

For specific BIOS implementations where KBFIX.EXE and KBDFIX.SYS will

not work, the INT9FIX.EXE utility was written. It loads as a TSR (288

resident bytes) and hooks the keyboard (09h) interrupt. Before passing

control to the BIOS keyboard handler, it masks IRQ0 (the timer) so the

keyboard ISR cannot be interrupted. When BIOS returns, it restores the

interrupt mask for IRQ0 to its original state. INT9FIX should normally

be the FIRST TSR loaded which hooks INT9, so that timer interrupts are

masked for as short a time as possible. No ill effects from using this

program have yet been noted, but since timer ticks are disabled,

operation of the Print-Screen key, the INT15 scan-code translation

hook, etc., may be affected in some software configurations.

End  KBFIX.DOC