README for M5.ZIP

December 8, 1992

SUMMARY

M5.ZIP provides 3270 Model 5 drivers that support seven additional display adapter

board types, for use with NetWare 3270 Workstation for DOS terminal emulators.

The additional board types are:

1. ATI VGA Wonder XL-24, and ATI VGA Wonder Charger

2. Trident's Trident Impact III (Uses Trident's VGA Chip)

3. Video 7's VRAM II (Uses Headland's VGA Chip.)

4. Ahead's VGA Wizard/Deluxe 5.0 (Uses Ahead's VGA Chip)

5. Cirrus's AVGA-3 (Uses Cirrus's CLGD 542x VGA Chip)

6. Diamond Computer Stealth VRAM board.  (Uses S3 Corp's VGA chip.)

7. Two boards based on Chips & Technology's 82C452 VGA chip

- Monolithic Systems Micro VGA 452

- Micro Pak 452 board

WARNINGS

- These drivers have only been tested with 3270 Workstation for DOS V3.0.  They may

or may not be compatible with earlier versions.

- If you use an "International Code Page Driver" the driver name on the command line

must be followed by the 3 digit code page number.

INSTALLATION INSTRUCTIONS

1. Check to make certain that the session is configured for Mod 5.

2. Copy the driver files you will need to the working directory of the workstation.

3. If you will be using the "International" code pages copy all EGA.CPI files from your

DOS directory to the workstations directory.

NOTE: Steps 4 - 6 must be repeated (perhaps by a .BAT file) each time a MOD 5

driver is loaded.

4.  Unload WSLAN if it is loaded.

5. Enter a LOAD command for the driver you will be using. (see below)

As with all 3270 Model 5 drivers provided by Novell, the drivers will then self install

resident, and extend PC BIOS support for INT 10H.  Little memory will be required,

typically around 500 bytes (the "International" versions of the drivers require more

memory because they contain a font definition).

6. Load the NetWare 3270 Workstation control program (WSLAN).

To unload Mod 5 drivers from memory:

- First unload WSLAN

- Then enter M5EXIT to unload the drivers

TO LOAD A MODEL 5 DRIVER

- To load a Mod 5 driver and utilize the ROM fonts for ASCII Code Page 437 (U.S.

English) use proper command "A" from the list below.

- To load a Mod 5 driver and utilize "International", ASCII Code Pages use the proper

command "B" from the list below.

If you use format B:

- The EGA.CPI file (or files) that comes with DOS (version 3.3 or later)

MUST be in the current directory at the time the command is executed.

The nnn parameter is an ASCII Code Page number.  Typical values for nnn

are 860 for Portugal, 863 for the French speaking parts of Canada, and 865

for Denmark/Norway.

1.  ATI VGA Wonder XL-24, and ATI VGA Wonder Charger

A. M5ATI

B. M5XATI nnn

2. Trident's Trident Impact III board

A. M5TRI

B. M5XTRI nnn

3.  Video 7's VRAM II board

A. M5VID7

B. M5XVID7 nnn

NOTE: this board occasionally leaves a residue border while switching from Model 5

session to DOS session.  ESU.COM, from the utility diskette provided with the board,

can be used to remove the border.

example: ESU /45<cr>

4. Ahead's VGA Wizard/Deluxe 5.0 board

A. M5AHD

B. M5XAHD nnn

5.  Cirrus's AVGA-3 board

A. M5CIRS

B. M5XCIRS nnn

NOTE: M5CIRS and M5XCIRS will only work with Cirrus's VGA BIOS v1.10 or

later.  If the you have an AVGA-3 board with a prior BIOS you should contact the

adapter manufacturer or Cirrus Logic to get the TSR version of the ver: 1.10 VGA

BIOS.  The TSR version of VGA Bios should be loaded prior to loading M5CIRS or

M5XCIRS.

6.  Diamond Computer's Stealth VRAM board

A. M5S3

B. M5XS3 nnn

NOTE: The Diamond Stealth VGA board will support concurrent addressing of two

character sets (fonts) when in 132 column mode only with the TSR version of VGA

BIOS supplied by Diamond Corp.  The User should contact the adapter manufacturer

to get the TSR version of the VGA_BIOS.  This TSR version of VGA Bios should be

loaded prior to loading M5S3 or M5XS3.

7.  Monolithic Systems Micro VGA 452 board & Micro Pak 452 board

A. M5C&T

B. M5XC&T nnn

The Micro VGA 452 board does not support concurrent addressing of two character sets

(fonts) when in 132 column mode.  Therefore, the special symbols for the OIA and APL

that are loaded by the 3270 Workstation control program do not display correctly.  The

OIA symbols (such as X () and the printer status symbols), display as miscellaneous PC

line drawing graphics.  The special OIA/APL symbol font is used by the 3270 emulator

whenever the VGA display adapter is in color mode and one or more host display

sessions are configured for Extended Data Stream (or Vector Graphics).  Otherwise,

standard PC graphics are used to approximate the OIA symbols.

There are two bypasses for this limitation.

A.  Configure all host display sessions for Base Data Stream.

OR

B.  Set the MONO MODE on the adapter (using the adapter's utility disk) BEFORE

installing the NetWare 3270 Workstation Control Program (WSLAN).  This will cause

the 3270 emulator to initialize in monochrome mode, and the standard PC graphics will

be used for the OIA symbols instead of the special OIA symbol font.  This takes

advantage of the fact that VGA display adapters can support monochrome text mode,

even when a color monitor is attached.

HISTORY

Another file, MOD5.ZIP, contains additional Model 5 drivers.

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to any

NetWare software, and specifically disclaims any express or implied warranties of

merchantability, title, or fitness for a particular purpose.

Distribution of any NetWare software is forbidden without the express

written consent of Novell, Inc.  Further, Novell reserves the right to discontinue

distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use of the

software, loss of data, costs of re-creating lost data, the cost of any substitute equipment

or program, or claims by any party other than you.  Novell strongly recommends a

backup be made before any software is

installed.  Technical support for this software may be provided at the

discretion of Novell.